3GPP TSG-RAN WG4 Meeting #110bis	R4-2405937

Changsha, China, Apr 15 – 19, 2024

Agenda item:	9.14.4

Source: 	Qualcomm Incorporated

Title: 		Scope of RRM core requirements for LP-WUS/WUR

Document for:	Discussion

# Introduction

For release 19, a new work item on low-power wakeup signal (WUS) and wakeup receiver (WUR) for NR has been agreed with the following objectives:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this paper, we discuss some aspects to be considered for initial discussion on RRM core requirements.

# Initial scope of RRM requirements for LP-WUR

According to the WID, the prime objective of this WI is to enable power savings in the IDLE/INACTIVE mode and use develop procedures to allow UE MR PDCCH monitoring triggered by LP-WUS in the Connected mode. As such, the optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. Moreover, in CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR. So, we think RAN4 should initially focus on specifying RRM requirements for  IDLE/INACTIVE mode and address issues, if any, related to CONNECTED mode at a later stage, unless an LS is received from RAN1/2 on CONNECTED mode operation

Observation 1: According to the WID, the optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

Observation 2: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR.

Proposal 1: RAN4 should initially focus on specifying RRM requirements for IDLE/INACTIVE mode and address issues, if any, related to CONNECTED mode at a later stage.

For the IDLE/INACTIVE mode, the main topics to be discussed are:

- Entry/exit condition for LP-WUS monitoring.

- LP-SS periodicity Y ms for LP-WUR, for synchronization and/or RRM for serving cell.

- RRM relaxation of UE MR for both serving and neighbor cell measurements, and

- UE serving cell RRM measurement offloaded from MR to LP-WUR.

In our understanding, RAN2 will work on defining the entry/exit condition for LP-WUS monitoring as well as conditions under which RRM relaxations and/or RRM measurement offloading can be performed while RAN4 will work on specifying the RRM relaxation/ offloading requirements.

Observation 3: It’s a common understanding that RAN2 specifies the entry/exit condition for LP-WUS monitoring as well as conditions under which RRM relaxations and/or RRM measurement offloading can be performed.

Proposal 2: RAN4 shall work on defining the RRM measurement relaxations and/or RRM measurement offloading mechanisms considering the criteria defined by RAN2.

Regarding the LP-SS periodicity Yms for synchronization and/or RRM measurements for the serving cell, we think RAN4 also needs to take into account the measurement accuracies of the LP-SS based measurements as well as the number of measurement samples needed for filtering to derive the metric needed for criteria evaluation. A good starting point could be to set the desired measurement accuracy levels and then work towards determining the number of samples needed for filtering. That could be used to determine a minimum LP-SS periodicity needed for RAN4 requirements.

Proposal 3: RAN4 to determine LP-SS periodicity Yms based on a desired measurement accuracy, a desired number of samples needed for filtering and a desired minimum SNR level.

# Conclusion

Observation 1: According to the WID, the optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

Observation 2: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR.

Proposal 1: RAN4 should initially focus on specifying RRM requirements for IDLE/INACTIVE mode and address issues, if any, related to CONNECTED mode at a later stage.

Observation 3: It’s a common understanding that RAN2 specifies the entry/exit condition for LP-WUS monitoring as well as conditions under which RRM relaxations and/or RRM measurement offloading can be performed.

Proposal 2: RAN4 shall work on defining the RRM measurement relaxations and/or RRM measurement offloading mechanisms considering the criteria defined by RAN2.

Proposal 3: RAN4 to determine LP-SS periodicity Yms based on a desired measurement accuracy, a desired number of samples needed for filtering and a desired minimum SNR level.

# References

[1] 	RP-240801, “Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR)”, vivo, RAN#103