3GPP TSG-RAN WG4 Meeting #110bis	R4-2405941

Changsha, China, Apr 15th – Apr 19th, 2024

Agenda item:	6.21.3.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on demodulation requirements for eRedCap

Document for:	Discussion

# Introduction

RAN4 completed the core part of the Rel-18 WI on enhanced support of reduced capability devices [1] during RAN4#109 and started working on specifying the performance requirements from RAN4#110 meeting.

In this contribution, we provide our views on the demod performance requirements based on the initial agreements reached in [2].

# UE demodulation requirements

During the last meeting, RAN4 agreed to define the PDSCH demodulation tests for FDD/HD-FDD with SCS 15kHz and TDD with SCS 30kHz for QPSK (MCS4) and 16QAM (MCS13) modulation schemes with new PRB sizes. Our initial simulation results for these cases are provided below:

PDSCH test cases for UE not supporting eRedCapNotReducedBB-BW-r18 (FG 48-1, supportOfERedCap-r18 only)

- FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | 70 | 1.72 |
| 16QAM | 1Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 70 | 9.87 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 25 | 9992 | TDLA30-10 | 70 | 13.44 |
| QPSK | 2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | 70 | -1.8 |
| 16QAM | 2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 70 | 5.89 |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 25 | 9992 | TDLA30-10 | 70 | 10.15 |

- TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | 70 | 1.9 |
| 16QAM | 1Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | 70 | 10.45 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 12 | 4736 | TDLA30-10 | 70 | 13.94 |
| QPSK | 2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | 70 | -1.53 |
| 16QAM | 2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | 70 | 6.31 |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 12 | 4736 | TDLA30-10 | 70 | 10.16 |

PDSCH test cases for UE supporting eRedCapNotReducedBB-BW-r18 (FG 48-2)

- FDD/HD-FDD SCS=15kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | 70 | 1.24 |
| 16QAM | 1Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 70 | 9.96 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 25 | 9992 | TDLA30-10 | 70 | 13.44 |
| QPSK | 2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | 70 | -2.15 |
| 16QAM | 2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 70 | 5.94 |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 25 | 9992 | TDLA30-10 | 70 | 10.15 |

- TDD SCS=30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | 70 | 1.18 |
| 16QAM | 1Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 70 | 9.91 |
| 64QAM | 1Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 12 | 4736 | TDLA30-10 | 70 | 13.94 |
| QPSK | 2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | 70 | -2.03 |
| 16QAM | 2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 70 | 5.98 |
| 64QAM | 2Rx | MCS19 in Table 1 | 0.5 | Rank 1 | 12 | 4736 | TDLA30-10 | 70 | 10.16 |

Proposal 1: RAN4 to consider the initial simulation results in the above tables for defining PDSCH demod performance.

There were two main open issues on PDSCH demod performance requirements for eRedCap UEs –

1) whether to specify requirements for 256 QAM and

2) whether to specify rank1 or rank2 tests for 64QAM tests.

We think 256QAM may not be very attractive modulation scheme for eRedCap UEs since number of PRBs needed to support the peak rate of 10Mbps is very low. So, we can deprioritize 256QAM. For the same reasons, we think 64QAM rank2 test can also be deprioritized and only rank1 tests can be specified.

Proposal 2: Consider deprioritizing 256QAM modulation scheme for PDSCH demod tests.

Proposal 3: Specify 64QAM test with rank1 configuration.

Since R18 eRedCap UEs can achieve the peak data rate of 10Mbps with or without baseband bandwidth reduction, RAN4 needs to specify the demod tests for both type of UEs.

# UE CSI reporting requirements

During the email discussion post meeting, a few companies brought up the issue of estimating CQI over wide-band CSI-RS when the PDSCH scheduling is limited to 25/12PRBs. Although baseband bandwidth reduction was considered only for PDSCH/PUSCH in the WID and not for other channels, we think it’s a valid issue to discuss and we are open to consider reduced CSI-RS BW in the CQI tests.

Proposal 4: RAN4 to discuss CSI-RS BW allocation in CQI tests for eRedCap UEs.

# Conclusion

Proposal 1: RAN4 to consider the initial simulation results in the above tables for defining PDSCH demod performance.

Proposal 2: Consider deprioritizing 256QAM modulation scheme for PDSCH demod tests.

Proposal 3: Specify 64QAM test with rank1 configuration.

Proposal 4: RAN4 to discuss CSI-RS BW allocation in CQI tests for eRedCap UEs.

# References

[1] RP-233637, “Revised WID on Enhanced support of reduced capability NR devices”, Ericsson, RAN#102

[2] R4-2402870, “Way forward on [110][325] NR_redcap_enh_demod”, Ericsson, RAN4#110