# 3GPP TSG-RAN WG4 Meeting # 110-bis                                                         R4-2405960

# Changsha, China, 15 – 19th April 2024

# Title:	PC2 CA_n25-n41 Rx Mixing

# Agenda Item:	5.18.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This contribution evaluates the receiver (Rx) harmonic mixing MSD of Band n41 due to power class 2 (PC2) operation in non-simultaneous Rx/Tx operation of uplink (UL) carrier-aggregation CA_n25-n41.

# Discussion

In CA_n25-n41, Band n41 is affected by UL4/DL3 Rx harmonic mixing. For PC3 operation, no MSD has been specified. Yet, for CA_n39-n41 a PC3 MSD test point has been specified based on text proposal [1] which is reproduced in Table 1.

Regarding MSD analysis of [1], we note that the MSD was calculated against a Band n41 REFSENS of -94.8dBm which is the level for 10MHz channel bandwidth (CBW). Yet the MSD was agreed for 5MHz CBW, we therefore propose that the Band n41 MSD for CA_n39_n41 is corrected to 11.1dB for 5MHz CBW as highlighted in yellow in Table 1.

Table 1 CA_n39-n41 power class 3 UL4/DL3 Rx harmonic mixing test points (Table 7.3A.4-4)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 11 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 2.1 | NOTE 11 | UL4/DL3 |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | [11.1] | NOTE 11 | UL4/DL3 |

Observation: For PC3 operation of CA_n39-n41, the Band n41 5MHz CBW MSD test point should be corrected to [11.1]dB.

Considering that the uplink Band n25 and Band n39 have similar frequency ranges, we propose to evaluate the PC2 CA_n25-n41 UL4/DL3 MSD based on the PC3 MSD analysis of CA_n39-n41 UL4/DL3 with the following corrections:

- The PC2 Band n25 fourth uplink harmonic (UL4) interference level is 3dB higher the PC3 H4 level. Measurement data in Figure 1 shows the 3dB power spectral density difference of the Band n25 UL4.

- We assume an 8dB better harmonic rejection by the Band n25 filter resulting in an overall 5dB difference with the interference level used for CA_n39-n41.

Figure 1: Measured PC3 and PC2 PA fourth uplink harmonic (UL4) PSD of Band n25 lower-most 5MHz channel for DFT-s-OFDM QPSK SCS 15kHz waveform with LCRB = 25 RB. Vertical axis: 3dB/division.

Based on these assumptions, we propose to the following MSD test point.

Proposal: For PC2 operation of CA_n25-n41, adopt the following Band n41 UL3/DL4 Rx harmonic mixing test point.

Table 2 CA_n25-n41 power class 2 UL4/DL3 Rx harmonic mixing test points (Table 7.3A.4-4a)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | 6.1 | NOTE 11 | UL4/DL3 |

# Conclusion

In this contribution, we observe that the UL4/DL3 Band n41 MSD of [1] for CA_n39-n41 PC3 operation should be corrected to 11.1dB. For the Band n41 UL4/DL3 MSD due to PC2 operation in CA_n25-n41, we assume 3dB higher UL4 harmonic interference level and 8dB harmonic rejection leading to 6.1dB MSD for 5MHz CBW.

Observation: For PC3 operation of CA_n39-n41, the Band n41 5MHz CBW MSD test point should be corrected to [11.1]dB.

Proposal: For PC2 operation of CA_n25-n41, adopt the following Band n41 UL3/DL4 Rx harmonic mixing test point.

Table 2 CA_n25-n41 power class 2 UL4/DL3 Rx harmonic mixing test points (Table 7.3A.4-4a)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | 6.1 | NOTE 11 | UL4/DL3 |

# Reference

[1] 	R4-2306550 TP for TR 38.894: Harmonic mixing MSD for simultaneous RxTx CA_n39-n41, 3GPP TSG-RAN WG4 Meeting #106bis-e, Electronic meeting, ZTE Corporation, Huawei, Hisilicon, Skyworks Solutions, Inc.