3GPP TSG-RAN WG4 Meeting #110-bis	R4-2405968

Changsha, China, April 15 – April 19, 2024

Agenda Item: 6.14.1.2

Source: MediaTek Inc.

Title: Discussion on the UE feature list of R18 NR mobility enhancement-part 2

Document for: Discussion

# Introduction

In the last meeting, RAN4 further discussed the core requirements of Rel-18 NR mobility enhancement part 2 as captured in the WF in [1]. In this paper, we address the remaining issues related to the UE feature list of R18 mobility enhancement for part 2 and provide our view.

# Discussion

In the previous meeting, the agreement on the UE feature list for R18 mobility enhancement part 2 was captured in the WF as below [1]:

Issue 1-1: UE feature list of R18 Further NR mobility enhancement part 2 Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-x1 | Measurement validation based on EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup | idleInactiveNR-MeasReport-r16 | Yes | N/A | UE does not support EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | FFS | N/A |  | Optional with capability signaling |
|  | 39-x2 | Measurement validation based on non-EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on non-EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup |  | Yes | N/A | UE does not support non-EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | FFS | N/A |  | Optional with capability signaling |

The remaining FFS part for the agreed feature list highlighted above need to be finalized. In our view, since the introduced capabilities of measurement validation based on EMR/non-EMR measurement during connection setup/resume are per UE capabilities and can be applicable to both FR1 and FR2, there should be no differentiation between FR1 and FR2 in the UE feature list.

Proposal 1: No need for FR1/FR2 differentiation in the UE feature list (39-x1 and 39-x2) for R18 mobility enhancement part 2.

# Summary

In this paper, the FFS part in the UE feature list for R18 mobility enhancement part 2 was addressed and the following proposal was concluded:

Proposal 1: No need for FR1/FR2 differentiation in the UE feature list (39-x1 and 39-x2) for R18 mobility enhancement part 2.

# Reference

- R4-2403548, WF on R18 Further NR mobility enhancement – part 2, Apple.