3GPP TSG-RAN WG4 Meeting #110bis   		                                  R4-2405972

# Changsha, China, April 15th - April 19th 2024

# Source:	THALES, Magister Solutions Ltd, Eutelsat Group, ESA, Inmarsat, Viasat, Novamint, EchoStar, Amazon

# Title:	On the ACS requirement issue for VSAT UE in above 10 GHz

# Type:	discussion

# Document for:	discussion

# Agenda Item:	6.16.2/6.16.5.2

Release:	Rel-18

# Introduction

The current paper is to discuss the ACS remaining issue for VSAT UE requirement definition in above 10 GHz applicable to TS 38.101-5.

At RAN4#109, the following agreements have been made:

- SAN ACLR:

  - [12] dB for both GEO and LEO

- UE ACLR:

  - [14] dB for both fixed VSAT and mobile VSAT

- SAN ACS:

  - For GEO: [18] dB,

  - For LEO: [24] dB.

- VSAT ACS:

  - Considering below candidate values as starting point for VSAT ACS

    - Option 1: 23 dB

    - Option 2: 30 dB

    - Option 3: 35 dB

  - FFS for the additional means to address the co-existence issue, candidate options for further discussion

    - Option 1: Limit the elevation angles on DL side for VSAT UE

    - Option 2: Configure additional guard-band

    - Other options not precluded

From all investigated scenarios, the only scenario that requiring very strict NTN VSAT UE implementation considering a non-realistic deployment scenario close to hypothetical TN deployment at 17 GHz is scenario #5.

(see RAN4#106)

Agreement 1: Study NTN-TN coexistence by assuming a reference frequency of 17 GHz for NTN DL cases and 27 GHz NTN UL cases, as well as the consideration of ACLR and ACS assumptions as following:

| No. | Combination | Aggressor | Victim | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |

Coexistence scenarios for use cases 5-8 (in above 10 GHz)

As explained below and in several parts of this technical document, VSAT UE ACS has to be relaxed to a more reasonable value supported by satellite ecosystem.

Then, at RAN4#110, the following agreement has been made:

With the following notes:

It has also been agreed that the respective notes should be included in the TR 38.863.

For the time being the complete text can be found in the approved Way Forward R4-2403092, “WF on [110][307] NR_NTN_enh_Part3” from RAN4#110:

Issue 3-5: UE ACS

Further investigate and check the current TN FR2 ACS test methodology/configuration and conclude a single ACS value from Option 1 and Option 2 in the next meeting

- Option 1: [27.5]dBc

- Option 2: [23-28]dBc

NOTE 1:              At the time of this 3GPP co-existence study, there is no TN band defined or planned near 17 GHz. The parameters are derived based on 3GPP coexistence scenarios in which a TN system is simulated to be operating in the band directly adjacent to the proposed NTN system as well as technical assumptions that may or may not be applicable in practice. The results of the study are not intended to address coexistence issues from a regulatory standpoint.

NOTE 2:             There are existing non-3GPP VSAT UE operating in Ka band at present and will likely continue operating in the future, with ACS performance lower than the values proposed.

NOTE 3:              Additional solutions could be further considered to address coexistence issues if and when TN is deployed in 17 GHz.

For a better reading, latest agreement/WF of Issue 3-2 to Issue 3-5 are summarized in the table below for reference.

|  | SAN |  | UE |  |
| --- | --- | --- | --- | --- |
|  | GEO | LEO | Fixed | Mobile |
| ACLR (dBc) | 12 | 12 | 14 | 14 |
| ACS (dBc) | 18 | 24 | [See Issue 3-5]1, 2, 3 | [See Issue 3-5]1, 2, 3 |
| NOTE 1: At the time of this 3GPP co-existence study, there is no TN band defined or planned near 17 GHz. The parameters are derived based on 3GPP coexistence scenarios in which a TN system is simulated to be operating in the band directly adjacent to the proposed NTN system as well as technical assumptions that may or may not be applicable in practice. The results of the study are not intended to address coexistence issues from a regulatory standpoint. NOTE 2: There are existing non-3GPP VSAT UE operating in Ka band at present and will likely continue operating in the future, with ACS performance lower than the values proposed. NOTE 3: Additional solutions could be further considered to address coexistence issues if and when TN is deployed in 17 GHz. |  |  |  |  |

# Proposals for ACS requirements for VSAT UE

Extra simulations and other alternatives are provided in a separate contribution to show that lower ACS values are possible and achievable. This proposal is also motivated by the fact that there is no TN deployment at 17 GHz and therefore it does not make sense to increase the protection with respect to an inexistent/fictitious potential interferer.

With respect to scenario 5 (see also contribution R4-2320970 - NTN-TN co-existence simulations results in above 10 GHz bands, THALES, Magister Solutions Ltd), several solutions have been investigated such as:

1/ VSAT UE alone/default (which requires/indicates an ACS of 30-35 dBs as also confirmed/found in the simulations by other companies as well, with the current hypothesis)

2/ Increasing Tx SAN power by 6 dB (e.g. 10 dBW/MHz EIRP density instead of 4 dBW/MHz)

3/ Increasing TN gNB antenna gain by increasing the number of antenna elements (8x16 AAS -> 16x32 AAS, which corresponds to about 6dB antenna gain);

4/ Decreasing TN gNB transmission power by 6dB (Total TXP – 6dB  = 35 dBm);

5/ Using a frequency guard band for a better protection from the TN gNB (at least 5dB interference decrease from TN BS to NTN UE VSAT can be easily obtained as OFDM spectrum decreases with the increase of the guard band);

6/ Increasing antenna gain of the VSAT UE (e.g. by using a higher aperture such as 0.6 m -> 1.2 m NTN VSAT antenna diameter for instance or lower; in practice lower aperture can be also considered e.g. 0.8m or lower).

It can be found that among all these (new) solutions, several are viable in combination or alone. For exemplification one may find a description of the solutions as explained below:

| Name of the solution | Detail | Comment |
| --- | --- | --- |
| sc5_22m | Default solution 22 m NTN UE height with VSAT antenna of 0.6 m | - |
| sc5_22m_ntn_txp+6dB | Tx SAN power by 6 dB (e.g. 10 dBW/MHz EIRP density instead of 4 dBW/MHz) | Increasing SAN transmission power/EIRP density |
| sc5_22m_tn_aas | Increasing TN gNB antenna gain by increasing the number of antenna elements (8x16 AAS -> 16x32 AAS, which corresponds to about 6dB antenna gain); | Peak gain increases from 26.5 to 32.6 dBi |
| sc5_22m_tn_aas_txp-6dB | Increasing TN gNB antenna gain by increasing the number of antenna elements (8x16 AAS -> 16x32 AAS, which corresponds to about 6dB antenna gain); Decreasing TN gNB transmission power by 6dB (Total TXP – 6dB = 35 dBm); | Combination of sc5_22m_aas with TN gNB power degrease |
| sc5_22m_vsat1.2m | 22 m NTN UE height with VSAT antenna of 1.2 m Increasing antenna gain of the VSAT UE (e.g. by using a higher aperture such as 0.6 m -> 1.2 m NTN VSAT antenna diameter). | In practice, 0.8 m could also be used |
| sc5_1.5m | Default solution 1.5 m NTN UE height with VSAT antenna of 0.6 m | This would correspond to a terminal mounted on car or other vehicle, which was not studied by all the companies. |
| sc5_1.5m_vsat1.2m | 1.5 m NTN UE height with VSAT antenna of 1.2 m Increasing antenna gain of the VSAT UE (e.g. by using a higher aperture such as 0.6 m -> 1.2 m NTN VSAT antenna diameter). |  |

Results with no isolation (without any guardband):

| LEO600 | Total ACIR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No guardband, no isolation (dB) | 90° |  | 25° |  | 45° |  |
|  | Mean | 5%-tile | Mean | 5%-tile | Mean | 5%-tile |
| sc5_22m | 28 | 30 | 30 | 36 | 28 | 32 |
| sc5_22m_ntn_txp+6dB | 22 | 30 | 28 | 34 |  |  |
| sc5_22m_tn_aas | 24 | 30 | 28 | 36 | 26 | 30 |
| sc5_22m_tn_aas_txp-6dB | 18 | 20 | 20 | 28 | 20 | 24 |
| sc5_22m_vsat1.2m | 18 | 24 | 20 | 20 | 20 | 26 |
| sc5_1.5m | 20 | 32 | 28 | 36 | 24 | 28 |
| sc5_1.5m_vsat1.2m | 10 | 18 | 18 | 22 | 16 | 24 |
|  | Required NTN UE ACS |  |  |  |  |  |
| TN gNB ACLR | 90° |  | 25° |  | 45° |  |
| 30 | Mean | 5%-tile | Mean | 5%-tile | Mean | 5%-tile |
| sc5_22m | 33 | #NUM! | #NUM! | #NUM! | 33 | #NUM! |
| sc5_22m_ntn_txp+6dB | 23 | #NUM! | 33 | #NUM! |  |  |
| sc5_22m_tn_aas | 26 | #NUM! | 33 | #NUM! | 29 | #NUM! |
| sc5_22m_tn_aas_txp-6dB | 19 | 21 | 21 | 33 | 21 | 26 |
| sc5_22m_vsat1.2m | 19 | 26 | 21 | 21 | 21 | 29 |
| sc5_1.5m | 21 | #NUM! | 33 | #NUM! | 26 | 33 |
| sc5_1.5m_vsat1.2m | 11 | 19 | 19 | 23 | 17 | 26 |
| GEO |  |  |  |  |  |  |
| TN gNB ACLR | 90° |  | 25° |  | 45° |  |
| 30 | Mean | 5%-tile | Mean | 5%-tile | Mean | 5%-tile |
| sc5_22m | 28 | 40 | 30 | 36 | 28 | 38 |
| sc5_22m_ntn_txp+6dB | 22 | 36 | 26 | 32 |  |  |
| sc5_22m_tn_aas | 24 | 30 | 28 | 34 | 26 | 36 |
| sc5_22m_tn_aas_txp-6dB | 18 | 30 | 20 | 26 | 20 | 30 |
| TN gNB ACLR | 90° |  | 25° |  | 45° |  |
| 30 | Mean | 5%-tile | Mean | 5%-tile | Mean | 5%-tile |
| sc5_22m | 33 | #NUM! | #NUM! | #NUM! | 33 | #NUM! |
| sc5_22m_ntn_txp+6dB | 23 | #NUM! | 29 | #NUM! |  |  |
| sc5_22m_tn_aas | 26 | #NUM! | 33 | #NUM! | 29 | #NUM! |
| sc5_22m_tn_aas_txp-6dB | 19 | #NUM! | 21 | 29 | 21 | #NUM! |

Results with isolation (extra guardband with 5dB ACIR decrease has been considered):

| LEO600 | Total ACIR with extra ACIR isolation |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Extra ACIR isolation (dB) | 90° |  | 25° |  | 45° |  |
| 5 | Mean | 5%-tile | Mean | 5%-tile | Mean | 5%-tile |
| sc5_22m | 23 | 25 | 25 | 31 | 23 | 27 |
| sc5_22m_ntn_txp+6dB | 17 | 25 | 23 | 29 |  |  |
| sc5_22m_tn_aas | 19 | 25 | 23 | 31 | 21 | 25 |
| sc5_22m_tn_aas_txp-6dB | 13 | 15 | 15 | 23 | 15 | 19 |
| sc5_22m_vsat1.2m | 13 | 19 | 15 | 15 | 15 | 21 |
| sc5_1.5m | 15 | 27 | 23 | 31 | 19 | 23 |
| sc5_1.5m_vsat1.2m | 5 | 13 | 13 | 17 | 11 | 19 |
|  | Required NTN UE ACS with extra ACIR isolation |  |  |  |  |  |
| TN gNB ACLR | 90° |  | 25° |  | 45° |  |
| 30 | Mean | 5%-tile | Mean | 5%-tile | Mean | 5%-tile |
| sc5_22m | 24 | 27 | 27 | #NUM! | 24 | 31 |
| sc5_22m_ntn_txp-6dB | 18 | 27 | 24 | 36 |  |  |
| sc5_22m_tn_aas | 20 | 27 | 24 | #NUM! | 22 | 27 |
| sc5_22m_tn_aas_txp+6dB | 14 | 16 | 16 | 24 | 16 | 20 |
| sc5_22m_vsat1.2m | 14 | 20 | 16 | 16 | 16 | 22 |
| sc5_1.5m | 16 | 31 | 24 | #NUM! | 20 | 24 |
| sc5_1.5m_vsat1.2m | 6 | 14 | 14 | 18 | 12 | 20 |
| GEO | Total ACIR with extra ACIR isolation |  |  |  |  |  |
| Extra ACIR isolation (dB) | 90° |  | 25° |  | 45° |  |
| 5 | Mean | 5%-tile | Mean | 5%-tile | Mean | 5%-tile |
| sc5_22m | 23 | 35 | 25 | 31 | 23 | 33 |
| sc5_22m_ntn_txp+6dB | 17 | 31 | 21 | 27 | -5 |  |
| sc5_22m_tn_aas | 19 | 25 | 23 | 29 | 21 | 31 |
| sc5_22m_tn_aas_txp-6dB | 13 | 25 | 15 | 21 | 15 | 25 |
|  | Required NTN UE ACS with extra ACIR isolation |  |  |  |  |  |
| TN gNB ACLR | 90° |  | 25° |  | 45° |  |
| 30 | Mean | 5%-tile | Mean | 5%-tile | Mean | 5%-tile |
| sc5_22m | 24 | #NUM! | 27 | #NUM! | 24 | #NUM! |
| sc5_22m_ntn_txp+6dB | 18 | #NUM! | 22 | 31 |  |  |
| sc5_22m_tn_aas | 20 | 27 | 24 | 36 | 22 | #NUM! |
| sc5_22m_tn_aas_txp-6dB | 14 | 27 | 16 | 22 | 16 | 27 |

Observation 1: For all these reasons, solutions to bring VSAT UE ACS down to [23-24] dB exist and are reasonable, for both LEO & GEO configurations, at different elevation angles.

Proposal 1: ACS value of VSAT NTN at 17 GHz (DL Ka-band) shall not be higher than [23] dB and include the agreed notes at RAN4#110 to provide context of the coexistence studies.

Proposal 2: The VSAT NTN ACS shall be tested with TDD type of interferer (to model interference of TN BS).

Moreover, in EN 303 978 for instance, the following information can be found for the existent terminals operating in harmonized Ka-band (Technical requirements specifications).

Therefore, previous design of VSAT UE operating at DL 17GHz is considering an ACS of 7 dBsd (ratio expressed in decibels relative to the spectral density). For this reason, many satellite companies consider that there is a huge discrepancy between 7 dB (of legacy existent VSAT UE) and proposed value of 27.5 dB ACS (from coexistence value with a non-existent TN system operating at 17 GHz).

Observation 2: Many satellite companies consider that there is a huge discrepancy between the 7 dB ACS applicable to legacy existing VSAT UE and a proposed value of 27.5 dB ACS (derived from coexistence value with hypothetical TN system operating at 17 GHz although there are no regulatory basis & no frequency band allocated to TN for the deployment of terrestrial network in these bands).

Observation 3: There are no regulatory basis to define ACS higher than 23 dB at 17 GHz for VSAT UE.

Moreover, similar as for the SAN ACS requirements (see endorsed R4-2402965), the following requirements may apply to NTN VSAT ACS (assuming 23dB + 10log10(133/64) = 26.17dB with e.g. 50 MHz interferer type instead of 200 MHz as in the coexistence studies, with other combination not precluded):

Table x.x.x.x-1: OTA ACS requirement for NTN VSAT

| NTN VSAT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6 dB (Note 1) | EISREFSENS_50M + 26.17+ ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is given in clause 10.3 |  |  |

Table x.x.x.x-2: OTA ACS interferer frequency offset for NTN VSAT

| NTN VSAT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper NTN VSAT RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 50 | ±24.29 |  |
| 100 | ±24.31 | 50 MHz OFDM TDD NR |
| 200 | ±24.29 | signal, 60 kHz SCS, 64 RBs |
| 400 | ±24.31 |  |

# Conclusions

Observation 1: Solutions to bring VSAT UE ACS down to [23-24] dB exist and are reasonable, for both LEO & GEO configurations, at different elevation angles.

Proposal 1: ACS value of VSAT NTN at 17 GHz (DL Ka-band) shall not be higher than [23] dB and include the agreed notes at RAN4#110 to provide context of the coexistence studies.

Observation 2: Many satellite companies consider that there is a huge discrepancy between the 7 dB ACS applicable to legacy existing VSAT UE and a proposed value of 27.5 dB ACS (derived from coexistence value with hypothetical TN system operating at 17 GHz although there are no regulatory basis & no frequency band allocated to TN for the deployment of terrestrial network in these bands).

Observation 3: There are no regulatory basis to define ACS higher than 23 dB at 17 GHz for VSAT UE.

Proposal 2: The VSAT NTN ACS shall be tested with TDD type of interferer (to model interference of TN BS).

Proposal 3: Discuss to use NTN VSAT EISREFSENS + 6 dB value to define wanted signal mean power (dBm) and NTN VSAT EISREFSENS_50M + 26.17 + NTN VSAT ΔFR2_REFSENS value to define interfering signal mean power (dBm).

Table x.x.x.x-1: OTA ACS requirement for NTN VSAT

| NTN VSAT channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 50, 100, 200, 400 | EISREFSENS + 6 dB (Note 1) | EISREFSENS_50M + 26.17+ ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is given in clause 10.3 |  |  |

Table x.x.x.x-2: OTA ACS interferer frequency offset for NTN VSAT

| NTN VSAT channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper NTN VSAT RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 50 | ±24.29 |  |
| 100 | ±24.31 | 50 MHz OFDM TDD NR |
| 200 | ±24.29 | signal, 60 kHz SCS, 64 RBs |
| 400 | ±24.31 |  |

# References

- R4-2402933, Remaining issues on VSAT UE requirements for above 10 GHz, THALES, Magister Solutions Ltd

i/ for GSO

- (ESOMP/ESIM)

EN 303 978 - V2.1.2 - Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU (etsi.org)

- (initially developed for fixed terminals)

EN 301 459 - V2.1.1 - Satellite Earth Stations and Systems (SES); Harmonised Standard for Satellite Interactive Terminals (SIT) and Satellite User Terminals (SUT) transmitting towards satellites in geostationary orbit, operating in the 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU (etsi.org)

- EN 301 360 - V2.1.1 - Satellite Earth Stations and Systems (SES); Harmonised Standard for Satellite Interactive Terminals (SIT) and Satellite User Terminals (SUT) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 29,5 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU (etsi.org)

ii/ for NGSO

- EN 303 979 - V2.1.2 - Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU (etsi.org)

- EN 303 699 - V1.1.1 - Satellite Earth Stations and Systems (SES); Fixed earth stations communicating with non-geostationary satellite systems in the 20 GHz and 30 GHz FSS bands; Harmonised Standard for access to radio spectrum (etsi.org)