3GPP TSG-RAN WG4 Meeting # 110bis											  R4-2406019

Changsha, China, 15 – 19 Apr. 2024

Title: 	Way forward on [110bis][333] NR_DSS_enh

Agenda Item:	6.25.1

Source: 	Ericsson

Document for:	Approval

# Topic #1: NR PDCCH reception in symbols with LTE CRS REs

Whether to change the current test case setup

Agreement:

- Agreed test cases for requirement:

| Test number | Duplex | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2X4 Low | 1 | TBD |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4X2 Low | 1 | TBD |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 4x4 Low | 1 | TBD |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 2x2 Low | 1 | TBD |

Payload

Agreement:

- Remain 39bits for all test cases unless any issue identified.

# Annex(for information)

## A.1 Simulation assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD/TDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |

## A.2 Work split

| Specification | Draft CR | Responsibility |
| --- | --- | --- |
| 38.101-4 | Introduction of PDCCH requirement for DSS enhancement -FDD 2Rx | MTK |
| 38.101-4 | Introduction of PDCCH requirement for DSS enhancement -TDD 2Rx | Apple |
| 38.101-4 | Introduction of PDCCH requirement for DSS enhancement -FDD 4Rx | Nokia |
| 38.101-4 | Introduction of PDCCH requirement for DSS enhancement -TDD 4Rx | ZTE |
| 38.101-4 | Applicability rule for DSS enhancement demodulation requirement | QC |
| 38.101-4 | Reference measurement channel for DSS enhancement demodulation requirement FDD and TDD | Huawei |
| 38.101-4 | Big draft CR to 38.101-4: Introduction of demodulation requirement for DSS enhancement | Ericsson |
| N/A | Simulation results collection for DSS enhancement demodulation requirement | Ericsson |