3GPP TSG-RAN4 Meeting # 110bis	R4-2406033

# Changsha, China,  15 - 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft CR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.101-4: Correction on the reference measurement channel for 8Rx requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correct the numbering of newly introduced reference measurement channel for 8Rx requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the numbering of reference measurement channel that is overlapping in rank 2, FDD 5~25MHz - Correct the order of the numbering for rank 8, FDD 30~50MHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The numbering of the reference measurement channel will remain confused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1------------------------------------------------------------

Table A.3.2.1.1-22: PDSCH Reference Channel for FDD CC and CA scenario

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-22.1 FDD | R.PDSCH.1-22.2 FDD | R.PDSCH.1-22.3 FDD | R.PDSCH.1-22. FDD |
| Channel bandwidth | MHz | 5 | 15 | 20 | 25 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 25 | 79 | 106 | 133 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 19 | 19 | 19 | 19 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.50 | 0.50 | 0.50 | 0.50 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 19968 | 63528 | 83976 | 106576 |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 3 | 8 | 10 | 13 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 37800 | 119448 | 160272 | 201096 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 29600 | 125136 | 167904 | 210672 |
| Max. Throughput averaged over 2 frames | Mbps | 18.970 | 60.352 | 79.777 | 101.247 |

-----------------------------------------------------------End of change 1---------------------------------------------------------------

----------------------------------------------------- Beginning of Change 2------------------------------------------------------------

Table A.3.2.1.1-21: PDSCH Reference Channel for FDD CC and CA scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-21.1 FDD | R.PDSCH.1-21.2 FDD | R.PDSCH.1-21.3 FDD | R.PDSCH.1-21.4 FDD | R.PDSCH.1-21.5 FDD |
| Channel bandwidth | MHz | 30 | 35 | 40 | 45 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Number of allocated resource blocks | PRBs | 160 | 188 | 216 | 242 | 270 |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 | 19 | 19 | 19 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 4 | 4 | 4 | 4 | 4 |
| Number of DMRS REs |  | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 196776 | 233608 | 262376 | 295176 | 335976 |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 24 | 28 | 32 | 36 | 40 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 10, 11 | Bits | 437760 | 514368 | 590976 | 662112 | 738720 |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 460800 | 541440 | 622080 | 696960 | 777600 |
| Max. Throughput averaged over 2 frames | Mbps | 186.937 | 221.928 | 249.257 | 280.417 | 319.177 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Two codewords and given per codeword |  |  |  |  |  |  |

-----------------------------------------------------------End of change 2---------------------------------------------------------------