3GPP TSG-RAN4 Meeting #110bis	R4-2406068

# Cahngsha, China, 15th April 2024 - 19th April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-4: Reference measurement channels of typeII-doppler-r18 codebook tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of the reference measurement channel settings used for PMI reporting test with typeII-doppler-r18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the reference measurement channel settings used for PMI reporting test with typeII-doppler-r18. FDD 15kHz; TDD 30kHz. MCS; 13; 13. MIMO layers; 2; 2. FRC periodicity; 20 slots (20ms); 40 slots (20ms). Non-CSI slots; mod(i, 10) = {1,3,5,7,8,9}; mod(i, 10) = {5,6}. CSI slots; mod(i, 10) = {0,2,4,6}; mod(i, 10) = {0,1,2,3}. DTX slots; mod(i, 10) = {4,7,8,9}. SSB slots; i=0; i=0. TRS slots; i=10 and i=11; i=20 and i=21. |
| Consequences if not approved: |  | Test equipment cannot configure the test setup. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.1.1, A.3.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

Table A.3.2.1.1-X: PDSCH Reference Channel for FDD PMI reporting requirements

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-X.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 52 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 12 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layer |  | 2 |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 10) = {0,2,4,6}, i={0,…,19} |  | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod (i, 10) ={1,3,5,7,8,9}, i={1,..19} | Bits | 24072 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 10) = {0,2,4,6}, i={0,…,19} |  | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod (i, 10) ={1,3,5,7,8,9}, i={1,..19} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 10) = {0,2,4,6}, i={0,…,19} |  | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod (i, 10) ={1,3,5,7,8,9}, i={1,..19} | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For CSI Slots i, if mod (i, 10) = {0,2,4,6}, i={0,…,19} |  | N/A |  |  |  |  |
| For Slots i = 11 | Bits | 47424 |  |  |  |  |
| For Non CSI-RS Slot i, if mod (i, 10) = {1,3,5,7,8,9}, i={0,..10,12,…,19} | Bits | 49920 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 14.443 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

Table A.3.2.2.2-Y: PDSCH Reference Channel for TDD PMI reporting requirements with UL-DL pattern FR1.30-1

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-Y.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 40 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 8 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 2 |  |  |  |  |
| Number of DMRS REs (Note 3) |  | 24 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0,1,2,3} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod(i, 10) = {5,6} for i from {1,…,39} | Bits | 49176 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0,1,2,3} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod(i, 10) = {5,6} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,7,8,9} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0,1,2,3} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod(i, 10) = {5,6} for i from {1,…,39} | CBs | 6 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {4,7,8,9} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0,1,2,3} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Non CSI-RS Slot i, if mod(i, 10) = {5,6} for i from {1,…,39} | Bits | 101760 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 19.670 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------