3GPP TSG-RAN WG4 Meeting #110bis 	R4-2406289

Changsha, China, 15th – 19th April, 2024

Agenda item:			6.12.4

Source:	Moderator (Ericsson)

Title:	Ad-hoc minutes #2 on RRM requirements for NR_pos_enh2

Document for:	Approval

# Introduction

The document contains discussions on the topics as indicated below.

Positioning RRM core requirements across the following 3 threads:

- Topic # 1: core [110bis][214] NR_pos_enh2_part1 (-)

- Topic # 2: core [110bis][215] NR_pos_enh2_part2 (-)

- Topic # 3: core [110bis][216] NR_pos_enh2_part3 (-)

Positioning RRM performance across the following 3 threads:

- Topic # 4: perf [110bis][214] NR_pos_enh2_part1 (Monday/Apr15: PRS BW aggregation)

- Topic # 5: perf [110bis][215] NR_pos_enh2_part2 (-)

- Topic # 6: perf [110bis][216] NR_pos_enh2_part3 (-)

Draft CRs:

- Topic # 7: core draft CRs for all threads (-)

- Topic # 8: perf draft CRs for all threads (-)

Work split:

- Topic # 9: Work split and testing principles for the performance part of the WI (Monday/Apr15)

# Topic #1: core [110bis][214] NR_pos_enh2_part1

## RedCap positioning (Agenda 6.12.2.4)

Issues recommended for discussion by moderator:

Issues prioritized during the AH: -

## PRS/SRS bandwidth aggregation (Agenda 6.12.2.4)

Issues recommended for discussion by moderator:

Issues prioritized during the AH: -

# Topic #2: core [110bis][215] NR_pos_enh2_part2

## SL positioning (Agenda 6.12.2.2)

Issues recommended for discussion by moderator:

Issues prioritized during the AH: -

## Carrier phase positioning (Agenda 6.12.2.2)

Issues recommended for discussion by moderator:

Issues prioritized during the AH: -

# Topic #3: core [110bis][216] NR_pos_enh2_part3

## LPHAP (Agenda 6.12.2.3)

Issues recommended for discussion by moderator:

Issues prioritized during the AH: -

# Topic #4: performance [110bis][214] NR_pos_enh2_part1

## RedCap positioning (Agenda 6.12.3.3)

Issues recommended for discussion by moderator:

For performance requirements performance requirements:

- RedCap positioning

  - Issue 4-1-1-1: Bandwidth for accuracy requirement for RedCap positioning.

  - Issue 4-1-1-2: Accuracy requirement for RedCap positioning.

  - Issue 4-1-1-3: Accuracy requirement for RedCap positioning (reduced number of samples).

  - Issue 4-1-1-4: Accuracy requirement for PRS-RSRP and PRS-RSRPP with Rx FH.

  - Issue 4-1-1-5: Baseline for performance requirement for RedCap positioning.

Recommended issues related to test cases for the online discussion in order of decreasing priority.

- RedCap positioning

  - Issue 4-1-2-2: Same test case for 1Rx and 2Rx RedCap UEs.

  - Issue 4-1-2-4: Separate test cases for measurement delay and accuracy requirements for RedCap positioning.

Issues prioritized during the AH: -

## PRS/SRS bandwidth aggregation (Agenda 6.12.3.4)

Issues recommended for discussion by moderator:

For performance requirements performance requirements:

- PRS/SRS bandwidth aggregation

  - Issue 5-1-1-1: Side condition for PRS aggregation accuracy requirements.

  - Issue 5-1-1-2: Bandwidth for PRS aggregation accuracy requirements.

  - Issue 5-1-1-7: Update to simulation assumption for PRS aggregation.

  - Issue 5-1-1-3: Accuracy requirements for PRS-RSRP and PRS-RSRPP measurements based on aggregated PRS resources.

  - Issue 5-1-1-8: Considerations for performance requirements for PRS aggregation.

Recommended issues related to test cases for the online discussion in order of decreasing priority.

- PRS/SRS bandwidth aggregation

  - Issue 5-1-2-1: Considerations for test cases for PRS aggregation.

Issues prioritized during the AH: 5-1-1-1, 5-1-1-2, 5-1-1-3, 5-1-1-(3), 5-1-1-7

### Issue 5-1-1-1: Side condition for PRS aggregation accuracy requirements.

- Proposals

  - Option 1: CATT, CMCC, HW

    - The Rel-17 side conditions for RSTD and UE Rx-Tx time difference can be reused for PRS based positioning measurement with bandwidth aggregation.

- Tentative agreement:

  - Rel-17 side conditions for RSTD and UE Rx-Tx time difference is reused for PRS based positioning measurement with bandwidth aggregation.

- Recommended WF

  - Agree on tentative agreement.

Discussion:

Agreement:

Rel-17 side conditions for RSTD and UE Rx-Tx time difference is reused for PRS based positioning measurement with bandwidth aggregation.

### Issue 5-1-1-2: Bandwidth for PRS aggregation accuracy requirements.

- Proposals

  - Option 1: CATT

    - The following total bandwidth (MHz) after aggregation in AWGN in FR1 can be the starting points for accuracy requirements:

      - 15 kHz SCS: 40, 60.

      - 30 kHz SCS: 100, 150.

      - 60 kHz SCS: 100, 150.

    - The following total bandwidth (MHz) after aggregation in AWGN in FR2 can be the starting points for accuracy requirements:

      - 60 kHz SCS: 100, 150.

      - 120 kHz SCS: 200, 400.

  - Option 2: E///

    - For RSTD and UE Rx-Tx measurements based on PRS aggregation, in addition to the agreed per-PFL BWs, requirements are also defined for per-PFL BW 400 MHz (272 PRBs, 120 kHz SCS) in FR2.

- Recommended WF

  - Discuss the option(s).

Discussion:

Per-PFL bandwidths in accuracy requirements: agreed already in the last meeting.

Agreement:

FFS: For RSTD and UE Rx-Tx measurements based on PRS aggregation, in addition to the agreed per-PFL BWs, requirements are also defined for per-PFL BW 400 MHz (272 PRBs, 120 kHz SCS) in FR2.

Option 1 is to be discussed together with issue 5-1-1-(3) below.

### Issue 5-1-1-(3): Accuracy requirements for 2 PFL and 3 PFL cases.

- Proposals

  - Option 1: E///, HW

    - Accuracy requirements are defined separately for the 2 PFL and 3 PFL cases.

- Proposal 2: CATT

  - The following total bandwidth (MHz) after aggregation in AWGN in FR1 can be the starting points for accuracy requirements:

    - 15 kHz SCS: 40, 60.

    - 30 kHz SCS: 100, 150.

    - 60 kHz SCS: 100, 150.

  - The following total bandwidth (MHz) after aggregation in AWGN in FR2 can be the starting points for accuracy requirements:

    - 60 kHz SCS: 100, 150.

    - 120 kHz SCS: 200, 400.

- Tentative agreement:

  - Accuracy requirements are defined separately for the 2 PFL and 3 PFL cases.

- Recommended WF

  - Agree on tentative agreement.

Discussion:

Conclusion: Further discussion is needed on whether within the same FR the same requirements can apply for a total BW, regardless of the combination of #PRBs.

### Issue 5-1-1-7: Update to simulation assumption for PRS aggregation.

- Proposals

  - Option 1: E///

    - Update simulation assumptions for PRS aggregation to also capture per-PFL BW 400 MHz (272 PRBs, 120 kHz SCS) in FR2.

  - Option 2: QC

    - Update the simulation assumptions for PRS BW aggregation to include the gap between adjacent intra-band contiguous PFLs.

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Gap between PFLs (num subcarriers) | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 84 | 1 | 64 |
|  | 30 | 132 | 82 | 1 | 32 |
|  |  | 272 | 68 | 1 | 16 |
|  | 60, FR1 | 64 | 65 | 1 | 32 |
|  |  | 132 | 82 | 1 | 16 |
|  | 60, FR2 | 64 | 64 | 1 | 32 |
|  |  | 132 | 82 | 1 | 16 |
|  | 120 | 64 | 65 | 1 | 16 |
|  |  | 128 | 82 | 1 | 8 |
| PRS comb size | 4 |  |  |  |  |
| PRS symbol size | 4 |  |  |  |  |
| Number of samples | 2, 4 |  |  |  |  |
| PRS periodicity | 40ms, 200ms |  |  |  |  |
| TOA estimation | Realistic |  |  |  |  |
| Path # | First path |  |  |  |  |

- Recommended WF

  - Discuss the option(s).

Discussion:

Agreement:

Companies further check in this meeting and conclude by the end of the meeting whether the update in simulation assumptions is needed or not.

      - Ericsson, Q: we followed Option 2 already, no need to update.

### Issue 5-1-1-3: Accuracy requirements for PRS-RSRP and PRS-RSRPP measurements based on aggregated PRS resources.

- Proposals

  - Option 1: E///

    - For a given side condition and propagation condition, existing accuracy requirement for the largest PRS BW applies to the PRS-RSRP/RSRPP measurements based on the PRS aggregation.

- Recommended WF

  - Discuss the option(s).

Discussion:

Agreements:

FFS:

      - Option 1: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement for the largest PRS BW among the aggregated PFLs applies to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation.

      - Option 2: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement applies for the total aggregated BW to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation.

# Topic #5: performance [110bis][215] NR_pos_enh2_part2

## SL positioning (Agenda 6.12.3.1)

Issues recommended for discussion by moderator: Sub topic 3-1: issue 3-1-1/3/4/6/7

Issues prioritized during the AH: -

## Carrier phase positioning (Agenda 6.12.3.5)

Issues recommended for discussion by moderator: Sub topic 4-1: issue 4-1-1/2/3/5/6/7

Issues prioritized during the AH: -

# Topic #6: performance [110bis][216] NR_pos_enh2_part3

## LPHAP (Agenda 6.12.3.2)

Issues recommended for discussion by moderator:

- Issue 1-2-2: combination of measurement types and RRC states

- Issue 1-2-1: whether to define accuracy TCs for LPHAP

- Issue 1-2-3: eDRX configurations

- Issue 1-2-7: UE types to be tested

Issues prioritized during the AH: -

- Topic #7: Core draft CRs for all threads

## General (AI 6.12.2.1)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2405196 | (NR_pos_enh2-Core) Modify positioning measurements related in RRC_INACTIVE state. | ZTE Corp. |  |  |

## SL positioning (AI 6.12.2.2)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2404950 | Draft CR on core requirement for SL positioning measurement | vivo |  |  |
| R4-2405528 | Draft CR for 38.133 on SL positioning RRM core requirements | Ericsson |  |  |
| R4-2405577 | draftCR on RRM requirements for SL positioning | Huawei, HiSilicon |  |  |

## Carrier Phase positioning (AI 6.12.2.2)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2405506 | DraftCR to TS38.133 Carrier phase based positioning core requirements | Ericsson |  |  |
| R4-2405578 | draftCR on RRM requirements for CPP | Huawei, HiSilicon |  |  |
| R4-2405884 | Draft CR 38.133 Corrections to measurement period requirements for NR CPP | Nokia |  |  |

## LPHAP (AI 6.12.2.3)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2404904 | (NR_pos_enh2-Core) Draft CR on UE transmit timing for positioning measurements | CMCC |  |  |
| R4-2405508 | DraftCR to TS38.133 LPHAP core requirements | Ericsson |  |  |
| R4-2405579 | draftCR on RRM requirements for LPHAP | Huawei, HiSilicon |  |  |

## RedCap positioning (AI 6.12.2.4)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2404422 | Draft CR on core requirements of RedCap UE positioning | CATT |  |  |
| R4-2405510 | DraftCR to TS38.133 RedCap positioning core requirements | Ericsson |  |  |
| R4-2405581 | draftCR on RRM requirements for RedCap positioning | Huawei, HiSilicon |  |  |
| R4-2405813 | Draft CR on bandwidth in measurement for RedCap positioning | Nokia |  |  |

## PRS/SRS bandwidth aggregation (AI 6.12.2.4)

| TDoc | Title | Source | Comments | Recommendation |
| --- | --- | --- | --- | --- |
| R4-2404423 | Draft CR on core requirements of BW aggregation positioning | CATT |  |  |
| R4-2405511 | DraftCR to TS38.133 Bandwidth aggregation for positioning core requirements | Ericsson |  |  |
| R4-2405582 | draftCR on RRM requirements for PRS CA | Huawei, HiSilicon |  |  |

| TDoc | Title | Source | Comments | Decision |
| --- | --- | --- | --- | --- |
| R4-2405513 | Skeleton for accuracy and report mapping for Rel. 18 positioning | Ericsson | 10.1A.X/Y/Z/X1, 10.1.X/Y/Z/X1/Y1/Z1 | Can be endorsed? |
| R4-2405530 | Draft CR for 38.133 on SL positioning performance requirements | Ericsson | 3, A.9A, B.4A | Can be endorsed? |

- Topic #8: Perf draft CRs for all threads

# Topic #9: Work split and testing principles for the performance part of the WI

Issues recommended for discussion by moderator:

- General aspects:

  - Issue 3-1-1: Timeline for DraftCR submissions.

  - Issue 3-1-2: Test case for carrier phase positioning.

  - Issue 3-1-3: Testing principles for Rel. 18 positioning.

Issues prioritized during the AH: 3-1-1

## Issue 3-1-1: Timeline for DraftCR submissions.

- Proposals

  - Option 1: E///

    - Phase I DraftCRs are submitted to RAN4#111.

    - Phase II DraftCRs are submitted to RAN4#112.

- Tentative agreement:

  - Phase I DraftCRs are submitted to RAN4#111.

  - Phase II DraftCRs are submitted to RAN4#112.

- Recommended WF

  - Agree on tentative agreement.

Discussion:

Agreements:

LPHAP, SL positioning, CPP: all draft CRs are to be provided in May meeting.

RedCap and PRS/SRS BW aggregation: phased approach applies

      - Phase I DraftCRs are submitted to RAN4#111

        - Accuracy requirements CRs

        - Selected test cases

      - Phase II DraftCRs are submitted to RAN4#112.

        - Selected test cases

Note: Discuss further during the meeting the selection of test cases for Phase I and Phase II.

# Annex: Remaining issues after ad hoc #1 which were not discussed

## Core part

Core issues are to be discussed in ad hoc #2, no core issue discussion is yet planned outside ad hoc #2.

## Performance part

### RedCap positioning (Agenda 6.12.3.3)

#### Issue 4-1-1-1: Bandwidth for accuracy requirement for RedCap positioning.

Proposals

  - Option 1: CATT

    - The bracketed values are proposed extended bandwidth values for RedCap UE in FR1:

      - 15 kHz: 5, 10, 20, [50, 80, 100] MHz

      - 30 kHz: 10, 20, [50, 80, 100] MHz

      - 60 kHz: 20, [50, 100] MHz.

    - The bracketed values are proposed extended bandwidth values for RedCap UE in FR2:

      - 60 kHz: 20, 50, 100, [200, 300, 500] MHz

      - 120 kHz: 50, 100, [200, 300, 500] MHz

    - The values outside brackets can be starting points for accuracy requirements without FH.

  - Option 2: HW

    - For RedCap with FH, RAN4 to discuss the per hop BW and total measurement BW across all hops for defining accuracy requirements.

  - Option 3: Nokia

    - RAN4 should define the different accuracy requirements depending on the number of hops within a single MG occasion.

  - Option 4: QC

    - Define measurement accuracy requirements for RSTD with Rx FH and 2Rx in AWGN based on the total PRS bandwidth, PRS bandwidth per hop and overlap between hops (1 RB) in the simulation assumptions. The requirement in each row applies for a multi-hop RSTD measurement provided:

- the minimum expected measured PRS BW is equal or larger than the total PRS BW listed in that row, and

- the max BW per hop (component 1 of FG 13-1) supported by the UE is equal or larger to the value listed in that row.

| FR | Total PRS BW (PRBs) | SCS (kHz) | PRS BW per hop (MHz) | sample rate (Tc) | Num. samples | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | ≥ 272 | 15 | ≥ 10 | 32 | 2 | ±X1 |
|  | ≥ 272 | 30 | ≥ 20 | 16 | 2 | ±X2 |
|  | ≥ 132 | 60 | ≥ 20 | 16 | 2 | ±X3 |
| FR2 | ≥ 272 | 60 | ≥ 50 | 8 | 2 | ±X4 |
|  | ≥ 272 | 120 | ≥ 100 | 4 | 2 | ±X5 |

| FR | Total PRS BW (PRBs) | SCS (kHz) | PRS BW per hop (MHz) | sample rate (Tc) | Num. samples | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | ≥ 272 | 15 | ≥ 10 | 32 | 4 | ±Y1 |
|  | ≥ 272 | 30 | ≥ 20 | 16 | 4 | ±Y2 |
|  | ≥ 132 | 60 | ≥ 20 | 16 | 4 | ±Y3 |
| FR2 | ≥ 272 | 60 | ≥ 50 | 8 | 4 | ±Y4 |
|  | ≥ 272 | 120 | ≥ 100 | 4 | 4 | ±Y5 |

- Define measurement accuracy requirements for RSTD with Rx FH and 1Rx in AWGN based on the total PRS bandwidth, PRS bandwidth per hop and overlap between hops (1 RB) in the simulation assumptions. The requirement in each row applies for a multi-hop RSTD measurement provided:

- the minimum expected measured PRS BW is equal or larger than the total PRS BW listed in that row, and

- the max BW per hop (component 1 of FG 13-1) supported by the UE is equal or larger to the value listed in that row.

| FR | Total PRS BW (PRBs) | SCS (kHz) | PRS BW per hop (MHz) | sample rate (Tc) | Num. samples | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | ≥ 272 | 15 | ≥ 10 | 32 | 2 | ±X1 |
|  | ≥ 272 | 30 | ≥ 20 | 16 | 2 | ±X2 |
|  | ≥ 132 | 60 | ≥ 20 | 16 | 2 | ±X3 |

| FR | Total PRS BW (PRBs) | SCS (kHz) | PRS BW per hop (MHz) | sample rate (Tc) | Num. samples | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | ≥ 272 | 15 | ≥ 10 | 32 | 4 | ±Y1 |
|  | ≥ 272 | 30 | ≥ 20 | 16 | 4 | ±Y2 |
|  | ≥ 132 | 60 | ≥ 20 | 16 | 4 | ±Y3 |

    - Define measurement accuracy requirements for UE Rx-Tx with Rx FH and 2Rx in AWGN based on the total PRS bandwidth, PRS bandwidth per hop and overlap between hops (1 RB) in the simulation assumptions. The requirement in each row applies for a multi-hop Rx-Tx measurement provided:

- the minimum expected measured PRS BW is equal or larger than the total PRS BW listed in that row, and

- the max BW per hop (component 1 of FG 13-1) supported by the UE is equal or larger to the value listed in that row.

| FR | Total PRS BW (PRBs) | SCS (kHz) | PRS BW per hop (MHz) | sample rate (Tc) | Num. samples | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
| FR1 | ≥ 272 | 15 | ≥ 10 | 32 | 2 | X1a | X1b |
|  | ≥ 272 | 30 | ≥ 20 | 16 | 2 | X2a | X2b |
|  | ≥ 132 | 60 | ≥ 20 | 16 | 2 | X3a | X3b |

| FR | Total PRS BW (PRBs) | SCS (kHz) | PRS BW per hop (MHz) | sample rate (Tc) | Num. samples | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | ≥ 272 | 15 | ≥ 10 | 32 | 2 | Y1a | Y1b |
|  | ≥ 272 | 30 | ≥ 20 | 16 | 2 | Y2a | Y2b |
|  | ≥ 132 | 60 | ≥ 20 | 16 | 2 | Y3a | Y3b |

- Recommended WF

  - Discuss the option(s).

Discussion:

Not discussed.

#### Issue 4-1-1-2: Accuracy requirement for RedCap positioning.

- Proposals

  - Option 1: CATT

    - RAN4 follow the following steps to define accuracy requirements for cases with and without FH:

      - Step 1: Define candidate accuracy requirements for both cases with and without FH separately.

      - Step 2: If possible, RAN4 try to merge two sets of accuracy requirements for cases with and without FH into one accuracy table to reduce specification redundancy.

  - Option 2:

    - Option 2a (CMCC, HW):

      - For RedCap with 2RX without FH, existing accuracy requirements are reused except for the applicable BW.

    - Option 2b (HW):

      - For RedCap with 1RX without FH, new accuracy requirements are defined based on the agreed Es/Iot side condition and related simulation results.

      - RAN4 to define new accuracy requirements for FH instead of re-using the requirements for non-FH.

  - Option 3: E///

      - Accuracy requirements are different for the cases with and without Rx FH for the scenario where the total PRS BW after all hops is larger than the PRS BW without Rx FH.

      - Accuracy requirements can be same for the cases with and without Rx FH for the scenario where the total PRS BW after all hops is equal to the PRS BW without Rx FH.

  - Option 4: Nokia

      - The accuracy requirements with FH and without FH should be different.

- Recommended WF

  - Discuss the option(s).

Discussion:

Not discussed.

#### Issue 4-1-1-3: Accuracy requirement for RedCap positioning (reduced number of samples).

- Proposals

  - Option 1: E///

    - Reduced sample RSTD accuracy requirement for 1 Rx RedCap UE with and without Rx FH in FR1 is defined for the AWGN propagation condition only. The requirement is defined for the side condition -3 dB for reference cell and -6 dB for target cell.

    - Reduced sample RSRP/UE Rx-Tx accuracy requirement for 1 Rx RedCap UE with and without Rx FH in FR1 is defined for the AWGN propagation condition only. The requirements are defined for 0 dB and -6 dB side conditions.

    - Reduced sample PRS-RSRPP accuracy requirement for 1 Rx RedCap UE with and without Rx FH in FR1 is defined for the Two-Tap channel model only. The requirements are defined for 0 dB and -6 dB side conditions.

    - Reduced sample RSTD/RSRP/UE Rx-Tx accuracy requirement for 2 Rx RedCap UE with and without Rx FH is only defined for the AWGN propagation condition. The requirement is defined for Rel. 17 side conditions.

    - Reduced sample PRS-RSRPP accuracy requirement for 2 Rx RedCap UE with and without Rx FH is only defined for the Two-Tap channel model only. The requirement is defined for Rel. 17 side conditions.

- Tentative agreement:

  - Reduced sample RSTD accuracy requirement for 1 Rx RedCap UE with and without Rx FH in FR1 is defined for the AWGN propagation condition only. The requirement is defined for the side condition -3 dB for reference cell and -6 dB for target cell.

  - Reduced sample RSRP/UE Rx-Tx accuracy requirement for 1 Rx RedCap UE with and without Rx FH in FR1 is defined for the AWGN propagation condition only. The requirements are defined for 0 dB and -6 dB side conditions.

  - Reduced sample PRS-RSRPP accuracy requirement for 1 Rx RedCap UE with and without Rx FH in FR1 is defined for the Two-Tap channel model only. The requirements are defined for 0 dB and -6 dB side conditions.

  - Reduced sample RSTD/RSRP/UE Rx-Tx accuracy requirement for 2 Rx RedCap UE with and without Rx FH is only defined for the AWGN propagation condition. The requirement is defined for Rel. 17 side conditions.

  - Reduced sample PRS-RSRPP accuracy requirement for 2 Rx RedCap UE with and without Rx FH is only defined for the Two-Tap channel model only. The requirement is defined for Rel. 17 side conditions.

- Recommended WF

  - Agree on tentative agreement.

Discussion:

Not discussed.

#### Issue 4-1-1-4: Accuracy requirement for PRS-RSRP and PRS-RSRPP with Rx FH.

- Proposals

  - Option 1: QC

    - For PRS-RSRP and PRS-RSRPP measurements with FH, apply the measurement accuracy requirements without FH defined for the corresponding PRS BW per hop.

- Recommended WF

  - Discuss the option(s).

Discussion:

Not discussed.

#### Issue 4-1-1-5: Baseline for performance requirement for RedCap positioning.

- Proposals

  - Option 1: Nokia

    - Using 4 measurement samples should be the baseline to define the performance requirements.

    - Using 1 UE Rx antenna for FR1 and 2 UE Rx antennas for FR2 should be the baseline to define the performance requirements.

- Recommended WF

  - Discuss the option(s).

Discussion:

Not discussed.

#### Issue 4-1-2-2: Same test case for 1Rx and 2Rx RedCap UEs.

- Proposals

  - Option 1: HW

    - Define a single set of TCs for 1RX and 2RX RedCap UEs.

  - Option 2: QC

    - For RedCap positioning measurement delay test cases in FR1, define common test cases for 1Rx and 2Rx, setting the side condition so that it applies to both types of devices.

- Recommended WF

  - Discuss the option(s).

Discussion:

Not discussed.

#### Issue 4-1-2-4: Separate test cases for measurement delay and accuracy requirements for RedCap positioning.

- Proposals

  - Option 1: HW

    - Use a single set of TCs to verify both delay and accuracy for RedCap UE with FH.

  - Option 2: E///

    - Test cases for measurement delay requirements needs to be defined for both without FH and with FH cases. Rel. 17 test cases are reused to define measurement delay test cases for without FH case.

    - Test cases for measurement accuracy requirements need to be defined for both without FH and with FH cases. Rel. 17 accuracy test cases are reused to define accuracy test case for 2Rx RedCap UE for the without FH case.

  - Option 3: CMCC

    - For RedCap positioning, it is proposed to define delay and accuracy tests for RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP.

- Recommended WF

  - Discuss the option(s).

Discussion:

Not discussed.

### PRS/SRS bandwidth aggregation (Agenda 6.12.3.4)

#### Issue 5-1-1-8: Considerations for performance requirements for PRS aggregation.

- Proposals

  - Option 1: Nokia

    - To define the performance requirements for PRS BW aggregation, use 4 measurement samples as a baseline.

    - The measurement accuracy requirement for RSTD/UE Rx-Tx time difference should depend on the baseband sampling rate as well as the number of PFLs.

- Recommended WF

  - Discuss the option(s).

Discussion:

Not discussed

#### Issue 5-1-2-1: Considerations for test cases for PRS aggregation.

- Proposals

  - Option 1: CATT

    - The existing test cases for positioning can be used as baseline. Different number of aggregated PFLs/total aggregated bandwidths should be considered when specifying configurations. The test case lists for delay requirements and accuracy requirements are provided in Table 5 and Table 6 respectively. Check Table 5 and Table 6 in R4-2404425 for details.

  - Option 2: HW

    - For RRM test for PRS CA, only set up the PRS resources for aggregate measurement.

- Recommended WF

  - Discuss the option(s).

Discussion:

Not discussed.

### SL positioning (Agenda 6.12.3.1)

#### Issue 3-1-1: SINR side conditions

- Proposals

For SL RSTD,

  - Option 1: (CATT, CMCC, vivo)

    - [-3, -6] dB for reference and target UE.

  - Option 2: (OPPO, Ericsson, Nokia)

    - [0, -6] dB for reference and target UE.

  - Option 3: (Huawei, Qualcomm)

    - (0, -3) dB for reference and target UE.

For SL UE Rx-Tx and SL RSRP(P),

  - Option 1: (CATT, CMCC, vivo, OPPO, Ericsson)

    - -6 dB for SL PRS RSRP/RSRPP and SL UE Rx-Tx.

  - Option 2: (Huawei)

    - -3 dB for SL PRS RSRP/RSRPP and SL UE Rx-Tx for the target UE.

  - Option 3: (Qualcomm)

    - Define two side conditions for SL Rx-Tx, SL PRS-RSRP and SL PRS-RSRPP:  0 dB and -3 dB.

  - Option 4: (Nokia)

    - Define SINR condition for SL Rx-Tx, SL-PRS RSRP and SL-PRS RSRPP: - 6 dB and 0 dB.

For SL AoA and SL RTOA,

  - Option 1: (OPPO, Ericsson)

    - [-6] dB for SL AoA, SL RTOA measurements.

- Recommended WF

  - Discuss the option(s).

Discussion:

Not discussed.

#### Issue 3-1-3: Assumptions to define SL PRS measurement accuracy requirements

- Proposals

  - Option 1: (CATT, Qualcomm)

    - One set of requirements is defined for all the comb configurations including both fully staggered and partially staggered (half comb).

  - Option 2: (OPPO, Huawei)

    - Reuse the channel of Uu counterparts for SL PRS measurement.

  - Option 3: (Qualcomm)

    - For measurement accuracy requirements with multiple samples (Nsample = 4), do not assume coherent combining between samples.

  - Option 4: (Nokia)

    - Define measurement accuracy requirements for SL-PRS based RSTD, SL-PRS based UE Rx-Tx time difference, SL-PRS based RSRP and SL-PRS based RSRPP

      - independent of SL-PRS comb size,

      - for measurements derived from SL-PRS transmissions/receptions on a single carrier and single numerology,

      - without coherent combining between samples,

      - for same channel profiles as defined for Uu

      - for measurement samples 1 or 4 depending on the RB number of PRS BW (lower/equal or higher than 48 RB)

- Recommended WF

  - Can option 4 be agreed?

Discussion:

Not discussed.

#### Issue 3-1-4: Measurement accuracy requirements

Agreements in RAN4#110: - Update the definition of Nsample as the following: - Nsample = 1 for SL-PRS BW > 48 PRBs, - Nsample = 4 for SL-PRS BW ≤ 48 PRBs - FFS whether for 48 PRBs SL-PRS BW a lower number of samples can be used based on performance results.

- Proposals

  - Option 1: (CATT)

    - Take table 1 as the structure for SL PRS based measurement accuracy requirements.

- Table 1: SL RSTD/RSRP/RSRPP/UE Rx-Tx accuracy in FR1

| Accuracy | SL PRS Ês/Iot | SL PRS SCS | SL PRS bandwidth | Nsample |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | RB |  |
| [TBD] | For RSTD: (Ês/Iot)ref ≥-3dB (Ês/Iot)i ≥-6dB For other measurements: (Ês/Iot)i ≥-6dB | 15 | ≤ 48 | 4 |
|  |  |  | > 48 | 1 |
| [TBD] |  | 30 | ≤ 48 | 4 |
|  |  |  | > 48 | 1 |
| [TBD] |  | 60 | ≤ 48 | 4 |
|  |  |  | > 48 | 1 |

    - The accuracy requirements can be defined based on the simulation results and a small implementation margin (clock drift).

  - Option 2: (OPPO)

    - Define one set of accuracy requirements depending on configuration of {SCS, Bandwidth, min SL resource repetition}, where SL PRS resource repetition is defined as

LSL-PRS/KcombSL-PRS .

  - Option 3: (vivo, Qualcomm)

    - RAN4 to define accuracy requirement for SL RSTD and SL Rx-Tx by considering the framework of ±(X+Y+Z) Tc and ±(X+) Tc, respectively.

      - X is the simulated measurement accuracy for a given propagation condition and number of measurement samples,

      - Y is the frequency/clock drift margin,

      - Z and  are the RF calibration margins.

  - Option 4: (vivo)

    - Define measurement accuracy requirements based on the following RB number configuration

      - 15kHz SCS: 48, 96

      - 30kHz SCS: 24, 48,

      - 60kHz SCS: 24

    - RAN4 to use lower number of samples for 48 PRBs SL-PRS BW, and at least 3 samples can be considered.

  - Option 5: (Ericsson)

    - Two sets of accuracy requirements are to be defined for:

      - 1 sample, and

      - 4 samples.

  - Option 6: (Huawei)

    - Accuracy requirements for SL PRS measurements are defined based on BW of

      - 15kHz SCS: 48 RB≤BW< 96 RBs, 96 RB≤BW

      - 30kHz SCS: 24 RB≤BW< 48 RBs, 48 RB≤BW<96 RBs

      - 60kHz SCS: 24 RB≤BW

    - Use Nsample = 1 for SL-PRS BW = 48 PRB.

  - Option 7: (Qualcomm)

    - Define SL position measurement accuracy using the following structure, aligned with the simulation assumptions.

| Accuracy (Tc) | SL PRS Ês/Iot (dB) | SL PRS SCS (kHz) | SL PRS bandwidth (num RB) | Nsample |
| --- | --- | --- | --- | --- |
| [TBD] | (Ês/Iot)ref ≥TBD (Ês/Iot)i ≥TBD | 15 | ≥ 48 | [1] |
|  |  |  | ≥ 96 | 1 |
| [TBD] |  | 30 | ≥ 24 | 4 |
|  |  |  | ≥ 48 | [1] |
|  |  |  | ≥ 96 | 1 |
| [TBD] |  | 60 | ≥ 24 | 4 |

- Recommended WF

  - Discuss the framework of accuracy requirements based on the following table:

- Table 1: SL RSTD/RSRP/RSRPP/UE Rx-Tx accuracy in FR1

| Accuracy | SL PRS Ês/Iot | SL PRS SCS | SL PRS bandwidth | Nsample |
| --- | --- | --- | --- | --- |
| Tc | dB | kHz | RB |  |
| [X+] | For RSTD: (Ês/Iot)ref ≥[-3]dB (Ês/Iot)i ≥[-6]dB For other measurements: (Ês/Iot)i ≥[-6]dB | 15 | [< 48] | [4] |
|  |  |  | [≥ 48] | [1] |
|  |  |  | [≥ 96] | [1] |
| [X+] |  | 30 | [< 48] | [4] |
|  |  |  | [≥ 48] | [1] |
|  |  |  | [≥ 96] | [1] |
| [X+] |  | 60 | [≥ 24] | [4] |

Discussion:

Not discussed during the meeting.

#### Issue 3-1-6: Test case list

Agreements in RAN4#110: - Define measurement delay test cases for SL RSTD, SL RTOA, SL Rx-Tx and SL-AoA/ZoA. - Define measurement accuracy test cases for SL RSTD, SL Rx-Tx. - Further discuss whether and how to define delay test cases for SL RSRP and SL RSRPP. - Further discuss whether and how to define accuracy test cases for SL RSRP and SL RSRPP.

- Proposals

  - Option 1: (CATT)

    - Define test cases as listed in table 2.

- Table 2: Test case list for sidelink positioning

| Set | SL PRS measurement test case scenarios | Impacted section in TS 38.133 |
| --- | --- | --- |
| SL PRS measurement delay tests in RRC_CONNECTED |  |  |
| 1-1 | SL RSTD together with SL RSRP reporting delay test case in FR1 |  |
| 1-2 | SL UE Rx-Tx reporting delay test case in FR1 |  |
| 1-3 | SL RTOA reporting delay test case in FR1 |  |
| 1-4 | SL AOA reporting delay test case in FR1 |  |
| SL PRS measurement accuracy tests in RRC_CONNECTED |  |  |
| 2-1 | SL RSTD together with SL RSRP accuracy test case in FR1 |  |
| 2-2 | SL UE Rx-Tx accuracy test case in FR1 |  |

  - Option 2: (CMCC)

    - It is proposed to define accuracy test cases for SL-PRS based RSRP and SL-PRS based RSRPP.

  - Option 3: (OPPO)

    - SL RSRP accuracy can be verified with the test case of Rx-Tx. And a dedicated test case with two-tap channel is needed for SL RSRPP accuracy.

  - Option 4: (vivo)

    - RAN4 to define accuracy test cases for SL RSRP reported with SL RSTD and SL RSRP reported with SL Rx-Tx.

    - RAN4 may need to define delay tests for sync source changes.

  - Option 5: (Ericsson)

    - Test cases for all SL positioning measurements are specified (SL RSTD, RSRP/RSRPP, Rx-Tx, AoA, and RTOA).

      - FFS: SL-RSRP and SL-RSRPP test cases are joint with other measurements.

    - No need to define test cases for additional paths.

  - Option 6: (Huawei)

    - RAN4 to define accuracy TCs for SL PRS-RSRP(P) jointly with other measurement, and only accuracy of SL PRS-RSRP(P) is verified (accuracy of other measurement is not verified).

    - RAN4 to consider the following scope for the test.

      - Resource pool: shared and dedicated

      - Coverage: in coverage and gNB as sync reference

  - Option 7: (Qualcomm)

    - Define SL positioning test cases for a single carrier and single resource pool.

    - Define measurement accuracy test cases for SL PRS-RSRP (reported with SL RSTD or Rx-Tx) and SL PRS-RSRPP (reported with SL RSTD or Rx-Tx).

    - No need to define test cases for additional paths.

    - Do not define SL positioning test cases with LoS/NLoS reporting.

    - Do not define SL positioning test cases with Tx/Rx ARP-ID reporting.

  - Option 8: (OPPO, vivo, Qualcomm)

    - Not define test case to verify delay requirements for SL RSRP and SL RSRPP.

- Recommended WF

  - Discuss the test case list based on the following table.

- Table 2: Test case list for sidelink positioning

| Set | SL PRS measurement test case scenarios | Impacted section in TS 38.133 |
| --- | --- | --- |
| General |  |  |
| 0-1 | SL PRS configuration | A.3.x |
| SL PRS measurement delay tests in RRC_CONNECTED |  |  |
| 1-1 | SL RSTD reporting delay test case in FR1 |  |
| 1-2 | SL UE Rx-Tx reporting delay test case in FR1 |  |
| 1-3 | SL RTOA reporting delay test case in FR1 |  |
| 1-4 | SL AOA reporting delay test case in FR1 |  |
| SL PRS measurement accuracy tests in RRC_CONNECTED |  |  |
| 2-1 | SL RSTD together with SL RSRP accuracy test case in FR1 |  |
| 2-2 | SL UE Rx-Tx together with SL RSRPP accuracy test case in FR1 |  |

Discussion:

Not discussed

#### Issue 3-1-7: Test configurations

- Proposals

  - Option 1: (CATT)

    - Define SL PRS configurations for test as listed in table 3 and the other configurations can use the existing V2X test as baseline.

- Table 3: SL PRS configurations for test

| PRS Parameters | Values |
| --- | --- |
| SL PRS transmission for different SL UE | TDMed in different slots |
| SCS | 15kHz, 30kHz |
| PRS comb size | 4, 2 |
| Number of PRS symbol | 4 |
| SL PRS resource time gap (slot) | 1 |
| RB numbers containing PRS within channel BW | 48，96 |
| Resource pool | dedicated |
| Note 1: Unless otherwise specified in the test case |  |

  - Option 2: (vivo)

    - RAN4 to define test propagation conditions as AWGN channel for:

Delay test cases: SL RSTD, SL RTOA, SL Rx-Tx, SL-AoA/ZoA and,

Accuracy test cases: SL RSTD, SL Rx-Tx, SL RSRP reported with SL RSTD and SL RSRP reported with SL Rx-Tx.

    - RAN4 to consider whether to define test propagation conditions as 2-tap channel for accuracy test cases: SL RSRPP reported with SL RSTD and SL RSRPP reported with SL Rx-Tx.

    - RAN4 to define the test cases for dedicated resource pool and shared resource pool as test configurations, and the SL PRS configurations could apply to both types of resource pool.

  - Option 3: (Ericsson)

    - For each SL positioning measurement type, test cases for core requirements for SL positioning measurements are defined at least for both:

      - 1 sample, and

      - 4 samples.

    - For each SL positioning measurement type (for which accuracy requirements are defined), test cases for measurement accuracy requirements are defined for both:

      - 1 sample, and

      - 4 samples.

  - Option 4: (Huawei)

    - RAN4 to consider the following SL PRS related parameters for the test.

      - (symbol num, comb size): (4, 4) and (2, 4)

      - BW: 48 RB for delay test

      - Es/Iot: 3dB (to ensure 100% PSCCH decoding)

    - RAN4 to consider the following SL PRS transmission pattern for the delay test.

      - TX UE1: slot n

      - TX UE2: slot n + 1 and slot n + 100ms

    - Define SL positioning test cases using only AWGN and 2-tap channel (for SL PRS RSRPP) propagation conditions.

  - Option 5: (Qualcomm)

    - Define SL positioning test cases only with AWGN propagation condition.

    - Define SL PRS configurations for SL positioning test cases that apply to both shared and dedicated resource pools

      - No TDM of SL PRS from different UEs in the same slot

      - No comb-based multiplexing of SL PRS from different UEs in the same slot

      - No FDM of SL PRS from different UEs in the same slot

    - FFS whether to use the 2-tap channel for testing SL PRS-RSRPP measurement accuracy.

- Recommended WF

  - Discuss the option(s) considering the following aspects.

    - SL PRS configurations

    - SL PRS transmission pattern

    - Propagation conditions

    - Type of resource pool

    - Sample number

    - Es/Iot condition

Discussion:

Not discussed.

### Carrier phase positioning (Agenda 6.12.3.5)

#### Issue 4-1-1: Side condition

Background:

Agreements from RAN4#109 [2]:

·       Define DL RSCPD measurement accuracy requirements for side condition [-3, -6] dB if RSTD measurement is done with reduced number of samples.

·       FFS: Define DL RSCPD measurement accuracy requirements for side condition [-6, -13] dB if RSTD measurement is done over 4 samples.

Proposals

- For DL RSCPD measurement:

  - Option 1: (CATT, Qualcomm, Nokia(P2 in R4-2405883))

    - Two sets of side conditions: [-6, -13]dB and [-3, -6]dB

  - Option 2: (Ericsson, Huawei)

    - [-6, -13] dB.

- For relative DL RSCP measurement:

  - Option 1: (CATT, Qualcomm)

    - Two sets of side conditions: [-6, -13]dB and [-3, -6]dB

  - Option 2: (Huawei)

    - [-3, -13]dB

  - Option 3: (Nokia)

    - Two sets of side conditions: [-3, -13]dB and [0, -6]dB

Recommended WF:

- Discuss the option(s).

Discussion:

Not discussed.

#### Issue 4-1-2: DL RSCPD absolute accuracy requirements

Proposals

- Proposal 1: (CATT, CMCC, OPPO, Ericsson, Huawei, Qualcomm)

  - Accuracy requirements for DL RSCPD and relative DL RSCP are defined using same RB numbers as used in existing RSTD and UE Rx-Tx accuracy requirements.

- Proposal 2: (OPPO)

  - On top of simulated CPP measurement results, additional margins for frequency drift and RF calibration should be considered when defining RSCPD and relative RSCP accuracy requirements.

- Proposal 3: (Qualcomm)

  - RAN4 to add margin to the RSCPD accuracy derived from RAN4 simulations to account for residual frequency errors

Recommended WF:

- Take proposal 1 as baseline, i.e., the following tables are used as baseline structure to define DL RSCPD absolute accuracy requirements.

  - Whether side condition [-3, -6] is needed depends on the discussion in issue 4-1-1.

- Discuss DL RSCP relative accuracy requirements in issue 4-1-3 after the side condition is decided.

- Discuss proposal 2.

- Table 1: DL RSCPD absolute accuracy in FR1 for AWGN channel

| Accuracy | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 |
| --- | --- | --- | --- |
| Tc Note 5 | dB | kHz | RB |
| [TBD] | (Ês/Iot)ref ≥-6dB (Ês/Iot)i ≥-13dB | 15 | ≥ 24 |
| [TBD] |  |  | ≥ 52 |
| [TBD] |  |  | ≥ 104 |
| [TBD] |  | 30 | ≥ 24 |
| [TBD] |  |  | ≥ 48 |
| [TBD] |  |  | ≥ 132 |
| [TBD] |  | 60 | ≥ 24 |
| [TBD] |  |  | ≥ 64 |
| [TBD] |  |  | ≥ 132 |
| [TBD] | (Ês/Iot)ref ≥-3dB (Ês/Iot)i ≥-6dB | 15 | ≥ 52 |
| [TBD] |  |  | ≥ 104 |
| [TBD] |  | 30 | ≥ 48 |
| [TBD] |  |  | ≥ 132 |
| [TBD] |  | 60 | ≥ 64 |
| [TBD] |  |  | ≥ 132 |

- Table 2: DL RSCPD absolute accuracy in FR2 for AWGN channel

| Accuracy | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 |
| --- | --- | --- | --- |
| Tc Note 5 | dB | kHz | RB |
| [TBD] | (Ês/Iot)ref ≥-6dB (Ês/Iot)i ≥-13dB | 60 | ≥ 24 |
| [TBD] |  |  | ≥ 64 |
| [TBD] |  |  | ≥ 132 |
| [TBD] |  | 120 | ≥ 32 |
| [TBD] |  |  | ≥ 64 |
| [TBD] |  |  | ≥ 128 |
| [TBD] | (Ês/Iot)ref ≥-3dB (Ês/Iot)i ≥-6dB | 60 | ≥ 64 |
| [TBD] |  |  | ≥ 132 |
| [TBD] |  | 120 | ≥ 64 |
| [TBD] |  |  | ≥ 128 |

- Table 3: DL RSCPD absolute accuracy in FR1 for Two-Tap channel

| Accuracy | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 |
| --- | --- | --- | --- |
| Tc Note 5 | dB | kHz | RB |
| [TBD] | (Ês/Iot)ref ≥-3dB (Ês/Iot)i ≥-6dB | 15 | ≥ 52 |
| [TBD] |  |  | ≥ 104 |
| [TBD] |  | 30 | ≥ 48 |
| [TBD] |  |  | ≥ 132 |
| [TBD] |  | 60 | ≥ 64 |
| [TBD] |  |  | ≥ 132 |

- Table 4: DL RSCPD absolute accuracy in FR2 for Two-Tap channel

| Accuracy | PRS Ês/Iot | PRS SCS | PRS bandwidth Note 1 |
| --- | --- | --- | --- |
| Tc Note 5 | dB | kHz | RB |
| [TBD] | (Ês/Iot)ref ≥-3dB (Ês/Iot)i ≥-6dB | 60 | ≥ 64 |
| [TBD] |  |  | ≥ 132 |
| [TBD] |  | 120 | ≥ 64 |
| [TBD] |  |  | ≥ 128 |

Discussion:

Not discussed.

#### Issue 4-1-3: DL RSCP relative accuracy requirements

Proposals

- Proposal 1: (CATT, OPPO)

  - Reuse DL RSCPD accuracy requirements if the side conditions are the same.

Recommended WF:

- Discuss the proposal (s).

Discussion:

Not discussed.

#### Issue 4-1-5: Test case list

Proposals

- Proposal 1: (CATT, Huawei, CMCC)

  - RAN4 to define TCs for CPP based on following combinations

    - RSTD with RSCPD, UE Rx-Tx with RSCP

    - RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE

    - FR1, FR2

    - Delay, accuracy

Recommended WF:

- Use the following table to discuss the test case list:

Table 5: Test list for CPP measurement

| Set | PRS measurement test case scenarios | Impacted section in TS 38.133 |
| --- | --- | --- |
| 0-1 | General: Configuration for the indicated time window | A.3.x |
| PRS measurement delay tests |  |  |
| 1-1 | DL RSCP reporting delay test case in FR1 in RRC_CONNECTED | A.6.6.x1 |
| 1-2 | DL RSCPD reporting delay test case in FR1 in RRC_CONNECTED | A.6.6.x2 |
| 1-3 | DL RSCP reporting delay test case in FR2 in RRC_CONNECTED | A.7.6.x1 |
| 1-4 | DL RSCPD reporting delay test case in FR2 in RRC_CONNECTED | A.7.6.x2 |
| 1-5 | DL RSCP reporting delay test case in FR1 in RRC_INACTIVE | A.6.8.x1 |
| 1-6 | DL RSCPD reporting delay test case in FR1 in RRC_INACTIVE | A.6.8.x2 |
| 1-7 | DL RSCP reporting delay test case in FR2 in RRC_INACTIVE | A.7.8.x1 |
| 1-8 | DL RSCPD reporting delay test case in FR2 in RRC_INACTIVE | A.7.8.x2 |
| PRS measurement accuracy tests |  |  |
| 2-1 | DL RSCP absolute accuracy test case in FR1 in RRC_CONNECTED | A.6.7.x1 |
| 2-2 | DL RSCPD relative accuracy test case in FR1 in RRC_CONNECTED | A.6.7.x2 |
| 2-3 | DL RSCP absolute accuracy test case in FR2 in RRC_CONNECTED | A.7.7.x1 |
| 2-4 | DL RSCPD relative accuracy test case in FR2 in RRC_CONNECTED | A.7.7.x2 |
| 2-5 | DL RSCP absolute accuracy test case in FR1 in RRC_INACTIVE | A.6.9.x1 |
| 2-6 | DL RSCPD relative accuracy test case in FR1 in RRC_INACTIVE | A.6.9.x2 |
| 2-7 | DL RSCP absolute accuracy test case in FR2 in RRC_INACTIVE | A.7.9.x1 |
| 2-8 | DL RSCPD relative accuracy test case in FR2 in RRC_INACTIVE | A.7.9.x2 |
| Note: In each test, the DL RSCPD/RSCP measurement is configured together with RSTD/UE Rx-Tx. |  |  |

Discussion:

Not discussed.

#### Issue 4-1-6: Test configurations

Proposals

- Proposal 1: (CATT)

  - The test configurations in existing UE Rx-Tx and RSTD tests can be reused.

- Proposal 2: (Huawei)

  - RAN4 to define the tests for CPP with periodic time window

    - Periodicity and offset: 2 times of PRS resource periodicity

    - Offset: same as PRS resource offset

    - Duration: covering all PRS resources from all TRPs

Recommended WF:

- Discuss the proposal(s).

Discussion:

Not discussed.

#### Issue 4-1-7: Test requirements

Proposals

- Option 1: (Ericsson)

  - For CPP, accuracy TCs are defined to validate the UE capability to report the carrier phase measurement together with the legacy measurements and meet the accuracy requirements for both measurements reported in the same measurement report.

- Option 2: (Huawei)

  - RAN4 to define accuracy TCs for RSCPD/RSCP jointly with another measurement, and only accuracy of RSCPD/RSCP is verified and accuracy of other measurement is not.

Recommended WF:

- Discuss whether and how to verify the accuracy requirements for legacy measurement.

Discussion:

Not discussed.

### LPHAP (Agenda 6.12.3.2)

#### Issue 1-2-1: whether to define accuracy TCs for LPHAP

- Proposals

  - Option 1 (CMCC, OPPO, E///, Nokia):

    - Yes

  - Option 2 (HW, QC):

    - No

- Recommended WF

  - Suggest to define accuracy TCs for RRC_IDLE based on majority view.

  - How to introduce the TCs and the testing applicability can be further discussed (as in Issue 1-2-2).

Discussion:

Not discussed.

#### Issue 1-2-2: combination of measurement types and RRC states

- Proposals

  - Option 1 (OPPO, HW, QC, Nokia):

    - Define TCs for all applicable measurement types for RRC INACTIVE and RRC_IDLE

  - Option 1a (HW):

    - UE supporting PRS measurement in both RRC_IDLE and RRC_INACTIVE does not need to pass the tests for RRC_INACTIVE if the corresponding tests for RRC_IDLE are defined.

  - Option 1b (QC):

    - if the UE supports measurements in RRC_INACTIVE and passes the test cases for RRC_INACTIVE, then it does not have to be tested in RRC_IDLE.

  - Option 2 (E///):

    - Define accuracy TCs for RSTD, PRS RSRP and PRS RSRPP measurement in RRC_IDLE state and accuracy TCs for UE Rx-Tx time difference in RRC_INACTIVE state.

- Recommended WF

  - Suggest to define TCs for all applicable measurement types for RRC INACTIVE and RRC_IDLE based on majority view.

  - Moderator suggests to use same set of TCs for RRC_IDLE and RRC_INACTIVE. Since the test setup would be same for RRC_IDLE and RRC_INACTIVE, using same set of TCs would avoid duplication and align test coverage.

  - Further discuss the test applicability as in P1a and P1b.

Discussion:

Not discussed.

#### Issue 1-2-3: eDRX configurations

- Proposals

  - Option 1 (CMCC)

    - RAN eDRX <= 10.24s, with both CN eDRX <= 10.24s and CN eDRX > 10.24s

    - RAN eDRX > 10.24s, with both Case 1 and Case 2

  - Option 2 (OPPO, QC):

    - RAN eDRX > 10.24s, with both Case 1 and Case 2

  - Option 3 (E///, HW, Nokia):

    - RAN eDRX > 10.24s, Case 2

- Recommended WF

  - Suggest to adopt option 2 as compromise.

Discussion:

Not discussed.

#### Issue 1-2-7: UE types to be tested

- Proposals

  - Option 1 (CMCC, E///, HW, Nokia):

    - Define test cases for both normal (non-RedCap) and RedCap type of devices.

  - Option 1a (HW, Nokia):

    - Define separate set TCs for non-RedCap and RedCap UEs

  - Option 1b (E///):

    - Use same set of TCs for non-RedCap and RedCap UEs

- Recommended WF

  - Suggest to define test cases for both normal (non-RedCap) and RedCap UEs, since all companies have same view on this.

  - Further discuss whether to define separate TCs or use same TCs for normal (non-RedCap) and RedCap UE. Align the principle with RedCap positioning discussion in [214].

Discussion:

Not discussed.

## Work split

### Issue 3-1-2: Test case for carrier phase positioning.

- Proposals

  - Option 1: E///

    - For carrier phase-based positioning, test cases are only defined for the case where the UE is configured to perform carrier phase measurement with legacy positioning measurements within the configured measurement time window.

- Tentative agreement:

  - For carrier phase-based positioning, test cases are only defined for the case where the UE is configured to perform carrier phase measurement with legacy positioning measurements within the configured measurement time window.

- Recommended WF

  - Agree on tentative agreement.

Discussion:

Not discussed

### Issue 3-1-3: Testing principles for Rel. 18 positioning.

- Proposals

  - Option 1: E///

    - Testing principles, to limit the number of test cases UE is expected to pass, are defined for Rel. 18 positioning.

- Tentative agreement:

  - Testing principles, to limit the number of test cases UE is expected to pass, are defined for Rel. 18 positioning. Companies to bring proposals on testing principles in RAN4#111.

- Recommended WF

  - Agree on tentative agreement.

Discussion:

Not discussed

### Issue X: List of test cases and accuracy requirements:

| TDoc | Title | Source | Comments | Decision |
| --- | --- | --- | --- | --- |
| R4-2405512 | Work split on performance requirements for Rel. 18 positioning features | Ericsson |  |  |

Discussion:

Discussed partly under specific topics. Needs further discussion.