3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2406299

# Changsha, CN, 15th Apr – 19st Apr, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | draft | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on test cases for RRC Re-establishment for NR NCR-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test cases for RRC Re-establishment for LA NCR-MT shall be introduced into TS 38.106. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following test cases in TS 38.106 for NCR-MT RRM requirements: - G.2.1.1.2 RRC Connection Release with Redirection for LA NCR-MT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for RRC Re-establishment for LA NCR-MT would be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New G.2.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

G.2.1.1.2	RRC Connection Release with Redirection for LA NCR-MT

G.2.1.1.1.1	Inter-frequency RRC Re-establishment in FR1 for LA NCR-MT

G.2.1.1.1.1.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR1 to an unknown target cell is within the specified limits. These tests will verify the requirements in clause 10.1.1. This test case is applicable only for local area NCR-MT and for NCR type 1-H.

The test parameters are given in table G.2.1.1.1.1.1-1, table G.2.1.1.1.1.1-2 and table G.2.1.1.1.1.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the NCR-MT shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the NCR-MT has the context of the carrier frequency of cell 2 by the end of T1.

Table G.2.1.1.1.1.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| Note: The NCR-MT is only required to be tested in one of the supported test configurations. |  |  |

Table G.2.1.1.1.1.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| Final condition | Active cell |  | 1, 2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1, 2 | 30000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
|  |  |  | 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | OFF |  |
| PRACH configuration |  |  | 1, 2 | FR1 PRACH configuration 1 | TBD |
| T1 |  | s | 1, 2 | 20 |  |
| T2 |  | ms | 1, 2 | 1000 | Time for the NCR-MT to detect RLF |
| T3 |  | s | 1, 2 | 20 |  |

Table G.2.1.1.1.1.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1, 2 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 2 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.1.1 FDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 2 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
|  |  | 3 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 2 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
|  |  | 3 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in TBD |  |  | OP.1 defined in TBD |  |  |
| TRS configuration |  | 1 | TRS.1.1 TDD |  |  | N/A |  |  |
|  |  | 2 | TRS.1.2 TDD |  |  |  |  |  |
|  |  | 3 | TRS.1.1 FDD |  |  |  |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0 |  |  | DLBWP.0 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0 |  |  | ULBWP.0 |  |  |
| Active DL BWP confgiuration |  | 1, 2 | DLBWP.1.1 | N/A | N/A | N/A | N/A | DLBWP.1.1 |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 | N/A | N/A | N/A | N/A | ULBWP.1.1 |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
|  | dB | 1, 2 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |  |  |
|  | dB | 1, 2 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
| SS-RSRP Note3 | dBm/SCS | 1 | -94 | -infinity | -infinity | -infinity | -infinity | -91 |
|  |  | 2 | -91 | -infinity | -infinity | -infinity | -infinity | -88 |
| Io | dBm/9.36 MHz | 1 | -64.59 | -70. 05 | -70. 05 | -70. 05 | -70. 05 | -62.26 |
|  | dBm/38.16 MHz | 2 | -58.50 | -63.94 | -63.94 | -63.94 | -63.94 | -56.15 |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

G.2.1.1.1.1.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the NCR-MT starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than 14.5 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay=TNCR-MT_re-establish_delay+TUL_grant

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception is used as a trigger for the completion of the test; hence TUL_grant is not used.

TNCR-MT_re-establish_delay=400 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2

Tidentify_intra_NR = 6400 ms

Tidentify_inter_NR = 6400 ms

TSI-NR = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 14495 ms, allow 14.5 s in the test case.

G.2.1.1.1.2	Intra-frequency RRC Re-establishment in FR1 without serving cell timing for LA NCR-MT

G.2.1.1.1.2.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 without serving cell timing is within the specified limits. These tests will verify the requirements in clause 10.1.1. This test case is applicable only for local area NCR-MT and for NCR type 1-H.

The test parameters are given in table G.2.1.1.1.2.1-1, table G.2.1.1.1.2.1-2 and table G.2.1.1.1.2.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table G.2.1.1.1.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| Note: The NCR-MT is only required to be tested in one of the supported test configurations. |  |

Table G.2.1.1.1.2.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| Final condition | Active cell |  | 1, 2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2 | 6000 | Radio link failure timer configured by RLF-TimersAndConstants |
| T311 |  | ms | 1, 2 | 15000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
|  |  |  | 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | OFF |  |
| PRACH configuration |  |  | 1, 2 | FR1 PRACH configuration 1 | TBD |
| T1 |  | s | 1, 2 | 10 |  |
| T2 |  | s | 1, 2 | 7 | Time for the NCR-MT to detect RLF |
| T3 |  | s | 1, 2 | 10 |  |

Table G.2.1.1.1.2.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 2 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |  | N/A |  |  |
|  |  | 2 | SR.2.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.1.1 FDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 2 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
|  |  | 3 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 2 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
|  |  | 3 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in TBD |  |  | OP.1 defined in TBD |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
|  | dB | 1, 2 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |  |  |  |
|  | dB | 1, 2 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
| SS-RSRP Note3 | dBm/SCS | 1 | -94 | -infinity | -infinity | -infinity | -infinity | -94 |
|  |  | 2 | -91 | -infinity | -infinity | -infinity | -infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.59 | -infinity | -infinity | -infinity | -infinity | -64.59 |
|  | dBm/9.36 MHz | 2 | -58.50 | -infinity | -infinity | -infinity | -infinity | -58.50 |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

G.2.1.1.1.2.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the NCR-MT starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell without serving cell timing shall be less than 8.1 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay=TNCR-MT_re-establish_delay+TUL_grant

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception is used as a trigger for the completion of the test; hence TUL_grant is not used.

TNCR-MT_re-establish_delay=400 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 6400 ms

TSI-NR = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 [2] for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 8095 ms, allow 8.1 s in the test case.

G.2.1.1.1.3	Inter-frequency RRC Re-establishment in FR2-1 for LA NCR-MT

G.2.1.1.1.3.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR2-1 without known target cell is within the specified limits. These tests will verify the requirements in clause 10.1.1. This test case is applicable only for local area NCR-MT and for NCR type 2-O.

The test parameters are given in table G.2.1.1.1.3.1-1, table G.2.1.1.1.3.1-2 and table G.2.1.1.1.3.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the NCR-MT shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the NCR-MT has the context of the carrier frequency of cell 2 by the end of T1.

Table G.2.1.1.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table G.2.1.1.1.3.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case in FR2-1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 30000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2-1 |  |
|  |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | FR2-1 PRACH configuration 1 | Table TBD |
| T1 |  | s | 1 | 10 |  |
| T2 |  | ms | 1 | 4800 | Time for the NCR-MT to detect RLF |
| T3 |  | s | 1 | 20 |  |

Table G.2.1.1.1.3.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR2-1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  | 1 | Setup 2 as specified in clause G.1.8.2 |  |  |  |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in TBD |  |  | OP.1 defined in TBD |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 8 |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -89 |  |  |  |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 8 |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -infinity | -infinity | -infinity | -infinity | -81 |
| Io | dBm/95.04 MHz | 1 | -53.82 | -infinity | -infinity | -infinity | -infinity | -51.37 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void |  |  |  |  |  |  |  |  |

G.2.1.1.1.3.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the NCR-MT starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than 18 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay=TNCR-MT_re-establish_delay+TUL_grant

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception is used as a trigger for the completion of the test; hence TUL_grant is not used.

TNCR-MT_re-establish_delay=400 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2

Tidentify_intra_NR = 8000 ms

Tidentify_inter_NR = 8000 ms

TSI-NR = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 17695 ms, allow 18 s in the test case.

G.2.1.1.1.4	Intra-frequency RRC Re-establishment in FR2-1 without serving cell timing for LA NCR-MT

G.2.1.1.1.4.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR2-1 without serving cell timing is within the specified limits. These tests will verify the requirements in clause 10.1.1. This test case is applicable only for local area NCR-MT and for NCR type 2-O.

The test parameters are given in table G.2.1.1.1.4.1-1, table G.2.1.1.1.4.1-2 and table G.2.1.1.1.4.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table G.2.1.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table G.2.1.1.1.4.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR2-1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 6000 | Radio link failure timer configured by RLF-TimersAndConstants |
| T311 |  | ms | 1 | 30000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2-1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | FR2-1 PRACH configuration 1 | Table TBD |
| T1 |  | s | 1 | 10 |  |
| T2 |  | s | 1 | 10800 | Time for the NCR-MT to detect RLF |
| T3 |  | s | 1 | 30 |  |

Table G.2.1.1.1.4.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR2-1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  | 1 | Setup 2 as specified in clause G.1.8.2 |  |  |  |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
|  |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| TCI state |  | 1 | CSI-RS.Config.0 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in TBD |  |  | OP.1 defined in TBD |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 defined in TBD |  |  | Setup 1 defined in TBD |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -89 |  |  |  |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| SS-RSRP Note3 | dBm/SCS | 1 | -93 | -infinity | -infinity | -infinity | -infinity | -93 |
| Io | dBm/95.04 MHz | 1 | -62.82 | -infinity | -infinity | -infinity | -infinity | -62.82 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void |  |  |  |  |  |  |  |  |

G.2.1.1.1.4.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the NCR-MT starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell without serving cell timing shall be less than 30 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay=TNCR-MT_re-establish_delay+TUL_grant

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception is used as a trigger for the completion of the test; hence TUL_grant is not used.

TNCR-MT_re-establish_delay=400 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 28160 ms

TSI-NR = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 [2] for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 29855 ms, allow 30 s in the test case.

<End of Change 1>