3GPP TSG-RAN WG4 Meeting # 110bis						       			        R4-2406301

Changsha, China, 15-19 April 2024

Agenda item:			6.24.5

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NR network energy saving

Document for:	Approval

- Topic #1: Core requirements maintenance

- Sub-topic 1-1 SSB-less related

Issue 1-1-2: QCL/TCI indication and reference Cell determination

- Agreement:

  - No requirement is applied for the case of more than one QCLtypeC source cells and without Rel-18 network indication.

Issue 1-1-3: Requirements applicability when multiple P-TRS are configured.

- Proposals

  - Option 1: For P-TRS based SSB-less SCell activation, do not define the SCell activation delay requirement if multiple TRSs are configured in SSB-less SCell. (Nokia)

Issue 1-1-5: Intra-band non-contiguous CA

- Proposals

  - Option 1 For FR1 intra-band non-contiguous CA, reuse the SSB-less SCell activation delay requirement defined for FR1 collocated inter-band CA with the same RTD side condition i.e. RTD within CP (Nokia, Intel)

    - Option 1a: Introduce the optional with capability signalling with per FS granularity for UE supporting intra-band NCCA SSB-less SCell operation in Rel-18. (Intel)

  - Option 2: The delay requirements of SSB-less SCell activation for intra-band non-contiguous CA can be same as that for legacy intra-band contiguous CA provided the UE impelmentation architecture are same for the two scenarios. The exact side conditions for intra-band non-contiguous CA can be further discussed. (ZTE)

  - Option 3: RAN4 will not consider SSBless SCell operation for intra-band NCCA in R18. (QC)

  - Option 4: (Apple, Huawei)

    - For FR1 intra-band NCCA with SSB-less operation, EPRE difference at UE side shall be NOT larger than 6dB

    - For FR1 intra-band NCCA with SSB-less operation, RTD between the target SCell and the intra-band NCCA collocated reference serving cell can be within CP

  - Option 5: (CMCC)

    - Define two sets of requirement and side condition, reuse the intra-band contiguous CA case and inter-band CA case respectively.

    - Define two UE capabilities for intra-band NCCA scenario, which corresponding to two set of requirements respectively, the granularity could be per FS indication

  - Option 6:

    - Specify only assuming separate chains and reuse largely the requirements specified for inter-band cases;

    - Specify UE optional capability signalling for intra-band NCCA SSB-less SCell operations in a similar way as for inter-band cases;

    - UE with single chain implementation does not indicate support for intra-band NCCA SSB-less SCell operations and does not need to meet the requirements.

  - Option 7:

    - Specify two sets of requirements respectively assuming single and separate chains, where one of the sets reuse largely the requirements specified for inter-band cases;

    - Specify UE optional capability signalling for two different implementations separately;

    - Different UE implementations are required to meet different sets of requirements correspondingly.

  - Other options are not precluded

Issue 1-1-6: Neighbour cells on carrier of SSB-less SCell

- Proposals

  - Option 1: If neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell (Apple)

  - Option 2: For the case of the neighbour cell(s) is on the carrier of SSB-less SCell and the SSB from neighbour cell(s) can be contained in the active BWP of SSB-less SCell, the measurement for such neighbour cell(s) can be treated as intra-frequency measurement. (CMCC, ZTE)

  - Option 3: Do not discuss the neighbor cell measurement on the carrier of SSB-less SCell. (Nokia, Huawei)

Issue 1-1-7: Multiple SSB-less SCells activation

Agreement:

- When to-be-activated SSB-less SCells are in different bands

  - Existing single CC requirement can apply to each to-be-activated SSB-less SCells respectively.

  - The reference cell to the multiple SSB-less Scells in difference bands may or may not be different.

- Sub-topic 1-2 NES-based CHO related

Issue 1-2-1: Whether to differentiate requirements wrt Cell turning off.

Agreement:

  - RAN4 will not define Cell switch off specific requirements/behaviour when receiving NES indication for CHO unless there is new agreement from other working group which can explicitly indicate “cell switch off”.

Issue 1-2-2: When CHO condition is not met anymore

Discuss in the CR directly.

- Topic #2: Performance part for NES – SSB-less

- Sub-topic 2-1 Requirements maintenance

Issue 2-1-1: Test configurations for SSB-less

- Agreement on Test configurations for SSB-less

  - Single P-TRS is configured in SSB-less SCell in the SSB-less SCell activation TCs

  - Don’t configure the parameter SCell measurement cycle (measCycleSCell), SSB configuration and SMTC configuration for SSB-less SCell

  - Set EPRE difference as [9 dB] + [ΔPL] in the test cases.

    - the [9] can be updated to the same number as in core requirement if any updates on the core requirements.

    - Pcell has [higher or lower] EPRE than Scell.

    - Further discuss whether and how [ΔPL] can be reflected in the TC.

      - ΔPL is the power difference caused by BW difference and/or frequency difference between two Cells.

      - Encourage concrete proposals on how ΔPL due to BW difference and PL difference can be reflected in the test.

  - The TCI.State.0 could be used for the TRS of the SSB-less SCell with an additional note that the reference signal is SSB0 from spCell.

- Topic #3: Perf: Performance part for NES - Others

Issue 3-1-1: Whether to define TC for Cell DTX

Agreement:

- Define TC: SA event triggered reporting test without gap under UE DRX and Cell DTX.

- A UE which fulfils the requirements in Cell DTX test case can skip the test case in SA event triggered reporting tests without gap under UE DRX.

Issue 3-2-1: Test configurations for NES-based CHO

Discuss in the CR directly.