# 3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2406314

# Changsha, China, 15th – 19th April, 2024

Agenda item:			7.3.6

Source:	Moderator (MediaTek inc.)

Title:	WF on R18 IoT NTN enhancement RRM requirements

Document for:	Approval

# Introduction

This document is to capture all agreements in email thread [110][233] IoT_NTN_enh.

# Topic #1: RRM core requirements

### Sub-Topic 1-1: Time-based measurement initiation in IDLE mode

#### Issue 1-1-1: Skipping serving cell measurement before t-service

<WayForward>

Further discuss

- Option 1: Proposal 1 with wording refinement.

- Option 2: no change is needed.

#### Issue 1-1-2: Measurement on inter-frequency neighbor cells associated to the same satellite

No agreement

### Sub-Topic 1-2: CONN mode neighbour cell measurements

#### Issue 1-2-1: For NB-IoT, frequencies to be measured

No agreement

#### Issue 1-2-2: For NB-IoT, time-based neighbor cell measurements

No agreement

### Sub-Topic 1-3: GNSS re-acquisition gap in connected mode

#### Issue 1-3-1: GNSS-MG with (e)DRX

<Agreement>

- When the GNSS-MG is shorter than the (e)DRX cycle and it is not colliding with the on Duration part of one (e)DRX cycle, the time to evaluate requirements is still applicable.

# Topic #2: RRM performance requirements

### Sub-Topic 2-1: Test case list

#### Issue 2-1-1: For NB-IoT, inter-frequency test cases for RRC Re-establishment

<Agreement>

Regarding inter-frequency test cases for NB-IoT RRC Re-establishment over NTN, introduce only one test for enhanced coverage. I.e. drop the test NB-3-1.

#### Issue 2-1-2a: For NB-IoT, test case for time-based measurement triggering in connected mode

<Agreement>

Do not test for time-based measurement triggering in connected mode for NB-IoT.

#### Issue 2-1-2b: For NB-IoT, test case for location-based measurement triggering in connected mode

<Agreement>

keep NB-CONN-2D

#### Issue 2-1-3: Test case upon t-ServiceStartNeigh

<Agreement>

- For eMTC, introduce new test case upon t-ServiceStartNeigh.

- For NB-IoT, do not introduce the test case.

#### Issue 2-1-4: For eMTC, test case with GNSS gap

<Agreement>

Do not introduce GNSS-MG test cases

### Sub-Topic 2-2: Test set-up

#### Issue 2-2-1 Test configuration on Quasi-Earth-Fixed cell or Earth-Moving cell

<Agreement>

Define test case with Quasi-Earth-Fixed cell as the baseline in Rel-18.

- It can be revisited until next meeting.

#### Issue 2-2-2 Test configuration with neighbor satellite

<Agreement>

For Idle and Connected mode mobility test cases, select a subset of the test cases to be tested with the neighbor cell associated to a different satellite.

### Sub-Topic 2-3: Updated Test cases and Worksplit

#### For NB-IoT

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Cell Re-Selection (A.13.1.1) | HD – FDD Inter frequency case for UE Category NB1 in normal coverage | NB-1-1 | MTK | Phase 1 (2024.04) Note: Draft available |
|  | HD – FDD Intra frequency case for UE Category NB1 in [enhanced] coverage, location-based triggering | NB-IDLE-1D |  |  |
|  | HD – FDD Inter frequency case for UE Category NB1 in [enhanced] coverage, time-based triggering | NB-IDLE-2T |  |  |
| RRC Re-establishment (A.13.3.1) | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | NB-3-1 |  |  |
|  | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | NB-3-2 |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Measurement Procedure in CONN (new) | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage | NB-CONN-1 | Huawei | Phase 2 (2024.05) Note: Draft available |
|  | HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage | NB-CONN-2 |  |  |
| Measurement Procedure in CONN with time/location triggering (new) | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, time-based triggering | NB-CONN-1T |  |  |
|  | HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, location-based triggering | NB-CONN-2D |  |  |

Add NGSO test configuration on the existing tests for for NB-IoT

| Category | Titlle | Volunteer | Draft target date |
| --- | --- | --- | --- |
| Cell Re-Selection (A.13.1.1) | A.13.1.1.1 HD – FDD Intra frequency case for UE Category NB1 in normal coverage | MTK | Phase 2 (2024.05) |
|  | A.13.1.1.2 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation |  |  |
|  | A.13.1.1.3 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX |  |  |
| RRC Re-establishment (A.13.3.1) | A.13.3.1.1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage |  |  |
|  | A.13.3.1.2 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage |  |  |

#### For eMTC

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Cell re-selection (A.14.1.1) | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M-1-1 | CMCC | Phase 1 (2024.04) Note: Draft available |
|  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M-1-2 |  |  |
|  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering | M-IDLE-1T |  |  |
|  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in [enhanced] coverage, time-based triggering | M-IDLE-2T |  |  |
|  | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering | M-IDLE-3D |  |  |
|  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering | M-IDLE-4D |  |  |
| RRC re-establishment (A.14.3.1) | E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M-3-1 | Ericsson | Phase 1 (2024.04) Note: Draft available |
|  | E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M-3-2 |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Handover (A.14.2.1) | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA | M-2-1 | Nokia | Phase 1 (2024.04) Note: Draft available |
|  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA | M-2-2 |  |  |
|  | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB | M-2-3 |  |  |
|  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB | M-2-4 |  |  |
|  | E-UTRAN FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M-2-5 |  |  |
|  | E-UTRAN HD-FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M-2-6 |  |  |
|  | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | M-2-7 |  |  |
|  | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | M-2-8 |  |  |
| Time/location based conditional Handover (A.14.2.1) | E-UTRAN FDD-FDD Intra frequency Time-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-11 | Huawei | Phase 2 (2024.05) |
|  | E-UTRAN HD-FDD Intra frequency Location-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-14 |  |  |
|  | E-UTRAN FDD-FDD Inter frequency Location-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-16 |  |  |
|  | E-UTRAN HD-FDD Inter frequency Time-based conditional handover for Cat-M1 UEs in CEModeA [for unknown target cell] | M1-2-17 |  |  |
|  | Note: for Time/location based conditional handover TCs, both time/location only-based conditional handover” tests and “time/location and RSRP based conditioned HO tests” are considered |  |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| UE measurement procedure in RRC-CONNECTED (A.14.5.1) | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M-5-1 | Huawei | Phase 2 (2024.05) |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M-5-2 |  |  |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M-5-3 |  |  |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M-5-4 |  |  |
|  | E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB | M-5-5 |  |  |
|  | E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB | M-5-6 |  |  |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells with burst gap | M-5-7 |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| UE measurement procedure in RRC-CONNECTED with time/location based triggering (A.14.5.1) | E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA, location-based triggering | M-CONN-1D | Nokia | Phase 2 (2024.05) |
|  | E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA, location-based triggering | M-CONN-2D |  |  |
|  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA, time-based triggering | M-CONN-3T |  |  |
|  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA, time-based triggering | M-CONN-4T |  |  |

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Measurement Performance Requirements (A.14.6.1) | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M-6-1 | MTK | Phase 1 (2024.04) Note: Draft available |
|  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M-6-2 |  |  |
|  | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB | M-6-3 |  |  |
|  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB | M-6-4 |  |  |

New Test case for eMTC

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| UE measurement procedure in RRC-CONNECTED with t-ServiceStartNeigh (A.14.5.1, A.14.5.2) | E-UTRAN FD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used | M-5-8 | Nokia | Phase 2 (2024.05) |
|  | E-UTRAN HD-FDD inter-frequency event triggered reporting in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB | M-5-9 |  |  |

Add NGSO test configuration on the existing tests for eMTC

| Category | Titlle | Volunteer | Draft target date | Draft target date |
| --- | --- | --- | --- | --- |
| RRC_IDLE state | Cell Re-Selection (A.14.1.1) | A.14.1.1.1 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage | MTK | Phase 2 (2024.05) |
|  |  | A.14.1.1.2 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage |  |  |
|  |  | A.14.1.1.3 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation |  |  |
|  |  | A.14.1.1.4 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation |  |  |
| RRC CONNECTED Mode Mobility | Handover and Conditional Handover (A.14.2.1) | A.14.2.1.1 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition |  |  |
|  |  | A.14.2.1.2 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition |  |  |
|  |  | A.14.2.1.3 E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA |  |  |
|  |  | A.14.2.1.4 E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA |  |  |
| RRC Connection Control | RRC Re-establishment (A.14.3.1) | A.14.3.1.1 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA |  |  |
|  |  | A.14.3.1.2 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA |  |  |

| Category | Titlle | Volunteer | Draft target date | Draft target date |
| --- | --- | --- | --- | --- |
| Measurement Procedure | Event triggered reporting (A.14.5.1) | A.14.5.1.1 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | CMCC | Phase 2 (2024.05) |
|  |  | A.14.5.1.2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |  |  |
|  |  | A.14.5.1.3 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA |  |  |
|  |  | A.14.5.1.4 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |  |  |
| Measurement Performance Requirements | RSRP accuracy (A.14.6.1) | A.14.6.1.1 FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |  |  |
|  |  | A.14.6.1.2 HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |  |  |