3GPP TSG-RAN WG4 Meeting #110bis	R4-2406320

Changsha, China, April 15 – April 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 36.133 on Conditional Inter-Frequency HO for Cat-M1 UEs in NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IOT_NTN_enh |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - New test cases for Inter-frequency Conditional HO for Cat-M1 UEs in NTN are needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce test cases for Inter-frequency Conditional HO for Cat-M1 UEs in NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test cases will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.2.1.11 (new), A.14.2.1.12 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of changes ************************************

#### A.14.2.1.11	E-UTRAN FDD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA

##### A.14.2.1.11.1	Test Purpose and Environment

This test is to verify the requirement for the FDD inter frequency conditional handover requirements.

The test configurations are given in Table A.14.2.1.11.1-1. The test scenario comprises of two E-UTRA FDD carrier and one cell in each carrier as given in tables A.14.2.1.11.1-2 and A.14.2.1.11.1-3 The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

E-UTRAN shall send a RRC message implying conditional handover to Cell 2 during period T1 at a time earlier than TRRC before the beginning of T2. The field sameSFN-Indication is not included in the handover command. At the start of T2, cell 2 becomes detectable and meets the handover condition.

During the test, UE is configured with measurement gap to enable inter-frequency measurement.

Table A.14.2.1.11.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, FD-FDD duplex mode |
| 2 | NGSO, FD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.11.1-2: General test parameters for E-UTRAN FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.11.1-3: Cell specific test parameters for E-UTRAN FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T1 | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.1 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.2 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.48 FDD | R.48 FDD |  | - | R.48 FDD |  |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.46 FDD |  |  | R.46 FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD |  | OP.7 FDD | OP.7 FDD |  | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 |  | -3.33 | -Infinity |  | 2.36 |
| Note 4 | dB | 8 |  | 8 | -Infinity |  | 11 |
| RSRP Note 4 | dBm/15 KHz | -90 |  | -90 | -Infinity |  | -87 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.12.2	Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution = [860 ms] from the start of T2 and interruption during T2 shall not exceed 50ms.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The conditional handover delay can be expressed as: TRRC + TDelayUncertainty + Tmeasure + TCHO_execution + Tinterrupt, where:

TRRC = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tmeasure = [800] ms in the test; Tmeasure is defined in clause 5.5A.2.3.2 without TDelayUncertainty.

TCHO_execution = 10 ms in the test; TCHO_execution is defined in clause 5.5A.2.3.3.

#### A.14.2.1.12	E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA

##### A.14.2.1.12.1	Test Purpose and Environment

This test is to verify the requirement for the HD-FDD inter frequency conditional handover requirements. The test configurations are given in Table A.14.2.1.12.1-1.

The test scenario comprises of two E-UTRA FDD carrier and one cell in each carrier as given in tables A.14.2.1.12.1-2 and A.14.2.1.12.1-3. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE shall have had the opportunity to acquire satellite assistance information for Cell 2, provided by Cell 1 in SystemInformationBlockType33.

E-UTRAN shall send a RRC message implying conditional handover to Cell 2 during period T1 at a time earlier than TRRC before the beginning of T2. The field sameSFN-Indication is not included in the handover command. At the start of T2, cell 2 becomes detectable and meets the handover condition.

During the test, UE is configured with measurement gap to enable inter-frequency measurement.

Table A.14.2.1.12.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Table A.14.2.1.12.1-2: General test parameters for E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration |  |  | PRACH_4CE | As specified in A.3.16 |
| PRACH initial CE level |  |  | 0 | Specified in the handover message |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| Gap pattern ID |  |  | 1 |  |

Table A.14.2.1.12.1-3: Cell specific test parameters for E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA without SFN acquisition test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| Satellite Information (Configuration 1)Note 1 |  | SSC.1 |  |  | NSC.1 |  |  |
| Satellite Information (Configuration 2)Note 1 |  | SSC.2 |  |  | NSC.2 |  |  |
| BWchannel | MHz | 1.4 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.1 |  | R.49 HD-FDD | R.49 HD-FDD | - | - | - | R.49 HD-FDD |
| MPDCCH Reference Channel in clause A.3.1.3.1 |  | R.47 HD-FDD |  |  | R.47 HD-FDD |  |  |
| OCNG Patterns in clause A.3.2.1 |  | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD | OP.7 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 12 | 12 |
| Note 4 | dB | 8 | -4.27 | -4.27 | -Infinity | 3.36 | 3.36 |
| RSRP Note 4 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -86 | -86 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x1 |  |  | 1x1 |  |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  |  | Based on Satellite Assistance information |  |  |
| Note 1: Satellite information is determined according to the testing principles for NTN determined in clause B.3.8. If satellite movement is applicable, it should be considered for the duration of the test case. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot and RSRP level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.14.2.1.12.2	Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution = [860 ms] from the start of T2 and interruption during T2 shall not exceed 50ms.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The conditional handover delay can be expressed as: TRRC + TDelayUncertainty + Tmeasure + TCHO_execution + Tinterrupt, where:

TRRC = 15 ms and is specified in clause 11.2 in TS 36.331 [2].

Tmeasure = [800] ms in the test; Tmeasure is defined in clause 5.5A.2.3.2 without TDelayUncertainty.

TCHO_execution = 10 ms in the test; TCHO_execution is defined in clause 5.5A.2.3.3.

Tinterrupt = 50 ms in the test; Tinterrupt is defined in clause 5.1.2.6.4.

5.5A.2.3