3GPP TSG-RAN WG4 Meeting #110bis	R4-2406329

Changsha, China, 12 ‒ 15 April, 2024

Title: 	WF on NR_FR1_lessthan_5MHz_BW

Agenda Item:	6.8.8

Source: 	Nokia

Document for:	Approval

# Topic 1 RRM core requirement maintenance

## Sub-topic 1-1 Handover Requirements

Issue 1-1-1: For HO requirements, should the additional SSB samples for SBI and MIB reading due to PBCH puncturing be included as part of T∆ or as part of Tsearch?

Agreement:

- For HO requirements, include the 2 additional SSB samples for SBI and MIB reading due to PBCH puncturing as part of T∆ instead of Tsearch.

  - For less than 5MHz requirement, include the additional time for T∆ in the spec.

- Further discuss whether to Capture in spec that in general the time for SBI and MIB reading is included in T∆.

Issue 1-1-2: For HO requirements, should the additional SSB samples for SBI and MIB reading due to PBCH puncturing be included for known target cell?

Agreement:

- For Issue 1-2-1/2/3/4/5 and Issue 1-3, RAN4 to not extend the cell identification delays for 12 PRB SSB but extend TSI-NR in the test-cases. Exact extension is FFS.

## Sub-topic 1-2 Extension of cell identification delays

Issue 1-2: For the following different scenarios, should the time to identify a target cell be extended to allow additional SSB samples for SBI and MIB reading due to PBCH puncturing:

Issue 1-2-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell (Table 6.2.1.2.1-1)?

Issue 1-2-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell (Table 6.2.1.2.1-2)?

Issue 1-2-3: Time to identify an unknown target NR inter-frequency cell for RRC connection release with re-direction, (Table 6.2.3.2.1-1)?

Issue 1-2-4: Time for RRC connection re-establishment shall be extended by 2xTSMTC for the unknown inter-frequency cell and by 3xTSMTC for the unknown intra-frequency cell, based on the target cell side condition of Es/Iot≥-4 dB for inter-frequency target cell and Es/Iot≥-6 dB for intra-frequency target cell?

Issue 1-2-5: Time for RRC connection release with re-direction shall be extended by 2xTSMTC for the unknown inter-frequency cell and by 3xTSMTC for the unknown intra-frequency cell, based on the target cell side condition of Es/Iot≥-4 dB for inter-frequency target cell and Es/Iot≥-6 dB for intra-frequency target cell?

Issue 1-3: Intra-frequency and inter-frequency cell reselection delays for a target cell with 12 PRB SSB by 2 DRX cycles?

Agreement:

- Issue 1-2-1/2/3/4/5 and Issue 1-3 are closed (Refer to issue 1.1.1).

# Topic 2 RRM performance requirements

## Sub-topic 2-1 Test cases

Issue 2-1: Test case discussion:

Agreement:

- Define RLM TCs with 12 RB and 15 RB. Define BFR TCs with 15 RB only. RLM TCs with 12 RB are only applicable for UE supporting band n100, and such UE need to pass the RLM TCs with 12RB and 15 RB.

      - Introduce RLM-1 and RLM-2 for 12RB

| RLM-1 | Radio Link Monitoring (SSB-based, FR1): Test 1 - DRX - Out-of-sync - 12 PRBs |
| --- | --- |
| RLM-2 | Radio Link Monitoring (SSB-based, FR1): Test 2 - DRX - In-sync - 12 PRBs |

- Not define HO-2 test case.

- Do not define new TCs for L3 measurement accuracy.

- Not define ‘Event-3’ test-case.

- Define one test case for cell reselection and not define the test case for RRC re-establishment.

Work split:

| TC # | Description | Company |
| --- | --- | --- |
| IDLE-1 | Cell reselection, Unknown target cell | Nokia |
| HO-1 | SA FR1-FR1 Handover, Intra-frequency, Unknown target cell | Apple |
| RLM-1 | Radio Link Monitoring (SSB-based, FR1): DRX, Out-of-sync, 12 PRBs | Nokia |
| RLM-2 | Radio Link Monitoring (SSB-based, FR1): DRX, In-sync, 12 PRBs | Qualcomm |
| RLM-3 | Radio Link Monitoring (SSB-based, FR1): Non-DRX, Out-of-sync, 15 PRBs | Nokia |
| RLM-4 | Radio Link Monitoring (SSB-based, FR1): Non-DRX, In-sync, 15 PRB | Qualcomm |
| BFD-1 | BFD and link recovery (SSB-based, FR1): DRX, 15 PRB | Ericsson |
| MR-1 | SA event triggered reporting, SSB based, Time period for time index detection: Intra-frequency, non-DRX, no gaps, 15 PRBs | Huawei |
| MR-2 | SA event triggered reporting, SSB based, Time period for time index detection: Inter-frequency, DRX, gaps, 15 PRBs | MTK |

Issue 2-3: Test case design:

Agreement:

- Discuss based on E///’s CR.