3GPP TSG-RAN WG4 Meeting #110bis															R4-2406364

Changsha, China, 15 – 19 April, 2024

Title: 	WF on core maintenance and performance part for BWP without restriction

Agenda Item:	6.3.5

Source: 	vivo, Vodafone

Document for:	Approval

- Agreements for core part maintenance

#### Issue 1-1: Requirements/UE behaviour for UE supporting both option B-1-1 and A

- For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour.

#### Issue 1-3: Requirements/UE behaviour for UE supporting both option C and B-1-1

- For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB, UE should perform L1 measurement on CD-SSB. No additional requirements will be defined for such scenario.

#### Issue 1-4: Applicability of NCD-SSB based L1 and L3 intra-frequency measurement requirements

- For UE supporting option C and configured with EN-DC or NR-DC, NCD-SSB based L1 and L3 intra-frequency measurement requirements are also applicable for the PSCell as long as the PSCell stays in active mode.

#### Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1

- FFS following options

  - Option 1:

    - LS RAN2 and RAN1 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1.

  - Option 2a:

    - RAN4 to confirm that SSB outside a BWP can be configured within CSI-ResourceConfig for the BWP if UE supports option B-1-1.

  - Option 2b:

    - Configuration of SSB resources outside active BWP is allowed within CSI-ResourceConfig settings for UEs supporting option B-1-1

      - No need to send an LS to RAN1/RAN2.

  - Option 2c:

    - RAN4 should further clarify what is the ambiguity part of the other WG spec before sending an LS requesting for further details.

  - Option 3:

    - Update FG53-1 related clarification in L1 RSRP measurement requirements.

#### Issue 1-6: Requirements for L3 intra-f measurement without gaps for option C

- Discuss in CR directly.

- Agreements for performance part

Issue 2-1: Selected test cases for RLM/BFD/BM requirements for option B-1-1

- Introduce test cases for both SA and EN-DC, and reuse the existing applicability rule if UE supports both SA and EN-DC.

- Test cases for SA:

  - TC1:	A.6.5.1.1: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM

  - TC2:	A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

  - TC5:	A.6.6.4.1: SSB-based L1-RSRP measurement

  - TC6:	A.7.6.3.1: SSB-based L1-RSRP measurement

- For EN-DC, the same set as for SA:

  - TC1:	A.4.5.1.1: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM

  - TC2:	A.5.5.1.1: Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

  - TC5:	A.4.6.4.1SSB-based L1-RSRP measurement

  - TC6:  A.5.6.3.1SSB-based L1-RSRP measurement

Issue 2-2: Selected test cases for intra-frequency measurements without gaps for option B-1-1

- Introduce test cases for both SA and EN-DC, and reuse the existing applicability rule if UE supports both SA and EN-DC.

- Test cases for SA:

  - TC1: A.6.6.1.1: SA event triggered reporting tests without gap under non-DRX (FR1)

  - TC2: A.7.6.1.1: SA event triggered reporting tests without gap under non-DRX (FR2)

- Test cases for EN-DC:

  - TC1: A.4.6.1.1: EN-DC event triggered reporting tests without gap under non-DRX (FR1)

  - TC2: A.5.6.1.1: EN-DC event triggered reporting tests without gap under non-DRX (FR2)

Issue 2-3: Additional Test cases for HO requirements for option C

- Introduce Test #1 and Test #4 to verify intra-frequency handover requirements based on NCD-SSB for UE supporting option C.

- Test #3 and Test #6 to verify inter-frequency handover requirements with unknown target cell will be removed.

Issue 2-4: Common test configuration for NCD-SSB for option C

- Introduce new SSB configurations and SMTC configurations for NCD-SSB.

Test cases for option C under EN-DC

- For the test case, apply the same principle for option C with EN-DC as that for Option A and B-1-1.

Issue 2-5: Selected test cases for RLM/BFD/BM requirements for option A

- Introduce test cases for both SA and EN-DC, and reuse the existing applicability rule if UE supports both SA and EN-DC.

- TC for SA:

  - TC1:	A.6.5.1.5: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM

  - TC2:	A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM

  - TC5:	A.6.6.4.3: CSI-RS based L1-RSRP measurement

  - TC6:	A.7.6.3.3: CSI-RS based L1-RSRP measurement

- TC for EN-DC:

  - TC1:	A.4.5.1.5: EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode

  - TC2:	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode

  - TC5:	A.4.6.4.3: CSI-RS based L1-RSRP measurement when DRX is not used

  - TC6:	A.5.6.3.3: CSI-RS based L1-RSRP measurement when DRX is not used

----EoD---