3GPP TSG-RAN WG4 Meeting # 110bis														R4-2406442

Changsha, China, 15th – 19th April 2024

Agenda item:			6.5.5

Source:	Intel Corporation

Title:	WF on NR_MG_enh2_part2

Document for:	Approval

# Topic #1 and #2: Core part maintenance

## Summary of discussions

### Sub-topic 1-1 DRX

Issue 1-1-1: Misalignment between DRX-on duration and SMTC for NFG measurements

- Proposals

  - Option 1: UE measures only within SMTC occasions once every DRX cycle and no interruption is allowed.

    - Option 1a: interruption can be allowed during the time extended due to drx-inactivityTimer.

  - Option 2: UE measures within SMTC occasions when DRX cycle is larger than 320ms and no interruption is allowed; but when DRX cycle is equal or smaller than 320ms interruption is allowed once every DRX cycle according to Tcycle, i.

    - Option 2a: 160ms instead of 320ms. And interruption can be allowed during the time extended due to drx-inactivityTimer.

  - Option 3: UE measurements can affect DRX-on duration. Interruption is allowed once every DRX cycle, and it is according to Tcycle,i.

  - Option 5: Interruptions may be allowed during DRX ON duration under the following conditions:

    - if DRX ON duration is smaller or equal to [5 or 10] ms and DRX cycle is larger than [40 or 80ms],

      - Interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer

    - Otherwise

      - interruptions are allowed, except for the last slot containing PDCCH in the DRX ON duration, excluding the time extended due to drx-inactivityTimer

  - Option 6: Interruptions may be allowed during DRX ON duration under the following conditions:

    - if DRX ON duration is smaller or equal to [5 or 10] ms and DRX cycle is larger than [40 or 80ms],

      - Interruptions are not allowed in the DRX ON duration, excluding the time extended due to drx-inactivityTimer

    - Otherwise

      - interruptions are allowed

Issue 1-1-2: Aligned DRX-on duration and SMTC for NFG measurements and DRX cycle is larger than 320ms

- Proposals

  - Option 1: UE does not measure within SMTC occasions and no interruption is expected.

  - Option 2: Interruption is always allowed, and it is according to Tcycle,i.

Issue 1-1-3: Aligned DRX-on duration and SMTC for NFG measurements and DRX cycle is equal or shorter than 320ms

- Proposals

  - Option 1: Interruption is always allowed, and it is according to Tcycle,i.

Issue 1-1-4: Consider DRX-on duration length as conditions to allow interruptions

- Proposals

  - Option 1: if DRX ON duration is larger than 10 ms, interruptions are allowed, except for the last slot containing PDCCH in the DRX ON duration.

Issue 1-1-5: Consider other DRX cycle length threshold as conditions to allow interruptions

- Proposals

  - Option 1: consider 40ms instead of 320ms.

Issue 1-1-6: Consider whether there is interruption on PDCCH reception as conditions to allow interruptions

- Proposals

  - Option 1:

    - if it is not possible to place an interruption without affecting first and last PDCCH occasion in the DRX ON duration, it is not allowed to cause interruptions during DRX ON durations.

    - Interruptions during inactivity time periods allowing both interruptions and PDCCH allocations shall be placed so that the last PDCCH occasion in the inactivity time is not-interrupted.

Issue 1-1-7: Scaling factor 1.5 when DRX is configured and equal or smaller than 320ms

- Background

  - 1.5 is to address frequent measurements in legacy releases when DRX cycle length is smaller than 320ms.

- Proposals

  - Option 1: Apply 1.5 to measurement and interruption requirements when DRX is configured and equal or smaller than 320ms.

### Sub-topic 1-2 MRDC interruption requirements

Issue 1-2-1: Interruption requirements in 8.2.2.2.19 apply also for NR-DC, EN-DC, and NE-DC

- Proposals

  - Option 1: Yes. Interruption requirements in 8.2.2.2.19 shall also apply for NR-DC, EN-DC, and NE-DC considering that operations in one cell group do not impact operations on another cell group.

    - Option 1a: Same interruption requirement defined at 7.8.2.22 in 36.133 and 8.2.2.2.19 in 38.133 are applied for each cell group for UE perform NR measurement without gap with interruption.

      - 7.8.2.22 in 36.133 and 8.2.2.2.19 in 38.133 is applied for MCG and SCG, respectively in EN-DC,

      - 7.8.2.22 in 36.133 and 8.2.2.2.19 in 38.133 is applied for SCG and MCG, respectively in NE-DC,

      - 8.2.2.2.19 in 38.133 is applied for both MCG and SCG in NR-DC.

  - Option 2: NFG requirements are applicable for NR SA only.

    - Option 2a: deprioritize MR_DC for NFG in objective 2 of the WI.

### Sub-topic 1-3 Others in NFG

Issue 1-3-1: further clarification on the measurement and interruption spec about gap configurations

- Proposals

  - Option 1: clarify in the spec that [no gap with interruption] requirements only apply when gap is not configured, or gap is fully non-overlapped with SMTC on any carriers which are measured with interruption.

Issue 1-3-2: NFG and NCSG capabilities

- Previous Agreements

  - No need to establish the mapping between UE’s indication for NeedForGaps and NCSG.

- Proposals

  - Option 1: NeedForGaps and NCSG are not expected to be enabled for the same UE at the same time.

Issue 1-3-3: capture in the spec that measurements without gap do not cause interruption when target SSB is completely contained within the active BWP

- Proposals

  - Option 1: Capture in the specification

    - the agreement from RAN4#106 that intra-frequency measurements without gaps don’t cause interruption when target SSB is completely contained withing the active BWP of the UE.

    - inter-frequency measurements without gaps do not cause interruption when target SSB is completely contained withing the active BWP of the UE.

### Sub-topic 2-1 Scheduling restriction

Issue 2-1-1: Scheduling restriction due to mixed numerology for case b-2

- Proposals

  - Option 1: The scheduling restriction shall be defined for inter-RAT LTE measurement case b-2 with mixed numerology, -- serving cell and target MO have mixed SCS and they are in the same band, and UE does not support mixed SCS between serving cell and target MO.

    - Option 1a: The scheduling restriction will be applied to the whole EMW if UE doesn’t support mix-numerology between LTE measurement and NR data reception.

Issue 2-1-2: Scheduling restrictions and UE capability reporting

- Proposals

  - Proposal 1a: For Cases b-1 and/or b-2: When the UE require NO scheduling restriction for a specific carrier:

    - the UE shall report to the NW that the UE can measure a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption;

    - the UE shall not report EMW (i.e. the UE can measure with at any reference signal occasion).

  - Proposal 1b: For Cases b-1 and/or b-2: When the UE require scheduling restriction for a specific carrier:

    - the UE reports to the NW that the UE can measure a given/specific carrier frequency of inter-RAT EUTRAN without gap and without interruption,

    - also, the UE shall report the support of effective measurement window (EMW) for inter-RAT EUTRAN capability (i.e. the UE can measure and cause scheduling restrictions only within the EMW occasions).

  - Proposal 1c: For Cases b-1 and/or b-2: When the UE reports for inter-RAT EUTRAN measurements without gap and without interruption and the UE doesn’t support EMW capability, the UE shall report NCSG, or gaps.

  - Proposal 2 : For case b-1, DL scheduling restriction is not needed for inter-RAT LTE measurement without gap due to mixed numerology.

Figure 1: Illustration of Option 1, UE behaviour for measuring inter-RAT EUTRAN without gaps and without interruption.

### Sub-topic 2-2 Measurement reporting period requirements

Issue 2-2-1: Overlap between Effective measurement window and SMTC/SSB

- Background

  - Previous Agreements

    - For case b-2, when EMW is configured overlapped with SMTC/SSB/CSI-RS measurement with scheduling restrictions, inter-RAT LTE measurement will be dropped.

    - For case b-1 and b-2, when EMW is partially overlapped with MG (EMW periodicity < MGRP), the EMW occasion colliding physically with MG will be dropped.

- Proposals to: For case b-1 and b-2 inter-RAT LTE measurement causing scheduling restriction, when EMW periodicity is smaller than MGRP,

  - Option 1: RAN4 to update the legacy agreements as: after considering EMW dropping rule if EMW is colliding with SMTC/SSB/CSI-RS, when the remaining EMW is fully overlapping with MG, the inter-RAT meas will be performed within MG.

Issue 2-2-1a: Overlap between Effective measurement window and MG

- Background

  - Agreements

    - For case b-1 and b-2, when EMW is partially overlapped with MG (EMW periodicity < MGRP), the EMW occasion colliding physically with MG will be dropped.

    - Note: The proximity rule in Rel-17 does not apply in this case.

- Proposals to: For case b-1 and b-2 inter-RAT LTE measurement causing scheduling restriction, when EMW periodicity is larger than MGRP and all EMW are covered by measurement gaps,

  - Option 1: inter-RAT LTE measurement will be dropped.

  - Option 2: No UE behaviour is specified.

  - Option 3: apply legacy gap-based measurement requirements, i.e. RAN4 requirements should NOT be defined based on EMW.

  - Option 4: UE measurement requirements are based on EMW-RP.

Issue 2-2-2: Tinter1 for EMW configuratios:

Table 2

| EMW Pattern Id | EMW Length (EMWL, ms) | EMW Repetition Period (EMWRP, ms) | Minimum available time for inter-RAT measurements during 480 ms period (Tinter1, ms) |
| --- | --- | --- | --- |
| 2 | 2 | 40 | [24] |
| 3 | 2 | 80 | [12] |

- Proposals

  - Option 1: Update the Tinter1 for EMW pattern 2/3 to 60/30ms.

| EMW Pattern Id | EMW Length (EMWL, ms) | EMW Repetition Period (EMWRP, ms) | Minimum available time for inter-RAT measurements during 480 ms period (Tinter1, ms) |
| --- | --- | --- | --- |
| 2 | 2 | 40 | [60] |
| 3 | 2 | 80 | [30] |

  - Option 2: When determining UE requirements using Tinter1 for EMW pattern IDs 2, 3, 4, 5, Tinter1 = 60 for gap pattern IDs 2, 4, and Tinter1 = 30 for gap pattern IDs 3 and 5 shall be used.

  - Option 3: When determining UE requirements using Tinter1 for EMW IDs 2 and 3,  Tinter1 = 60 for EMW ID 2 and Tinter1 = 30 for EMW ID 3 shall be used if EMW dropping rule is not applied specified in clause X. Otherwise, Tinter1 specified in table 2 is applied.

Issue 2-2-4: Tinter1 without EMW configuration

- Proposals

  - Option 1: Define Tinter1 without EMW configuration as 60ms (based on EMW pattern 0).

  - Option 2: For UE can perform inter-RAT LTE measurement without gap and does not require a scheduling restrictions,  Tinter1 = 60ms is applied for the inter-RAT LTE measurement without gap.

Issue 2-2-3: Scaling factor for case a-1: Nfreq definition

- Background

  - The principles are different between NR MO outside gap and LTE inter-frequency without MG, where all inter-frequency MOs, regardless if they are measured with or without MG, are counted in the same Nfreq.

- Proposals

  - Option 1: number of NR MOs that are measured outside MG (same principle as NR SA).

  - Option 2: total number of LTE and NR MOs (same principle as LTE SA).

### Sub-topic 2-3 UE capabilities

Issue 2-3-1: case a-1: The issue with UE capability interRAT-NeedForIntrNR-r18

- Proposals

  - Option 1: Interruptions due to interRAT NR measurements without gaps must be explicitly allowed by the network (via SIB or other means). Send LS to RAN2.

  - Option 2: Do not change current interRAT-NeedforIntrNR-r18 capability design. Not to make reporting of interRAT-NeedForIntrNR-r18 based on NW control. Do not further discuss how to report UE capability interRAT-NeedForIntrNR-r18.

  - Option 3: interRAT-NeedForIntrNR-r18 capability is based on network request. Send LS to RAN2.

Issue 2-3-1a: Relations between interRAT-NeedForGaps-r16 and interRAT-NeedForIntrNR-r18 and UE behaviours

- Previous agreements

Issue 1-1-1: Scenario 1, LTE – NR inter-RAT measurement Tentative agreements - interRAT-NeedForGaps-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - FFS mandatory report of the UE capability R18 interRAT-NeedForIntrNR-r18 from Rel-18 UE if the UE reports interRAT-NeedForGaps-r16=FALSE - FFS the UE behavior if the Rel-18 UE does not support interRAT-NeedForIntrNR-r18

- Proposals

  - Option 1: A Rel-18 UE indicating support of interRAT-NeedForGapsNR-r16 shall also indicate support of interRAT-NeedForInterruptionNR-r18.

  - Option 2: When a Rel-18 UE only supports Rel-16 NFG capability but not supports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE.

    - Option 2a: When a Rel-18 UE supports both Rel-16 NFG and Rel-18 NFI capabilities, but NW doesn’t configure Rel-18 needForInterruptionConfigNR, the UE’s behaviour is the same as Rel-16 UE.

Issue 2-3-1b: Relations between nr-NeedForGap-Reporting-r16 and nr-NeedForInterruptionReport-r18 and UE behaviours

- Previous agreements

Issue 1-1-2: Scenario 2, NR measurements without gaps Tentative agreements - “no-gap” as part of NeedForGapsInfoNR-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - FFS the UE behavior if the Rel-18 UE does not support NeedForInterruptionNR-r18

- Proposals

  - Option 1: A Rel-18 UE indicating support of nr-NeedForGap-Reporting-r16 shall also indicate support of nr-NeedForInterruptionReport-r18.

  - Option 2: When a Rel-18 UE only supports Rel-16 NFG capability but not supports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE.

    - Option 2a: When a Rel-18 UE supports both Rel-16 NFG and Rel-18 NFI capabilities, but NW doesn’t configure Rel-18 needForInterruptionConfigNR, the UE’s behaviour is the same as Rel-16 UE.

Issue 2-3-2: Introduction of new general UE capabilities (feature groups) for case b-1: [32-6 inter-RAT EUTRAN measurement without gap and outside active DL BWP]

- Agreements

  - EMW is only needed when scheduling restriction is expected. For measurement delay, RAN4 needs to define a default Tinter1 when EMW is not configured.

  - Introduce FG 32-6

Issue 2-3-4: UE supporting Case b-1 or Case b-2 needs to support EMW

Issue 2-3-5: Introduction of new UE capabilities (feature groups) for mixed numerology for case b-1 and b-2: [32-10 UE support concurrent inter-RAT measurement on EUTRAN and reception from NR serving cell with a different numerology than 15kHz]

Issue 2-3-6: Introduction of new UE capabilities (feature groups) for EUTRAN feature: [general support of inter-RAT NR measurements without gap with or without interruption]

Issue 2-3-7: Introduction of new UE capabilities (feature groups) for EUTRAN feature: [UE support concurrent inter-RAT NR measurement with a different numerology than 15kHz and reception from LTE serving cell]

Issue 2-3-8: Feature list tables for inter-RAT measurements without gap

- Agreements

  - For UE support 32-4, no scheduling restriction is needed due to mix-numerology for CRS fully non-overlapping with UE’s active BWP

  - FFS whether to add a FG for scheduling restriction for FG x-y

- Agreed feature list:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gaps with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Details in Clause [9.1.x.2] of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-2 | Support for dynamic collisions | Support RRM requirements for handling dynamic collisions between a Pre-MG and another measurement gap or Pre-MG. | 32-1 | Yes | No | UE is not expected to meet RRM requirements for dynamic collisions | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-3 | Concurrent gaps with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Details in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-4 | Inter-RAT EUTRAN measurements without gap and outside active DL BWP | Support inter-RAT EUTRAN measurements outside active DL BWP for nogap-noncsg | 19-1b | Yes | NA | UE does not meet the requirements of inter-RAT EUTRAN measurements without gap in TS 38.133 and the UE behavior is unknown to network | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-5 | Inter-RAT EUTRAN measurement without gap and within active DL BWP | Support of inter-RAT EUTRAN measurements without gap when CRS is contained within UE’s active DL BWP | [32-8] | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-6 | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | 32-4 or 32-5 | Yes | No | UE is not allowed to cause scheduling restriction defined in TS 38.133 for 32-6 or 32-7 | Per UE | No | No | N.A | - A bitmap for 6 effective measurement window (EMW) patterns defined in TS 38.133. - #0 and #1 are mandatory, if UE supports EMW feature. Other patterns are optional Note: If UE supports 32-6 or 32-7 and UE requires scheduling restriction, UE should support this FG | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-7 | Simultaneous reception of NR data and EUTRAN CRS with different numerology | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS and PDCCH or PDSCH reception from the serving cell with a different numerology | 32-4 or 32-5 | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |

Table 2: Rel-18 LTE UE features for NR_MG_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | x-y | interRAT-NeedForInterruptionNR-r18 | Support of inter-RAT NR measurements without gap with or without interruption when the interRAT-NeedForGapsNR-r16 is false. Note: This feature already has a defined UE capability: ‘interRAT-NeedForInterruptionNR-r18’. The intention of adding this FG is only keep consistency between 38.822 and 36.306. | interRAT-NeedForGapsNR-r16 | Yes | NA | The UE does not support inter-RAT NR measurements without gap with or without interruption for performing inter-RAT NR measurement without gap | [Per target band per BC] Note: the same granularity as interRAT-NeedForGapsNR-r16 | No | No | NA | # Candidate value: “{no-gap-with-interruption, no-gap-no-interruption}” | Optional with capability signalling |

# Topic #3: Performance part requirements for measurements without gap

## Summary of discussions

Up to this meeting, all agreed using scenarios for inter-RAT NR/LTE measurements without gap can summarized as:

- the inter-RAT NR measurements without gap in Rel18 includes the two scenarios below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

- the inter-RAT LTE measurements without gap in Rel18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

## Test cases list and responsibilities

### NFG

Test list agreed

| No. # | Item of core requirements | Type of test cases | Frequency range of serving cell | SMTC configuration | MG configuration | DRX configuration | Subclause | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NFG1 | Event triggered reporting and interruptions | Intra-frequency measurements without gap config but with DRX Note: The testability needs to be checked before introducing the test case | FR1 | 20ms | No gap config | [DRX pattern TBD] | A.6.6.1.X | Nokia |
| NFG2 |  | Intra-frequency measurements without gap or DRX configuration | FR2 | 160ms20ms | gap is configured Note: UE can skip corresponding Rel-15 test cases | No DRX | A.7.6.1.X | Intel |
| NFG3 |  | Inter-frequency measurements without gap and without DRX | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | QC |
| NFG4 |  | Inter-frequency measurements without gap config but without DRX Note: The testability needs to be checked before introducing the test case | All in FR1 | 160ms | without gap | No DRX config | A.7.6.1.X | HW |
| NFG5 | Event triggered reporting [without interruption] | Inter-frequency measurements without gap with non-DRX | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | Ericsson |
| NFG6 |  | Inter-frequency measurements without gap with non-DRX | All in FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X | CATT |
| NFG7 |  | Intra-frequency measurements without gap with non-DRX | FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | CMCC |
| NFG8 |  | Intra-frequency measurements without gap with non-DRX | FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X | MTK |

- Agreements

  - For all intra-freq without gap TC, companeis are encourage to check the testability, e.g., FFS L1 measurement configuration for active BWP not containing serving cell SSB.

  - FFS whether to change the test case list remove TC among NFG5 to NFG8 will be confirmed in RAN4#111 meeting

### Inter-RAT

Test list for information

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | EMW configuration | MG configuration | DRX configuration | Subclause | Responsibility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IR1 | Event triggered reporting and interruptions | Inter-RAT EUTRAN measurements case b-1 with MG and DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | [DRX pattern TBD] | A.6.6.3.X | CATT |
| IR2 |  | Inter-RAT EUTRAN measurements case b-1 with MG but no DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | No DRX config | A.6.6.3.X | xiaomi |
| IR3 | Event triggered reporting [without interruption] | Inter-RAT EUTRAN measurements case b-2 without MG or DRX configuration | NR FR1 | [Pattern TBD] | No gap configuration | No DRX configuration | A.6.6.3.X | CMCC |
| IR4 | Event triggered reporting | Inter-RAT NR measurements case a-1with MG | NR FR2 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | HW |
| IR5 | Event triggered reporting and interruptions | Inter-RAT NR measurements case a-1 with MG | NR FR1 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | Nokia |