# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2406485

# Changsha, China, 15 April – 19 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Core) draft CR on core maintenance for R18 NTN mobility and service continuity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-03-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The reply LS on RAN2 agreements for satellite switch with resync (R4-2403493) was agreed in last RAN4 meeting. The feasibility of soft satellite switch with resync was agreed as following: - It is feasible that a soft satellite switch capable UE can perform downlink synchronization with the target NGSO satellite and keep the connection (DL and UL) with the source NGSO satellite simultaneously under the following conditions. - Only if SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side. - If the UE starts synchronization to a different satellite from T-ServiceStart, scheduling restriction is expected, if: - the UE is not capable of parallelMeasurementWithoutRestriction-r17, or - the UE is not capable of simultaneousRxDataSSB-DiffNumerology but the SSB has a different SCS than PDCCH/PDSCH - Note: RAN4 is investigating how to minimize the duration of the scheduling restriction. However, the time domain seperation of SSBs from the two satellites is not captured as a side condition in the current soft satellite switch requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the time domain seperation of SSBs from the two satellites as a side condition in the current soft satellite switch requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The time domain seperation of SSBs from the two satellites is not captured as a side condition in the current soft satellite switch requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of Change 1

### 6.1C.3	NR SAN Satellite switching with re-synchronization

#### 6.1C.3.1	Introduction

The purpose of NR SAN Satellite switching with re-synchronization is to switch satellite in the same SSB frequency and same gNB without PCI changing. The requirements in this clause are applicable to SA NR SAN.

#### 6.1C.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization

The requirements in this clause are applicable to both hard and soft switch over in quasi-earth fixed scenario from NR SAN FR1 cell to NR SAN FR1 cell. The requirements in this clause apply provided that UE has the valid and applicable parameters of ephemeris information, common TA, DL and UL Polarization information, Koffset, and Kmac for target NR SAN cell during Dswitch_unchangedPCI, otherwise interruption time may be longer than the requirements in clause 6.1C.3.2.2 for hard satellite switch and clause 6.1C.3.2.3 for soft satellite switch respectively.

##### 6.1C.3.2.1	Satellite switching delay

When the UE receives a broadcast message implyingsatellite switching within re-synchronization, the UE shall be ready to start the transmission of the new uplink PRACH channel or transmission of the new uplink PUSCH channel within Dswitch_unchangedPCI msec.

Where:

-	Dswitch_unchangedPCI equals the interruption time stated in clause 6.1C.3.2.2 and clause 6.1C.3.2.3 for hard satellite switch and soft satellite switch respectively.

##### 6.1C.3.2.2	Interruption time for hard satellite switch

The interruption time is the time between t-service and the time the UE starts transmission of the new PRACH for RACH-based case or first UL transmission on PUSCH for RACH-less case if the UE only supports the feature for hard satellite switch and the hardSatelliteSwitch-Resync-NTN-r18 is enabled.

When intra-frequency hard switch to NR SAN cell is commanded,

the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Otherwise, no interruption time requirement is applied.

Where:

-	Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = [Tfirst_SSB] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	T∆ is same as the one defined in section 6.1C.2.2.2.1.

-	Tprocessing is time for UE processing. Tprocessing can be up to 10 ms.

-	Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

-	TIU is the interruption uncertainty in acquiring the first UL transmission resource, which can be a configured grant based PUSCH, dynamic grant based PUSCH, SR on PUCCH, according to NW configuration and scheduling, or PRACH if TA timer is not running and there is no PUCCH SR

-	Tfirst_SSB is the time to the end of the first complete SSB burst indicated by

-	SMTC of target satellite (SMTC of the source satellite serving cell + ssb-TimeOffset) + PDD propagation difference

##### 6.1C.3.2.3	Satellite switch delay for soft satellite switch

The Satellite switch delay is from t-serviceStart to the time instance for the first UL transmissionon of the new PRACH for RACH-based case if TA timer is not running and there is no PUCCH SR or configured grant based PUSCH, dynamic grant based PUSCH, SR on PUCCH for RACH-less based case, if the UE supports the feature for soft satellite switch and the softSatelliteSwitch-Resync-NTN-r18 is enabled.

When intra-frequency soft switch to NR SAN cell is commanded,

the satellite switch time shall be less than Tsoft_switch

Tsoft_switch = max(t-service - t-seviceStart, Tsearch + T∆ + Tmargin) + TIU + Tprocessing  ms

Otherwise, no satellite switch time requirement is applied.

Where:

-	Tsearch is the time required to search the target NR SAN cell when the target cell is not already known when the handover command is received by the UE. [If the target cell is known, then Tsearch = 0 ms.] If the target cell is an unknown intra-frequency cell and the target cell Es/Iot ≥ -2 dB, then Tsearch = Tfirst_SSB ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

-	T∆ is same as the one defined in section 6.1C.2.2.2.1.

-	Tprocessing is same as the one defined in section 6.1C.2.2.2.1.

-	Tmargin is same as the one defined in section 6.1C.2.2.2.1.

-	TIU is the interruption uncertainty in acquiring the first UL transmission resource, which can be a configured grant based PUSCH, dynamic grant based PUSCH, SR on PUCCH, according to NW configuration and scheduling, or PRACH if TA timer is not running and there is no PUCCH SR

-	Tfirst_SSB is is the time to the end of the first complete SSB burst indicated by

-	SMTC of serving cell + ssb-TimeOffset + PDD propagation difference

[In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds before UE starts synchronizing with target satellite. Otherwise it is unknown. Relevant cell identification requirements are described in Clause 9.2.5 for intra-frequency handover.]

The requirement in this clause applies and UE is required to keep the connection (DL and UL) with the source NGSO satellite, under the following conditions:

- SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side.

End of Change 1