3GPP TSG-RAN WG4 Meeting #110bis 											R4-2406598

# Changsha, China, April 15 – 19, 2024

Agenda item:			6.20.5

Source:	LG Electronics

Title:	WF on SL enhancement for remaining NS values

Document for:	Approval

# Way forward for completing the A-MPR requirements for remaining NS values

In Rel-18 NR SL-U A-MPR requirements were defined for 5 NS-values covering PSCCH, PSSCH, PSFCH and S-SSB. To achieve global footprint A-MPR requirements need to be defined for the remaining 12 NS values:

The status for A-MPR work for all 17 NS-values is shown in the table below:

| # | NS | Status | A-MPR simulation results available |
| --- | --- | --- | --- |
| 1 | NS_28 | Way forward below in 2. | YES |
| 2 | NS_29 |  | YES |
| 3 | NS_30 |  | YES |
| 4 | NS_31 | Already completed | YES |
| 5 | NS_53 | Already completed | YES |
| 6 | NS_54 |  | YES |
| 7 | NS_58 | Already completed | YES |
| 8 | NS_59 |  | YES |
| 9 | NS_60 | Already completed | YES |
| 10 | NS_61 | Already completed | YES |
| 11 | NS_63 |  | YES |
| 12 | NS_64 |  | YES |
| 13 | NS_65 |  | YES |
| 14 | NS_66 |  | YES |
| 15 | NS_67 |  | YES |
| 16 | NS_68 |  | YES |
| 17 | NS_69 |  | YES |

RAN4 targets to complete the A-MPR requirements for SL-U in RAN4#111 meeting (May) based on the available A-MPR simulation results.

Following the suggestion from the Chairman it is agreed to request for a new spectrum related Rel-19 WI in upcoming RAN plenary, if the work cannot be completed in May meeting (RAN4#111).

# A-MPR for NS_28

A-MPR simulation results for NS_28 have been provided and these have been/are being captured into the TR 38.786. Based on these results following A-MPR is agreed for PSCCH, PSSCH, PSFCH and S-SSB.

---- CLIP ---- CLIP ---- CLIP ----

#### 6.2E.3F.8	A-MPR for NS_28

When NS_28 is indicated by the network or pre-configured radio parameters for NR sidelink UE, this clause specifies the allowed Maximum Power Reduction (MPR) power for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed A-MPR is specified in Table 6.2F.3F.8-1 for power class 5 NR sidelink UE.

Table 6.2E.3F.8-1: A-MPR for NS_28 NR SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 3) |  |  |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 7.0 | ≤ 4.0 | ≤ 5.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 6.0 | ≤ 7.5 | ≤ 4.0 | ≤ 5.0 |  |
|  | 64 QAM | ≤ 6.5 | ≤ 7.5 | ≤ 4.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 8.0 | ≤ 8.5 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, 5520, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.2E.2F-3 apply. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed A-MPR is specified in Table 6.2E.3F.8-2 for power class 5 NR sidelink UE.

Table 6.2E.3F.8-2: A-MPR for NS_28 for PSFCH transmission for NR SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |
| --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer/Inner RB sets |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 15.0 | ≤ 12.5 | Table 6.2E.2F-4(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz hannels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, 5530, and 5610 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |

For S-SSB transmission, the allowed A-MPR is specified in Table 6.2E.3F.8-3 for power class 5 NR sidelink UE.

Table 6.2E.3F.8-3: A-MPR for NS_28 for S-SSB transmission for NR SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |
| --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer/Inner RB sets |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |  |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 | Table 6.2E.2F-5(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |

---- CLIP ---- CLIP ---- CLIP ----