3GPP TSG RAN WG4 #110BIS meeting                                                            	         R4-2406611

Changsha, China, 15th – 19th April, 2023

Agenda item:			9.9.4

Source:	OPPO, Meta Ireland

Title:	WF on NR_SL_ intraB_CA_ITS

Document for:	Approval

Introduction

This WF try to capture some agreements in the on-line session. Mostly the sub-topic 2-1 and 3-1 are covered.

For the MPR simulation assumption, we give some architecture clarifications together with the background of Rel-17 UL CA contiguous/non-contiguous baseline assumptions and try to reach some WF of the baseline assumption for Rel-19 intra-band PC2 contiguous CA and intra-band PC3/PC2 non-contiguous CA. Hence for sub-topic 2-2 and 3-2, we try to reach some consensus in this WF.

For the UE TX/RX requirements covered by sub-topic 2-3 and 3-3, it is left as open issues since it is only the first meeting and we hope to focus and agree on the simulation assumptions first. Companies can further investigate and input in future meetings.

Background

This background gives instruction on Rel-17 PC2 NR UL contiguous and non-contiguous CA work, the following architecture and baseline is agreed. This might give further instruction for issue 2-2-1 and issue 3-2-2.

For PC2 contiguous UL CA:

- 200MHz PA + 1 LO is assumed as baseline:

Figure 1 Baseline assumption for PC2 contiguous UL CA in Rel-17

For PC2 non-contiguous UL CA:

- 2*100MHz PA + 2 LO is assumed as baseline:

Figure 2 Baseline assumption for PC2 non-contiguous UL CA in Rel-17

Note that the architecture is discussed based on 200MHz channel bandwidth.

Topic #2: Intra-band non-contiguous CA

Sub-topic 2-1 General

- Issue 2-1-1: Channel bandwidth

Agreement:

- RAN4 should specify the RF requirements based on the channel bandwidth combinations with two component carrier in Table 2.1-1:

Table 2.1-1 channel bandwidth combinations for SL intra-band non-contiguous CA

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47(2A) | SL_n47(2A) | 10 | 10, 20 |  |  | 30 | 0 |

Sub-topic 2-2 MPR/A-MPR Related

- Issue 2-2-1: UE RF architecture

- Proposal:

  - Proposal 1: (Huawei, OPPO)

    - Single PA structure for intra-band non-contiguous SL CA is sufficient in Rel-19.

  - Proposal 2: (Xiaomi)

    - MPR for PC3 SL intra-band non-contiguous CA should be specified based on both of 1Tx antenna architecture and 2Tx antenna architecture.

    - MPR for PC2 SL intra-band non-contiguous CA should be first specified based on 2Tx antenna architecture, it needs further discuss whether MPR for PC2 SL intra-band non-contiguous CA should be specified based on 1Tx antenna architecture.

  - Proposal 3: (LGE, Meta)

    - Specify PC2 MPR considering both single Tx and dual Tx, and PC3 MPR.

  - Proposal 4: (VIVO)

    - To start the work for PC2/PC3 SL intra-band non-contiguous CA, decide the RF architectures first.

Further clarification and WF:

Similarly, for Rel-19 sidelink CA, based on the assumption of both 1TX and 2TX for intra-band non-contiguous NR CA, with different implementation of 1LO and 2LO for 2TX cases, below   different cases are listed:

- Option 1: 1PA + 1LO

Figure 2-a PC2/PC3 intra-band non-contiguous CA assumption of 1PA+1LO

- Option 2: 2PA + 2LO,

Figure 2-b PC2/PC3 intra-band non-contiguous CA assumption of 2PA+2LO

- Option 3: 2PA + 1LO

Figure 2-c PC2/PC3 intra-band non-contiguous CA assumption of 2PA+1LO

Similar as previous issue, as only 70MHz needs to be supported, hence the following WF is proposed:

WF:

  - For PC3 and PC2 intra-band non-contiguous CA, it is proposed to use Option1 and Option 2 as a starting point assumption for MPR/A-MPR simulation.

  - RAN4 can consider all candidate RF architectures to derive PC3/PC2 MPR/A-MPR requirements for intra-band non-contiguous CA.

  - Further discuss these options in the next meeting.

- Issue 2-2-2: Simulation assumption

- Proposals:

  - Proposal 1: (Meta)

- Table 1. Simulation assumption for NR SL CA operation in Rel-19

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | Intra-band contiguous CA: per CC (10/20/30/40MHz), Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) Intra-band non-contiguous CA : 10MHz+10MHz or 10MHz +20MHz |
| Maximum output power for aggregated CBW | Intra-band contiguous SL-CA: 26dBm Intra-band non-contiguous SL-CA: 23dBm/26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 31dBc for PC2 30dBc for PC3 |
| Carrier leakage | 34dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

  - Proposal 2 (Huawei): For the MPR evaluation of intra-band non-contiguous CA with PC2/PC3, the simulation assumptions as follows should be agreed

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 23dBm and 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 34dBc or 25dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

  - Proposal 3 (Xiaomi, OPPO): Using the MPR/A-MPR simulation assumptions in Table 2.2-1 as starting point for PC2/PC3 SL intra-band non-contiguous:

- Table 2.2-1 MPR/A-MPR simulation assumptions

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20MHz aggregated CBW: 10+10MHz/ 10+20MHz |
| Maximum output power for aggregated CBW | 23dBm/26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 31dBc for PC2, 30dBc for PC3 |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dB ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

  - Proposal 4: MPR for PC2/PC3 SL intra-band non-contiguous CA should be specified for PSSCH/PSCCH, PSFCH and S-SSB.

- WF:

  - Agree on below table for simulation assumption

  - The carrier leakage can be further discussed

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20MHz aggregated CBW: 10+10MHz/ 10+20MHz |
| Maximum output power for aggregated CBW | 23dBm/26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 30dBc(PC3) / 31dBc(PC2) |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dB ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

- Issue 2-2-3: Frequency gap

- Proposals:

  - Proposal 1: (Meta)

    - On top of above MPR simulation assumptions in Table 1, RAN4 should consider the frequency gap between CC1 and CC2 to derive MPR requirements for intra-band non-contiguous SL CA in ITS spectrum.

    - On top of above MPR simulation assumptions in Table 1 and additional spectrum emission mask in Table 6.5E.2.3.1-1 in TS38.101-1 and additional spurious emission requirements in Table 6.5E.3.4.2-1 and Table 6.5E.3.4.2-2 in TS38.101-1, RAN4 should consider the frequency gap between CC1 and CC2 to derive A-MPR requirements to comply European regulation for intra-band non-contiguous SL CA for both power class 3 and power class 2 in ITS spectrum.

- Moderator WF:

  - for the intra-band non-contiguous SL CA, the frequency gap should be determined based on the real deployment scenarios in global countries. The detail frequency gap is FFS.

Topic #3: Intra-band contiguous CA

Open issues summary

Sub-topic 3-1 General

- Issue 3-1-1: System Parameter

Agreement:

- RAN4 define the PC2 RF requirements for sidelink CA SL_n47B with channel bandwidth combinations in Table 2.1-1:

Table 2.1-1 channel bandwidth combinations for PC2 SL intra-band contiguous CA

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47B | SL_n47B | 10 | 10, 20,30 |  |  | 70 | 0 |
|  |  | [20] | [20], [30] |  |  |  |  |
|  |  | 30 | 30,40 |  |  |  |  |

Sub-topic 3-2 MPR/A-MPR related

- Issue 3-2-1: UE RF architecture

- Proposals

  - Proposal 1: (Xiaomi)

    - MPR for PC2 SL intra-band contiguous CA should be specified for PSSCH/PSCCH, PSFCH and S-SSB considering both of 1Tx antenna architecture and 2Tx antenna architecture

  - Proposal 2: (LGE, Meta)

    - Specify PC2 MPR considering both single Tx and dual Tx.

  - Proposal 3: (VIVO)

    - To adopt 1Tx architecture for PC2 SL intra-band contiguous CA and further discuss whether to support 2Tx architecture.

Further clarification and WF:

Similarly, for Rel-19 sidelink CA, based on the assumption of both 1TX and 2TX, with different implementation of 1LO and 2LO for 2TX cases, below   different cases are listed:

Firstly, for PC2 intra-band contiguous CA:

- Option 1: 1PA + 1LO.

Figure 3-a PC2 intra-band contiguous CA assumption of 1PA+1LO

- Option 2: 2PA + 2LO,

Figure 3-b PC2 intra-band contiguous CA assumption of 2PA+2LO

- Option 3: 2PA + 1LO

Figure 3-c PC2 intra-band contiguous CA assumption of 2PA+1LO

Based on some offline discussion, WF is proposed:

WF:

It is proposed to use option 1 for 1PA+1LO (Figure 3-a) as the baseline assumption for PC2 intra-band contiguous CA.	It is proposed to use option 1 and option 3 as the starting point assumption for PC2 intra-band contiguous CA.

  - RAN4 can consider all candidate RF architectures to derivePC2 MPR/A-MPR requirements for intra-band contiguous CA.

  - Further discuss these options in the next meeting.

- Issue 3-2-2: Simulation assumption

- Proposals

  - Proposal 1: (Huawei)

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

  - Proposal 2: (Xiaomi, OPPO)

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 34dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

  - Proposal 3: (Meta)

- Table 1. Simulation assumption for NR SL CA operation in Rel-19

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | Intra-band contiguous CA: per CC (10/20/30/40MHz), Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | Intra-band contiguous SL-CA: 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 34dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

  - Proposal 4: MPR requirements for PC2 SL intra-band contiguous CA should be specified for PSSCH/PSCCH, PSFCH and S-SSB.

- WF:

  - Agree on below table for simulation assumption

  - The carrier leakage can be further discussed

| - Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |