3GPP TSG-RAN WG4 Meeting #110bis											R4-246615

Changsha, China 15th ‒ 19th April, 2024

Title: 	WF on IMT parameters for other frequency ranges

Agenda Item:	9.3.6

Source: 	CATT

Document for:	Approval

# 1 Background

For the Rel-19 study item aiming at replying the LS from ITU-R WP5D requesting IMT parameters for the following three frequency ranges:

- 4400 to 4800 MHz

- 7125 to 8400 MHz

- 14800 to 15350 MHz

RAN4#110bis has started the discussion [1], and the agreements on the first frequency range 4400 – 4800 MHz are captured in [2], while the other two frequency ranges in this way forward for facilitating the discussions in subsequent meetings.

# 2 IMT parameters for Frequency 7125 – 8400 MHz

Way forward:

-	n104 as starting point

o	Identify which parameters need further discussion and could differ from n104

Issue 2-1 Duplex Method

- Option 1: Align with 4GHz outcome, i.e., TDD as baseline, check SBFD

- Option 2: Follow n104

Issue 2-2 Typical Channel Bandwidth (100MHz is maximum bandwidth)

- n104 (100MHz) baseline

  - FFS higher bandwidths (e.g. 200MHz)

Issue 2-3 Transmit bandwidth configuration (Signal bandwidth)

- n104 spectrum utilization assumed

  - FFS check for higher bandwidth

Issue 2-4 Power dynamic range (UE)

- Use n104, as appropriate for maximum output power

Issue 2-5 Spectrum mask

- BS:

  - n104 as basis for emissions levels

    - ΔfOBUE and ΔfOOB for the BS side can be considered further.

- UE:

  - n104 as basis for emissions levels

Issue 2-6 ACLR

- BS

  - Use n104

- UE

  - Option 1:  26dB, 27dB (study) for PC3

  - Option 2:  30dB (n104) for PC3, 31dB (n104) for PC2

Issue 2-7 Spurious Emissions

- BS:

  - Use n104

- UE:

  - Use n104

Issue 2-8 Maximum output power (UE)

- Option 1: 23dBm only.

- Option 2: 20dBm

- Option 3: Use n104 (23 and 26dBm)

- Option 4: 29dBm

Issue 2-9 Noise Figure

- UE

  - Option 1: Follow n104 noise figure (12dB)

  - Option 2: Be consistent with information sent previously IMT-2020 28GHz, e.g. 10dB

  - Option 3: Be consistent with Previous LS to ITU-R on 6, 10GHz, NF was 9-13dB

- BS

  - Option 1: Follow 38.820, i.e., 6dB WA, 11dB MR, 14dB LA

  - Option 2: Follow n104 if different

Issue 2-10 Sensitivity

- To be discussed further

Issue 2-11 Blocking response

- BS:

  - Follow n104

    - Discuss ΔfOOB

- UE:

  - FFS

Issue 2-12 ACS

- UE:

  - Follow n104 or follow previous studies

- BS:

  - Follow n104 (42dB)

Issue 2-13 SINR operating range

- To be discussed further

Issue 2-14: Deployment scenarios to consider

- Include urban and sub-urban macro, but double check coverage

# 3 IMT parameters for Frequency 14800 – 15350 MHz

Way forward

-	Further discussing the following issues

Issue 3-1 Common understanding on UE antenna array options

- Option 1: “UE beamforming” (FR2 like)

  - Assumed to be FR2 like UE, with two panels similar to Rel-14 NR study 38.803

  - Analogue like beamforming

  - Option 1a: 2x2 antennas per panel

- Option 2: FR1 like

  - Assumed to be FR1 like UE with more than one TX/RX antenna

  - Isotropic antennas

  - Option 2a: 2 TX

  - Option 2b: 4 TX

  - Option 2c: 4 RX

  - Option 2d: 6 RX

  - Option 2e: 8 RX

Issue 3-2 Initial deployment scenarios

- Option 1: Follow 38.921

  - Indoor 20m ISD

  - [0.2 - 0.45]km Urban

  - [<=0.9]km sub-urban

- Option 2: 38.803 FR2 assumptions

  - Indoor

  - Dense Urban

  - Urban macro

- Other options such as smaller ISD not precluded depending on antenna array assumptions

Issue 3-3 Initial coordinated operation

- Option 1:

  - Co-ordinated for indoor

  - Both co-ordinated and un-co-ordinated for outdoor

Issue 3-4 Initial views on BS array size

- Option 1: 1024 elements

- Option 2: 2048 elements

- Option 3: Other sizes

Issue 3-5 Initial views on BS sub-array architecture

  - Option 1: No sub-arrays

  - Option 2: Sub array size 2

  - Option 3: Sub array size 3

  - Option 4: Sub-array size 4

  - Option 5: Sub-array size 6

  - Option 6: Sub-array size 8

  - Option 7: Sub-array size 12

# Reference

- R4-2405283, “Ad-hoc minutes [110bis][130] FS_NR_IMT”, Moderator (Ericsson)

- R4-2406612, “WF on study of IMT parameters for 4400 ~4800 MHz frequency range”, Ericsson