# 3GPP TSG-RAN WG4 Meeting #110bis                                                           R4-2406635

# Changsha, China, 15th - 19th April 2024

# Title:	TP for TR38.718-03-01 Support of CA_n1-n77-n79

# Source:	Softbank

# Agenda Item:	5.11.2

# Document for:	Approval

- Introduction

This contribution is a text proposal for TR 38.718-03-01[5] to include the following NRCA combinations as requested in RAN4#109. This proposal adds DL CA_n1A-n77(2A)-n79A with ULCA_n77(2A). DL CA_n1A-n77(2A)-n79A with UL CA_n1A-n77A, CA_n1A-n79A, CA_n77A-n79A are already specified in the spec.

- CA_n1-n77-n79

- Reference

[1] 3GPP TR 21.905 Vocabulary for 3GPP Specifications (Release 17) V17.1.0

[5] TR38.718-03-01, Rel-18 NR inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL(x=1,2) V0.10.0

- Text Proposal

- -- Start of TP –

-- Unaffected parts omitted –

#### 5.x	CA_n1-n77-n79

## This proposal adds DL CA_n1A-n77(2A)-n79A with ULCA_n77(2A).

## 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n77-n79 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n1+n77+n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n77(2A)-n79A | CA_n1A-n77A CA_n1A-n79A CA_n77A-n79A CA_ n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

Since 3DL/1UL has already been specified, ∆TIB,c and ∆RIB,c values have also been specified.

- 5.x.2	Specific for 2 bands UL CA

- 5.x.2.1	UE co-existence studies

MSD analysis can be skipped. This is because the impact of ULCA CA_n77(2A) has already been investigated in fallback.

- 5.x.2.2	REFSENS requirements

Based on co-existence studies on 5.x.2.1, no need to define exceptional REFSENS requirements.

-- Unaffected parts omitted --

-- End of TP --