3GPP RAN WG4 Meeting #110bis		R4-2406701

Changsha, China, April 15th – 19th, 2024

Title:	WF on Corrections to UL harmonic MSD

Agenda item:	4.3

WI/SI:	TEI18

Release:	Rel-18

Source:	Skyworks Solutions Inc.

Document for:	Approval

# 1	Background

- This WF captures the improvements explained in [1] to the guidelines on MSD test points for UL harmonic interference MSD [2]. It should be noted that the changes proposed to the UL configuration are minor since they were already formulated in a similar way in [2]. These guidelines are aligned with the agreed principles for Rx Harmonic mixing of [4].

# Way Forward on Guidelines for MSD Test Points due to Harmonic Interference

## 2.1 Way Forward Proposal

Proposal 1: Adopt the changes highlighted in light blue to the guidelines [2].

Guidelines for the DL band configuration:

- For both Tx harmonic direct hit and for Rx harmonic mixing, a maximum of two test points can be specified [1]:

  - one test point to capture the lowest DL channel bandwidth (CBW) MSD,

  - one test point to capture the highest DL channel bandwidth (CBW) MSD. For this second test points, other DL configurations are not precluded.

- For near miss harmonic interference:

  - When the DL band may not be affected by a direct-hit collision, one near-miss MSD test point may be considered whenever applicable, and the MSD should be specified for the lowest DL band CBW.

  - When the DL band may be affected by an UL2/DL1 direct-hit collision, the near-miss MSD test point is not needed. Footnote 2 may be modified to inform RAN5 that the DL band may be affected by near-miss interference for which the MSD is not specified.

- The “lowest” and the “highest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

Guidelines for the UL band configuration.

For all MSD test points, adopt a unique and common UL:

- The UL band should be configured with the smallest UL CBW and the smallest SCS supported by the UL band, and with the UL RB allocation “Lcrb” and “RBstart” which ensures that the DL affected band RX CBW entirely overlaps the harmonic interference power spectral density (PSD) [3].

- This ensures that the UL harmonic interference level is agnostic of the affected DL band CBW.

- The “lowest” and the “highest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

Guidelines for the UL/DL harmonic order:

- The generic notation “ULn/DLm” [4] should be used, where “m” indicates the DL band harmonic order, “n” indicates the UL band harmonic order.

- For Tx harmonic interference: the collision type is specified either as “direct-hit” or as “near-miss”. For example, “UL2/DL1 direct-hit” is used to indicate a DL band MSD test point due to direct hit of the UL band 2nd harmonic,

- For Rx harmonic mixing: the collision type is not needed. For example, “UL3/DL2” is used to indicate a DL band MSD test point is due to 2nd harmonic Rx mixing with third UL Tx harmonic.

There is no need to specify the UL band and the DL band carrier-centre frequencies.

## 2.2 Example on handling near-miss test points for combinations affected by direct-hit interference.

The example of CA_n1-n77 is used in Table 1 below to illustrate in yellow the removal of un-necessary near-miss MSD test point in case the DL band may be affected by a direct-hit UL harmonic interference [1]. Table 1 also captures proposal 1 to add new text to footnote 2.

Table 1 Handling of near-miss MSD test points (Table 7.3A.4-1).

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order | Clean-up suggestion |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |  |
| n1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit | Keep 1st test point |
| n1 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit | - Keep this 2nd optional test point - Align the UL configuration with 1st test point |
| n1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss | Remove |
| NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. This DL band may be affected by near-miss interference for which the MSD is not specified. |  |  |  |  |  |  |  |  |  |

The corrections to the UL configuration of the 2nd MSD test points are highlighted in blue in in Table 1:

- The UL CBW should be changed to 5MHz, and Lcrb changed to Lcrb=25(RBstart=0).

## 2.3 Way Forward on Corrections to UL MSD

Proposal 2: Interested companies are invited to verify if MSD needs to be re-evaluated for band combinations where the UL configuration of the 2nd direct-hit MSD test point needs to be aligned with the UL configuration of the first direct-hit test point.

# References

R4-2405961 "Corrections to UL harmonic MSD", 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th 2024, Skyworks Solutions Inc.

R4-2210565 “WF on criteria on Rel-17 enhanced MSD table format”, 3GPP TSG-RAN WG4 Meeting #103-e, Electronic meeting, Skyworks Solutions, Inc.

R4-2107822, “WF for possible improvements on MSD in relation to BCS4”, 3GPP TSG-RAN WG4 Meeting # 99-e, Electronic Meeting, 19 – 27 May, 2021, Huawei, HiSilicon.

R4-2405453 “Harmonic Mixing clean-up”, 3GPP TSG RAN WG4 Meeting #110bis, Changsha, China, April 15th – 19th 2024, Qualcomm France.