3GPP TSG-RAN WG4 Meeting #110-bis	R4-2406703

Changsha, China, April 15 – April 19, 2024

Source:	Murata Manufacturing Co Ltd.

Title:	TP for TR 38.894: CA_n28A-n39A-n41A

Agenda item:			5.21.2

Document for:	Approval

Introduction

This contribution is a text proposal for TR 38.894 to update the requirements of CA_n28A-n39A-n41A.

Text Proposal

<Start of Text Proposal>

5.5	CA_n28A-n39A-n41A

5.5.1	Status of the band combination

5.5.1.1	Operating bands for CA

The operating bands for CA_n28-n39-n41 are specified in Table 5.5.1.1-1.

Table 5.5.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n28-n39-n41 | n28, n39, n41 |  |
| NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n28-n39-n41, the current spec only considers scenarios without simultaneous Rx/Tx for n39 and n41.

5.5.1.2	∆TIB and ∆RIB

The ΔTIB,c due to CA_n28-n39-n41 are specified in Table 5.5.1.2-1

Table 5.5.1.2-1: ΔTIB,c due to CA_n28-n39-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n28-n39-n41 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

The ΔRIB,c due to CA_n28-n39-n41 are specified in Table 5.5.1.2-2.

Table 5.5.1.2-2: ΔRIB,c due to CA_n28-n39-n41

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n28-n39-n41 | - | 0.2 | 0.2 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.5.1.3	Existing MSD requirement

Reference sensitivity exceptions due to intermodulation interference due to 2UL CA were specified for UL of n39 and n41 to DL of n28, without simultaneous Rx/Tx for n39 and n41.

Table 5.5.1.3-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n39-n41 | n28 | N/A | 5 | N/A | 762 | 29.3 | FDD | IMD2 |
|  | n39 | 1923 | 5 | 25 | 1923 | N/A | TDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |

5.5.2	MSD analysis for simultaneous Rx/Tx

5.5.2.1	Reference UE architecture

The following two reference architectures can be considered for CA_n28A-n39A-n41A for simultaneous Rx/Tx operation.

Figure 5.5.2.1-1: Reference architecture using shared antenna (R4-2400583, Murata)

Figure 5.5.2.1-2: Reference architecture using separate antenna (R4-2400583, Murata)

5.5.2.2	RF component assumptions

IMD calculation was performed based on the below linearity parameters, filter attenuation, diplexer isolation and PCB and antenna isolation (R4-2400583, Murata).

Table 5.5.2.2-1: Intercept point parameters

|  | IP2 | IP4 | IP5 |
| --- | --- | --- | --- |
| PA forward | 27 | 31 | 27 |
| PA reverse | 38 | 33 | 32 |
| Switch | 112 | 55 | 53 |
| Filter/Duplexer | 100 | 55 | 53 |
| Diplexer | 115 | 55 | 53 |
| LNA | 6 | -5 | -10 |

Table 5.5.2.2-2: Filter attenuation/isolation, PCB and antenna isolation

| n41 filter attenuation at n28 | 35 |
| --- | --- |
| n41 filter attenuation at n39 | 30 |
| n39 filter attenuation at n28 | 30 |
| n39 filter attenuation at n41 | 35 |
| n28 filter attenuation at n39 | 45 |
| n28 filter attenuation at n41 | 45 |
| n79 filter attenuation at n41 | 40 |
| Diplexer isolation | 20 |
| Antenna isolation | 15 |
| PCB isolation | 60 |

5.5.2.3	Calculated MSD values

Proposed Option 1: MSD was calculated as below for shared and separate antenna architecture respectively (R4-2400583, Murata).

Table 5.5.2.3-1: MSD due to 2UL IMD for simultaneous Rx/Tx using shared antenna structure (R4-2400583)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 32.6 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 32.6 | TDD | IMD2 |

Table 5.5.2.3-2: MSD due to 2UL IMD for simultaneous Rx/Tx using seperate antenna structure (R4-2400583)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 45.0 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 42.7 | TDD | IMD2 |

Table 5.5.2.3-3: MSD due to 2UL IMD for simultaneous Rx/Tx with averaging MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 42.2 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 40.1 | TDD | IMD2 |

Proposed Option 2: n39 MSD 32.2dB, n41 MSD 31.5dB (Qualcomm)

5.5.3	Requirements for simultaneous Rx/Tx

5.5.3.1	∆TIB and ∆RIB values

Table 5.5.3.1-1: ΔTIB,c due to CA_n28-n39-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n28-n39-n41 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.5.3.1-2: ΔRIB,c due to CA_n28-n39-n41

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n28-n39-n41 | - | 0.2 | 0.2 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.5.3.2	Reference sensitivity requirements

Table 5.5.3.2-1: The MSD value for CA_n28A-n39A-n41A with simultaneous Rx-Tx

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n28-n39-n41 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | N/A | 5 | N/A | 1895 | 39.6 | TDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n39 | 1985 | 5 | 25 | 1895 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2620 | 37.6 | TDD | IMD2 |

CA_n28A-n39A-n41A is only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx.

<End of Text Proposal>