3GPP TSG-RAN WG4 Meeting #110-bis	R4-2404116

Changsha, China, 15 – 19 April, 2024

# 3GPP TSG-RAN WG2 Meeting #125	R2-2401969

# Athens, Greece, 26th February – 1st March 2024

Title:	Reply LS on Rel-18 UL Tx switching for parallel switching on four bands

Response to:	R2-2311751/R4-2317609

Release:	Rel-18

Work Item:	NR_MC_enh-Core

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact Person:

#### Name:	Qianxi Lu

E-mail Address:	qianxi.lu@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN2 thanks RAN4 for the LS on UL Tx switching for parallel switching on four bands.

In RAN2#124 and RAN2#125 meeting, RAN2 discussed RAN4 LS. RAN2 realize RAN4 asked RAN2 for “Supporting the advanced capability of the switching period can be improved to min{max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}” and thus agreed on the solution that:

Rely on switchingAdditionalPeriodDualUL-r18 to report min{max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)}.

i.e., to address the intention to support the improved switching period by reporting the period length of min{max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} via switchingAdditionalPeriodDualUL-r18 (i.e., the field introduced for uplinkTxSwitching1T1Tto1T1T upon R4 request in LS R4-2310495).

RAN2 also realize RAN4 asked RAN2 to “Introduce optional per-BC UE capability to distinguish the case-1 and case-2 based on scheduled order of uplink grants and report the preferred case by UE as illustrated in the attachment[1]”:

On one hand, this agreement was made by assuming no need for the network to be aware of those switching cases based on the description in the RAN4 LS. Instead, it is sufficient to signal the improved switching period itself as mentioned above. Such solution also has the benefit that common capability signalling framework can be used for different use cases.

On the other hand, there is also concern raised in RAN2 on this solution. I.e., the RAN2 agreement only implements part of the technical solution illustrated in the attachment of the LS, i.e., the first key point of the three key points of the solution which are 1) UE indicating its advanced capability of this solution; 2) UE reports its preferred switching case; 3) the network configures which switching case applied. However, there is also concern raised on overlooking the other two key points in this solution.

Pending RAN4 response to the above concern, RAN2 will revisit the agreement if needed.

2. Actions:

To RAN4

ACTION: 	RAN2 respectfully asks RAN4 to take the above RAN2 agreements and discussion into account and feedback if any concern on the solution.

3. Date of Next TSG-RAN2 Meetings:

3GPP TSG RAN WG2#125bis	2024-04-15 ~ 2024-04-19			Changsha, CN

3GPP TSG RAN WG2#126	2024-05-20 ~ 2024-05-24			Fukuoka, JP