3GPP TSG-RAN WG4 Meeting #110bis	R4-2406579

Changsha, China, April 15 – 19, 2024

Title:	LS on 3Tx SAR solution for inter-band CA with PC1.5

Response to:

Release:	Rel-18

Work Item:	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: 	Hu Dan

E-mail Address: hudan11@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R4-2406580

1. Overall Description:

In current TS 38.101-1, for 3Tx inter-band UL CA in power class 1.5 with either UL MIMO or Tx diversity in one band, the UE capability maxUplinkDutyCycle-interBandCA-PC2 is re-used from power class 2 and applied as a duty threshold in the SAR solution in Rel-18 (latest update of PC1.5 SAR solution can be found in R4-2406580). From RAN4’s perspective, the reuse of the duty cycle value indicated by the capability is considered more convenient than introducing another capability, without causing technical issue in RAN4 specification.

- Question#1: From RAN2’s view, is there any technical problem of reusing the capability of maxUplinkDutyCycle-interBandCA-PC2 to the SAR solution in PC1.5?

In RAN4#110bis, it is pointed out that per TS 38.306, the expected UE behavior when the capability maxUplinkDutyCycle-interBandCA-PC2-r17 is absent is that the UE ‘shall work on power class 2’ by default. However, per TS 38.101-1, when the capability is absent, UE maximum transmit power will not be impacted by this maxUplinkDutyCycle-interBandCA-PC2-r17 IE. In this case, it is RAN4’s understanding the capability maxUplinkDutyCycle-interBandCA-PC2-r17 description in TS 38.306 may need to be updated.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-interBandCA-PC2-r17 Indicates the maximum average percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. The average percentage of uplink symbols is specified in 6.2A.1.3 in TS 38101-1[2] and the capability applies to the CA combinations listed in table 6.2A.1.3-1 in TS 38101-1[2]. If the field is absent, UE may use P-MPRc as defined in 6.2.4 in TS 38101-1[2] if necessary. Value n50 corresponds to 50%, value n60 corresponds to 60% and so on. NOTE: Specific targeted UL duty cycle percentage is not assumed if the field is absent. | BC | No | N/A | FR1 only |

Question#2: If there is no other technical problem in Q1, would RAN2 consider the approach above as a reference to update TS38.306?

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to assess Question#1, and update the capability of maxUplinkDutyCycle-interBandCA-PC2 to accommodate the SAR solution for 3Tx inter-band UL CA in power class 1.5, with either UL MIMO or Tx diversity in one band, if needed.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#111						20 - 24 May 2024							  Fukuoka, Japan

TSG-RAN4 Meeting#112  	 				19 – 23 August 2024				   Maastricht, Netherlands