# 3GPP TSG-RAN WG4 Meeting #111	R4-2409556

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source: 	Huawei, HiSilicon

Title: 		Further discussion on test conditions for NTN SAN (Ka band)

Agenda Item:	7.16.4

Document for:	Discussion

# Introduction

During RAN4#110bis meeting, WF on extreme conditions testing in [2] was approved. WF content was extracted below:

Way forward 1: Consider the following options (with other not precluded) for SAN extreme testing conditions decision next meeting: - Option 1: Do not consider extreme conditions testing; - Option 2: Consider to introduce new dedicated set of optional manufacturer declarations in TS 38.181, table 4.6-1, and Annex B as below (Track Changes): Condition; Minimum; Maximum. Barometric pressure; D.106; D.107. Relative humidity; D.108; D.109. Vibration; D.110; D.111. Additional conditions; D.112; D.112. NOTE: Additional extreme test conditions might be decided outside 3GPP, according to the specific mission requirements, together with operator and manufacturer customizations.. Way forward 2: If Option 2 in Way Forward 1 is agreed, agree that extreme conditions testing for NTN SAN is a in band-agnostic topic, i.e. not to re-open this discussion for new NTN frequency range or band(s), e.g. Ku, etc.

In this contribution, we provide further inputs to the discussion on extreme conditions testing for Ka band.

# Discussion

Normal as well as extreme test conditions were used for the BS testing in legacy specifications. However, discussion on NTN SAN testing conditions was not as obvious due to completely different NTN deployment scenarios. Therefore, in Rel-17 TS 38.181 SAN test specification, an informative annex B on Environmental requirements for the SAN equipment was captured. However, there is no explicit requirement captured in the normative part of TS 38.108, nor in 38.181 specification.

Observation 1: Extreme conditions testing is only implicitly captured in the annex of SAN test specification.

Furthermore, as there are not band-specific aspects related to Ka band testing, the existing text of TS 38.181 can be transparently reused for Rel-18 WI, and TS 38.181 v 18.1.0 already captures the same technical content as developed for Environmental requirements for SAN in Rel-17.

Observation 2: Rel-18 specification TS 38.181 already captures the same technical content as defined for Environmental requirements for SAN in Rel-17.

Regarding the WF content in [1], we provide the following feedback:

- Way forward 1, Option 1:

while the extreme conditions testing was introduced in Rel-17 WI, it would introduce another level of ambiguity if we now change the approach for Rel-18 NTN WI. Therefore, we suggest to keep Rel-17 approach and Rel-18 approach aligned for FR1-NTN and for FR2-NTN.

- Way forward 1, Option 2:

Modifications proposed in Option 2 should be considered for both Rel-17 and Rel-18 as package. There are not Ka band specific issues in that proposal. We suggest to discuss related improvements to the extreme conditions testing under Rel-17 Maintenance agenda.

- Way forward 2:

We do agree that extreme conditions testing for NTN SAN is a in band-agnostic topic.

Based on the above discussion, the following proposal was formulated:

Proposal 1: As both the Rel-17 and Rel-18 versions of TS 38.181 are already capturing the same approach to the Environmental requirements for SAN, there is no need for further updates related to Ka band extreme conditions testing.

Still, there seems to be ambiguity related to the completeness of the extreme conditions testing, therefore we suggest to handle it outside this WI:

Proposal 2: Any updates to the test conditions for NTN shall be aligned among FR1-NTN and for FR2-NTN, and covered under Rel-17 Maintenance agenda.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: As both the Rel-17 and Rel-18 versions of TS 38.181 are already capturing the same approach to the Environmental requirements for SAN, there is no need for further updates related to Ka band extreme conditions testing.

Proposal 2: Any updates to the test conditions for NTN shall be aligned among FR1-NTN and for FR2-NTN, and covered under Rel-17 Maintenance agenda.

# References

[1]		R4-2406002	Way Forward for [110bis][306] NR_NTN_enh_Part2 Extreme conditions and MU, RAN4#110bis