| 3GPP TR 38.899 V0.10.0 (2024-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power UE for FR1 NR inter-band CA/DC or NR SUL band combination with y (1<y<=6) bands DL and x (x=1, 2) bands UL and power class m (m<3) and high power on TDD band(s); (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	13

1	Scope	15

2	References	15

3	Definitions of terms, symbols and abbreviations	16

3.1	Terms	16

3.2	Symbols	16

3.3	Abbreviations	16

4	Background	16

4.1	TR Maintenance	17

5	High Power UE CA of 2 bands DL and 1 or 2 bands UL	17

5.x	CA_nX-nY	17

5.x.1	Configurations	17

5.x.2	Maximum output power	17

5.x.3	REFSENS requirements	17

5.x.4	∆TIB and ∆RIB values	18

5.1	CA_n25-n41C	18

5.1.1	Configurations	18

5.1.2	Maximum output power	18

5.1.3	REFSENS requirements	18

5.1.4	∆TIB and ∆RIB values	19

5.2	CA_n41C-n66	19

5.2.1	Configurations	19

5.2.2	Maximum output power	19

5.2.3	REFSENS requirements	20

5.2.4	∆TIB and ∆RIB values	20

5.3	CA_n41C-n71	20

5.3.1	Configurations	20

5.3.2	Maximum output power	21

5.3.3	REFSENS requirements	21

5.3.4	∆TIB and ∆RIB values	21

5.4	CA_n41C-n77A	21

5.4.1	Configurations	21

5.4.2	Maximum output power	22

5.4.3	REFSENS requirements	22

5.4.4	∆TIB and ∆RIB values	22

5.5	CA_n77-n79	23

5.5.1	Configurations	23

5.5.2	Maximum output power	23

5.5.3	REFSENS requirements	23

5.5.4	∆TIB and ∆RIB values	24

5.6	CA_n78-n79	25

5.6.1	Configurations	25

5.6.2	Maximum output power	25

5.6.3	REFSENS requirements	25

5.6.4	∆TIB and ∆RIB values	27

5.7	CA_n3-n41	27

5.7.1	Configurations	27

5.7.2	Maximum output power	28

5.7.3	REFSENS requirements	28

5.7.4	∆TIB and ∆RIB values	28

5.8	CA_n3-n77	28

5.8.1	Configurations	28

5.8.2	Maximum output power	29

5.8.3	REFSENS requirements	29

5.8.4	∆TIB and ∆RIB values	31

5.9	CA_n8-n78	31

5.9.1	Configurations	31

5.9.2	Maximum output power	31

5.9.3	REFSENS requirements	31

5.9.4	∆TIB and ∆RIB values	33

5.10	CA_n40-n77	33

5.10.1	Configurations	33

5.10.2	Maximum output power	33

5.10.3	REFSENS requirements	33

5.10.4	∆TIB and ∆RIB values	35

5.11	CA_n28-n77	35

5.11.1	Configurations	35

5.11.2	Maximum output power	36

5.11.3	REFSENS requirements	36

5.11.4	∆TIB and ∆RIB values	37

5.12	CA_n1-n41	37

5.12.1	Configurations	37

5.12.2	Maximum output power	38

5.12.3	REFSENS requirements	38

5.12.4	∆TIB and ∆RIB values	39

5.13	CA_n1-n77	39

5.13.1	Configurations	39

5.13.2	Maximum output power	39

5.13.3	REFSENS requirements	39

5.14	CA_n25-n78	41

5.14.1	Configurations	41

5.14.2	Maximum output power	41

5.14.3	REFSENS requirements	41

5.14.4	∆TIB and ∆RIB values	43

5.15	CA_n66-n78	43

5.15.1	Configurations	43

5.15.2	Maximum output power	44

5.15.3	REFSENS requirements	44

5.15.4	∆TIB and ∆RIB values	45

5.16	CA_n7-n77	45

5.16.1	Configurations	45

5.16.2	Maximum output power	45

5.16.3	REFSENS requirements	45

5.16.4	∆TIB and ∆RIB values	47

5.17	CA_n1-n79	48

5.17.1	Configurations	48

5.17.2	Maximum output power	48

5.17.3	REFSENS requirements	48

5.17.4	∆TIB and ∆RIB values	49

5.18	CA_n8-n79	49

5.18.1	Configurations	49

5.18.2	Maximum output power	49

5.18.3	REFSENS requirements	49

5.18.4	∆TIB and ∆RIB values	51

5.19	CA_n13-n77	51

5.19.1	Configurations	51

5.19.2	Maximum output power	51

5.19.3	REFSENS requirements	51

5.19.4	∆TIB and ∆RIB values	52

5.20	CA_n5-n77	52

5.20.1	Configurations	52

5.20.2	Maximum output power	52

5.20.3	REFSENS requirements	53

5.21	CA_n66-n77	53

5.21.1	Configurations	53

5.21.2	Maximum output power	54

5.21.3	REFSENS requirements	54

5.22	CA_n25-n77	55

5.22.1	Configurations	55

5.22.2	Maximum output power	56

5.22.3	REFSENS requirements	56

5.23	CA_n71-n78	56

5.23.1	Configurations	56

5.23.2	Maximum output power	57

5.23.3	REFSENS requirements	57

5.23.4	∆TIB and ∆RIB values	57

5.24	CA_n5-n78	57

5.24.1	Configurations	57

5.24.2	Maximum output power	58

5.24.3	REFSENS reqsuirements	58

5.25	DL CA_n1-n78, UL n78A	59

5.25.1	Configurations	59

5.25.2	Maximum output power	59

5.25.3	REFSENS requirements	59

5.25.4	∆TIB and ∆RIB values	61

5.26	DL CA_n3-n78, UL n78A	61

5.26.1	Configurations	61

5.26.2	Maximum output power	61

5.26.3	REFSENS requirements	62

5.26.4	∆TIB and ∆RIB values	63

5.27	DL CA_n28-n78, UL n78A	63

5.27.1	Configurations	63

5.27.2	Maximum output power	64

5.27.3	REFSENS requirements	64

5.27.4	∆TIB and ∆RIB values	65

5.28	CA_n8-n77	66

5.28.1	Configurations	66

5.28.2	Maximum output power	66

5.9.3	REFSENS requirements	66

5.28.4	∆TIB and ∆RIB values	67

5.29	CA_n3-n79	67

5.29.1	Configurations	67

5.29.2	Maximum output power	68

5.29.3	REFSENS requirements	68

5.29.4	∆TIB and ∆RIB values	68

5.30	CA_n28-n79	68

5.30.1	Configurations	68

5.30.2	Maximum output power	68

5.30.3	REFSENS requirements	69

5.30.4	∆TIB and ∆RIB values	69

5.31	CA_n41-n79	69

5.31.1	Configurations	69

5.31.2	Maximum output power	70

5.31.3	REFSENS requirements	70

5.31.4	∆TIB and ∆RIB values	70

5.32	CA_n18-n41	71

5.32.1	Configurations	71

5.32.2	Maximum output power	71

5.32.3	REFSENS requirements	71

5.32.4	∆TIB and ∆RIB values	72

5.33	CA_n18-n77	72

5.33.1	Configurations	72

5.33.2	Maximum output power	72

5.33.3	REFSENS requirements	72

5.34	CA_n28-n41	73

5.34.1	Configurations	73

5.34.2	Maximum output power	73

5.34.3	REFSENS requirements	74

5.34.4	∆TIB and ∆RIB values	74

5.35	CA_n7-n78	74

5.35.1	Configurations	74

5.35.2	Maximum output power	74

5.35.3	REFSENS requirements	75

5.35.4	∆TIB and ∆RIB values	76

5.36	CA_n71-n77	76

5.36.1	Configurations	76

5.36.2	Maximum output power	76

5.36.3	REFSENS requirements	76

5.36.4	∆TIB and ∆RIB values	77

5.37	DL CA_n77A-n85A, UL n77, UL CA_n77A-n85A	77

5.37.1	Configurations	77

5.37.2	Maximum output power	77

5.37.3	REFSENS requirements	77

5.37.4	Void	79

5.38	CA_n3A-n41C	79

5.38.1	Configurations	79

5.38.2	Maximum output power	79

5.38.3	REFSENS requirements	79

5.38.4	∆TIB and ∆RIB values	82

5.39	CA_n40A-n41A	82

5.39.1	Configurations	82

5.39.2	Maximum output power	82

5.39.3	REFSENS requirements	82

5.39.4	REFSENS requirements	83

5.40	CA_n41-n77, UL CA_n77(2A)	83

5.40.1	Configurations	83

5.40.2	Maximum output power	84

5.40.3	REFSENS requirements	84

5.40.4	∆TIB and ∆RIB values	85

5.41	CA_n40-n78	85

5.41.1	Configurations	85

5.41.2	Maximum output power	85

5.41.3	REFSENS requirements	86

5.41.4	∆TIB and ∆RIB values	87

5.42	CA_n39-n41	87

5.42.1	Configurations	87

5.42.2	Maximum output power	88

5.42.3	REFSENS requirements	88

5.42.4	∆TIB and ∆RIB values	92

5.43	CA_n28A-n40A	92

5.43.1	Configurations	92

5.43.2	Maximum output power	93

5.43.3	REFSENS requirements	93

5.43.4	∆TIB and ∆RIB values	94

5.44	CA_ n3A-n40A	94

5.44.1	UE maximum output power	94

5.44.2	Maximum output power	94

5.44.3	Reference sensitivity requirements	95

5.44.4	∆TIB and ∆RIB values	95

6	High Power UE CA of 3 bands DL and 1 or 2 bands UL	95

6.x	CA_nX-nY-nZ	95

6.x.1	Configurations	95

6.x.2	Maximum output power	96

6.x.3	REFSENS requirements	96

6.x.4	∆TIB and ∆RIB values	96

6.1 	DL CA_n25-n41-n66, UL CA_n25A-n41A	96

6.1.1	Configurations	96

6.1.2	Maximum output power	97

6.1.3	REFSENS requirements	97

6.1.4	Void	97

6.2	DL CA_n25-n41-n66, UL CA_n41A-n66A	97

6.2.1	Configurations	97

6.2.2	Maximum output power	98

6.2.3	REFSENS requirements	98

6.2.4	Void	99

6.3	DL CA_n25-n41-n71, UL CA_n25A-n41A	100

6.3.1	Configurations	100

6.3.2	Maximum output power	100

6.3.3	REFSENS requirements	100

6.3.4	Void	100

6.4	DL CA_n25-n41-n71, UL CA_n41A-n71A	101

6.4.1	Configurations	101

6.4.2	Maximum output power	101

6.4.3	REFSENS requirements	101

6.4.4	Void	102

6.5	DL CA_n25-n41-n71, UL CA_n41C	102

6.5.1	Configurations	102

6.5.2	Maximum output power	102

6.5.3	REFSENS requirements	102

6.5.4	Void	102

6.6	DL CA_n25-n41-n77, UL CA_n25A-n41A	102

6.6.1	Configurations	102

6.6.2	Maximum output power	103

6.6.3	REFSENS requirements	103

6.6.4	∆TIB and ∆RIB values	104

6.7	DL CA_n25-n41-n77, UL CA_n25A-n77A	104

6.7.1	Configurations	104

6.7.2	Maximum output power	105

6.7.3	REFSENS requirements	105

6.7.4	∆TIB and ∆RIB values	106

6.8	DL CA_n25-n41-n77, UL CA_n41A-n77A	106

6.8.1	Configurations	106

6.8.2	Maximum output power	106

6.8.3	REFSENS requirements	106

6.8.4	∆TIB and ∆RIB values	107

6.9	DL CA_n25-n66-n77, UL CA_n25A-n77A	108

6.9.1	Configurations	108

6.9.2	Maximum output power	108

6.9.3	REFSENS requirements	108

6.9.4	∆TIB and ∆RIB values	109

6.10	DL CA_n25-n66-n77, UL CA_n66A-n77A	109

6.10.1	Configurations	109

6.10.2	Maximum output power	110

6.10.3	REFSENS requirements	110

6.10.4	∆TIB and ∆RIB values	111

6.11	DL CA_n25-n71-n77, UL CA_n25A-n77A	111

6.11.1	Configurations	111

6.11.2	Maximum output power	112

6.11.3	REFSENS requirements	112

6.11.4	∆TIB and ∆RIB values	112

6.12	DL CA_n25-n71-n77, UL CA_n71A-n77A	112

6.12.1	Configurations	112

6.12.2	Maximum output power	112

6.12.3	REFSENS requirements	113

6.12.4	∆TIB and ∆RIB values	114

6.13	DL CA_n41-n66-n77, UL CA_n41A-n66A	114

6.13.1	Configurations	114

6.13.2	Maximum output power	114

6.13.3	REFSENS requirements	114

6.13.4	∆TIB and ∆RIB values	115

6.14	DL CA_n41-n66-n77, UL CA_n41A-n77A	115

6.14.1	Configurations	115

6.14.2	Maximum output power	116

6.14.3	REFSENS requirements	116

6.14.4	∆TIB and ∆RIB values	117

6.15	DL CA_n41-n66-n77, UL CA_n66A-n77A	117

6.15.1	Configurations	117

6.15.2	Maximum output power	117

6.15.3	REFSENS requirements	118

6.15.4	∆TIB and ∆RIB values	118

6.16	DL CA_n41-n71-n77, UL CA_n41A-n71A	119

6.16.1	Configurations	119

6.16.2	Maximum output power	119

6.16.3	REFSENS requirements	119

6.16.4	∆TIB and ∆RIB values	121

6.17	DL CA_n41-n71-n77, UL CA_n41A-n77A	121

6.17.1	Configurations	121

6.17.2	Maximum output power	121

6.17.3	REFSENS requirements	122

6.17.4	∆TIB and ∆RIB values	123

6.18	DL CA_n41-n71-n77, UL CA_n71A-n77A	123

6.18.1	Configurations	123

6.18.2	Maximum output power	123

6.18.3	REFSENS requirements	123

6.18.4	∆TIB and ∆RIB values	124

6.19	DL CA_n66-n71-n77, UL CA_n66A-n77A	125

6.19.1	Configurations	125

6.19.2	Maximum output power	125

6.19.3	REFSENS requirements	125

6.19.4	∆TIB and ∆RIB values	126

6.20	DL CA_n66-n71-n77, UL CA_n71A-n77A	126

6.20.1	Configurations	126

6.20.2	Maximum output power	127

6.20.3	REFSENS requirements	127

6.20.4	∆TIB and ∆RIB values	128

6.21	DL CA_n41-n66-n71, UL CA_n41A-n71A	128

6.21.1	Configurations	128

6.21.2	Maximum output power	128

6.21.3	REFSENS requirements	129

6.21.4	Void	129

6.22	DL CA_n41-n66-n71, UL CA_n41A-n71A	129

6.22.1	Configurations	129

6.22.2	Maximum output power	129

6.22.3	REFSENS requirements	130

6.22.4	Void	130

6.23	CA_n3-n28-n41	130

6.23.1	Configurations	130

6.23.2	Maximum output power	130

6.23.3	REFSENS requirements	130

6.24	CA_n3-n41-n77	131

6.24.1	Configurations	131

6.24.2	Maximum output power	131

6.24.3	REFSENS requirements	132

6.24.4	∆TIB and ∆RIB values	132

6.25	CA_n3-n28-n77	132

6.25.1	Configurations	132

6.25.2	Maximum output power	133

6.25.3	REFSENS requirements	133

6.25.4	∆TIB and ∆RIB values	133

6.26	CA_n28-n41-n77	134

6.26.1	Configurations	134

6.26.2	Maximum output power	134

6.26.3	REFSENS requirements	134

6.26.4	∆TIB and ∆RIB values	135

6.27	CA_n2-n48-n77	135

6.27.1	Configurations	135

6.27.2	Maximum output power	135

6.27.3	REFSENS requirements	136

6.27.4	Void	136

6.28	CA_n5-n48-n77	136

6.28.1	Configurations	136

6.28.2	Maximum output power	136

6.28.3	REFSENS requirements	136

6.28.4	Void	137

6.29	CA_n48-n66-n77	137

6.29.1	Configurations	137

6.29.2	Maximum output power	137

6.29.3	REFSENS requirements	137

6.29.4	Void	138

6.30	DL CA_n2-n5-n77, UL_ CA_n2A-n77A	138

6.30.1	Configurations	138

6.30.2	Maximum output power	138

6.30.3	REFSENS requirements	138

6.30.4	∆TIB and ∆RIB values	138

6.31	DL CA_n2-n5-n77, UL_CA_n5A-n77A	139

6.31.1	Configurations	139

6.31.2	Maximum output power	139

6.31.3	REFSENS requirements	139

6.31.4	∆TIB and ∆RIB values	139

6.32	CA_n1-n3-n41	139

6.32.1	Configurations	139

6.32.2	Maximum output power	140

6.32.3	REFSENS requirements	140

6.32.4	∆TIB and ∆RIB values	140

6.33	CA_n7-n66-n78	140

6.33.1	Configurations	140

6.33.2	Maximum output power	140

6.33.3	REFSENS requirements	141

6.33.4	∆TIB and ∆RIB values	141

6.34	CA_n25-n66-n78	141

6.34.1	Configurations	141

6.34.2	Maximum output power	142

6.34.3	REFSENS requirements	142

6.34.4	∆TIB and ∆RIB values	142

6.35	CA_n5-n7-n77	142

6.35.1	Configurations	142

6.35.2	Maximum output power	143

6.35.3	REFSENS requirements	143

6.35.4	∆TIB and ∆RIB values	144

6.36	CA_n7-n25-n77	145

6.36.1	Configurations	145

6.36.2	Maximum output power	146

6.36.3	REFSENS requirements	146

6.36.4	∆TIB and ∆RIB values	146

6.37	CA_n5-n25-n77	147

6.37.1	Configurations	147

6.37.2	Maximum output power	147

6.37.3	REFSENS requirements	147

6.37.4	∆TIB and ∆RIB values	148

6.38	CA_n5-n25-n78	148

6.38.1	Configurations	148

6.38.2	Maximum output power	148

6.38.3	REFSENS requirements	149

6.38.4	∆TIB and ∆RIB values	149

6.39	CA_n7-n66-n77	150

6.39.1	Configurations	150

6.39.2	Maximum output power	152

6.39.3	REFSENS requirements	152

6.39.4	∆TIB and ∆RIB values	152

6.40	CA_n7-n71-n77	153

6.40.1	Configurations	153

6.40.2	Maximum output power	153

6.40.3	REFSENS requirements	153

6.40.4	∆TIB and ∆RIB values	154

6.41	CA_n13-n25-n77	154

6.41.1	Configurations	154

6.41.2	Maximum output power	154

6.41.3	REFSENS requirements	155

6.41.4	∆TIB and ∆RIB values	155

6.42	CA_n1-n3-n77	155

6.42.1	Configurations	155

6.42.2	Maximum output power	156

6.42.3	REFSENS requirements	156

6.42.4	∆TIB and ∆RIB values	156

6.43	DL CA_n3-n77-n79, UL CA_n3A-n77A	156

6.43.1	Configurations	156

6.43.2	Maximum output power	157

6.43.3	REFSENS requirements	157

6.43.4	Void	157

6.44	DL CA_n3-n77-n79, UL CA_n3A-n79A	157

6.44.1	Configurations	157

6.44.2	Maximum output power	158

6.44.3	REFSENS requirements	158

6.44.4	Void	158

6.45	DL CA_n3-n77-n79, UL CA_n77A-n79A	158

6.45.1	Configurations	158

6.45.2	Maximum output power	158

6.45.3	REFSENS requirements	159

6.45.4	∆TIB and ∆RIB values	160

6.46	DL CA_n28-n77-n79, UL CA_n28A-n77A	160

6.46.1	Configurations	160

6.46.2	Maximum output power	160

6.46.3	REFSENS requirements	160

6.46.4	Void	161

6.47	DL CA_n28-n77-n79, UL CA_n28A-n79A	161

6.47.1	Configurations	161

6.47.2	Maximum output power	161

6.47.3	REFSENS requirements	161

6.47.4	Void	162

6.48	DL CA_n28-n77-n79, UL CA_n77A-n79A	162

6.48.1	Configurations	162

6.48.2	Maximum output power	162

6.48.3	REFSENS requirements	162

6.48.4	∆TIB and ∆RIB values	163

6.49	DL CA_n3-n28-n79, UL CA_n3A-n79A	164

6.49.1	Configurations	164

6.49.2	Maximum output power	164

6.49.3	REFSENS requirements	164

6.49.4	∆TIB and ∆RIB values	165

6.50	DL CA_n3-n28-n79, UL CA_n28A-n79A	165

6.50.1	Configurations	165

6.50.2	Maximum output power	166

6.50.3	REFSENS requirements	166

6.50.4	∆TIB and ∆RIB values	167

6.51	CA_n1-n28-n41	167

6.51.1	Configurations	167

6.51.2	Maximum output power	167

6.51.3	REFSENS requirements	167

6.51.4	∆TIB and ∆RIB values	168

6.52	CA_n18-n28-n77	168

6.52.1	Configurations	168

6.52.2	Maximum output power	168

6.52.3	REFSENS requirements	168

6.52.4	∆TIB and ∆RIB values	169

6.53	CA_n3-n18-n77	169

6.53.1	Configurations	169

6.53.2	Maximum output power	169

6.53.3	REFSENS requirements	170

6.53.4	∆TIB and ∆RIB values	170

6.54	CA_n18-n41-n77	170

6.54.1	Configurations	170

6.54.2	Maximum output power	171

6.54.3	REFSENS requirements	171

6.54.4	∆TIB and ∆RIB values	172

6.55	CA_n13-n66-n77	172

6.55.1	Configurations	172

6.55.2	Maximum output power	173

6.55.3	REFSENS requirements	173

6.55.4	∆TIB and ∆RIB values	174

6.56	CA_n1-n28-n77	174

6.56.1	Configurations	174

6.56.2	Maximum output power	174

6.56.3	REFSENS requirements	175

6.56.4	∆TIB and ∆RIB values	175

6.57	CA_n1-n41-n77	175

6.57.1	Configurations	175

6.57.2	Maximum output power	176

6.57.3	REFSENS requirements	176

6.57.4	∆TIB and ∆RIB values	176

6.58	CA_n7-n26-n78	177

6.58.1	Configurations	177

6.58.2	Maximum output power	177

6.58.3	REFSENS requirements	177

6.58.4	∆TIB and ∆RIB values	178

6.59	DL CA_n1-n3-n79, UL CA_n1A-n79A	178

6.59.1	Configurations	178

6.59.2	Maximum output power	179

6.59.3	REFSENS requirements	179

6.59.4	∆TIB and ∆RIB values	179

6.60	DL CA_n1-n3-n79, UL CA_n3A-n79A	179

6.60.1	Configurations	179

6.60.2	Maximum output power	179

6.60.3	REFSENS requirements	180

6.60.4	∆TIB and ∆RIB values	181

6.61	DL CA_n1-n28-n79, UL CA_n1A-n79A	181

6.61.1	Configurations	181

6.61.2	Maximum output power	181

6.61.3	REFSENS requirements	181

6.61.4	∆TIB and ∆RIB values	182

6.62	DL CA_n1-n28-n79, UL CA_n28A-n79A	183

6.62.1	Configurations	183

6.62.2	Maximum output power	183

6.62.3	REFSENS requirements	183

6.62.4	∆TIB and ∆RIB values	184

6.63	CA_n1-n3-n78	184

6.63.1	Configurations	184

6.63.2	Maximum output power	185

6.63.3	REFSENS requirements	185

6.63.4	∆TIB and ∆RIB values	185

6.64	CA_n1-n28-n78	185

6.64.1	Configurations	185

6.64.2	Maximum output power	186

6.64.3	REFSENS requirements	186

6.64.4	∆TIB and ∆RIB values	187

6.65	CA_n3-n7-n78	187

6.65.1	Configurations	187

6.65.2	Maximum output power	187

6.65.3	REFSENS requirements	188

6.65.4	∆TIB and ∆RIB values	188

6.66	CA_n3-n28-n78	189

6.66.1	Configurations	189

6.66.2	Maximum output power	189

6.66.3	REFSENS requirements	190

6.66.4	∆TIB and ∆RIB values	190

6.67	CA_n7-n28-n78	191

6.67.1	Configurations	191

6.67.2	Maximum output power	191

6.67.3	REFSENS requirements	191

6.67.4	∆TIB and ∆RIB values	191

6.68	CA_n1-n77-n79	192

6.68.1	Configurations	192

6.68.2	Maximum output power	192

6.68.3	REFSENS requirements	192

6.68.4	∆TIB and ∆RIB values	193

6.69	CA_n1-n-n78	193

6.69.1	Configurations	193

6.69.2	Maximum output power	194

6.69.3	REFSENS requirements	194

6.69.4	∆TIB and ∆RIB values	195

7	Power class 2 CA with SUL	195

7.x	CA_nW-nX_SUL_nY-nZ	195

7.x.1	Configurations	195

7.x.2	Maximum output power	195

7.x.3	REFSENS requirements	195

7.x.4	∆TIB and ∆RIB values	195

Annex <A> (informative): Change history	196

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for high power UE for NR inter-band Carrier Aggregation or Dual Connection and NR SUL band combinations with y (1<y<=6) bands downlink and x (x=1, 2) bands uplink under Rel-18 time-frame. The purpose is to gather the relevant background information and studies in order to complete the band-combination specific requirements for the newly requested band combinations for power class 2 and power class 1.5 UE. Table 1-1 lists the three types of band combinations covered by this TR.

Table 1-1 High power UE inter-band CA/DC and SUL band combinations

| # | Band combination | Power class for uplink |
| --- | --- | --- |
| 1 | Power class 2 NR Inter-band CA/DC for y bands DL with x bands UL (x=1, 2) | 1UL(TDD): PC2 on TDD band 2UL (FDD+TDD, TDD+FDD, TDD+TDD): PC3 on FDD band, PC2 or PC3 on TDD band |
| 2 | Power class 1.5 NR Inter-band CA for y bands DL with 1 band UL | 1UL (TDD): PC1.5 on TDD band |
| 3 | Power class 2 SUL band combinations with or without CA for y bands DL with 2 bands UL | SUL: PC3 on SUL band NUL(TDD): PC2 on TDD band NUL = Normal Uplink in contrast to SUL. |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-222623: “New WID: High power UE for FR1 NR inter-band CA/DC or NR SUL (supplementary uplink) band combination with y bands downlink (1<y<=6) and x bands uplink (x=1,2) and power class m (m<3) and high power on TDD band(s)”.

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TR 37.865-01-01: “NR Carrier Aggregation for intra-band (m Down Link (DL) / 1 Up Link (UL) bands) and inter-band (n Down Link (DL) / 1 Up Link (UL) bands)”

[5]	3GPP TR38.717-02-01 V17.0.0: “Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) (Release 17)”

[6]	3GPP TR38.716-02-00 V16.0.0: “NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) (Release 16)”

[7]	R4-2321924 AdHoc report [109][104] NR_Baskets_Part_1, RAN4#109

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

BS				Base Station

BCS				Bandwidth Combination Set

CA				Carrier Aggregation

CC				Component Carriers

DC				Dual Connectivity

DL				Downlink

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

REFSENS				Reference Sensitivity power level

SCS				Subcarrier spacing

TDD				Time Division Duplex

UE				User Equipment

UL				Uplink

# 4	Background

At 3GPP RAN4#97-e meeting, a basket Work Item on “High power UE for FR1 NR inter-band CA/DC or NR SUL (supplementary uplink) band combination with y bands downlink (1<y<=6) and x bands uplink (x=1,2) and power class m (m<3) and high power on TDD band(s)” [2] was approved for Rel-18. The objectives of the core part are as follows:

Specify the band-combination specific RF requirements for all listed band combinations as defined in attached excel file of this WI.

The requirements that need to analyse and specify include

-	Maximum output power.

-	Analysing combinations that have self-desensitization, applicable ∆TIB, c and ∆RIB, c and reference sensitivity exceptions including MSD test cases.

-	Other additional impact to the requirements due to the high power on UL, if necessary

The present document is a technical report for this basket Work Item.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

# 5	High Power UE CA of 2 bands DL and 1 or 2 bands UL

## 5.x	CA_nX-nY

### 5.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.1-1x unless additional clarification>

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY | nX8,9, nY8,9, CA_nX-nY8 | nX |  |  |
|  |  | nY |  |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.x.2	Maximum output power

<Editor’s note: In table 5.x.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

#### 5.x.3.1	Power class 2 case a

#### 5.x.3.2	Power class 2 case b

#### 5.x.3.3	Power class 2 case c

#### 5.x.3.4	Power class 1.5 for single uplink nX

#### 5.x.3.5	Power class 1.5 for single uplink nY

### 5.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

## 5.1	CA_n25-n41C

### 5.1.1	Configurations

Table 5.1.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25(2A)-n41C | n418, 9 CA_n25A-n41A8 CA_n41C8 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n25A-n41C | n418, 9 CA_n25A-n41A8 CA_n41C8 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.1.2	Maximum output power

Table 5.1.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.1.3	REFSENS requirements

#### 5.1.3.1	Power class 2 case a, b, c, d

The MSD for PC3 CA_n41C into n25:

Table 5.1.3-1 PC3 MSD

| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |

For PC2 there should be no difference because for these we have assumed that MPR is applied to meet the SEM mask and thus IMD3 is -13dBm, IMD5 is -25dBm for NS04 and -30dBm for NS01 and then we decay the power of the higher IMDs

The proposed value for PC2 UL CA MSD can be found in Table 5.1.3-2:

Table 5.1.3-2 PC2 MSD

| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |

### 5.1.4	∆TIB and ∆RIB values

Void

## 5.2	CA_n41C-n66

### 5.2.1	Configurations

Table 5.2.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n66A | n418, 9 n418, 9 CA_n41C8 CA_n41A-n66A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A) | n418, 9 CA_n41C8 CA_n41A-n66A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.2.2	Maximum output power

Table 5.2.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.2.3	REFSENS requirements

#### 5.2.3.1	Power class 2 case a, b, c, d

The MSD for PC3 CA_n41C into n66:

Table 5.2.3-1 PC3 MSD

| CA_n41-n66 | CA_n41C | n4112 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  |  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD | IMD5 |

For PC2 there should be no difference because for these we have assumed that MPR is applied to meet the SEM mask and thus IMD3 is -13dBm, IMD5 is -25dBm for NS04 and -30dBm for NS01 and then we decay the power of the higher IMDs

The proposed value for PC2 UL CA MSD can be found in Table 5.2.3-2:

Table 5.2.3-2 PC2 MSD

| CA_n41-n66 | CA_n41C | n4112 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |
|  |  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD |

### 5.2.4	∆TIB and ∆RIB values

Void

## 5.3	CA_n41C-n71

### 5.3.1	Configurations

Table 5.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n71A | n418, 9 CA_n41C8 CA_n41A-n71A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71(2A) | n418,9 CA_n41C8 CA_n41A-n71A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41C-n71B | n418,9 CA_n41C8 CA_n41A-n71A8 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

Note: DL CA_n41C-n71(2A) with UL CA_n41C is not in 38.101-1 17.7.0, but it was in agreed CR R4-2214650 From RAN4#104-e.

### 5.3.2	Maximum output power

Table 5.3.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.3.3	REFSENS requirements

#### 5.3.3.1	Power class 2 case a, b, c, d

For  PC3 CA_n41C there is no MSD into n71.

For  PC2 CA_n41C there should be no MSD into n71.

### 5.3.4	∆TIB and ∆RIB values

Void

## 5.4	CA_n41C-n77A

### 5.4.1	Configurations

Table 5.4.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41C-n77A | n418,9 n778,9 CA_n41A-n77A8 CA_n41C8 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.4.2	Maximum output power

Table 5.4.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.4.3	REFSENS requirements

#### 5.4.3.1	Power class 2 case a, b, c, d

The MSD for PC3 CA_n41C into n77:

Table 5.4.3-1 PC3 MSD

| CA_n41-n77 | n4112 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | FDD | IMD9 |

For PC2 there should be no difference because for these we have assumed that MPR is applied to meet the SEM mask and thus IMD3 is -13dBm, IMD5 is -25dBm for NS04 and -30dBm for NS01 and then we decay the power of the higher IMDs

The proposed value for PC2 UL CA MSD can be found in Table 5.4.3-2.

Table 5.4.3-2 PC2 MSD

| CA_n41-n77 | n4112 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | FDD | IMD9 |

### 5.4.4	∆TIB and ∆RIB values

Void

## 5.5	CA_n77-n79

### 5.5.1	Configurations

The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. These are shown in Table 5.2A.2.1-1 of TS 38.101-1 [3].

Table 5.5.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n77A-n79A | n77A8,9, n79A8,9 CA_n77A-n79A8 | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n77(2A)-n79A | n77A8,9, n79A8,9 CA_n77(2A)8, 12 CA_n77A-n79A8 | n77 | CA_n77(2A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n77(3A)-n79A | n77A8,9, n79A8,9 CA_n77(2A)8,12 CA_n77A-n79A8 | n77 | CA_n77(3A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination NOTE 12: UL configurations are for non simultaneous Rx/Tx operation. |  |  |  |  |

### 5.5.2	Maximum output power

Table 5.5.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n77-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.5.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink. For PC3 CA_n77-n79 with single uplink, the co-existence study is provided in TR 37.865-01-01 [4]. For PC2 CA_n77-n79 with UL CA_n77-n79, the co-existence study is provided in TR38.717-02-01[5]. For PC2 and PC1.5 CA_n77-n79 with single uplink, the co-existence study is provided in TR 37.865-01-01[4]. Analysis is based on these studies.

Additionally, analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n77(2A) UL for PC2.

#### 5.5.3.0	Power class 2 case a, b, c and d

Considering both n77 and n79 are TDD bands and synchronous operation for CA_n77-n79 is assumed, no MSD is needed for dual UL of CA_n77-n79.

CA_n77(2A) UL for CA_n77(2A)-n79A DL and CA_n77(3A)-n79A DL are already defined for PC3. PC2 is also defined for CA_n77(2A)-n79A DL and CA_n77(3A)-n79A DL but not yet for CA_n77(2A) UL. However, non-simultaneous Rx-Tx operation for CA_n77-n79 is assumed, therefore no MSD is needed for CA_n77(2A) UL.

#### 5.5.3.1	Power class 2 for single uplink n77

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

Therefore, there is no MSD issue for this CA configuration.

#### 5.5.3.2	Power class 2 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n77.

Therefore, there is no MSD issue for this CA configuration.

#### 5.5.3.3	Power class 1.5 for single uplink n77

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

Therefore, there is no MSD issue for this CA configuration.

#### 5.5.3.4	Power class 1.5 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n77.

Therefore, there is no MSD issue for this CA configuration.

### 5.5.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.6	CA_n78-n79

### 5.6.1	Configurations

For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. These are shown in Table 5.2A.2.1-1 of TS 38.101-1 [3].

Table 5.6.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n78A-n79A | n78A8,9, n79A8,9 n78A-n79A8 | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.6.2	Maximum output power

Table 5.6.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n78-n79 | CA power class | Carrier n78 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n78-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.6.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink. For PC3 CA_n78-n79 with single uplink, the co-existence study is provided in TR 37.865-01-01 [4]. In addition, MSD due to cross band isolation is needed for UEs supporting the inter-band carrier aggregation with simultaneous Rx/Tx capability.

#### 5.6.3.0	Power class 2 case a, b, c and d

#### Considering both n78 and n79 are TDD bands, no MSD is needed for dual UL of CA_n78-n79.5.6.3.1	Power class 2 for single uplink n78

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

For MSD due to cross band isolation, MSD value of PC2 case will be 3dB higher than that of PC3 case. New MSD values are shown in Table 5.6.3-1 below.

Table 5.6.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78X1 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 5 | >ACLR2 |
| n78X1 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 5 | >ACLR2 |
| NOTE X1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

#### 5.6.3.2	Power class 2 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n78.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n78.

For MSD due to cross band isolation, MSD value of PC2 case will be 3dB higher than that of PC3 case. New MSD values are shown in Table 5.6.3-2 below.

Table 5.6.3-2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n79 | n78X1 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 5.6 | >ACLR2 |
| n79 | n78X1 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 5.6 | >ACLR2 |
| NOTE X1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

#### 5.6.3.3	Power class 1.5 for single uplink n78

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n79.

For MSD due to cross band isolation, MSD value of PC1.5 case will be 6dB higher than that of PC3 case. New MSD values are shown in Table 5.6.3-3 below.

Table 5.6.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78X1 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 8 | >ACLR2 |
| n78X1 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4450 | 100 | 8 | >ACLR2 |
| NOTE X1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

#### 5.6.3.4	Power class 1.5 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n78.

-	the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n78.

For MSD due to cross band isolation, MSD value of PC2 case will be 3dB higher than that of PC3 case. New MSD values are shown in Table 5.6.3-2 below.

Table 5.6.3-4: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n79 | n78X1 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 8.6 | >ACLR2 |
| n79 | n78X1 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 8.6 | >ACLR2 |
| NOTE X1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

### 5.6.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.7	CA_n3-n41

### 5.7.1	Configurations

Table 5.7.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A | n418,9 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 3 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.7.2	Maximum output power

This band combination does not support high power UL CA, so this section is omitted.

### 5.7.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.7.3.1	Power class 1.5 for single uplink n41

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n3.

-		 the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n3.

-		MSD due to cross band isolation should be defined, PC2 	and PC3 of this band combination also suffers this kind of interference.

Table 5.7.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 3.9 | >ACLR2 |

### 5.7.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.8	CA_n3-n77

### 5.8.1	Configurations

Table 5.8.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A | n778,9 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n77(2A) | n778,9 CA_n77(2A)8 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n3A-n77(3A) | n778,9 CA_n77(2A)8 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.8.2	Maximum output power

Table 5.8.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.8.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.8.3.1	Power class 2 case a, b

For UL CA_n3-n77, based on calculation, IMD 2, 4, 5 of dual UL falls into n3 DL, the MSD exception is defined as below, the values are reused from CA_n3-n78.

Additionally, IMD 5, 6, 7 and 8 of CA_n77(2A) UL falls into n3 DL. And for this combination PC3 with CA_n77(2A) UL is specified, with MSD defined for IMD7. Possible problematic 2UL IMD cases are:

-  IMD5: There is no need to specify the MSD because for IMD5 to fall on n3 DL, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [3].

-  IMD6:  There is no need to specify the MSD because of the same reason.

-  IMD7: The MSD can be evaluated using the same test configuration as for PC3

-  IMD8. Because test points for IMD2 and IMD4 exist already, hence it is not recommended to add third test point for even order IMD.

As such, we recommend to specify IMD7 MSD using the same test configuration as was used for PC3. The MSD exception is defined as in Table below.

Table 5.8.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n774 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 | 1 (RBstart=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBstart=0) | 3945 |  |  |  |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

#### 5.8.3.2	Power class 2 for single uplink n77

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n3.

-		the 2th order harmonic mixing falls into Rx frequencies of n3.

Therefore, MSD issue due to harmonic mixing from PC2 n77 UL falling into n3 DL should be defined, the value is reused from CA_n3-n78.

Table 5.8.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 1 | NOTE 4 | UL1/DL2 |

#### 5.8.3.3	Power class 1.5 for single uplink n77

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n3.

-		the 2th order harmonic mixing falls into Rx frequencies of n3.

Therefore, MSD issue due to harmonic mixing from PC1.5 n77 UL falling into n3 DL should be defined.

Table 5.8.3.3-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 11.1 | NOTE 4 | UL1/DL2 |
| n77 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 4 | NOTE 4 | UL1/DL2 |

### 5.8.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.9	CA_n8-n78

### 5.9.1	Configurations

Table 5.9.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n78A | n788,9 CA_n8A-n78A8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.9.2	Maximum output power

Table 5.9.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n8-n78 | CA power class | Carrier n8 power class | Carrier n78 power class |
| --- | --- | --- | --- | --- |
| CA_n8-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.9.3	REFSENS requirements

For PC2 CA_n8-n78, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n8-n78, where:

4th harmonic frequency of band 8 UL may fall into band n78 DL Rx

4th IMD frequencies (i.e. 3*f8-fn78) may fall into band 8 DL Rx

4th Harmonic mixing issue, i.e. Band n78 UL may fall into 4th receiver harmonic mixing frequency of band n8

It shall be noted that the MSD values for the above two types of MSD have already been defined for PC3 CA_n8-n78 configuration in TS38.101-1 specification.

#### 5.9.3.1	Power class 2 case a

For PC2 case a, since the power configuration for each band is 23dBm, therefore:

For the harmonic issue, comparing with PC3 CA_n8A-n78A, no additional MSD are expected for this PC2 CA_n8A-n78A with 1 uplink carrier.

For the harmonic mixing issue, comparing with PC3 CA_n8A-n78A, no additional MSD are expected for this PC2 CA_n8A-n78A.

For the IMD issue, comparison of the NOTE 1 in Table 7.3A.5-1 (for PC3 2UL/2DL) and Table 7.3A.5-1a (for PC2 2UL/2DL) in TS38.101-1, it can be found that the set for both transmitters is changed from min(+20 dBm, PCMAX_L,f,c) to min(+23 dBm, PCMAX_L,f,c), which means the IMD4 MSD for PC2 2UL/2DL CA_n8A-n78A should be defined additionally.

The IMD4 MSD for PC2 2UL/2DL CA_n8A-n78A is proposed to be defined in table 5.9.3.1-1, by reusing the same IMD4 MSD of PC2 2UL/2DL DC_8A_n78A.

Table 5.9.3.1-1 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 15.5 | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |

#### 5.9.3.2	Power class 2 case b

For PC2 case b, similar with case a, no additional harmonic MSD are expected for this PC2 CA_n8A-n78A with 1 uplink carrier since the power of band n8 is kept as 23dBm.

For PC2 case b, the harmonic mixing issue, comparing with PC3 CA_n8A-n78A, additional MSD are expected for this PC2 CA_n8A-n78A since the power of band n78 is increased to 26dBm. The harmonic mixing MSD for PC2 CA_n8A-n78A is defined in table 5.9.3.2-1 by referring to the same MSD of PC2 CA_n5A-n77A.

Table 5.9.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 20 | 15 | 20 (RBstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Regarding the power configuration of 23dBm+26dBm in case b, due to the set for both transmitters for MSD calculation is the same, i.e. min(+23 dBm, PCMAX_L,f,c), as 23+23dBm power configuration in case a, so the IMD4 MSD value defined in table 5.9.3.1-1 can be applied.

#### 5.9.3.3	Power class 1.5 for single uplink n78

Based on above,

-    4th harmonic mixing fall into Rx frequencies of band n8.

Therefore, MSD issue due to harmonic mixing from PC1.5 n79 UL falling into n8 DL should be defined.

Table 5.9.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 10.5 | NOTE 5 | UL1/DL4 |
| n78 | n8 | 20 | 15 | 20 (RBstart=0) | 20 | 6.7 | NOTE 5 | UL1/DL4 |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

### 5.9.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n8A-n78A requirements are applied for PC2 CA_n8A-n78A.

## 5.10	CA_n40-n77

### 5.10.1	Configurations

Table 5.10.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n77A | n40A8,9 n77A8,9 CA_n40A-n77A8 | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.10.2	Maximum output power

Table 5.10.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n40-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.10.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.10.3.1	Power class 2 and 1.5 case a, b, c, d

Based on TS 38.101-1 Table 7.3A.4-1, there is no UL harmonic interference issue for this band combination.

Based on TS 38.101-1 Table 7.3A.4-4, there is harmonic mixing issue for n77 to n40 this is treated under the single band exceptions.

Based on TS 38.101-1 Table 7.3A.5-1, there is no IMD issues for this band combination.

Based on TS 38.101-1 Table 7.3A.6-1, there is cross band isolation issues for n77 to n40 this is treated under the single band exceptions.

#### 5.10.3.2	Power class 2 for single uplink n77

-		The 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n40.

-		 The 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n40.

-		n40 DL suffers UL2/DL3 harmonic mixing from n77.

-		 Cross band isolation of n77 UL falls into n40 DL.

Therefore, MSD for cross band isolation and harmonic mixing should be defined.

Table 5.10.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 6.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 1.2 | >ACLR2 |

Table 5.10.3.2-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 4.4 | NOTE 3 | UL2/DL3 |

#### 5.10.3.3	Power class 1.5 for single uplink n77

Similar as for PC2 MSD should be defined.

Table 5.10.3.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 9.0 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 2.2 | >ACLR2 |

Table 5.10.3.3-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 16.1 | NOTE 3 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 6.7 | NOTE 3 | UL2/DL3 |

#### 5.10.3.4	Power class 2 and 1.5 for single uplink n40

According to previous co-existence studies there are no issues.

### 5.10.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.11	CA_n28-n77

### 5.11.1	Configurations

Table 5.11.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A | n778,9 CA_n28A-n77A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n77(2A) | n778,9 CA_n77(2A)8 CA_n28A-n77A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n77(3A) | n778,9 CA_n77(2A) 8 CA_n28A-n77A8 | n28 | 5, 10 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.11.2	Maximum output power

Table 5.11.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY or (CA_nC) | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.11.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.11.3.1	Power class 2 case a, b

For UL CA_n28A-n77A, based on calculation, IMD5 of dual UL falls into n28 DL, the MSD exception is defined as below.

For UL CA_n77(2A), based on calculation, IMD 2, 4, 6, 7 and 8 of dual n77 UL falls into n28 DL. Possible problematic cases are

-  IMD2: For IMD2 to fall on n28, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [3].

-  IMD4

-  IMD6: Considered to be covered by lower even order case of IMD4. A note should be added to clarify that IMD6 is present but the corresponding MSD does not need to be specified (e.g. NOTE X as in Table below).

-  IMD7: For IMD7 to fall on n28, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [3].

-  IMD8: Considered to be covered by lower even order case of IMD4

The MSD exception is defined as below.

Table 5.11.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 | 19.2 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
|  | n28 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 18.5 | FDD | IMD4X |
|  | n7712 | 3510 | 10 | 1 RBSTART=25 | 3510 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 RBSTART=25 | 3900 | N/A | TDD | N/A |
| NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD2 MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD6 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.11.3.2	Power class 2 for single uplink n77

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

- the 5th order harmonic mixing falls into Rx frequencies of n28.

Therefore, MSD issue due to harmonic mixing from PC2 n77 UL falling into n28 DL should be defined, the value is reused from CA_n28-n78.

Table 5.11.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 11.7 | NOTE 1 | UL1/DL5 |

#### 5.11.3.3	Power class 1.5 for single uplink n77

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

- the 5th order harmonic mixing falls into Rx frequencies of n28.

Therefore, MSD issue due to harmonic mixing from PC1.5 n77 UL falling into n28 DL should be defined,.

Table 5.11.3.3-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n77 | n28 | 30 | 15 | 160 (RBstart=0) | 30 | 14.7 | NOTE 1 | UL1/DL5 |

### 5.11.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.12	CA_n1-n41

### 5.12.1	Configurations

Table 5.12.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n41A | n418,9 CA_n1A-n41A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.12.2	Maximum output power

Table 5.12.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.12.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.12.3.1	Power class 2 case a, b

Based on calculation, no IMD of dual uplink falls into n1 DL, the MSD exception is not needed.

#### 5.12.3.2	Power class 2 for single uplink n41

-    the harmonic mixing interference of n41 does not fall into Rx frequencies of n1.

-    Cross band isolation interference of PC2 n41 falls into n1

The PC2 MSD due to cross band isolation is specified in following table.

Table 5.12.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 20.8 | >ACLR2 |

#### 5.12.3.3	Power class 1.5 for single uplink n41

-    the harmonic mixing interference of n41 does not fall into Rx frequencies of n1.

-    Cross band isolation interference of PC1.5 n41 falls into n1

The PC1.5 MSD due to cross band isolation is specified in following table.

Table 5.12.3.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 23.5 | >ACLR2 |

### 5.12.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.13	CA_n1-n77

### 5.13.1	Configurations

Table 5.13.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n77A | n778,9 CA_n1A-n77A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n77(2A) | n778,9 CA_n1A-n77A8 CA_n77(2A)8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n1A-n77(3A) | n778,9 CA_n77(2A)8 | n77 | CA_n77(2A)_BCS0 | 0 |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.13.2	Maximum output power

Table 5.13.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.13.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

(The analysis for PC3 is addressed in R4-2313802)

#### 5.13.3.1	Power class 2 case a, b

For UL CA_n1A-n77A) based on calculation, IMD2/4/5 of dual UL falls into n1 DL, the MSD exception is defined as below, which is reused from DC_1_n77.

For UL CA_n77(2A), based on calculation, IMD 5, 6 and 7 of dual intra-band n77 UL falls into n1 DL. Possible problematic cases are:

- IMD5

-  IMD6: For IMD6 to fall on n1 DL, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [3].

-  IMD7: This is not an issue either since it can be covered by IMD5, the lowest odd order in this case. A note should be added to clarify that IMD7 is present but the corresponding MSD does not need to be specified, e.g. NOTE X as in Table below.

Table 5.13.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n774 | n1 | 1950 | 5 | 25 | 2140 | 35.8 | FDD | IMD2 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 17.8 | FDD | IMD4 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2130 | 31 | FDD | IMD5X |
|  | n7712 | 3310 | 10 | 1 RBSTART=25 | 3310 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 RBSTART=25 | 3900 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.13.3.2      Power class 2 for single uplink n77

-   No harmonic mixing of n77 falls into Rx frequencies of n1.

-   No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

#### 5.13.3.3	Power class 1.5 for single uplink n77

-   No harmonic mixing of n77 falls into Rx frequencies of n1.

-   No cross band isolation issue.

Therefore, no MSD exception needs to be defined.5.13.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.14	CA_n25-n78

### 5.14.1	Configurations

Table 5.14.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n78A | n788,9 CA_n25A-n78A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n78(2A) | n788,9 CA_n25A-n78A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n78(2A)8 | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.14.2	Maximum output power

Table 5.14.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.14.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.14.3.1	Power class 2 case a, b

Based on calculation, IMD2 and IMD5 of dual UL falls into n25 DL. PC2 MSD value is reused from CA_n2-n77.

Table 5.14.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.14.3.2	Power class 2 for single uplink n78

Table 5.14.3.2-1 lists up to 7th harmonics for n25A-n78A which shows that there are 2nd harmonics issues from UL n25 into DL n78.

Table 5.14.3.2-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 | 11100 | 11490 | 12950 | 13405 |
| n78 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 | 19800 | 22800 | 23100 | 26600 |

Table 5.14.3.2-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 5.14.3.2-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

No MSD issue due to harmonic or harmonic mixing from PC2 n78 UL need to be defined

#### 5.14.3.3	Power class 1.5 for single uplink n78

No MSD issue due to harmonic or harmonic mixing from PC1.5 n78 UL need to be defined

#### 5.14.3.4	Power class 2 for non-contiguous uplink n78

Table 5.14.3.4-1 lists up to 7th order IMD from UL CA_n78(2A) UE-to-UE coexistence analysis.

Table 5.14.3.4-1: CA_n78(2A) IMD products

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3780 | 3800 | 3780 | 3320 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Ranges | 20 | 480 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Ranges | 2820 | 4280 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Ranges | 40 | 960 | 6120 | 8080 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Ranges | 2340 | 4760 | 9420 | 11880 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Ranges | 60 | 1440 | 5640 | 8560 | 12720 | 15680 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Ranges | 1860 | 5240 | 8940 | 12360 | 16020 | 19480 |

As can be seen in the co-existence analysis above there are IMD7 products from CA_n78(2A) affecting band n25 DL. MSD value is reused from PC2 CA_n25-n77 and test points are reused from PC3 CA_n25-n78.

Table 5.14.3.4-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n78 | n25 | N/A | 5 | N/A | 1980 | 13.6 | FDD | IMD7 |
|  | n7812 | 3315 | 10 | 1 (RBSTART=7) | 3315 | N/A | TDD | N/A |
|  |  | 3760 | 10 | 1 (RBSTART=0) | 3760 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

### 5.14.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.15	CA_n66-n78

### 5.15.1	Configurations

Table 5.15.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n78A | n788,9 CA_n66A-n78A8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n78(2A) | n788,9 CA_n66A-n78A8 | n66 | 5, 10, 15, 20, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.15.2	Maximum output power

Table 5.15.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.15.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.15.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL falls into n66 DL. PC2 MSD value is reused from CA_n66-n77.

Table 5.15.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n78 | n66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

#### 5.15.3.2	Power class 2 for single uplink n78

Table 5.15.3.2-1 lists up to 7th harmonics for n66A-n78A which shows that there are 2nd harmonics issues from UL n66 into DL n78.

Table 5.15.3.2-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n66 | 1710 | 1780 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 | 8550 | 8900 | 10260 | 10680 | 11970 | 12460 |
| n78 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 | 19800 | 22800 | 23100 | 26600 |

Table 5.15.3.2-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 5.15.3.2-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

No MSD issue due to harmonic or harmonic mixing from PC2 n78 UL need to be defined

#### 5.15.3.3	Power class 1.5 for single uplink n78

No MSD issue due to harmonic or harmonic mixing from PC1.5 n78 UL need to be defined

### 5.15.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.16	CA_n7-n77

### 5.16.1	Configurations

Table 5.16.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n77A | n778, 9 CA_n7A-n77A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n77A | n778, 9 CA_n7A-n77A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n77(2A) | n778, 9 CA_n77(2A) 8 CA_n7A-n77A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n77(2A) | n778, 9 CA_n77(2A) 8 CA_n7A-n77A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n77(3A) | n778, 9 CA_n77(2A)8 CA_n7A-n77A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7(2A)-n77(3A) | n778, 9 CA_n77(2A)8 CA_n7A-n77A8 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.16.2	Maximum output power

Table 5.16.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.16.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.16.3.1	Power class 2 case a, b

Based on co-existence calculations, IMD4 of dual UL and IMD5 of non-contiguous UL falls into n7 DL.

The PC3 IMD4 MSD value for CA_n7-n77 is 7.1

The PC3 IMD5 MSD value from non-contiguous UL CA_n7-n77 is 15.

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD4 interference power increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed IMD4 and IMD5 MSD values for PC2 based on above calculations can be found in Table 5.16.3-1.

Table 5.16.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 18.2 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2687.5 | 29.9 | FDD | IMD5x |
|  | n7712 | 3455 | 10 | 1 (RBSTART=0) | 3455 | N/A | TDD | N/A |
|  |  | 3835 | 10 | 1 (RBSTART=7) | 3835 | N/A | TDD | N/A |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.16.3.2	Power class 2 for single uplink n77

Table 5.16.3.2-1 lists up to 7th harmonics for n7A-n77A which shows that there are no harmonics issues.

Table 5.16.3.2-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 | 15000 | 15420 | 17500 | 17990 |
| n77 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 | 19800 | 25200 | 23100 | 29400 |

Table 5.16.3.2-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 5.16.3.2-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

No MSD issue due to harmonic or harmonic mixing from PC2 n77 UL need to be defined.

The cross band isolation needed is defined in PC2 table below. Values from CA_n7-n78 is reused.

Table 5.16.3.2-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 6.5 | >ACLR2 |

#### 5.16.3.3	Power class 1.5 for single uplink n77

No MSD issue due to harmonic or harmonic mixing from PC1.5 n77 UL need to be defined.

The cross band isolation needed is defined in PC1.5 table below. Values from CA_n41-n77 is reused.

Table 5.16.3.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n77 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 9.0 | >ACLR2 |

### 5.16.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.17	CA_n1-n79

### 5.17.1	Configurations

Table 5.17.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n79A | n798, 9 CA_n1A-n79A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.17.2	Maximum output power

Table 5.17.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.17.3	REFSENS requirements

### Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.17.3.1	Power class 2 case a, b

For PC2 CA_n1-n79, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n1-n79.

Based on TR38.716-02-00[6] Table 6.6.1.3-1, there is no harmonic issue for the band combination of n1 and n79.

Based on TR38.716-02-00[6] Table 6.6.1.3-2, there is no harmonic mixing issue for the band combination of n1 and n79.

#### 5.17.3.2	Power class 2 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

#### Therefore, there is no MSD issue for this CA configuration.

#### 5.17.3.3	Power class 1.5 for single uplink n79

Based on above,

-	the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n79.

Therefore, there is no MSD issue for this CA configuration.

### 5.17.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n1A-n79A requirements are applied for PC2 CA_n1A-n79A and PC1.5 CA_n1A-n79A.

## 5.18	CA_n8-n79

### 5.18.1	Configurations

Table 5.18.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n79A | n798, 9 CA_n8A-n798 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 10, 20, 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.18.2	Maximum output power

Table 5.18.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n8-n79 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.18.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

For PC2 and PC1.5 CA_n8-n79, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n8-n79.

According to the study results in TR 37.865-01-01[4] Table 8.14.2-1,

-	5th order IMD may fall into Rx frequencies of band n8.

It shall be noted that the MSD values for the above two types of MSD have already been defined for PC3 CA_n8-n79 configuration in TS38.101-1[3] Table 7.3A.5-1.

#### 5.18.3.1	Power class 2 case a, b

For PC2 case a, since the power configuration for each band is 23dBm, therefore:

For the harmonic issue, comparing with PC3 CA_n8A-n79A, no additional MSD are expected for this PC2 CA_n8A-n79A with 1 uplink carrier.

For the harmonic mixing issue, comparing with PC3 CA_n8A-n79A, no additional MSD are expected for this PC2 CA_n8A-n79A.

For the IMD issue, comparison of the NOTE 1 in Table 7.3A.5-1 (for PC3 2UL/2DL) and Table 7.3A.5-1a (for PC2 2UL/2DL) in TS38.101-1, it can be found that the set for both transmitters is changed from min(+20 dBm, PCMAX_L,f,c) to min(+23 dBm, PCMAX_L,f,c), which means the IMD5 MSD for PC2 2UL/2DL CA_n8A-n79A should be defined additionally.

Table 5.18.3.1-1 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

|  | NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| CA_n8-n79 |  | n8 | 897.5 | 5 | 25 | 942.5 | 21.5 | FDD | IMD5 |
|  |  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |

Regarding the power configuration of 23dBm+26dBm in case b, due to the set for both transmitters for MSD calculation is the same, i.e. min(+23 dBm, PCMAX_L,f,c), as 23+23dBm power configuration in case a, so the IMD5 MSD value defined in table 5.18.3.1-1 can be applied.

#### 5.18.3.2	Power class 2 for single uplink n79

Based on above,

- 2nd, 3rd, 4th and 5th order harmonic do not fall into Rx frequencies of n8.

- 5th order harmonic mixing falls into Rx frequencies of n8.

Therefore MSD issue due to harmonic mixing from PC2 n79 UL falling into n8 DL should be defined.

Table 5.18.3.2-1 Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL bands for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 28.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

#### 5.18.3.2	Power class 1.5 for single uplink n79

Based on above,

- 2nd, 3rd, 4th and 5th order harmonic do not fall into Rx frequencies of n8.

- 5th order harmonic mixing falls into Rx frequencies of n8.

Therefore MSD issue due to harmonic mixing from PC2 n79 UL falling into n8 DL should be defined.

Table 5.18.3.2-1 Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL bands for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 31.0 | NOTE 1 | UL1/DL5 |
| NOTE 1: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

### 5.18.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n8A-n79A requirements are applied for PC2 CA_n8A-n79A and PC1.5 CA_n8A-n79A.

## 5.19	CA_n13-n77

### 5.19.1	Configurations

Table 5.19.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n13A-n77(2A) | CA_n77(2A)8 | n13 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.19.2	Maximum output power

Table 5.19.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY or (CA_nC) | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.19.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.19.3.1	Power class 2 case a, b

PC3 DL_n13A-n77(2A)_UL_n77(2A) is specified while IMD4/6 are missing. We submit a draft CR R4-2315469 to correct the error. For PC2 case, IMD 4/6 of dual n77 UL falls into n13 DL, which is the same as PC3. The MSD exception is defined as below.

Table 5.19.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n13-n77 | n13 | 781 | 5 | 25 | 750 | 18.5 | FDD | IMD414 |
|  | n7712 | 3510 | 10 | 1 RBSTART=25 | 3510 | N/A | TDD | N/A |
|  |  | 3885 | 10 | 1 RBSTART=25 | 3885 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 14: This band is subject to IMD6 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 5.19.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.20	CA_n5-n77

### 5.20.1	Configurations

To enable CA_n77(2A) for PC2, below yellow marked update to the existing 38.10-1 configuration tables is needed.

PC1.5 is already enabled for CA_n5A-n77A, and below yellow marked update also enable PC1.5 for these configurations.

Table 5.20.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n77(2A) | n778, 9 CA_n5A-n77A8 CA_n77(2A) 8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n5A-n77(3A) | n778, 9 CA_n77(2A) 8 CA_n5A-n77A 8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |

### 5.20.2	Maximum output power

Table 5.20.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.20.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n77(2A) UL for PC2.

#### 5.20.3.1	Power class 2 case a, b

CA_n77(2A) UL for CA_n5A-n77(2A) DL is already defined for PC3. PC2 is also defined for CA_n5A-n77(2A) DL but not yet for CA_n77(2A) UL.

PC2 test points for the now added CA_n77(2A) UL is reused from PC3, and the MSD value is reused from PC2 CA_n28-n77.

Table 5.20.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n774 | n5 | N/A | 5 | N/A | 880 | 18.5 | FDD | IMD4 |
|  | n7712 | 3410 | 10 | 1 RBSTART=25 | 3410 | N/A | TDD | N/A |
|  |  | 3850 | 10 | 1 RBSTART=25 | 3850 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

#### 5.20.3.2      Power class 2 for single uplink n77

This section is omitted since PC2 single uplink for CA_n5A-n77(2A) DL is already specified.

#### 5.20.3.3	Power class 1.5 for single uplink n77

This section is omitted since PC1.5 single uplink for CA_n5A-n77(2A) DL is already specified.

## 5.21	CA_n66-n77

### 5.21.1	Configurations

To enable CA_n77(2A) for PC2, below yellow marked update to the existing 38.10-1 configuration tables is needed.

PC1.5 is already enabled for CA_n66A-n77A, and below yellow marked update also enable PC1.5 for these configurations.

Table 5.21.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n77(2A) | n778,9 CA_n66A-n77A8 CA_n77(2A) 8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n66A-n77(3A) | n778,9 CA_n77(2A) 8 CA_n66A-n77A 8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |

### 5.21.2	Maximum output power

Table 5.21.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.21.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n77(2A) UL for PC2.

#### 5.21.3.1	Power class 2 case a, b

CA_n77(2A) UL for CA_n66A-n77(2A) DL is already defined for PC3. PC2 is also defined for CA_n66A-n77(2A) DL but not yet for CA_n77(2A) UL.

PC2 test points for the now added CA_n77(2A) UL is reused from PC3, and the MSD value is reused from PC2 CA_n1-n77.

Table 5.21.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n774 | n66 | N/A | 5 | N/A | 2197.5 | 31 | FDD | IMD5x |
|  | n7712 | 3305 | 10 | 1 (RBstart=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBstart=8) | 3855 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.21.3.2      Power class 2 for single uplink n77

This section is omitted since PC2 single uplink for CA_n66A-n77(2A) DL is already specified.

#### 5.21.3.3	Power class 1.5 for single uplink n77

This section is omitted since PC1.5 single uplink for CA_n66A-n77(2A) DL is already specified.

## 5.22	CA_n25-n77

### 5.22.1	Configurations

To enable CA_n77(2A) for PC2, below yellow marked update to the existing 38.10-1 configuration tables is needed.

PC1.5 is already enabled for CA_n25A-n77A, and below yellow marked update also enable PC1.5 for these configurations.

Table 5.22.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n77(2A) | n778,9 CA_n77(2A) 8 CA_n25A-n77A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n77(2A) | n778,9 CA_n77(2A) 8 CA_n25A-n77A8 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n77(3A) | n778,9 CA_n77(2A) 8 CA_n25A-n77A 8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |

### 5.22.2	Maximum output power

Table 5.22.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.22.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n77(2A) UL for PC2.

#### 5.22.3.1	Power class 2 case a, b

CA_n77(2A) UL for CA_n25A-n77(2A) DL is already defined for PC3. PC2 is also defined for CA_n25A-n77(2A) DL but not yet for CA_n77(2A) UL.

PC2 test points for the now added CA_n77(2A) UL is reused from PC3, and the MSD value is reused from PC2 CA_n3-n77.

Table 5.22.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n774 | n25 | N/A | 5 | N/A | 1987.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 RBSTART=10 | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 RBSTART=0 | 3945 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

#### 5.22.3.2      Power class 2 for single uplink n77

This section is omitted since PC2 single uplink for CA_n25A-n77(2A) DL is already specified.

#### 5.22.3.3	Power class 1.5 for single uplink n77

This section is omitted since PC1.5 single uplink for CA_n25A-n77(2A) DL is already specified.

## 5.23	CA_n71-n78

### 5.23.1	Configurations

Table 5.23.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n71A-n78A | n788,9 CA_n71A-n78A8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71A-n78(2A) | n788,9 CA_n71A-n78A8 | n71 | 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.23.2	Maximum output power

Table 5.23.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY or (CA_nC) | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.23.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.23.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n71-n78 falls into n71, the MSD values are defined as below.

Table 5.23.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 11.4 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |

#### 5.23.3.2	Power class 2 for single uplink n78

Based on calculation, there is no harmonic issues for Rx of n71.

#### Therefore, no MSD issue due to harmonic or harmonic mixing from PC2 n78 UL need to be defined.

#### 5.23.3.3	Power class 1.5 for single uplink n78

Based on calculation, there is no harmonic issues for Rx of n71.

Therefore, no MSD issue due to harmonic or harmonic mixing from PC1.5 n78 UL need to be defined.

### 5.23.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.24	CA_n5-n78

### 5.24.1	Configurations

To enable CA_n78(2A) for PC2, below yellow marked update to the existing 38.10-1 configuration tables is needed.

Table 5.24.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n78(2A) | n788, 9 CA_n5A-n78A8 CA_n78(2A) 8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |

### 5.24.2	Maximum output power

Table 5.24.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.24.3	REFSENS reqsuirements

Analysis of REFSENS exceptions or MSD requirements is needed due to now added CA_n78(2A) UL for PC2.

#### 5.24.3.1	Power class 2 case a, b

CA_n78(2A) UL for CA_n5A-n78(2A) DL is already defined for PC3. PC2 is also defined for CA_n5A-n78(2A) DL but not yet for CA_n78(2A) UL.

PC2 test points for the now added CA_n78(2A) UL is reused from PC3, and the MSD value is reused from PC2 CA_n28-n78.

Table 5.24.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n784 | n5 | N/A | 5 | N/A | 880 | 18.5 | FDD | IMD4 |
|  | n7812 | 3340 | 10 | 1 RBSTART=25 | 3340 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 RBSTART=25 | 3780 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

#### 5.24.3.2      Power class 2 for single uplink n78

This section is omitted since PC2 single uplink for CA_n5A-n78(2A) DL is already specified.

## 5.25	DL CA_n1-n78, UL n78A

### 5.25.1	Configurations

Table 5.25.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A | n788,9 CA_n1A-n78A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 3 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n78(2A) | n788,9 CA_n1A-n78A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  | n788,9 CA_n1A-n78A8 CA_n78(2A)8 | n1 | 5, 10, 15, 20 | 2 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.25.2	Maximum output power

Table 5.25.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n1-n78 | CA power class | Carrier n1 power class | Carrier n78 power class |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.25.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.25.3.1	Power class 2 case a

This PC2 band combination of CA_n1A-n78A is already defined in TS38.101-1, so this section is omitted.

#### 5.25.3.2	Power class 2 case b

This PC2 band combination of CA_n1A-n78A is already defined in TS38.101-1, so this section is omitted.

#### 5.25.3.3	Power class 2 for single uplink n78

This band combination is already defined in TS38.101-1, so this section is omitted.

#### 5.25.3.4	Power class 1.5 for single uplink n78

For PC3 CA_n1-n78 with single uplink, the co-existence study is provided in TR 38.716-02-00 [6].

Based on above, there is no harmonic issue for the band combination of n1 and n78.

Based on above, there is no harmonic mixing issue for the band combination of n1 and n78.

For PC1.5 CA_n1-n78 with single uplink, there is no harmonic and harmonic mixing issue.

#### 5.25.3.5	Power class 2 for non-contiguous uplink n78

Table 5.25.3.5-1 lists up to 7th order IMD from UL CA_n78(2A) UE-to-UE coexistence analysis.

Table 5.25.3.5-1: CA_n78(2A) IMD products

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3780 | 3800 | 3780 | 3320 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Ranges | 20 | 480 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Ranges | 2820 | 4280 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Ranges | 40 | 960 | 6120 | 8080 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Ranges | 2340 | 4760 | 9420 | 11880 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Ranges | 60 | 1440 | 5640 | 8560 | 12720 | 15680 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Ranges | 1860 | 5240 | 8940 | 12360 | 16020 | 19480 |

As can be seen in the co-existence analysis above there are IMD7 products from CA_n78(2A) affecting band n1 DL. MSD value is reused from PC2 CA_n25-n77.

Table 5.25.3.5-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 | N/A | 5 | N/A | 2140 | 13.6 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=0) | 3305 | N/A | TDD | N/A |
|  |  | 3675 | 10 | 1 (RBSTART=44) | 3675 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

### 5.25.4	∆TIB and ∆RIB values

There is no change by comparing to the value for PC3 CA.

## 5.26	DL CA_n3-n78, UL n78A

### 5.26.1	Configurations

Table 5.26.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n78A | n788,9 CA_n3A-n78A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n78(2A) | n788,9 CA_n3A-n78A8 CA_n78(2A)8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.26.2	Maximum output power

Table 5.26.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n78 | CA power class | Carrier n3 power class | Carrier n78 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n78A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.26.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.26.3.1	Power class 2 case a

This PC2 band combination of CA_n3A-n78A is already defined in TS38.101-1[3], so this section is omitted.

#### 5.26.3.2	Power class 2 case b

This PC2 band combination of CA_n3A-n78A is already defined in TS38.101-1[3], so this section is omitted.

#### 5.26.3.3	Power class 2 for single uplink n78

This band combination is already defined in TS38.101-1[3], so this section is omitted.

#### 5.26.3.4	Power class 1.5 for single uplink n78

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n78.

-		the 2nd order harmonic mixing falls into Rx frequencies of n3.

Therefore, MSD issue due to harmonic mixing from PC1.5 n78 UL falling into n3 DL should be defined, the value is reused from CA_n3-n77.

Table 5.26.3.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 11.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 4 | NOTE 4 | UL1/DL2 |

#### 5.26.3.5	Power class 2 for non-contiguous uplink n78

Table 5.26.3.5-1 lists up to 7th order IMD from UL CA_n78(2A) UE-to-UE coexistence analysis.

Table 5.26.3.5-1: CA_n78(2A) IMD products

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3780 | 3800 | 3780 | 3320 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Ranges | 20 | 480 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Ranges | 2820 | 4280 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Ranges | 40 | 960 | 6120 | 8080 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Ranges | 2340 | 4760 | 9420 | 11880 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Ranges | 60 | 1440 | 5640 | 8560 | 12720 | 15680 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Ranges | 1860 | 5240 | 8940 | 12360 | 16020 | 19480 |

As can be seen in the co-existence analysis above there are IMD7 products from CA_n78(2A) affecting band n3 DL. MSD value is reused from PC2 CA_n3-n77.

Table 5.26.3.5-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78 | n3 | N/A | 5 | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7812 | 3305 | 10 | 1 (RBSTART=3) | 3305 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 (RBSTART=0) | 3780 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

### 5.26.4	∆TIB and ∆RIB values

There is no change by comparing to the value for PC3 CA.

## 5.27	DL CA_n28-n78, UL n78A

### 5.27.1	Configurations

Table 5.27.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n78A | n788,9 CA_n28A-n78A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n78(2A) |  | n788,9 CA_n78(2A)8 CA_n28A-n78A8 |  | n28 | 5, 10, 15, 20 |  | 0 |
|  |  |  |  | n78 | CA_n78(2A)_BCS0 |  |  |
|  |  |  |  | n28 | 5, 10, 15, 20 |  | 1 |
|  |  |  |  | n78 | CA_n78(2A)_BCS2 |  |  |
|  |  |  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  |  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.27.2	Maximum output power

Table 5.27.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n78 | CA power class | Carrier n28 power class | Carrier n78 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n78A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.27.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.27.3.1	Power class 2 case a

This PC2 band combination of CA_n28A-n78A is already defined in TS38.101-1[3], so this section is omitted.

#### 5.27.3.2	Power class 2 case b

This PC2 band combination of CA_n28A-n78A is already defined in TS38.101-1[3], so this section is omitted.

#### 5.27.3.3	Power class 2 for single uplink n78

This band combination is already defined in TS38.101-1[3], so this section is omitted.

#### 5.27.3.4	Power class 1.5 for single uplink n78

-		the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n78.

-		the 5th order harmonic mixing falls into Rx frequencies of n28.

Therefore, MSD issue due to harmonic mixing from PC1.5 n78 UL falling into n28 DL should be defined, the value is reused from CA_n28-n77.

Table 5.27.3.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| n78 | n28 | 30 | 15 | 160 (Rbstart=0) | 30 | 14.7 | NOTE 1 | UL1/DL5 |

#### 5.27.3.5	Power class 2 for non-contiguous uplink n78

Table 5.27.3.5-1 lists up to 7th order IMD from UL CA_n78(2A) UE-to-UE coexistence analysis.

Table 5.27.3.5-1: CA_n78(2A) IMD products

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3780 | 3800 | 3780 | 3320 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Ranges | 20 | 480 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Ranges | 2820 | 4280 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Ranges | 40 | 960 | 6120 | 8080 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Ranges | 2340 | 4760 | 9420 | 11880 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Ranges | 60 | 1440 | 5640 | 8560 | 12720 | 15680 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Ranges | 1860 | 5240 | 8940 | 12360 | 16020 | 19480 |

As can be seen in the co-existence analysis above there are IMD4 products from CA_n78(2A) affecting band n28 DL. MSD value is reused from PC2 CA_n28-n77.

Table 5.27.3.5-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n78 | n28 | N/A | 5 | N/A | 780 | 18.5 | FDD | IMD414 |
|  | n7812 | 3310 | 10 | 1 (RBSTART=7) | 3310 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=0) | 3700 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 14: This band is subject to IMD6 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 5.27.4	∆TIB and ∆RIB values

There is no change by comparing to the value for PC3 CA.

## 5.28	CA_n8-n77

### 5.28.1	Configurations

Table 5.28.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n77A | n778,9 CA_n8A-n77A8 | n8 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.28.2	Maximum output power

Table 5.28.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n8-n77 | CA power class | Carrier n8 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n8A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.9.3	REFSENS requirements

For PC2 CA_n8-n77, the co-existence studies on the harmonic issue and intermodulation issues are the same with the PC3 CA_n8-n77, where:

4th harmonic frequency of band 8 UL may fall into band n77 DL Rx

4th IMD frequencies (i.e. 3*f8-fn77) may fall into band 8 DL Rx

4th Harmonic mixing issue, i.e. Band n77 UL may fall into 4th receiver harmonic mixing frequency of band n8

It shall be noted that the MSD values for the above two types of MSD have already been defined for PC3 CA_n8-n77 configuration in TS38.101-1[3] specification.

#### 5.28.3.1	Power class 2 case a

For PC2 case a, since the power configuration for each band is 23dBm, therefore:

For the harmonic issue, comparing with PC3 CA_n8A-n77A, no additional MSD are expected for this PC2 CA_n8A-n77A with 1 uplink carrier.

For the harmonic mixing issue, comparing with PC3 CA_n8A-n77A, no additional MSD are expected for this PC2 CA_n8A-n77A.

For the IMD issue, comparison of the NOTE 1 in Table 7.3A.5-1 (for PC3 2UL/2DL) and Table 7.3A.5-1a (for PC2 2UL/2DL) in TS38.101-1[3], it can be found that the set for both transmitters is changed from min(+20 dBm, PCMAX_L,f,c) to min(+23 dBm, PCMAX_L,f,c), which means the IMD4 MSD for PC2 2UL/2DL CA_n8A-n77A should be defined additionally.

The IMD4 MSD for PC2 2UL/2DL CA_n8A-n77A is proposed to be defined in table 5.28.3.1-1, by reusing the same IMD4 MSD of PC2 2UL/2DL CA_n8A_n78A.

Table 5.28.3.1-1 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n8-n77 | n8 | 897.5 | 5 | 25 | 942.5 | 15.5 | IMD4 |
|  | n77 | 3635 | 10 | 50 | 3635 | N/A | N/A |

#### 5.28.3.2	Power class 2 case b

For PC2 case b, similar with case a, no additional harmonic MSD are expected for this PC2 CA_n8A-n77A with 1 uplink carrier since the power of band n8 is kept as 23dBm.

For PC2 case b, the harmonic mixing issue, comparing with PC3 CA_n8A-n77A, additional MSD are expected for this PC2 CA_n8A-n77A since the power of band n77 is increased to 26dBm. The harmonic mixing MSD for PC2 CA_n8A-n77A is defined in table 5.28.3.2-1 by referring to the same MSD of PC2 CA_n8A-n78A.

Table 5.28.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n8 | 10 | 15 | 25 (Rbstart=0) | 5 | 8.1 | NOTE 5 | UL1/DL4 |
| n77 | n8 | 20 | 15 | 20 (Rbstart=0) | 20 | 4.3 | NOTE 5 | UL1/DL4 |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Regarding the power configuration of 23dBm+26dBm in case b, due to the set for both transmitters for MSD calculation is the same, i.e. min(+23 dBm, PCMAX_L,f,c), as 23+23dBm power configuration in case a, so the IMD4 MSD value defined in table 5.28.3.1-1 can be applied.

### 5.28.4	∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n8A-n77A requirements are applied for PC2 CA_n8A-n77A.

## 5.29	CA_n3-n79

### 5.29.1	Configurations

Table 5.29.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n79A | N798,9 CA_n3A-n79A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 10, 20, 40, 50, 60, 80, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.29.2	Maximum output power

This band combination does not support high power UL CA, so this section is omitted.

### 5.29.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.29.3.1	Power class 2 case a, b

This band combination does not support high power UL CA, so this section is omitted.

#### 5.29.3.2	Power class 2 for single uplink n79

This band combination does not support high power UL CA, so this section is omitted.

#### 5.29.3.3	Power class 1.5 for single uplink n79

Based on MSD analysis for PC3, there’re no harmonic/harmonic mixing/cross-band isolation issues. Therefore, no MSD exception needs to be defined.

### 5.29.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.30	CA_n28-n79

### 5.30.1	Configurations

Table 5.30.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | n798, 9 CA_n28A-n79A8 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.30.2	Maximum output power

Table 5.30.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n79 | CA power class | Carrier n28 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.30.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.30.3.1	Power class 2 case a, b

For PC2 CA_n28-n79, the co-existence studies on the harmonic issue and intermodulation issues have already been defined in TS38.101-1[3] specification.

#### 5.30.3.2	Power class 2 for single uplink n79

For PC2 CA_n28-n79 for single uplink n79, the co-existence studies on the harmonic issue and intermodulation issues have already been defined in TS38.101-1[3] specification.

#### 5.30.3.3	Power class 1.5 for single uplink n79

For PC1.5 CA_n28-n79 for single uplink n79,

- the 2nd, 3rd, 4th and 5th order harmonic do not fall into Rx frequencies of n28.

- the 2nd, 3rd, 4th and 5th order harmonic mixing do not fall into Rx frequencies of n28.

Therefore, no MSD exception needs to be defined.

### 5.30.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.31	CA_n41-n79

### 5.31.1	Configurations

Table 5.31.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n79A | n418,9 n798,9 CA_n41A-n79A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90,100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 2 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 5.31.2	Maximum output power

Table 5.31.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n41-n79 | CA power class | Carrier n41 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.31.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.31.3.1	Power class 2 case a, b, c, d

Based on TS38.101-1[3] Table 7.3A.5-1 and Table 7.3A.5-1b, there is no harmonic issue for this band combination of n41 and n79.

Based on the same tables, there is no harmonic mixing issue for this band combination of n41 and n79.

#### 5.31.3.2	Power class 2 for single uplink n79

For PC2 CA_n41-n79 for single uplink n79 is already defined in TS38.101-1[3].

So, this section is omitted.

#### 5.31.3.3	Power class 1.5 for single uplink n79

Based on above,

- No harmonic mixing issue.

- No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

#### 5.31.3.4	Power class 2 for single uplink n41

For PC2 CA_n41-n79 for single uplink n41 is already defined in TS38.101-1[3].

So, this section is omitted.

#### 5.31.3.5	Power class 1.5 for single uplink n41

Based on above,

- No harmonic mixing issue.

- No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

### 5.31.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.32	CA_n18-n41

### 5.32.1	Configurations

Table 5.32.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n41A | n41A 8 CA_n18A-n41A 8 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.32.2	Maximum output power

Table 5.32.2-1: UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.32.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink inter-band CA.

- the 3rd order harmonic mixing may fall into Rx frequencies of n18.

- IMD3 of inter-band UL CA, CA_n18A-n41A, may fall into Rx frequencies of n18.

- No crossband isolation interference.

#### 5.32.3.1	Power class 2 case a, b

MSD due to IMD3 of inter-band UL CA to downlink n18 is defined as below.

Table 5.32.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n41 | n18 | 820 | 5 | 25 | 865 | 24.6 | FDD | IMD3 |
|  | n41 | 2505 | 5 | 25 | 2505 | N/A | TDD | N/A |

#### 5.32.3.2	Power class 2 for single uplink n41

MSD due to harmonic mixing is defined as below.

Table 5.32.3-2: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n18X | 5 | 15 | 25 (RBstart=0) | 5 | 27.3 | NOTE Y | UL1/DL3 |
| n41 | n18X | 10 | 15 | 25 (RBstart=0) | 5 | 27.3 | NOTE Y | UL1/DL3 |
| NOTE X: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE Y: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

### 5.32.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

## 5.33	CA_n18-n77

### 5.33.1	Configurations

Table 5.33.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n77A | CA_n18A-n77A 8 | n18 | 5, 10, 15 | 0 |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.33.2	Maximum output power

Table 5.33.2-1: UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

Unaffected parts are omitted.

### 5.33.3	REFSENS requirements

Comparing the 3 results, for both IMD4 and IMD5, the biggest difference among the corresponding MSD values are 2.3dB for IMD4 and 2.5dB for IMD5, hence the data from 3 companies are quite consistent. The test points are the same.  During RAN4#109 meeting in Chicago, experts from the 3 Companies agree to average the corresponding results to the value for PC2 [7] as shown in Table 5.33.3.1

Table 5.33.3-1. MSD values for PC2 HPUE CA_n18-n77

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR-CA Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n18-n77 | n18 | 827.5 | 5 | 25 | 872.5 | 17.5 | IMD4 X |
|  | n77 | 3355 | 10 | 50 | 3355 | N/A | N/A |
|  | n18 | 817.5 | 5 | 25 | 862.5 | 10.5 | IMD5 X |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | N/A |
| NOTE X: In Japan, n77 band is restricted to 3400 – 4100 MHz frequency range, and there are no valid MSD test points when using this restricted frequency range. |  |  |  |  |  |  |  |

## 5.34	CA_n28-n41

### 5.34.1	Configurations

Table 5.34.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41A | n418,9 CA_n28A-n41A8, 13 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 30 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 5.34.2	Maximum output power

Table 5.34.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n41 | CA power class | Carrier n28 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.34.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.34.3.1	Power class 2 case a, b

Based on calculation of previous release, no IMD of dual uplink falls into n28 DL, the MSD exception is not needed.

#### 5.34.3.3	Power class 2 for single uplink n41

Based on calculation of previous release,

No harmonic mixing of n41 fall into Rx frequencies of n28.

No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

#### 5.34.3.4	Power class 1.5 for single uplink n41

Based on above,

No harmonic mixing of n41 fall into Rx frequencies of n28.

No cross band isolation issue.

Therefore, no MSD exception needs to be defined.

### 5.34.4	∆TIB and ∆RIB values

There is no change by compairing to the values for PC3 CA.

## 5.35	CA_n7-n78

### 5.35.1	Configurations

Table 5.35.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| CA_n7A-n78(2A) | n788,9 CA_n7A-n78A8 CA_n78(2A) 8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.35.2	Maximum output power

Already defined.

### 5.35.3	REFSENS requirements

Table 5.35.2.1-1 lists up to 7th order IMD from UL CA_n78(2A) UE-to-UE coexistence analysis

Table 5.35.2.1-1: CA_n78(2A) IMD products

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3780 | 3800 | 3780 | 3320 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Ranges | 20 | 480 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Ranges | 2820 | 4280 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Ranges | 40 | 960 | 6120 | 8080 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Ranges | 2340 | 4760 | 9420 | 11880 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Ranges | 60 | 1440 | 5640 | 8560 | 12720 | 15680 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Ranges | 1860 | 5240 | 8940 | 12360 | 16020 | 19480 |

As can be seen in the co-existence analysis above there are IMD5 and IMD7 products from CA_n78(2A) affecting band n7 DL. MSD value and is reused from CA_n7-n77 and test points are reused from CA_n7-n78 PC3.

Table 5.35.2.1-2: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n78 | n7 | N/A | 5 | N/A | 2650 | 29.9 | FDD | IMD515 |
|  | n7812 | 3350 | 10 | 1 (RBSTART=25) | 3350 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 (RBSTART=25) | 3700 | N/A | TDD | N/A |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 15: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.35.3.2	Power class 2 for single uplink n78

Already defined.

#### 5.35.3.3	Power class 1.5 for single uplink n78

Already defined.

### 5.35.4	∆TIB and ∆RIB values

Already defined.

## 5.36	CA_n71-n77

### 5.36.1	Configurations

Table 5.36.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n71A-n77(2A) | CA_n77(2A)8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71A-n77(3A) | CA_n77(2A)8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

### 5.36.2	Maximum output power

Table 5.36.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY or (CA_nC) | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n77(2A) | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.36.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.36.3.1	Power class 2 case a, b

Based on calculation, IMD4/6 of dual UL CA_n77(2A) falls into n71 DL, the MSD values are defined as below.

Table 5.36.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71-n77 | n71 | N/A | 5 | N/A | 640 | 18.5 | FDD | IMD414 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 14: This band is subject to IMD6 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 5.36.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.37	DL CA_n77A-n85A, UL n77, UL CA_n77A-n85A

### 5.37.1	Configurations

Table 5.37.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n77A-n85A | n778,9 CA_n77A-n85A8 | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.37.2	Maximum output power

Table 5.37.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n77A-n85A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 26dBm | 23dBm |

### 5.37.3	REFSENS requirements

#### 5.37.3.1	Power class 2 case a

Since there is intermodulation MSD for PC2 CA_n12A-n77A, it is proposed that the same MSD apply for PC2 CA_n77A-n85A.

Table 5.37.3.1-1 Intermodulation MSD for PC2 CA_n12-n77

| CA_n12-n77 | 12 | 702 | 5 | 20 | 732 | 11.7 | FDD | IMD5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |

Table 5.37.3.1-2 Proposed Intermodulation MSD for PC2 CA_n77-n85

| CA_n77-n85 | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n85 | 702 | 5 | 20 | 732 | 11.7 | FDD | IMD5 |

#### 5.37.3.2	Power class 2 case b

The same MSD applies as proposed in Table 5.37.3-2.

#### 5.37.3.3	Power class 2 case c

#### N/A

#### 5.37.3.4	Power class 2 for single uplink n77

Table 5.37.3.4-1 Harmonic Mixing MSD for PC2 n77 with DL CA_n12-n77

| n77 | n12 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | n12 | 15 | 15 | 75 (RBstart=0) | 15 | 29.2 | NOTE 1 | UL1/DL5 |

Table 5.37.3.4-2 Proposed Harmonic Mixing MSD for PC2 n77 with DL CA_n77-n85

| n77 | n85 | 10 | 15 | 25 (RBstart=0) | 5 | 34 | NOTE 1 | UL1/DL5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | n85 | 15 | 15 | 75 (RBstart=0) | 15 | 29.2 | NOTE 1 | UL1/DL5 |

There is no cross-band isolation MSD for PC2 n77 into n12, so we propose no cross-band isolation MSD for PC2 n77 into n85.

#### 5.37.3.5	Power class 1.5 for single uplink n77

Table 5.37.3.4-1 Harmonic Mixing MSD for PC1.5 n77 with DL CA_n12-n77

| n77 | n12 | 10 | 15 | 25 (RBstart=0) | 5 | 37 | NOTE 1 | UL1/DL5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | n12 | 15 | 15 | 75 (RBstart=0) | 15 | 32.2 | NOTE 1 | UL1/DL5 |

Table 5.37.3.4-2 Proposed Harmonic Mixing MSD for PC1.5 n77 with DL CA_n77-n85

| n77 | n85 | 10 | 15 | 25 (RBstart=0) | 5 | 37 | NOTE 1 | UL1/DL5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | n85 | 15 | 15 | 75 (RBstart=0) | 15 | 32.2 | NOTE 1 | UL1/DL5 |

There is no cross-band isolation MSD for PC1.5 n77 into n12, so we propose no cross-band isolation MSD for PC1.5 n77 into n85.

### 5.37.4	Void

## 5.38	CA_n3A-n41C

### 5.38.1	Configurations

Table 5.38.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A | n418,9 CA_n3A-n41A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 3 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41B | CA_n3A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
| CA_n3A-n41C | n418 CA_n41C8 CA_n3A-n41A8 CA_n3A-n41C8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

PC2 and PC1.5 for n41 have been supported for CA_n3A-n41A. Hence, no new MSD analysis is needed for n41 single UL for CA_n3A_n41C.

### 5.38.2	Maximum output power

Table 5.38.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n41 | CA power class | Carrier n3 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

In addition, the power class for UL CA_n41C is PC2 , and PC1.5 n41 single carrier for this band combination.

### 5.38.3	REFSENS requirements

For PC3 CA_n3-n41, there are cross band isolation MSD and intermodulation MSD for this band combination.

Due to PC2 UL CA_n41C are supported, the IMDs issue caused by intra-band UL contiguous CA_n41C can be found in table 5.38.3-1: (By using the template in TR38.718-02-01, where up to 7th IMD is included.)

Table 5.38.3-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 190 | 2496 | 2690 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 2496 | 2516 | 2686 | 2690 | 2670 | 2500 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 190 | 5012 | 5360 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2306 | 2880 | 7508 | 8050 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 380 | 4802 | 5570 | 10004 | 10740 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2116 | 3070 | 7298 | 8260 | 12500 | 13430 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 570 | 4612 | 5760 | 9794 | 10950 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1926 | 3260 | 7108 | 8450 | 12290 | 13640 |

It can be found that there are no co-existence issues up to 7th caused by CA_n41C impact band n3 DL.

#### 5.38.3.1	Power class 2 case a, b

Since the power configuration for each band is 23dBm in Case a, comparing with PC3 there are no additional cross band isolation MSD or intermodulation MSD including triple beat IMD for this PC2 CA_n3A-n41C.

Since the power configuration for band n41 is 26dBm in Case b, comparing with PC2 there is no additional cross band isolation MSD for this PC2 CA_n3A-n41C.

For the intermodulation caused by UL inter-band CA:

it is proposed to use the approaches of PC3 UL CA_n3A-n41A and UL_CA_n3A-n41C, in which:

- For PC2 UL CA_n3A-n41A, the IMD requirement have already been defined in the spec.

- For PC2 UL CA_n3A-n41C, the 1st triple beat MSD should be defined due to the power for n41C is increased.

Based on the existing PC3 MSD requirements in TS38.101-1 (Table 5.38.3.1-1), it can be seen that CA_n41C may causes IMD9 falling into band n3 DL. And CA_n3A-n41C may have triple-beat product falling into band n3 DL.

Table 5.38.3.1-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | N/A | FDD | IMD914 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |
|  | n3 | 1747.5 | 5 | 25 (RBSTART= 0) | 1842.5 | 15.3 | FDD | IMD3 |
|  | n41 | 2560 | 60 | 1 (RBSTART= 30) | 2560 | N/A | TDD | N/A |
|  |  | 2620 | 60 | 1 (RBSTART= 127) | 2620 |  |  |  |

The MSD for PC3 triple-beat is specified as 15.3dB. For PC2. Hence, the MSD for PC2 is estimated as 23.1dB.

Table 5.38.3.1-2: MSD due to IMD issue caused by triple beat

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n3-n41 | n3 | 1747.5 | 5 | 25 (RBSTART= 0) | 1842.5 | 23.3 | FDD | IMD3 |
|  | n41 | 2560 | 60 | 1 (RBSTART= 30) | 2560 | N/A | TDD | N/A |
|  |  | 2620 | 60 | 1 (RBSTART= 127) | 2620 |  |  |  |

#### 5.38.3.2	Power class 2 for UL CA_n41C

Due to PC2 UL CA_n41C is supported in this band combination, in terms of the PC3 UL CA_n41C in the TR38.718-02-01, there are IMD9 MSD defined but with N.A value considering of the ‘Applicable when n41 spectrum is restricted to 2515-2675MHz’.

Thus, for PC2 UL CA_n41C, same approach as PC3 can be applied, the IMD value caused by PC2 n41C is defined in the following:

Table 5.38.3.2-1 MSD due to IMD issue  caused by PC2 intra-band contiguous CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n3-n41 | n3 | N/A | 5 | N/A | 1877.5 | N/AX | FDD | IMD9 |
|  | n41 | 2545 | 60 | 1 RBSTART= 0 | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 RBSTART= 272 | 2625 |  |  |  |
| NOTE X: Applicable when n41 spectrum is restricted to 2515-2675MHz |  |  |  |  |  |  |  |  |

### 5.38.4	∆TIB and ∆RIB values

void

## 5.39	CA_n40A-n41A

### 5.39.1	Configurations

Table 5.39.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n41A | n408,9 n418,9 CA_n40A-n41A8 | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n39 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

### 5.39.2	Maximum output power

void

### 5.39.3	REFSENS requirements

For PC3 and PC2 CA_n40A-n41A, there are cross band isolation MSD for this band combination.

#### 5.39.3.1	Power class 2 for single uplink n40

The cross band isolation MSD for this PC2 CA_n40A-n41A with UL n40 has already been specified in TS38.101-1.

#### 5.39.3.2	Power class 1.5 for single uplink n40

Since the MOP for n40 is increased to 29dBm, cross band isolation MSD caused by UL PC1.5 n40 should be defined.

The proposed value for PC1.5 UL CA MSD can be found in Table 5.39.3.2-1:

Table 5.39.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | [37.9] | ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

#### 5.39.3.3	Power class 1.5 for single uplink n41

Since the MOP for n41 is increased to 29dBm, cross band isolation MSD caused by UL PC1.5 n41 should be defined.

The proposed value for PC1.5 UL CA MSD can be found in Table 5.39.3.3-1:

Table 5.39.3.3-1-: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | [41.2] | ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

### 5.39.4	REFSENS requirements

Void

## 5.40	CA_n41-n77, UL CA_n77(2A)

### 5.40.1	Configurations

Table 5.40.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n77(2A) | n418,9 n778,9 CA_n77(2A)8 CA_n41A-n77A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  | n418,9 n778,9 CA_n41A-n77A8 | n41 | n41 channel bandwidth in Table5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.40.2	Maximum output power

Table 5.40.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.40.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink. For PC3 CA_n41-n77 with single uplink and UL CA_n41-n77, the co-existence study is provided in TR38.717-02-01[5]. For PC3 CA_n41-n77 with UL CA_n77(2A), the co-existence study is provided in TR38.718-02-01[9]. For PC2 and PC1.5 CA_n41-n77 with single uplink, the co-existence study is provided in TR 38.841[10]. Analysis is based on these studies.

For UL CA_n77(2A), IMD3, IMD5 and IMD7 fall into n41 downlink. IMD5 test point was chosen instead of IMD3, as for IMD3 to fall on n41 the frequency separation would need to be over 600 MHz which is not allowed based on clause 5.3A.5 of 38.101-1.

For PC3 CA_n41-n77 with UL CA_n77(2A), analysis of REFSENS exceptions is already defined in the Table 7.3A.5-1a of TS38.101-1[3] as follows.

Table 5.40.3-1 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n77 | n41 | N/A | 10 | N/A | 2565 | 17 | TDD | IMD516 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 16: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD5 increases by 3*5=15dB.

The proposed value for PC2 UL CA MSD can be found in Table 5.40.3-2.

Table 5.40.3-2 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n77 | n41 | N/A | 10 | N/A | 2565 | 32 | TDD | IMD516 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 16: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 5.40.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 5.41	CA_n40-n78

### 5.41.1	Configurations

Table 5.41.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n78A | n40A8,9 n78A8,9 CA_n40A-n78A8 | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.41.2	Maximum output power

Table 5.41.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n40-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.41.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.41.3.1	Power class 2 and 1.5 case a, b, c, d

Based on TS 38.101-1 Table 7.3A.4-1, there is no UL harmonic interference issue for this band combination.

Based on TS 38.101-1 Table 7.3A.4-4, there is harmonic mixing issue for n78 to n40 this is treated under the single band exceptions.

Based on TS 38.101-1 Table 7.3A.5-1, there is no IMD issues for this band combination.

Based on TS 38.101-1 Table 7.3A.6-1, there is cross band isolation issues for n78 to n40 this is treated under the single band exceptions.

#### 5.41.3.2	Power class 2 for single uplink n78

-		The 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n40.

-		 The 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of n40.

-		n40 DL suffers UL2/DL3 harmonic mixing from n78.

-		 Cross band isolation of n78 UL falls into n40 DL.

Therefore, MSD for cross band isolation and harmonic mixing should be defined.

Table 5.41.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 6.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 1.2 | >ACLR2 |

Table 5.41.3.2-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 13.2 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 4.4 | NOTE 3 | UL2/DL3 |

#### 5.41.3.3	Power class 1.5 for single uplink n78

Similar as for PC2 MSD should be defined for PC1.5.

Table 5.41.3.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2395 | 10 | 9.0 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 2.2 | >ACLR2 |

Table 5.41.3.3-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 16.1 | NOTE 3 | UL2/DL3 |
| n78 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 6.7 | NOTE 3 | UL2/DL3 |

#### 5.41.3.4	Power class 2 and 1.5 for single uplink n40

According to previous co-existence studies there are no issues.

### 5.41.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 5.42	CA_n39-n41

### 5.42.1	Configurations

Table 5.42.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n39A-n41A | n398 n418,9 CA_n39A-n41A8 | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n41C | n398 n418,9 CA_n41C8 CA_n39A-n41A8 CA_n39A-n41C8 | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

### 5.42.2	Maximum output power

Table 5.42.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n39-n41 | CA power class | Carrier n39 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n39A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |
| CA_n39A-n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 5.42.3	REFSENS requirements

PC2 UL single carrier

For PC3 CA_n39A-n41A, due to simultaneous Rx/Tx operation is supported, there are two types of MSD for this band combination, which are:

1: cross-band isolation MSD

2: Harmonic mixing MSD, where:

- 3rd harmonic of band n41 UL may fall into 4th harmonic of band n39 DL

- 4th harmonic of band n39 UL may fall into 3rd harmonic of band n41 DL.

For PC2 CA_n39A-n41A, by using the same co-existence study as PC3, and due to either PC2 single band n39 or PC2 single band n41 is supported, the above two types MSD should be defined.

PC2 UL CA_n41C

Due to PC2 UL CA_n41C are supported, the IMDs issue caused by intra-band UL contiguous CA_n41C can be found in table 5.42.3-1:

Table 5.42.3-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 190 | 2496 | 2690 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 2496 | 2516 | 2686 | 2690 | 2670 | 2500 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 190 | 5012 | 5360 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2306 | 2880 | 7508 | 8050 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 380 | 4802 | 5570 | 10004 | 10740 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2116 | 3070 | 7298 | 8260 | 12500 | 13430 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 570 | 4612 | 5760 | 9794 | 10950 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1926 | 3260 | 7108 | 8450 | 12290 | 13640 |

It can be found that there are no co-existence issues caused by CA_n41C impact band n39 DL.

PC2 UL CA_n39A-41A

For PC2 UL CA_n39A-n41A, except the cross band isolation and harmonic mixing MSD issue as mentioned as above, additional IMD MSD need to be checked. However, due to both n39 and n41 are TDD bands, there are no IMD issue due to the UL and DL will not operate at the same time.

PC2 UL CA_n39A-41C

For PC2 UL CA_n39A-n41A, similar with PC2 UL CA_n39A-n41A, the cross band isolation and harmonic mixing MSD issue have already been included in the PC2 UL single carrier.

For the impacts of PC2 UL CA_n41C, the analysis are included in the PC2 UL_CA_n41C as above.

For the triple beat IMD issue, due to both n39 and n41 are TDD bands, there are no IMD issue due to the UL and DL will not operate at the same time. Therefore, no additional triple beat IMD MSD are foreseen to be be defined to support PC2 UL CA_n39A-n41C.

To summarize, to support PC2 power class for the UL configuration of single carrier, CA_n41C, CA_n39A-n41A and CA_n39A-n41C, only the cross-band isolation MSD and Harmonic mixing MSD should be defined, which are given in the following sub-clauses. There are no IMD issue caused by UL CA_n39-n41 for the CA_n39-n41 band combination.

#### 5.42.3.1	Power class 2 for case a

For PC2 case a, since the power configuration for each band is 23dBm, therefore comparing with PC3 CA_n39A-n41A, no additional cross-band isolation MSD or harmonic mixing MSD are expected for this PC2 CA_n39A-n41A with 1 uplink carrier.

#### 5.42.3.2	Power class 2 for case b

For PC2 case b, due to the power class for n39 is still kept as PC3(i.e. 23dBm), comparing with PC3 CA_n39A-n41A, there are no additional cross-band isolation MSD or harmonic mixing MSD caused by PC3 UL n39 are expected.

However, due to the power class for n41 is increased to 26dBm, therefore comparing with PC3 CA_n39A-n41A,  cross-band isolation MSD or harmonic mixing MSD caused by UL PC2 n41 shall be defined.

the harmonic mixing MSD caused by UL PC2 n39 are proposed in table 5.42.3.2-1.

Table 5.42.3.2-1. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 5 | 6.1 | NOTE X | UL3/DL4 |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 40 | 1.5 | NOTE X | UL3/DL4 |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Similarly, the cross band isolation caused by UL PC2 n41 is proposed in table 5.42.3.2-2.

Table 5.42.3.2-2. Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 2.7 | >ACLR2 |

NOTE： Cross-band isolation between n41C UL (PC3 and PC2) may need to be evaluated and specified, pending on the general MSD framework for cross-band interference between intra-band contiguous UL CA and the victim DL band.

#### 5.42.3.3	Power class 2 for case c

For PC2 case c, due to the power class for n41 is still kept as PC3(i.e. 23dBm), comparing with PC3 CA_n39A-n41A, there are no additional cross-band isolation MSD or harmonic mixing MSD caused by PC3 UL n41 are expected.

However, due to the power class for n39 is increased to 26dBm, comparing with PC3 CA_n39A-n41A, cross-band isolation MSD or harmonic mixing MSD caused by UL PC2 n39 shall be defined.

Therefore, the harmonic mixing MSD caused by UL PC2 n39 are proposed in table 5.42.3.3-1.

Table 5.42.3.3-1. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 5 | 10.6 | NOTE X | UL4/DL3 |
| n39 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 4.9 | NOTE X | UL4/DL3 |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Similarily, the cross band isolation caused by UL PC2 n39 is proposed in table 5.42.3.3-2.

Table 5.42.3.3-2. Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 4.7 | >ACLR2 |

#### 5.42.3.4	Power class 2 for case d

For PC2 case d, the power class for both bands n39 and n41 are increased to 26dBm, therefore comparing with PC3 CA_n39A-n41A, cross-band isolation MSD or harmonic mixing MSD caused by UL PC2 n39 or PC2 n41 shall be defined.

Due to the cross-band isolation MSD or harmonic mixing MSD are the situation of one single band impact another single band, therefore the cross-band isolation MSD or harmonic mixing MSD caused UL PC2 n39 or PC2 n41 are already included in sub-clause 5.42.3.2 and sub-clause 5.42.3.3.

#### 5.42.3.5	Power class 1.5 for single uplink n41

Since the MOP for n41 is increased to 29dBm, harmonic mixing MSD and cross band isolation MSD caused by UL PC1.5 n41 should be defined.

The proposed value for PC1.5 UL CA MSD can be found in Table 5.42.3.5-1 and Table 5.42.3.5-2:

Table 5.42.3.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 5 | 10.5 | NOTE X | UL3/DL4 |
| n41 | n39 | 10 | 15 | 25 (RBstart=0) | 40 | 3.0 | NOTE X | UL3/DL4 |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 5.42.3.5-2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 6.7 | >ACLR2 |

### 5.42.4	∆TIB and ∆RIB values

Void

## 5.43	CA_n28A-n40A

### 5.43.1	Configurations

Table 5.43.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n40A | n408,9 CA_n28A-n40A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

### 5.43.2	Maximum output power

Table 5.43.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n40 | CA power class | Carrier n28 power class | Carrier n40 power class |
| --- | --- | --- | --- | --- |
| CA_n28-n40 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

In addition, the power class for PC2 and PC1.5 n40 single carrier for this band combination.

### 5.43.3	REFSENS requirements

For PC3 CA_n28A-n40A, there are harmonic mixing MSD for this band combination:

band n41 UL may fall into 3rd harmonic of band n28 DL.

#### 5.43.3.1	Power class 2 case a

For PC2 case a, since the power configuration for each band is 23dBm, therefore comparing with PC3 CA_n28A-n40A, no additional harmonic mixing MSD are expected for this PC2 CA_n28A-n40A with 1 uplink carrier.

#### 5.43.3.2	Power class 2 case b

For PC2 case b, due to the MOP for n28 is still kept as 23dBm. Since the MOP for n40 is increased to 26dBm, harmonic mixing MSD caused by UL PC2 n40 shall be defined.

The proposed value for PC2 UL CA MSD can be found in Table 5.43.3.2-1:

Table 5.43.3.2-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n40 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 40.8 | NOTE 7 | UL1/DL3 |
| n40 | n28 | 20 | 15 | 100 (RBstart=0) | 20 | 33.3 | NOTE 7 | UL1/DL3 |
| NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

#### 5.43.3.3	Power class 1.5 for single uplink n40

Since the MOP for n40 is increased to 29dBm, harmonic mixing MSD caused by UL PC1.5 n40 should be defined.

The proposed value for PC1.5 UL CA MSD can be found in Table 5.43.3.3-1:

Table 5.43.3.3-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n40 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 43.8 | NOTE X | UL1/DL3 |
| n40 | n28 | 20 | 15 | 100 (RBstart=0) | 20 | 36.3 | NOTE X | UL1/DL3 |
| NOTE X: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

### 5.43.4	∆TIB and ∆RIB values

Void

## 5.44	CA_ n3A-n40A

### 5.44.1	UE maximum output power

Table 5.44.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier |  | NR Band |  | Channel bandwidth (MHz) |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n40A | n408,9 CA_n3A-n40A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 2 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |  |  |  |  |  |

### 5.44.2	Maximum output power

Table 5.44.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n40 | CA power class | Band n3 power class | Band n40 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n40A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 5.44.3	Reference sensitivity requirements

#### 5.44.3.1	Power class 2 for case A

As existing requirements for PC3 CA_n3-n40, there is no MSD issue.

#### 5.44.3.2	Power class 2 for case B

Cross band isolation interference of PC2 n40 falls into n3.

The PC2 MSD due to cross band isolation are specified in following table 5.44.3.2-1.

Table 5.44.3.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC2 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n3 | 2350 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 0.6 | >ACLR2 |

#### 5.44.3.3	Power class 1.5 for n40

Cross band isolation interference of PC1.5 n40 falls into n3.

The PC1.5 MSD due to cross band isolation are specified in following table 5.44.3.3-1.

Table 5.44.3.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n3 | 2350 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 1.2 | >ACLR2 |

### 5.44.4	∆TIB and ∆RIB values

Void

# 6	High Power UE CA of 3 bands DL and 1 or 2 bands UL

## 6.x	CA_nX-nY-nZ

### 6.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.2-1x unless additional clarification>

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY-nZ | nX7,9, nY7,9, CA_nX-nY7 | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.x.2	Maximum output power

<Editor’s note: In table 6.x.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. The requirements in this TR are intended to be power class 2 cases based, however how to address in the spec will be further discussed. >

#### 6.x.3.1	Power class 2 case a

### 6.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

## 6.1 	DL CA_n25-n41-n66, UL CA_n25A-n41A

### 6.1.1	Configurations

Table 6.1.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66A | CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A | CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.1.2	Maximum output power

Table 6.1.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.1.3	REFSENS requirements

#### 6.1.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n25A-n41A into CA_n25-n41-n66, none is proposed for PC2.

#### 6.1.3.2	Power class 2 case b

Since there is no MSD for PC3 UL CA_n25A-n41A into CA_n25-n41-n66, none is proposed for PC2.

### 6.1.4	Void

## 6.2	DL CA_n25-n41-n66, UL CA_n41A-n66A

### 6.2.1	Configurations

Table 6.2.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.2.2	Maximum output power

Table 6.2.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.2.3	REFSENS requirements

#### 6.2.3.1	Power class 2 case a

There is 11.0 dB of MSD for PC3 CA_n41A-n66A into n25:

Table 6.2.3-1 PC3 MSD

| CA_n25-n41-n66 | n25 | 1860 | 5 | 25 | 1940 | 11.0 | FDD | IMD4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD4 interference power increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.x.3-2.

Table 6.x.3-2 PC2 MSD

| CA_n25-n41-n66 | n25 | 1860 | 5 | 25 | 1940 | 22.7 | FDD | IMD4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |

#### 6.2.3.2	Power class 2 case c

The same MSD applies for Power class 2 case c as power class 2 case a.

### 6.2.4	Void

## 6.3	DL CA_n25-n41-n71, UL CA_n25A-n41A

### 6.3.1	Configurations

Table 6.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n71A | CA_n25A-n41A7 CA_n41A-n71A CA_n25A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71A | CA_n25A-n41A7 CA_n41A-n71A CA_n25A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n71A | CA_n25A-n41A7 CA_n41A-n71A CA_n25A-n71A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.3.2	Maximum output power

Table 6.3.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.3.3	REFSENS requirements

#### 6.3.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n25A-n41A into n71, none is proposed for PC2.

#### 6.3.3.2	Power class 2 case b

Since there is no MSD for PC3 UL CA_n25A-n41A into -n71, none is proposed for PC2.

### 6.3.4	Void

## 6.4	DL CA_n25-n41-n71, UL CA_n41A-n71A

### 6.4.1	Configurations

Table 6.4.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n71A | CA_n25A-n41A CA_n41A-n71A7 CA_n25A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71A | CA_n25A-n41A CA_n41A-n71A7 CA_n25A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n71A | CA_n25A-n41A CA_n41A-n71A7 CA_n25A-n71A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.4.2	Maximum output power

Table 6.4.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.4.3	REFSENS requirements

#### 6.4.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n41A-n71A into CA_n25-n41-n71, none is proposed for PC2.

#### 6.4.3.2	Power class 2 case c

Since there is no MSD for PC3 UL CA_n41A-n71A into CA_n25-n41-n71, none is proposed for PC2.

### 6.4.4	Void

## 6.5	DL CA_n25-n41-n71, UL CA_n41C

### 6.5.1	Configurations

Table 6.5.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41C-n71A | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.5.2	Maximum output power

Table 6.5.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41C | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.5.3	REFSENS requirements

#### 6.5.3.1	Power class 2 case a, case b, case c, case c

Any MSD for UL CA_n41C PC2 is handled in the 2 band DL fallbacks.

### 6.5.4	Void

## 6.6	DL CA_n25-n41-n77, UL CA_n25A-n41A

### 6.6.1	Configurations

Table 6.6.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n77A | CA_n25A-n41A7 CA_n25A-n77A CA_n41A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n77A | CA_n25A-n41A7 CA_n25A-n77A CA_n41A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.6.2	Maximum output power

Table 6.6.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.6.3	REFSENS requirements

#### 6.6.3.1	Power class 2 case a

Power class 3 CA for UL CA_n25A-n41A:

Table 6.6.3.1-1 Power class 3 MSD for 2 bands UL CA_n25A-n41A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n77 | 3470 | 10 | 50 | 3470 | 14.8 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 | 25 | 2645 | N/A | TDD | N/A |
|  | n77 | 3775 | 10 | 50 | 3775 | 4.2 | TDD | IMD5 |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.6.3.1-1.

Table 6.6.3.1-2 Proposed Power class 2 MSD for UL CA_n25-n41-n71

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n77 | 3470 | 10 | 50 | 3470 | 23.7 | TDD | IMD3 |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n41 | 2525 | 5 | 25 | 2645 | N/A | TDD | N/A |
|  | n77 | 3775 | 10 | 50 | 3775 | 17.2 | TDD | IMD5 |

#### 6.6.3.2	Power class 2 case b

The same MSD for case a applies to case b.

### 6.6.4	∆TIB and ∆RIB values

Void.

## 6.7	DL CA_n25-n41-n77, UL CA_n25A-n77A

### 6.7.1	Configurations

Table 6.7.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n77A | CA_n25A-n41A CA_n25A-n77A7 CA_n41A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  | CA_n25A-n41A CA_n25A-n77A7 CA_n41A-n77A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
| CA_n25(2A)-n41A-n77A |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.7.2	Maximum output power

Table 6.7.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.7.3	REFSENS requirements

#### 6.7.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n25A-n77A:

Table 6.7.3.1-1 Power class 3 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2640 | 5 | 25 | 2640 | 5.3 | TDD | IMD5ZZ |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD5 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.7.3.1-2.

Table 6.7.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2640 | 5 | 25 | 2640 | 18.8 | TDD | IMD55 |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.7.3.2	Power class 2 case c

The same MSD for case a applies to case b.

### 6.7.4	∆TIB and ∆RIB values

Void

## 6.8	DL CA_n25-n41-n77, UL CA_n41A-n77A

### 6.8.1	Configurations

Table 6.8.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
| CA_n25(2A)-n41A-n77A |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.8.2	Maximum output power

Table 6.8.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.8.3	REFSENS requirements

#### 6.8.3.1	Power class 2 case a, case b, case c, case c

The MSD is the same for case a, case b, case c and case d.

Power class 3 MSD for UL CA_n41A-n77A:

Table 6.8.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n77A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | 17.6 | FDD | IMD3ZZ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2675 | 5 | 25 | 2675 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 8.6 | FDD | IMD4 |
|  | n41 | 2550 | 5 | 25 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

If the input signal increases by 3 dB, the IMD4 increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.8.3.1-2.

Table 6.8.3.1-1 Proposed power class 2 MSD for 2 bands UL CA_n41A-n77A

| CA_n25-n41-n77 | n25 | 1870 | 5 | 25 | 1950 | 26.5 | FDD | IMD35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n41 | 2675 | 5 | 25 | 2675 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n25 | 1870 | 5 | 25 | 1950 | 20.0 | FDD | IMD4 |
|  | n41 | 2550 | 5 | 25 | 2685 | N/A | TDD | N/A |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.8.4	∆TIB and ∆RIB values

Void

## 6.9	DL CA_n25-n66-n77, UL CA_n25A-n77A

### 6.9.1	Configurations

Table 6.9.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.9.2	Maximum output power

Table 6.9.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.9.3	REFSENS requirements

#### 6.9.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n25A-n77A:

Table 6.9.3.1-1 Power class 3 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n66-n77 | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1715 | 5 | 25 | 2115 | 29.2 | FDD | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | 10 | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*4=12 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.9.3.1-2.

Table 6.9.3.1-2 Power class 3 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n66-n77 | n25 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1715 | 5 | 25 | 2115 | 35.2 | FDD | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 22.0 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 16.9 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |

#### 6.9.3.2	Power class 2 case b

The same MSD for case a applies to case b.

### 6.9.4	∆TIB and ∆RIB values

Void.

## 6.10	DL CA_n25-n66-n77, UL CA_n66A-n77A

### 6.10.1	Configurations

Table 6.10.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A-n77A | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n77(2A) | CA_n25A-n66A CA_n25A-n77A CA_n66A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.10.2	Maximum output power

Table 6.10.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.10.3	REFSENS requirements

#### 6.10.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n66A-n77A:

Table 6.10.3.1-1 Power class 3 MSD for 2 bands UL CA_n25A-n77A

| CA_n25-n66-n77 | n25 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4ZZ |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5ZZ |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*4=12 dB.

If the input signal increases by 3 dB, the IMD5 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.10.3.1-2.

Table 6.10.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n66A-n77A

| CA_n25-n66-n77 | n25 | 1880 | 5 | 25 | 1960 | 38.1 | FDD | IMD2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 20.6 | FDD | IMD45 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 13.2 | FDD | IMD55 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.10.3.2	Power class 2 case c

The same MSD for case a applies to case b.

### 6.10.4	∆TIB and ∆RIB values

Void

## 6.11	DL CA_n25-n71-n77, UL CA_n25A-n77A

### 6.11.1	Configurations

Table 6.11.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.11.2	Maximum output power

Table 6.11.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.11.3	REFSENS requirements

#### 6.11.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n25A-n77A, so there should be none for PC2.

#### 6.11.3.2	Power class 2 case b

The same MSD for case a applies to case b.

### 6.11.4	∆TIB and ∆RIB values

Void.

## 6.12	DL CA_n25-n71-n77, UL CA_n71A-n77A

### 6.12.1	Configurations

Table 6.12.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n71A-n77A | CA_n25A-n71A CA_n25A-n77A CA_n71A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.12.2	Maximum output power

Table 6.12.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.12.3	REFSENS requirements

#### 6.12.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n71A-n77A:

Table 6.12.3.1-1 Power class 3 MSD for 2 bands UL CA_n71A-n77A

| CA_n25-n71-n77 | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD32,5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.12.3.1-2.

Table 6.12.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n71A-n77A

| CA_n25-n71-n77 | n25 | 1874 | 5 | 25 | 1954 | 25.4 | FDD | IMD32,5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.12.3.2	Power class 2 case c

The same MSD for case a applies to case b.

### 6.12.4	∆TIB and ∆RIB values

Void

## 6.13	DL CA_n41-n66-n77, UL CA_n41A-n66A

### 6.13.1	Configurations

Table 6.13.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | CA_n41A-n66A7 CA_n41A-n77A CA_n66A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n77A | CA_n41A-n66A7 CA_n41A-n77A CA_n66A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.13.2	Maximum output power

Table 6.13.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.13.3	REFSENS requirements

#### 6.13.3.1	Power class 2 case a

Power class 3 CA for UL CA_n25A-n41A:

Table 6.13.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n66A

| CA_n41-n66-n77 | n41 | 2600 | 5 | 25 | 2600 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3470 | 10 | 50 | 3470 | 16.1 | TDD | IMD31,2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.13.3.1-1.

Table 6.13.3.1-2 Proposed Power class 2 MSD for UL CA_n41-n66

| CA_n41-n66-n77 | n41 | 2600 | 5 | 25 | 2600 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3470 | 10 | 50 | 3470 | 25.0 | TDD | IMD31,2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 6.13.3.2	Power class 2 case b

The same MSD for case a applies to casec.

### 6.13.4	∆TIB and ∆RIB values

Void.

## 6.14	DL CA_n41-n66-n77, UL CA_n41A-n77A

### 6.14.1	Configurations

Table 6.14.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | CA_n41A-n66A CA_n41A-n77A7 CA_n66A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A7 CA_n66A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.14.2	Maximum output power

Table 6.14.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.14.3	REFSENS requirements

#### 6.14.3.1	Power class 2 case a, case b, case c, case d

Power class 3 MSD for UL CA_n41A-n77A:

Table 6.14.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n77A

| CA_n41-n66-n77 | n41 | 2640 | 5 | 25 | 2640 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1760 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD4 increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.14.3.1-2.

Table 6.14.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_41A-n77A

| CA_n41-n66-n77 | n41 | 2640 | 5 | 25 | 2640 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1760 | 5 | 25 | 2160 | 20.5 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

### 6.14.4	∆TIB and ∆RIB values

Void

## 6.15	DL CA_n41-n66-n77, UL CA_n66A-n77A

### 6.15.1	Configurations

Table 6.15.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.15.2	Maximum output power

Table 6.15.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.15.3	REFSENS requirements

#### 6.15.3.1	Power class 2 case a, case b, case c, case c

The MSD is the same for case a, case b, case c and case d.

Power class 3 MSD for UL CA_n66A-n77A:

Table 6.15.3.1-1 Power class 3 MSD for 2 bands UL CA_n66A-n77A

| CA_n41-n66-n77 | n41 | 2670 | 5 | 25 | 2670 | 5.2 | TDD | IMD55 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*5=15 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.15.3.1-2.

Table 6.15.3.1-1 Proposed power class 2 MSD for 2 bands UL CA_n66A-n77A

| CA_n41-n66-n77 | n41 | 2670 | 5 | 25 | 2670 | 18.7 | TDD | IMD55 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.15.4	∆TIB and ∆RIB values

Void

## 6.16	DL CA_n41-n71-n77, UL CA_n41A-n71A

### 6.16.1	Configurations

Table 6.16.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A-n77A | CA_n41A-n71A7 CA_n41A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77A | CA_n41A-n71A7 CA_n41A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n77A | CA_n41A-n71A7 CA_n41A-n77A CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.16.2	Maximum output power

Table 6.16.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.16.3	REFSENS requirements

#### 6.16.3.1	Power class 2 case a

Power class 3 CA for UL CA_n41A-n71A:

Table 6.16.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n71A

| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 29.1 | TDD | IMD21,5 |
|  | n41 | 2564 | 5 | 25 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | 16.3 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 10.3 | TDD | IMD41 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

If the input signal increases by 3 dB, the IMD4 increases by 3*4=12 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.16.3.1-1.

Table 6.16.3.1-2 Proposed Power class 2 MSD for UL CA_n41-n71

| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 35.1 | TDD | IMD21,5 |
|  | n41 | 2564 | 5 | 25 | 2564 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | 25.2 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 21.9 | TDD | IMD41 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.16.3.2	Power class 2 case c

The same MSD for case a applies to case c.

### 6.16.4	∆TIB and ∆RIB values

Void.

## 6.17	DL CA_n41-n71-n77, UL CA_n41A-n77A

### 6.17.1	Configurations

Table 6.17.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A-n77A | CA_n41A-n71A CA_n41A-n77A7 CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77A | CA_n41A-n71A CA_n41A-n77A7 CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A7 CA_n71A-n77A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.17.2	Maximum output power

Table 6.17.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.17.3	REFSENS requirements

#### 6.17.3.1	Power class 2 case a, case b, case c, case d

Power class 3 MSD for UL CA_n41A-n77A:

Table 6.17.3.1-1 Power class 3 MSD for 2 bands UL CA_n41A-n77A

| CA_n41-n71-n77 | 41 | 2680 | 5 | 25 | 2680 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 686 | 5 | 25 | 640 | 30.8 | FDD | IMD25 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.17.3.1-2.

Table 6.17.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_41A-n77A

| CA_n41-n71-n77 | 41 | 2680 | 5 | 25 | 2680 | N/A | TDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 686 | 5 | 25 | 640 | 36.8 | FDD | IMD25 |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.17.4	∆TIB and ∆RIB values

Void

## 6.18	DL CA_n41-n71-n77, UL CA_n71A-n77A

### 6.18.1	Configurations

Table 6.18.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.18.2	Maximum output power

Table 6.18.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.18.3	REFSENS requirements

#### 6.18.3.1	Power class 2 case a, case b, case c, case c

The MSD is the same for case a, case b, case c and case d.

Power class 3 MSD for UL CA_n71A-n77A:

Table 6.18.3.1-1 Power class 3 MSD for 2 bands UL CA_n71A-n77A

| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | 28.7 | TDD | IMD25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2564 | 5 | 25 | 2564 | 15.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD2 increases by 3*2=6 dB.

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.18.3.1-2.

Table 6.18.3.1-1 Proposed power class 2 MSD for 2 bands UL CA_n71A-n77A

| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | 34.7 | TDD | IMD25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2564 | 5 | 25 | 2564 | 24.4 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.18.4	∆TIB and ∆RIB values

Void

## 6.19	DL CA_n66-n71-n77, UL CA_n66A-n77A

### 6.19.1	Configurations

Table 6.19.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71A-n77(2A) | CA_n66A-n71A CA_n66A-n77A7 CA_n71A-n77A | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.19.2	Maximum output power

Table 6.19.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.19.3	REFSENS requirements

#### 6.19.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n66A-n77A:

Table 6.19.3.1-1 Power class 3 MSD for 2 bands UL CA_n66A-n77A

| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 686 | 5 | 25 | 640 | 15.3 | FDD | IMD35 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.19.3.1-2.

Table 6.19.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n66A-n77A

| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 686 | 5 | 25 | 640 | 24.2 | FDD | IMD35 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

#### 6.19.3.2	Power class 2 case b

The same MSD for case a applies to case b.

### 6.19.4	∆TIB and ∆RIB values

Void.

## 6.20	DL CA_n66-n71-n77, UL CA_n71A-n77A

### 6.20.1	Configurations

Table 6.20.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n71A-n77A | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A7 | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71A-n77(2A) | CA_n66A-n71A CA_n66A-n77A CA_n71A-n77A7 | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.20.2	Maximum output power

Table 6.20.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.20.3	REFSENS requirements

#### 6.20.3.1	Power class 2 case a

Power class 3 MSD for UL CA_n71A-n77A:

Table 6.20.3.1-1 Power class 3 MSD for 2 bands UL CA_n71A-n77A

| CA_n66-n71-n77 | n66 | 1750 | 5 | 25 | 2150 | 15.5 | FDD | IMD32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 690 | 5 | 25 | 644 | N/A | FDD | N/A |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.20.3.1-2.

Table 6.20.3.1-2 Proposed power class 2 MSD for 2 bands UL CA_n71A-n77A

| CA_n66-n71-n77 | n66 | 1750 | 5 | 25 | 2150 | 24.4 | FDD | IMD32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 690 | 5 | 25 | 644 | N/A | FDD | N/A |
|  | n77 | 3530 | 10 | 50 | 3530 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 6.20.3.2	Power class 2 case c

The same MSD for case a applies to case b.

### 6.20.4	∆TIB and ∆RIB values

Void

## 6.21	DL CA_n41-n66-n71, UL CA_n41A-n71A

### 6.21.1	Configurations

Table 6.21.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n71A | CA_n41A-n71A7 CA_n66A-n71A CA_n41A-n66A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n71A | CA_n41A-n71A7 CA_n66A-n71A CA_n41A-n66A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71A | CA_n41A-n71A7 CA_n66A-n71A CA_n41A-n66A | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.21.2	Maximum output power

Table 6.21.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.21.3	REFSENS requirements

#### 6.21.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n41A-n71A into CA_n41-n66-n71, none is proposed for PC2.

#### 6.21.3.2	Power class 2 case c

Since there is no MSD for PC3 UL CA_n41A-n71A into CA_n41-n66-n71, none is proposed for PC2.

### 6.21.4	Void

## 6.22	DL CA_n41-n66-n71, UL CA_n41A-n71A

### 6.22.1	Configurations

Table 6.22.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n71A | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n71A | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A7 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71A | CA_n41A-n71A CA_n66A-n71A CA_n41A-n66A7 | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.22.2	Maximum output power

Table 6.22.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |

### 6.22.3	REFSENS requirements

#### 6.22.3.1	Power class 2 case a

Since there is no MSD for PC3 UL CA_n41A-n66A into CA_n41-n66-n71, none is proposed for PC2.

#### 6.22.3.2	Power class 2 case c

Since there is no MSD for PC3 UL CA_n41A-n66A into CA_n41-n66-n71, none is proposed for PC2.

### 6.22.4	Void

## 6.23	CA_n3-n28-n41

### 6.23.1	Configurations

Table 6.23.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n41A | CA_n3A-n28A CA_n3A-n41A7 CA_n28A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.23.2	Maximum output power

Table 6.23.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.23.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.23.3.1	Power class 2 case a, b

Based on calculation, IMD2 and IMD3 of dual UL CA_n3-n41 fall into n28 DL, IMD2 of dual UL CA_n28-n41 falls into n3 DL.

Table 6.23.3-13 DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n41 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 32 | FDD | IMD24 |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | TDD | N/A |
|  | n3 | 1737.5 | 5 | 25 | 1832.5 | 32 | FDD | IMD2 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.23.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.24	CA_n3-n41-n77

### 6.24.1	Configurations

Table 6.24.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A-n77A | n417 n777 CA_n3A-n41A7 CA_n3A-n77A7 CA_n41A-n77A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n41A-n77(2A) | n417 n777 CA_n3A-n41A7 CA_n3A-n77A7 CA_n41A-n77A7 CA_n77(2A) 7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.24.2	Maximum output power

Table 6.24.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nC | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.24.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.24.3.1	Power class 2 case a, b, c, d

Based on calculation, IMD3/5 of dual UL CA_n3-n41 fall into n77 DL; IMD3/4 of dual UL CA_n41-n77 falls into n3 DL; IMD5 of dual UL CA_n3-n77 falls into n41 DL.

Table 6.24.3-13 DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | 25.6 | TDD | IMD31 |
|  | n3 | 1745 | 5 | 25 | 1840 | 25.1 | FDD | IMD32 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2640 | 5 | 25 | 2640 | 13 | TDD | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.24.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.25	CA_n3-n28-n77

### 6.25.1	Configurations

Table 6.25.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n77A | n777 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n77(2A) | n777 CA_n77(2A)7 CA_n3A-n28A CA_n3A-n77A7 CA_n28A-n77A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.25.2	Maximum output power

Table 6.25.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.25.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.25.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL CA_n3-n77 fall into n28 DL, IMD3/4 of dual UL CA_n28-n77 falls into n3 DL.

For UL CA_n77(2A) with DL CA_n3A-n77(2A), based on calculation, IMD 2, 4, 5 of dual UL falls into n3 DL, the MSD exception are also already defined in Table 7.3A.5-1a of the current TS 38.101-1 [2]. For UL CA_n77(2A), with DL CA_n28A-n77(2A), IMD 2, 4, 6, 7 and 8 of dual n77 UL falls into n28 DL. The MSD exception have also been defined in Table 7.3A.5-1a of the current TS 38.101-1 [2].

Table 6.25.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n77 | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | 24.2 | FDD | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 25.8 | FDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.25.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.26	CA_n28-n41-n77

### 6.26.1	Configurations

Table 6.26.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41A-n77A | n417,9 n777,9 CA_n28A-n41A7 CA_n28A-n77A7 CA_n41A-n77A7 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41A-n77(2A) | n417,9 n777,9 CA_n28A-n41A7 CA_n28A-n77A7 CA_n41A-n77A7 CA_n77(2A) 7 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.26.2	Maximum output power

Table 6.26.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.26.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.26.3.1	Power class 2 case a, b, c, d

Based on calculation, IMD2/3/4 of dual UL CA_n28-n41 fall into n77 DL; IMD2/3/5 of dual UL CA_n41-n77 falls into n28 DL; IMD2/3 of dual UL CA_n28-n77 falls into n41 DL.

Table 6.26.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n77 | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n77 | 3323 | 10 | 50 | 3323 | 34.2 | TDD | IMD22,4 |
|  | n28 | 743 | 5 | 25 | 798 | 36.8 | FDD | IMD21,4 |
|  | n41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 35.5 | TDD | IMD24 |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 6.26.3.2	Power class 1.5 for single UL n41 and n77

The PC 1.5 on single band UL n41 only does not cause further IMD issues, and the harmonic/harmonic mixing issues have been covered by the two band cases, hence no additional MSD is needed.

### 6.26.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.27	CA_n2-n48-n77

### 6.27.1	Configurations

Table 6.27.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n48A-n77A | n777, 9 CA_n2A-n48A CA_n2A-n77A7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.27.2	Maximum output power

Table 6.27.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n2A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.27.3	REFSENS requirements

Analysis performed for the PC3 CA_n2A-n48A-n77A as captured in TR 38.717-03-02, the following co-existence issues due to dual uplink are addressed:

-	Simultaneous Tx/Rx operation between NR n48 and n77 carriers are not supported hence, no MSD is needed for IMD4 and IMD5 products produced by Band n2 and n48 that might fall in Rx of band n77.and IMD4 and IMD5 products produced by Band n2 and n77 that might fall in Rx of band n48.

-	CA_n48-n77 is limited for downlink, hence no UL configuration exists.

#### 6.27.3.1	Power class 2 case a, b

No additional MSD is needed, since uplink configuration CA_n2A-n77A PC2 will still not affect band n48, since n48 and n77 are not simultaneous RX/TX.

### 6.27.4	Void

## 6.28	CA_n5-n48-n77

### 6.28.1	Configurations

Table 6.28.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n48A-n77A | n777, 9 CA_n5A-n48A CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.28.2	Maximum output power

Table 6.28.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.28.3	REFSENS requirements

Analysis performed for the PC3 CA_n5A-n48A-n77A as captured in TR 38.717-03-02, the following co-existence issues due to dual uplink are addressed:

-    Simultaneous Tx/Rx operation between NR n48 and n77 carriers are not supported hence.

-	CA_n48-n77 is limited for downlink, hence no UL configuration exists.

#### 6.28.3.1	Power class 2 case a, b

No additional MSD is needed

### 6.28.4	Void

## 6.29	CA_n48-n66-n77

### 6.29.1	Configurations

Table 6.29.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48A-n66A-n77A | n777, 9 CA_n48A-n66A CA_n66A-n77A7 | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.29.2	Maximum output power

Table 6.29.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n48A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.29.3	REFSENS requirements

Analysis performed for the PC3 CA_n48A-n66A-n77A as captured in TR 38.717-03-02, the following co-existence issues due to dual uplink are addressed:

-	Simultaneous Tx/Rx operation between NR n48 and n77 carriers are not supported hence, no MSD is needed for IMD4 and IMD5 products produced by Band n48 and n66 that might fall in Rx of band n77 and IMD4 and IMD5 products produced by Band n66 and n77 that might fall in Rx of band n48.

-	CA_n48-n77 is limited for downlink, hence no UL configuration exists.

#### 6.29.3.1	Power class 2 case a, b

No additional MSD is needed, since uplink configuration CA_n66A-n77A PC2 will still not affect band n48, since n48 and n77 are not simultaneous RX/TX.

### 6.29.4	Void

## 6.30	DL CA_n2-n5-n77, UL_ CA_n2A-n77A

### 6.30.1	Configurations

Table 6.30.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n5A-n77C | CA_n2A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.30.2	Maximum output power

Table 6.30.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n2-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.30.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements had been studied in R4-2117251 for both case a and b, and the requirements were specified in 38.101-1 due to higher power uplink. Thus, there is no more additional requirement.

### 6.30.4	∆TIB and ∆RIB values

The values had been specified in 38.101-1. There is no change by comparing to the values, so this section is omitted.

## 6.31	DL CA_n2-n5-n77, UL_CA_n5A-n77A

### 6.31.1	Configurations

Table 6.31.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n5A-n77C | CA_n5A-n77A7 | n2 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n5 | 5, 10, 15, 20, 251 |  |
|  |  | n77 | CA_n77C_BCS1 |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.31.2	Maximum output power

Table 6.31.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.31.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements had been studied in R4-2117251 for both case a and b, and the requirements were specified in 38.101-1 due to higher power uplink. Thus, there is no more additional requirement.

### 6.31.4	∆TIB and ∆RIB values

The values had been specified in 38.101-1. There is no change by comparing to the values, so this section is omitted.

## 6.32	CA_n1-n3-n41

### 6.32.1	Configurations

Table 6.32.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n41A | CA_n1A-n41A7 CA_n3A-n41A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n41 | 10, 15, 20, 30 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.32.2	Maximum output power

Table 6.32.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n3-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.32.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.32.3.1	Power class 2 case a, b

Based on calculation, no MSD issue for dual UL CA_n1-n41 falls into n3 DL or dual UL CA_n3-n41 falls into n1 DL.

### 6.32.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.33	CA_n7-n66-n78

### 6.33.1	Configurations

Table 6.33.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n66A-n78A | n787, 9 CA_n7A-n78A7 CA_n66A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.33.2	Maximum output power

Table 6.33.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n66-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.33.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

### 6.33.3.1	Power class 2 case a, b

Based on calculation, IMD4 of dual UL CA_n7-n78 fall into n66 DL, MSD values reused from CA_n41-n66-n77.

Table 6.33.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n66-n78 | n7 | 2540 | 5 | 25 | 2660 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 20.5 | FDD | IMD4 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.33.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.34	CA_n25-n66-n78

### 6.34.1	Configurations

Table 6.34.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A-n78A | n787, 9 CA_n25A-n78A7 CA_n66A-n78A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n78(2A) | n787, 9 CA_n25A-n78A7 CA_n66A-n78A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.34.2	Maximum output power

Table 6.34.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n66-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.34.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

### 6.34.3.1	Power class 2 case a, b

IMD4 of dual UL CA_n25-n78 falls into n66 DL. MSD value n66 is reused from PC2 DC_2A-66A_n77A.

IMD2/4/5 of dual UL CA_n66-n78 falls into n25 DL, MSD value n25 is reused from PC2 CA_n2-n66-n77.

Table 6.34.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n66-n78 | n25 | 1880 | 5 | 25 | 1960 | M/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 21.1 | FDD | IMD4 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 37.6 | FDD | IMD21,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.34.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.35	CA_n5-n7-n77

### 6.35.1	Configurations

Table 6.35.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n7A-n77A | n77A7,9 CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n7A-n77(2A) | n77A7,9 CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n5A-n7A-n77(3A) | n77A7,9 CA_n5A-n77A7 CA_n7A-n77A7 | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.35.2	Maximum output power

Table 6.35.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.35.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.35.3.1	Power class 2 case a, b

Based on calculation, IMD2 of dual UL CA_n5-n77 falls into n7 DL and IMD2/3/5 of dual UL CA_n7-n77 falls into n5 DL respectively, the MSD values are defined as below.

Table 6.35.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n7-n77 | n5 | N/A | 5 | N/A | 879 | 35.2 | FDD | IMD21,4 |
|  | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n77 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 35.1 | FDD | IMD2 |
|  | n77 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.35.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.36	CA_n7-n25-n77

### 6.36.1	Configurations

Table 6.36.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n25A-n77A | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25(2A)-n77A | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n25A-n77(2A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n25A-n77(3A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n25(2A)-n77(2A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25A-n77A | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25(2A)-n77A | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n25A-n77(2A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n25(2A)-n77(2A) | n77A7,9 CA_n7A-n77A7 CA_n25A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.36.2	Maximum output power

Table 6.36.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n25-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.36.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.36.3.1	Power class 2 case a, b

Based on calculation, IMD4 of dual UL CA_n7-n77 falls into n25 DL and IMD5 of dual UL CA_n25-n77 falls into n7 DL respectively. The MSD values are defined as below.

Table 6.36.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n25-n77 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1950 | 20.0 | FDD | IMD4 |
|  | n77 | 3525 | 10 | 50 | 3525 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2640 | 18.8 | FDD | IMD5 |
|  | n25 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n77 | 4125 | 10 | 50 | 4125 | N/A | TDD | N/A |

### 6.36.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.37	CA_n5-n25-n77

### 6.37.1	Configurations

Table 6.37.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n25A-n77A | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n77A | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n77(2A) | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n5A-n25A-n77(3A) | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n5A-n25(2A)-n77(2A) | n777, 9 CA_n5A-n77A7 CA_n25A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.37.2	Maximum output power

Table 6.37.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n25-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.37.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.37.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL n5-n77 fall into n25 DL, MSD values reused from CA_n2-n5-n77.

Based on calculation, IMD5 of dual UL n25-n77 fall into n5 DL, MSD values reused from CA_n2-n5-n77.

Table 6.37.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n25-n77 | n5 | N/A | 5 | N/A | 889 | 13.6 | FDD | IMD55 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987 | 24.8 | FDD | IMD3 |
|  | n77 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

### 6.37.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.38	CA_n5-n25-n78

### 6.38.1	Configurations

Table 6.38.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n25A-n78A | n787, 9 CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25(2A)-n78A | n787, 9 CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n25A-n78(2A) | n787, 9 CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n5A-n25(2A)-n78(2A) | n787, 9 CA_n5A-n78A7 CA_n25A-n78A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.38.2	Maximum output power

Table 6.38.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n25-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.38.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.38.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL n5-n78 fall into n25 DL, MSD values reused from CA_n2-n5-n77.

Based on calculation, IMD5 of dual UL n25-n78 fall into n5 DL, MSD values reused from CA_n2-n5-n77.

Table 6.38.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n25-n78 | n5 | N/A | 5 | N/A | 889 | 13.6 | FDD | IMD5 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987 | 24.8 | FDD | IMD3 |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |

### 6.38.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.39	CA_n7-n66-n77

### 6.39.1	Configurations

Table 6.39.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n66A-n77A | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66(2A)-n77A | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n66A-n77(2A) | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n66A-n77(3A) | n77A7,9 CA_n77(2A)7 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n66(2A)-n77(2A) | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66A-n77A | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66(2A)-n77A | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n66A-n77(2A) | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n66(2A)-n77(2A) | n77A7,9 CA_n7A-n66A CA_n7A-n77A7 CA_n66A-n77A7 | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.39.2	Maximum output power

Table 6.39.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n66-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.39.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.39.3.1	Power class 2 case a, b

Based on calculation, IMD4 of dual UL CA_n7-n77 falls into n66 DL and IMD5 of dual UL CA_n66-n77 falls into n7 DL respectively. The MSD values are defined as below.

Table 6.39.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n66-n77 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2150 | 20.5 | FDD | IMD4 |
|  | n77 | 3625 | 10 | 50 | 3625 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2640 | 18.8 | FDD | IMD5 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |

### 6.39.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.40	CA_n7-n71-n77

### 6.40.1	Configurations

Table 6.40.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n71A-n77A | n77A7,9 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n71A-n77(2A) | n77A7,9 CA_n77(2A) CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
| CA_n7A-n71A-n77(3A) | n77A7,9 CA_n77(2A) CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n7A-n71A-n77A | n77A7,9 CA_n7A-n71A CA_n7A-n77A7 CA_n71A-n77A7 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20, 25, 30, 35 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.40.2	Maximum output power

Table 6.40.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n71-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.40.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.40.3.1	Power class 2 case a, b

Based on calculation, IMD2 of dual UL CA_n71-n77 falls into n7 DL respectively. The MSD values are defined as below.

Table 6.40.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n71-n77 | n7 | N/A | 5 | N/A | 2670 | 34.6 | FDD | IMD2 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |

### 6.40.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.41	CA_n13-n25-n77

### 6.41.1	Configurations

Table 6.41.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n13A-n25A-n77A | n77A7,9 CA_n13A-n25A CA_n13A-n77A7 CA_n25A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n25A-n77(2A) | n77A7,9 CA_n77(2A) CA_n13A-n25A CA_n13A-n77A7 CA_n25A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.41.2	Maximum output power

Table 6.41.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n13-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n25-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.41.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.41.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL CA_n13-n77 falls into n25 DL respectively. The MSD values are defined as below.

Table 6.41.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n13-n25-n77 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n25 | N/A | 5 | N/A | 1960 | 24.0 | FDD | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | TDD | N/A |

### 6.41.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.42	CA_n1-n3-n77

### 6.42.1	Configurations

Table 6.42.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n77A | CA_n1A-n77A7 CA_n3A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n3A-n77(2A) | CA_n1A-n77A7 CA_n3A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.42.2	Maximum output power

Table 6.42.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.42.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.42.3.1	Power class 2 case a, b

Based on calculation, IMD2/4/5 of dual UL CA_n1A-n77A falls into n3 DL and IMD2/5 of dual UL CA_n3A-n77A falls into n1 DL respectively, the MSD exceptions are defined as below, which are reused from DC_1-3_n77.

Table 6.42.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1807.5 | 37.5 | FDD | IMD21,2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 37.0 | FDD | IMD21 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.42.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.43	DL CA_n3-n77-n79, UL CA_n3A-n77A

### 6.43.1	Configurations

Table 6.43.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A-n79A4 | CA_n3A-n77A7 CA_n3A-n79A CA_n77A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n3A-n77(2A)-n79A4 | CA_n3A-n77A7 CA_n3A-n79A CA_n77A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.43.2	Maximum output power

Table 6.43.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n77 | CA power class | Carrier n3 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.43.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.43.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n3A-n77A into CA n3-n77-n79, none is proposed for PC2.

#### 6.43.3.2	Power class 2 case b

There is no MSD for PC3 UL CA_n3A-n77A into CA n3-n77-n79, none is proposed for PC2.

### 6.43.4	Void

## 6.44	DL CA_n3-n77-n79, UL CA_n3A-n79A

### 6.44.1	Configurations

Table 6.44.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A-n79A4 | CA_n3A-n77A CA_n3A-n79A7 CA_n77A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n3A-n77(2A)-n79A4 | CA_n3A-n77A CA_n3A-n79A7 CA_n77A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.44.2	Maximum output power

Table 6.44.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n79 | CA power class | Carrier n3 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.44.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.44.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n3A-n79A into CA n3-n77-n79, none is proposed for PC2.

#### 6.44.3.2	Power class 2 case b

There is no MSD for PC3 UL CA_n3A-n79A into CA n3-n77-n79, none is proposed for PC2.

### 6.44.4	Void

## 6.45	DL CA_n3-n77-n79, UL CA_n77A-n79A

### 6.45.1	Configurations

Table 6.45.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n77A-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A7 | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n3A-n77(2A)-n79A4 | CA_n3A-n77A CA_n3A-n79A CA_n77A-n79A7 | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.45.2	Maximum output power

Table 6.45.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n77-n79 | CA power class | Carrier n77 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n77A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.45.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.45.3.1	Power class 2 case a, b, c and d

The MSD is same for case a, b ,c and d.

Power class 3 MSD for UL CA_n77A-n79A:

Table 6.45.3.1-1 Power class 3 MSD for 2 bands UL CA_n77A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n77-n79 | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n79 | 4840 | 40 | 216 | 4840 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | FDD | IMD31, 2 ¦2*fBn77-fBn79¦ |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD3 increases by 3*3=9dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.45.3.1-2.

Table 6.45.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n77-n79 | n77 | 3350 | 10 | 50 | 3350 | N/A | FDD | N/A |
|  | n79 | 4840 | 40 | 216 | 4840 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 24.2 | TDD | IMD31, 2 ¦2*fBn77-fBn79¦ |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.45.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.46	DL CA_n28-n77-n79, UL CA_n28A-n77A

### 6.46.1	Configurations

Table 6.46.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A-n79A4 | CA_n28A-n77A7 CA_n28A-n79A CA_n77A-n79A | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n28A-n77(2A)-n79A4 | CA_n28A-n77A7 CA_n28A-n79A CA_n77A-n79A | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.46.2	Maximum output power

Table 6.46.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n77 | CA power class | Carrier n28 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.46.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.46.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n28A-n77A into CA n28-n77-n79, none is proposed for PC2.

#### 6.46.3.2	Power class 2 case b

There is no MSD for PC3 UL CA_n28A-n77A into CA n28-n77-n79, none is proposed for PC2.

### 6.46.4	Void

## 6.47	DL CA_n28-n77-n79, UL CA_n28A-n79A

### 6.47.1	Configurations

Table 6.47.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A-n79A4 | CA_n28A-n77A CA_n28A-n79A7 CA_n77A-n79A | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n28A-n77(2A)-n79A4 | CA_n28A-n77A CA_n28A-n79A7 CA_n77A-n79A | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.47.2	Maximum output power

Table 6.47.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n79 | CA power class | Carrier n28 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.47.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.47.3.1	Power class 2 case a

There is no MSD for PC3 UL CA_n28A-n79A into CA n28-n77-n79, none is proposed for PC2.

#### 6.47.3.2	Power class 2 case b

There is no MSD for PC3 UL CA_n28A-n79A into CA n28-n77-n79, none is proposed for PC2.

### 6.47.4	Void

## 6.48	DL CA_n28-n77-n79, UL CA_n77A-n79A

### 6.48.1	Configurations

Table 6.48.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77A-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A7 | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | n77 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| CA_n28A-n77(2A)-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A7 | n28 | n28 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.48.2	Maximum output power

Table 6.48.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n77-n79 | CA power class | Carrier n77 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n77A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.48.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.48.3.1	Power class 2 case a, b, c and d

The MSD is same for case a, b ,c and d.

Power class 3 MSD for UL CA_n77A-n79A:

Table 6.48.3.1-1 Power class 3 MSD for 2 bands UL CA_n77A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 52 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 16.2 | FDD | IMD21, 2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD2 increases by 3*2=6dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.48.3.1-2.

Table 6.48.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 52 | 3620 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 22.7 | FDD | IMD21, 2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.48.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.49	DL CA_n3-n28-n79, UL CA_n3A-n79A

### 6.49.1	Configurations

Table 6.49.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n79A | CA_n3A-n79A7 CA_n28A-n79A | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n28 | n28 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.49.2	Maximum output power

Table 6.49.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n3-n79 | CA power class | Carrier n3 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.49.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.49.3.1	Power class 2 case a, b

The MSD is same for case a, b.

Power class 3 MSD for UL CA_n3A-n79A:

Table 6.49.3.1-1 Power class 3 MSD for 2 bands UL CA_n77A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n79 | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 10.3 | FDD | IMD4 ¦3*fBn3-fBn79¦ |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD4 increases by 3*4=12dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.49.3.1-2.

Table 6.49.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n79 | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 780 | 21.5 | FDD | IMD4 ¦3*fBn3-fBn79¦ |

### 6.49.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.50	DL CA_n3-n28-n79, UL CA_n28A-n79A

### 6.50.1	Configurations

Table 6.50.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n79A | CA_n3A-n79A CA_n28A-n79A7 | n3 | n3 channel bandwidth in Table 5.3.5-1 | 0 |
|  |  | n28 | n28 channel bandwidth in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidth in Table 5.3.5-1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.50.2	Maximum output power

Table 6.50.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28-n79 | CA power class | Carrier n28 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.50.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.50.3.1	Power class 2 case a, b

The MSD is same for case a, b.

Power class 3 MSD for UL CA_n28A-n79A:

Table 6.50.3.1-1 Power class 3 MSD for 2 bands UL CA_n28A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n79 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 5.7 | FDD | IMD5 ¦4*fBn28-fBn79¦ |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD5 increases by 3*5=15dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.50.3.1-2.

Table 6.50.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n79 | n28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1870 | 20.5 | FDD | IMD5 ¦4*fBn28-fBn79¦ |

### 6.50.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.51	CA_n1-n28-n41

### 6.51.1	Configurations

Table 6.51.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n28A-n41A | CA_n1A-n41A7 CA_n28A-n41A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.51.2	Maximum output power

Table 6.51.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.51.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.51.3.1	Power class 2 case a, b

Based on calculation, IMD2/5 of dual UL CA_n1A-n41A falls into n28 DL and no IMD issue for dual UL CA_n28-n41 falls into n1 DL, the MSD exceptions are defined as below

Table 6.51.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n41 | n1 | 1923 | 5 | 25 | 2113 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 762 | 36.6 | FDD | IMD21 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.51.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.52	CA_n18-n28-n77

### 6.52.1	Configurations

Table 6.52.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n28A-n77A | n77A7 CA_n18A-n77A7 CA_n28A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.52.2	Maximum output power

Table 6.52.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.52.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.52.3.1	Power class 2 case a, b

Based on calculation, IMD5 and IMD6 of dual UL CA_n18A-n77A fall into n18 DL and IMD5 and IMD7 of dual UL CA_n28A-n77A falls into n28 DL, the same as the two band cases. For UL CA_n18A-n77A MSD exceptions are defined [4-5] and are reflected as Table 6.52.3-1. For DL and UL CA_n28A-n77A, and victim n28, the MSD exceptions are already captured in the specification [1]. Hence the necessary MSD values for the two bands fallback cases are available, only 3 bands case need to be investigated.

Based on calculation, IMD5 of dual UL CA_n18A-n77A falls into n28 DL and IMD5 and IMD7 of dual UL CA_n28A-n77A falls into n18 DL, the MSD exceptions are defined as in yellow in Table 6.52.3-1. The MSD exception for UL CA_n18A-n77A falls into n28 DL is borrowed from Table 5.60.3-1 of 3GPP TR 38.898 V0.6.0 (2023-10)[6].

Table 6.52.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n28-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 778 | 17.5 | FDD | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 865 | 12.1 | FDD | IMD5 |
|  | n28 | 723 | 5 | 25 | 778 | N/A | FDD | N/A |
|  | n77 | 3757 | 10 | 52 | 3757 | N/A | TDD | N/A |

### 6.52.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.53	CA_n3-n18-n77

### 6.53.1	Configurations

Table 6.53.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n18A-n77 | n77A7 CA_n3A-n77A7 CA_n18A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n18A-n77(2A) | n77A7 CA_n3A-n77A7 CA_n18A-n77A7 | n3 | 5, 10, 15 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.53.2	Maximum output power

Table 6.53.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.53.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

The MSD exceptions to n3 downlink and n18 downlink due to single UL n77 need not to defined because it drops no IMD and no harmonic mixing into the n3 or n18 downlink.

#### 6.53.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL CA_n18A-n77A falls into n3 DL and IMD6 and 7 of dual UL CA_n3A-n77A falls into n18 DL respectively, IMD6 and IMD7 interferences are lower than REFESENS, so we don’t need to define the MSD for IMD6 and IMD7. The MSD exceptions due to CA_n18A-n77A falls into n3 DL are defined as below.

Table 6.53.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n18-n77 | n18 | 820 | 5 | 25 | 865 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1865 | 24.2 | FDD | IMD3 |
|  | n77 | 3505 | 10 | 50 | 3505 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.53.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA

## 6.54	CA_n18-n41-n77

### 6.54.1	Configurations

Table 6.54.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n41A-n77A | CA_n18A-n41A7 CA_n18A-n77A7 CA_n41A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n41A-n77(2A) | CA_n18A-n41A7 CA_n18A-n77A7 CA_n41A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.54.2	Maximum output power

Table 6.54.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.54.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.54.3.1	Power class 2 case a, b, c, d

Based on calculation, IMD 2/3/4/ of dual UL CA_n18A-n41A falls into n77 DL; IMD 2 and 3 of dual UL CA_n18A-n77A falls into n41 DL; and IMD 2/3/5 of dual UL CA_n41A-n77A falls into n18 DL respectively, the MSD exceptions are defined as below.

Table 6.54.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n41-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2570 | 5 | 25 | 2570 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3390 | 33.0 | TDD | IMD22,4 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | TDD | N/A |
|  | n41 | N/A | 5 | N/A | 2630 | 32.6 | TDD | IMD24 |
|  | n41 | 2590 | 10 | 50 | 2590 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 33.8 | FDD | IMD21,4 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

For information and for reference, the omitted MSD values for IMD3, IMD4 and IMD5 are shown in the following table.

They are the MSD values corresponding to the following:

1	 IMD 2, IMD3(2, 1), IMD3(1, -2), and IMD4(2, -2) of dual UL CA_n18A-n41A falls into n77 DL;

2	IMD3(2, 1) of dual UL CA_n18A-n77A falls into n41 DL;

3	 IMD3(2, -1), IMD5(3, -2) of dual UL CA_n41A-n77A falls into n18 DL.

Table: Additional 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n41-n77 | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | TDD | N/A |
|  | n77 | 4155 | 10 | N/A | 4155 | 22.9 | TDD | IMD3(2,1) |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2500 | 5 | 25 | 2500 | N/A | TDD | N/A |
|  | n77 | 4180 | 10 | N/A | 4180 | 23.0 | TDD | IMD3(1, -2) |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | TDD | N/A |
|  | n77 | 3390 | 10 | N/A | 3390 | 18.0 | TDD | IMD4(2,2) |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n77 | 4155 | 10 | 50 | 4155 | N/A | TDD | N/A |
|  | n41 | N/A | 5 | N/A | 2515 | 18.3 | TDD | IMD3(2, -1) |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | TDD | N/A |
|  | n77 | 4165 | 10 | 50 | 4165 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 865 | 21.6 | FDD | IMD3(2, -1) |
|  | n41 | 2640 | 5 | 25 | 2640 | N/A | TDD | N/A |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 865 | 11.4 | FDD | IMD5(3, -2) |

### 6.54.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.55	CA_n13-n66-n77

### 6.55.1	Configurations

Table 6.55.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n13A-n66A-n77A | n777, 9 CA_n13A-n66A CA_n13A-n77A7 CA_n66A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.55.2	Maximum output power

Table 6.55.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n13-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n66-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.55.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.55.3.1	Power class 2 case a, b

Based on coexistence studies, IMD3 of dual UL n13-n77 fall into n66 DL. Test points are reused from PC3 and PC3 MSD value n66 of 17.1 is recalculated using formula below.

Based on coexistence studies, IMD3 of dual UL n66-n77 fall into n13 DL, Test points are reused from PC3 and PC3 MSD value n13 of 15.2 is recalculated using formula below.

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 interference power increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

Formula above gives IMD3 PC2 MSD for n66 as 26.025 and IMD3 PC2 MSD for n13 as 24.082.

Table 6.55.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n13-n66-n77 | n13 | 782 | 5 | 25 | 751 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2146 | 26.0 | FDD | IMD3 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n13 | N/A | 5 | N/A | 750 | 24.1 | FDD | IMD3 |
|  | n66 | 1710 | 5 | 25 | 2110 | N/A | FDD | N/A |
|  | n77 | 4170 | 10 | 50 | 4170 | N/A | TDD | N/A |

### 6.55.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.56	CA_n1-n28-n77

### 6.56.1	Configurations

Table 6.56.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n28A-n77A | CA_n1A-n77A7 CA_n28A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n77(2A) | CA_n1A-n77A7 CA_n28A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.56.2	Maximum output power

Table 6.56.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.56.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.56.3.1	Power class 2 case a, b, c, d

Based on calculation, IMD5 of dual UL CA_n1A-n77A falls into n28 DL and IMD3 of dual UL CA_n28A-n77A falls into n1 DL respectively, the MSD exceptions are defined as below

Table 6.56.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 16.5 | FDD | IMD5 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2150 | 24.7 | FDD | IMD3 |

### 6.56.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.57	CA_n1-n41-n77

### 6.57.1	Configurations

Table 6.57.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n41A-n77A | CA_n1A-n41A7 CA_n1A-n77A7 CA_n41A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n41A-n77(2A) | CA_n1A-n41A7 CA_n1A-n77A7 CA_n41A-n77A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.57.2	Maximum output power

Table 6.57.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n1-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n41-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.57.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.57.3.1	Power class 2 case a, b, c, d

Based on calculation, IMD3/4/5 of dual UL CA_n1A-n41A falls into n77 DL, IMD4/5 of dual UL CA_n1A-n77A falls into n41 DL, and IMD4 of dual UL CA_n41A-n77A falls into n1 DL respectively, the MSD exceptions are defined as below

Table 6.57.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n41-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3330 | 28.2 | TDD | IMD31,2 |
|  | n1 | 1975 | 5 | 10 | 2165 | N/A | FDD | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 22.0 | TDD | IMD41 |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 21.4 | FDD | IMD4 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.57.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.58	CA_n7-n26-n78

### 6.58.1	Configurations

Table 6.58.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| CA_n7A-n26A-n78A | CA_n7A-n26A CA_n7A-n78A7 CA_n26A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7B-n26A-n78A | CA_n7A-n26A CA_n7A-n78A7 CA_n26A-n78A7 CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.58.2	Maximum output power

Table 6.58.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n26-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.58.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.58.3.1	Power class 2 case a, b

Based on coexistence studies, IMD2 of dual UL n7-n78 fall into n26 DL. Test points are reused from PC3 and the PC3 MSD value n26 of 30.2 is recalculated using formula below. The result gives a PC2 IMD2 n26 MSD value of 36.2.

Based on coexistence studies, IMD5 of dual UL n7-n78 fall into n26 DL. Test points are reused from PC3 and the PC3 MSD value n26 of 3.3 is recalculated using formula below. The result gives a PC2 IMD5 n26 MSD value of 15.7.

Based on coexistence studies, IMD2 of dual UL n26-n78 fall into n7 DL, Test points are reused from PC3 and the PC3 MSD value n7 of 30.1 is recalculated using formula below. The result gives a PC2 IMD2 n7 MSD value of 36.1.

MSD for PC2 UL CA is calculated from MSD for PC2 as follows:

If the input signal increases by 3 dB, the IMD3 interference power increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

Table 6.58.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n26-n78 | n7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 879 | 36.2 | FDD | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | TDD | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 875 | 15.7 | FDD | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2645 | 36.1 | FDD | IMD2 |
|  | n26 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | TDD | N/A |

### 6.58.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.59	DL CA_n1-n3-n79, UL CA_n1A-n79A

### 6.59.1	Configurations

Table 6.59.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n79A | CA_n1A-n3A CA_n1A-n79A7 CA_n3A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.59.2	Maximum output power

Table 6.59.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.59.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.59.3.1	Power class 2 case a, b

The MSD is same for case a, b.

Since there is no MSD for PC3 UL CA_n1A-n79A into CA_n1A-n3A-n79A, none proposed for PC2.

### 6.59.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.60	DL CA_n1-n3-n79, UL CA_n3A-n79A

### 6.60.1	Configurations

Table 6.60.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n79A | CA_n1A-n3A CA_n1A-n79A CA_n3A-n79A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.60.2	Maximum output power

Table 6.60.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.60.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.60.3.1	Power class 2 case a, b

The MSD is same for case a, b.

Power class 3 MSD for UL CA_n3A-n79A:

Table 6.60.3.1-1 Power class 3 MSD for 2 bands UL CA_n3A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n79 | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 3.6 | FDD | IMD5 |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD5 increases by 3*5=15dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.60.3.1-2.

Table 6.60.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n79 | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 18.7 | FDD | IMD5 |

### 6.60.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.61	DL CA_n1-n28-n79, UL CA_n1A-n79A

### 6.61.1	Configurations

Table 6.61.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n28A-n79A | CA_n1A-n28A CA_n1A-n79A7 CA_n28A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.61.2	Maximum output power

Table 6.61.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.61.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.61.3.1	Power class 2 case a, b

The MSD is same for case a, b.

Power class 3 MSD for UL CA_n1A-n79A:

Table 6.61.3.1-1 Power class 3 MSD for 2 bands UL CA_n1A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n79 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 788 | 15.2 | FDD | IMD32 |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD3 increases by 3*3=9dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.61.3.1-2.

Table 6.61.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n79 | n1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 788 | 24.2 | FDD | IMD32 |
| NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.61.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.62	DL CA_n1-n28-n79, UL CA_n28A-n79A

### 6.62.1	Configurations

Table 6.62.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n28A-n79A | CA_n1A-n28A CA_n1A-n79A CA_n28A-n79A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.62.2	Maximum output power

Table 6.62.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.62.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.62.3.1	Power class 2 case a, b

The MSD is same for case a, b.

Power class 3 MSD for UL CA_n28A-n79A:

Table 6.62.3.1-1 Power class 3 MSD for 2 bands UL CA_n28A-n79A

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n79 | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167.5 | 1.2 | FDD | IMD41 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3dB, the IMD4 increases by 3*4=12dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

The proposed value for PC2 UL CA MSD can be found in Table 6.62.3.1-2.

Table 6.62.3.1-2 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink  configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n79 | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2167.5 | 13.4 | FDD | IMD41 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

### 6.62.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.63	CA_n1-n3-n78

### 6.63.1	Configurations

Table 6.63.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n78A | n787,9 CA_n1A-n3A CA_n1A-n78A7 CA_n3A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3A-n78(2A) | n787,9 CA_n78(2A)7 CA_n1A-n3A CA_n1A-n78A7 CA_n3A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.63.2	Maximum output power

Table 6.63.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n3-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.63.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.63.3.1	Power class 2 case a, b

Based on calculation, IMD2 of dual UL CA_n1-n78 may fall into n3 DL, MSD value is already defined in TS 38-101-1.

Coexistence analysis for UL n78 and CA_n78(2A) for lower order fallbacks are performed in R4-2409353.

### 6.63.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.64	CA_n1-n28-n78

### 6.64.1	Configurations

Table 6.64.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n28A-n78A | n787,9 CA_n1A-n28A CA_n1A-n78A7 CA_n28A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n28A-n78(2A) | n787,9 CA_n78(2A)7 CA_n1A-n28A CA_n1A-n78A7 CA_n28A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.64.2	Maximum output power

Table 6.64.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.64.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.64.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n1-n78 fall into n28 DL, IMD3 of dual UL CA_n28-n78 falls into n1 DL.

Coexistence analysis for UL n78 and CA_n78(2A) for lower order fallbacks are performed in R4-2409353.

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3 dB, the IMD3 interference power increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

Table 6.25.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n78 | n1 | N/A | 5 | N/A | 2150 | 24.6 | FDD | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 794 | 17.2 | FDD | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | TDD | N/A |

### 6.64.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.65	CA_n3-n7-n78

### 6.65.1	Configurations

Table 6.65.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n7A-n78A | n787,9 CA_n3A-n7A CA_n3A-n78A7 CA_n7A-n78A7 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n7A-n78(2A) | n787,9 CA_n78(2A)7 CA_n3A-n7A CA_n3A-n78A7 CA_n7A-n78A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.65.2	Maximum output power

Table 6.65.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n7-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.65.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.65.3.1	Power class 2 case a, b

Based on calculation, IMD3 and IMD4 of dual UL CA_n7-n78 falls into n3 DL.

Coexistence analysis for UL n78 and CA_n78(2A) for lower order fallbacks are performed in R4-2409353.

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3 dB, the IMD3 interference power increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

Table 6.65.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n7-n78 | n3 | N/A | 5 | N/A | 1820 | 26.6 | FDD | IMD3 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1820 | 17 | FDD | IMD4 |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | TDD | N/A |

### 6.65.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.66	CA_n3-n28-n78

### 6.66.1	Configurations

Table 6.66.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n28A-n78A | n787,9 CA_n3A-n28A CA_n3A-n78A7 CA_n28A-n78A7 | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n28 | 5, 10 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n3A-n28A-n78(2A) | n787,9 CA_n3A-n28A CA_n3A-n78A7 CA_n28A-n78A7 | n3 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 202 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  | n787,9 CA_n78(2A)7 CA_n3A-n28A CA_n3A-n78A7 CA_n28A-n78A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.66.2	Maximum output power

Table 6.66.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.66.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.66.3.1	Power class 2 case a, b

Based on calculation, IMD3 of dual UL CA_n28-n78 may fall into n3 DL.

Coexistence analysis for UL n78 and CA_n78(2A) for lower order fallbacks are performed in R4-2409354.

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3 dB, the IMD3 interference power increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

Table 6.66.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n28-n78 | n3 | N/A | 5 | N/A | 1850 | 29.2 | FDD | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |

### 6.66.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.67	CA_n7-n28-n78

### 6.67.1	Configurations

Table 6.67.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n28A-n78A | n787,9 CA_n7A-n78A7 CA_n28A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  | n787,9 CA_n7A-n28A CA_n7A-n78A7 CA_n28A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 90, 100 |  |
| CA_n7A-n28A-n78(2A) | n787,9 CA_n78(2A)7 CA_n7A-n28A CA_n7A-n78A7 CA_n28A-n78A7 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.67.2	Maximum output power

Table 6.67.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n7-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.67.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.67.3.1	Power class 2 case a, b

Coexistence studies for UL CA_n28-n78 and CA_n7-n78 are already included in TS 38.101-1.

UL n78 and  CA_n78(2A) are already defined for lower order fallbacks in TS 38.101-1.

### 6.67.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

## 6.68	CA_n1-n77-n79

### 6.68.1	Configurations

Table 6.68.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n77A-n79A4 | CA_n1A-n77A7 CA_n1A-n79A7 CA_n77A-n79A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n1A-n77(2A)-n79A4 | CA_n1A-n77A7 CA_n1A-n79A7 CA_n77A-n79A7 CA_n77A(2A)7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A) BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.68.2	Maximum output power

Table 6.68.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n1A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n77A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

### 6.68.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.68.3.1	Power class 2 case a, b, c, d

MSD analysis of ULCA_n77(2A) can be skipped. Because the impact of it has already been investigated in PC3’s fallback combination.

Based on PC3’s co-existence studies of 2UL/2DL CA_n1-n77, CA_n1-n79, and CA_n77-n79 in TR38.717-03-02[9], own Rx impact of the 3rd band is the followings

-3rd, 4th and 5th order IMD generated by dual uplink of CA_n1-n77 may fall into part of own band n79.

=> There is no need to have MSD added because non-simultaneous Rx-Tx operation for CA_n77-n79 is assumed, therefore no MSD is needed for CA_n1-n77.

-5th order IMD generated by dual uplink of CA_n1-n79 may fall into part of own band n77.

=> There is no need to have MSD added because non-simultaneous Rx-Tx operation for CA_n77-n79 is assumed, therefore no MSD is needed for CA_n1-n79.

-3rd, 4th and 5th order IMD generated by dual uplink of CA_n77-n79 may fall into part of own band n1.

=> The MSD values are shown in the following table. This MSD value is the average of the analysis results of the two companies. Also, we define only the highest MSD, and omit the MSDs due to IM4 and IM5. This is described in NOTE2 and NOTE1.

Table 6.68.3-1: MSD for the CA configuration

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n1A-n77A-n79A | n1 | 1950 | 5 | 25 | 2140 | 24.6 | IMD31,2 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

### 6.68.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.69	CA_n1-n-n78

### 6.69.1	Configurations

Table 6.69.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n7A-n78A | n787,9 CA_n1A-n7A CA_n1A-n78A7 CA_n7A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 901, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 901, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7A-n78(2A) | n787,9 CA_n1A-n7A CA_n1A-n78A7 CA_n7A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | n787,9 CA_n78(2A)7 CA_n1A-n7A CA_n1A-n78A7 CA_n7A-n78A7 | n1 | 5, 10, 15, 20 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.69.2	Maximum output power

Table 6.69.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n7-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.69.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.69.3.1	Power class 2 case a, b

Based on calculation, IMD4 of dual UL CA_n1-n78 may fall into n7 DL, IMD4 of dual UL CA_n7-n78 falls into n1 DL.

Coexistence analysis for UL n78 and CA_n78(2A) for lower order fallbacks are performed in R4-2409353.

MSD for PC2 UL CA is calculated from MSD for PC3 as follows:

If the input signal increases by 3 dB, the IMD3 interference power increases by 3*3=9 dB.

MSD due to interference power is given by

where N is the noise spectral density and BW is the bandwidth of the carrier. If the initial MSD is known,

then we have

If is increased by dB, then is given by

Table 6.69.3-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n7-n78 | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |
|  | n7 | N/A | 5 | N/A | 2627.5 | 20.6 | FDD | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 20.1 | FDD | IMD4 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |

### 6.69.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA, so this section is omitted.

# 7	Power class 2 CA with SUL

## 7.x	CA_nW-nX_SUL_nY-nZ

### 7.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5C-4 unless additional clarification>

Table 7.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink configuration | NR Band | Channel Bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nW-nX_SUL_nY-nZ | SUL_nY-nZ | nW |  |  |
|  |  | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |

### 7.x.2	Maximum output power

### 7.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

### 7.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

# Annex <A> (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-11 | RAN4 #105 | R4-2219272 |  |  |  | TR skeleton | 0.0.1 |
| 2022-11 | RAN4#105 | R4-2220844 |  |  |  | The following TPs are approved: R4-2220470 TP for HPUE CA_n77-n79 with 1UL for TR 38.899, NTT DOCOMO Inc R4-2220471 TP for HPUE CA_n78-n79 with 1UL for TR 38.899, NTT DOCOMO Inc R4-2220472 TP for TR38.899 for DL CA_n25-n41-n66 with PC2 2BUL, T-Mobile USA R4-2220004 TP for TR38.899 for DL CA_n25-n41-n71 with PC2 2BUL, T-Mobile USA R4-2220475 TP for TR38.899 for DL CA_n25-n41-n77 with PC2 2BUL, T-Mobile USA R4-2220476 TP for TR38.899 for DL CA_n25-n66-n77 with PC2 2BUL, T-Mobile USA R4-2220477 TP for TR38.899 for DL CA_n25-n71-n77 with PC2 2BUL, T-Mobile USA R4-2220008 TP for TR38.899 for DL CA_n41-n66-n71 with PC2 2BUL, T-Mobile USA R4-2220478 TP for TR38.899 for DL CA_n41-n66-n77 with PC2 2BUL, T-Mobile USA R4-2220479 TP for TR38.899 for DL CA_n41-n71-n77 with PC2 2BUL, T-Mobile USA R4-2220480 TP for TR38.899 for DL CA_n66-n71-n77 with PC2 2BUL, T-Mobile USA R4-2220012 TP for TR38.899 for DL CA_n25-n41C with PC2 UL CA_n41C, T-Mobile USA R4-2220013 TP for TR38.899 for DL CA_n41C-n66 with PC2 UL CA_n41C, T-Mobile USA R4-2220014 TP for TR38.899 for DL CA_n41C-n71 with PC2 UL CA_n41C, T-Mobile USA R4-2220015 TP for TR38.899 for DL CA_n41C-n77 with PC2 UL CA_n41C, T-Mobile USA | 0.1.0 |
| 2023-03 | RAN4#106 | R4-2300720 |  |  |  | The following TPs are approved: R4-2303453 TP for HPUE CA_n3-n41 with 1UL for TR 38.899, Samsung, KDDI R4-2303456 TP for HPUE CA_n3-n77 with 1UL and 2UL for TR 38.899, Samsung, KDDI R4-2303528 TP for TR38.899 PC2 CA_n8A-n78A, ZTE Corporation, China Unicom, CHTTL R4-2303454 TP for HPUE CA_n40-n77 with 1UL for TR 38.899, Samsung, KDDI R4-2301130 TP for HPUE CA_n3-n28-n41 with 2UL for TR 38.899, Samsung, KDDI R4-2301132 TP for HPUE CA_n3-n41-n77 with 2UL for TR 38.899, Samsung, KDDI | 0.2.0 |
| 2023-04 | RAN4#106bis-e | R4-2304522 |  |  |  | The following TPs are approved: R4-2305020 TP for HPUE CA_n28-n77 with 1UL and 2UL for TR 38.899 Samsung, KDDI, Qualcomm R4-2305018 TP for HPUE CA_n3-n28-n77 with 1UL and 2UL for TR 38.899 Samsung, KDDI, Qualcomm R4-2305019 TP for HPUE CA_n28-n41-n77 with 1UL and 2UL for TR 38.899 Samsung, KDDI, Qualcomm | 0.3.0 |
| 2023-05 | RAN4#107 | R4-2307920 |  |  |  | The following TPs are approved: R4-2308074 TP to TR 38.899 Addition of CA_n2-n48-n77, Nokia, Verizon R4-2308075 TP to TR 38.899 Addition of CA_n5-n48-n77, Nokia, Verizon R4-2308076 TP to TR 38.899 Addition of CA_n48-n66-n77, Nokia, Verizon R4-2308139 TP for HPUE CA_n1-n41 with 1UL and 2UL for TR 38.899, Samsung, KDDI R4-2308140 TP for HPUE CA_n1-n77 with 1UL and 2UL for TR 38.899, Samsung, KDDI R4-2309379 TP for HPUE CA_n2A-n5A-n77C with 2UL for TR 38.899, Samsung, Verizon, Nokia | 0.4.0 |
| 2023-08 | RAN4#108 | R4-2313487 |  |  |  | The following TPs are approved: R4-2311967 TP for HPUE CA_n1-n3-n41 with 2UL for TR 38.899, Samsung, KDDI R4-2313322 TP for 38.899 adding HPUE for CA_n25-n78, Ericsson, Bell Mobility R4-2313330 TP for 38.899 adding HPUE for CA_n66-n78, Ericsson, Bell Mobility R4-2313324 TP for 38.899 adding HPUE for CA_n7-n66-n78, Ericsson, Bell Mobility R4-2314637 TP for 38.899 adding HPUE for CA_n7-n77, Ericsson, Bell Mobility R4-2314638 TP for 38.899 adding HPUE for CA_n25-n66-n78, Ericsson, Bell Mobility R4-2314639 TP for adding UL CA_n77(2A) to DL CA_n1A-n77(2A) for PC2 HPUE in TR 38.899, KDDI, Samsung, Qualcomm R4-2314640 TP for adding UL CA_n77(2A) to DL CA_n3A-n77(2A) for PC2 HPUE in TR 38.899, KDDI, Samsung, Qualcomm R4-2314689 TP for adding UL CA_n77(2A) to DL CA_n28A-n77(2A) for PC2 HPUE in TR 38.899, KDDI, Samsung, Qualcomm R4-2314901 TP for TR38.899: CA_n77-n79, CA_n3-n77, CA_n28-n77, CA_n1-n77, CA_n1-n79, CA_n8-n79, SoftBank, LG Electronics | 0.5.0 |
| 2023-10 | RAN4#108bis | R4-2316479 |  |  |  | The following TPs are approved: R4-2315470 TP for TR 38.899 to include HPUE CA_n13-n77 Samsung, TELUS, Bell Mobility R4-2316418 TP for 38.899 adding non-contiguous UL to CA_n7-n77 Ericsson, Bell Mobility, Huawei, HiSiicon R4-2316419 TP for 38.899 adding PC1.5 single UL and PC2 non-contiguous UL to CA_n5-n77 Ericsson, Bell Mobility R4-2316420 TP for 38.899 adding PC1.5 single UL and PC2 non-contiguous UL to CA_n66-n77 Ericsson, Bell Mobility R4-2316421 TP for 38.899 adding PC1.5 single UL and PC2 non-contiguous UL to CA_n25-n77 Ericsson, Bell Mobility R4-2317581 TP for TR 38.899 to include HPUE CA_n5-n7-n77 Samsung, TELUS, Bell Mobility R4-2317582 TP for TR 38.899 to include HPUE CA_n7-n25-n77 Samsung, TELUS, Bell Mobility R4-2317584 TP for 38.899 adding HPUE CA_n5-n25-n77 Ericsson, Bell Mobility R4-2317585 TP for 38.899 adding HPUE CA_n5-n25-n78 Ericsson, Bell Mobility R4-2317676 TP for 38.899 to include HPUE CA_n7-n66-n77 Nokia, TELUS, Bell Mobility R4-2317677 TP for 38.899 to include HPUE CA_n7-n71-n77 Nokia, TELUS, Bell Mobility R4-2317678 TP for 38.899 to include HPUE CA_n13-n25-n77 Nokia, TELUS, Bell Mobility | 0.6.0 |
| 2023-11 | RAN#109 | R4-2320676 |  |  |  | The following TPs are approved: R4-2319769 TP for TR 38.899 to include HPUE CA_n71-n78 Samsung, TELUS, Bell Mobility R4-2319771 TP for HPUE CA_n1-n3-n77 with 2UL for TR 38.899 Samsung, KDDI, Qualcomm R4-2320313 TP for 38.899 adding PC2 non-contiguous UL to CA_n5-n78 Ericsson, Bell Mobility R4-2321704 TP for TR38.899 PC2 NRCA for FR1 2DL 1or2UL and PC1.5 NRCA for FR1 2DL and 1UL SoftBank, LG Electronics R4-2321705 TP for HPUE CA_n1-n28-n41 with 2UL for TR 38.899 Samsung, KDDI, LGE R4-2321707 TP for adding UL CA_n77(2A) to DL CA_n3A-n28A-n77(2A) for PC2 HPUE in TR 38.899 Samsung, KDDI, Qualcomm R4-2321708 TP for PC2 HPUE CA_n18-n41 with 2UL for TR 38.899 Samsung, KDDI, Qualcomm R4-2321709 TP for PC2 HPUE CA_n18-n77 with 2UL in TR 38.899 KDDI, Samsung, LGE, Murata, Skyworks R4-2321710 TP for PC2 HPUE CA_n18-n28-n77 with 2UL in TR 38.899 KDDI, Samsung, Murata R4-2321978 TP for HPUE CA_n3-n18-n77 with 2UL in TR 38.899 KDDI, Samsung, LGE | 0.7.0 |
| 2024-03 | RAN#110 | R4-2402217 |  |  |  | The following TPs are approved: R4-2403618 [HPUE_FR1_TDD_NR_CADC_SUL_R18] TP for adding 1cc UL to CA_n28A-n41A-n77A for PC1.5 HPUE in TR 38.899, KDDI R4-2403620 [HPUE_FR1_TDD_NR_CADC_SUL_R18] TP for inter-band 2UL to CA_n18A-n41A-n77A for PC2 HPUE in TR 38.899, KDDI R4-2400331 TP for TR38.899 to include new HP-NRCA combinations for FR1, SoftBank R4-2403847 TP for TR38.899: Addition of uplink configurations to CA_n8A-n78A, SoftBank R4-2403621 TP for 38.899 adding CA_n78(2A) PC2 UL to CA_n7A-n78(2A), Ericsson, Bell Mobility, TELUS R4-2401474 TP for 38.899 adding CA_n78(2A) PC2 UL to CA_n66A-n78(2A), Ericsson, Bell Mobility, TELUS R4-2401475 TP for 38.899 adding CA_n78(2A) PC2 UL to CA_n25A-n78(2A), Ericsson, Bell Mobility, TELUS R4-2401476 TP for 38.899 adding CA_n77(2A) PC2 UL to CA_n25(2A)-n77(2A), Ericsson, Bell Mobility, TELUS R4-2403826 TP for 38.899 adding PC2 UL to CA_n13A-n66A-n77A, Ericsson, Bell Mobility, TELUS R4-2402356 TP for TR 38.899 to include HPUE CA_n71-n77 with UL CA_n77(2A), Samsung, TELUS, Bell Mobility R4-2402362 TP for HPUE CA_n1-n28-n77 with 2UL for TR 38.899, Samsung, KDDI Corporation, Qualcomm Incorporated, SoftBank Corp. R4-2403622 TP for HPUE CA_n1-n41-n77 with 2UL for TR 38.899, Samsung, KDDI Corporation, Qualcomm Incorporated, LGE R4-2403623 TP for TR38.899 for DL CA_n77A-n85A with UL PC2 CA_n77A-n85A and PC2 and PC1.5 UL n77, T-Mobile USA | 0.8.0 |
| 2024-04 | RAN4#110bis | R4-2404883 |  |  |  | The following TPs are approved: R4-2406559 TP for TR38.899 to add HP-NRCA n28-n77, n41-n77 and n77-n79, SoftBank Corp. R4-2406560 TP for TR38.899 to add HP NRCA n1-n41, n1-n3-n79 and n1-n28-n79, SoftBank Corp., LG Electronics R4-2406561 TP to TR 38.899 Addition of PC2 and PC1.5 for CA_n40A-n77A, Nokia, NBN R4-2406562 TP to TR 38.899 Addition of PC2 and PC1.5 for CA_n40A-n78A, Nokia, NBN R4-2406563 (HPUE_FR1_TDD_NR_CADC_SUL_R18) TP for TR 38.899 to introduce PC2 and PC1.5 CA_n39-n41, CMCC, ZTE Corporation, Huawei, HiSilicon, Murata Manufacturing Co Ltd. R4-2406564 (HPUE_FR1_TDD_NR_CADC_SUL_R18) TP for TR 38.899 to introduce PC2 and PC1.5 CA_n28A-n40A, CMCC, ZTE Corporation, Huawei, HiSilicon, Murata Manufacturing Co Ltd. R4-2405372 (HPUE_FR1_TDD_NR_CADC_SUL_R18) TP for TR 38.899 to introduce PC2 CA_n3A-n41C, CMCC, ZTE Corporation, Huawei, HiSilicon, Murata Manufacturing Co Ltd. R4-2405373 (HPUE_FR1_TDD_NR_CADC_SUL_R18) TP for TR 38.899 to introduce PC2 and PC1.5 CA_n40A-n41A, CMCC, ZTE Corporation, Huawei, HiSilicon, Murata Manufacturing Co Ltd. | 0.9.0 |
| 2024-05 | RAN4#111 | R4-2409642 |  |  |  | The following TPs are approved: R4-2407169 TP for TR38.899 to add new HP-NRCA combinations for FR1, SoftBank Corp. R4-2410541 TP for TR38.899 to add new HP-NRCA 1-77-79 with 2UL, SoftBank Corp. LGE R4-2409239 TP for adding UL CA_n77(2A) to HPUE CA_n3-n41-n77(2A) for TR 38.899, KDDI, Samsung, Qualcomm R4-2409298 TP for adding UL CA_n77(2A) to HPUE CA_n28-n41-n77(2A) for TR 38.899, KDDI R4-2409346 TP for 38.899 to add PC2 and PC1.5 UL to CA_n1-n3-n78, Ericsson, BT plc R4-2410542 TP for 38.899 to add PC2 and PC1.5 UL to CA_n1-n7-n78, Ericsson, BT plc R4-2409348 TP for 38.899 to add PC2 and PC1.5 UL to CA_n1-n28-n78, Ericsson, BT plc R4-2409349 TP for 38.899 to add PC2 and PC1.5 UL to CA_n3-n7-n78, Ericsson, BT plc R4-2409350 TP for 38.899 to add PC2 and PC1.5 UL to CA_n3-n28-n78, Ericsson, BT plc R4-2409351 TP for 38.899 to add PC2 and PC1.5 UL to CA_n7-n28-n78, Ericsson, BT plc R4-2410543 TP for 38.899 to add CA_n78(2A) PC2 UL to CA_n1A-n78(2A), Ericsson, BT plc R4-2410544 TP for 38.899 to add CA_n78(2A) PC2 UL to CA_n3A-n78(2A), Ericsson, BT plc R4-2409354 TP for 38.899 to add CA_n78(2A) PC2 UL to CA_n28A-n78(2A), Ericsson, BT plc R4-2407948 (HPUE_FR1_TDD_NR_CADC_SUL_R18) TP for TR 38.899 to introduce PC2 and PC1.5 CA_n3A-n40A, CMCC, Murata Manufacturing Co Ltd., ZTE Corporation | 0.10.0 |