3GPP TSG-RAN WG4 Meeting #112	R4-2411019

Maastricht, Netherlands, 19 Aug 2024 - 23 Aug 2024

Source:	Charter Communications

Title:	Adding a summary sentence in sub-clause 12.2.1

Agenda item:	10.12 Evolution of NR duplex operation: Sub-band full duplex [NR_duplex_evo]

Document for:	Agreement

Introduction

In TR 38.858 paragraph 12.2.1 North America, we propose to add a summary sentence regarding SBFD: “Therefore, it is expected that new SBFD operators in AMBIT band or in C-Band will seek a fair coexistence with legacy TDD operating in CBRS band.”

Discussion

A recent coexistence study [1] between two adjacent carriers showed that both the SBFD network and the legacy-TDD network will face some degradation due to adjacent-channel interference.

The following diagram shows that in North America the operation of CBRS band is limited in power to 35 dB below its adjacent bands (AMBIT and C-Band)

Observation 1:   New SBFD operation in locations nearby legacy CBRS operators may cause adjacent channel interference to the legacy networks. Hence, in TR 38.858 paragraph 12.2.1 North America, we propose to add a summary sentence regarding SBFD: “Therefore, it is expected that new SBFD operators in AMBIT band or in C-Band will seek a fair coexistence with legacy TDD operating in CBRS band.”

Observation 2:   In the recent 3GPP RAN4 way forward [3], the below proposal is agreed.

Observation 3:   Since TR 38.858 is owned by RAN1 and paragraph 12.2.1 is owned by RAN4, we propose in both RAN1 [4] and in RAN4 [5] to agree to change the text below.

Proposal:   In TR 38.858 paragraph 12.2.1 North America, we propose to add a summary sentence regarding SBFD: “Therefore, it is expected that new SBFD operators in AMBIT band or in C-Band will seek a fair coexistence with legacy TDD operating in CBRS band.”

Conclusion

The following Observations and Proposals were made:

Observation 1:   New SBFD operation in locations nearby legacy CBRS operators may cause adjacent channel interference to the legacy networks. Hence, in TR 38.858 paragraph 12.2.1 North America, we propose to add a summary sentence regarding SBFD: “Therefore, it is expected that new SBFD operators in AMBIT band or in C-Band will seek a fair coexistence with legacy TDD operating in CBRS band.”

Observation 2:   In the recent 3GPP RAN4 way forward [3], the below proposal is agreed.

Observation 3:   Since TR 38.858 is owned by RAN1 and paragraph 12.2.1 is owned by RAN4, we propose in both RAN1 [4] and in RAN4 [5] to agree to change the text below.

Proposal:   In TR 38.858 paragraph 12.2.1 North America, we propose to add a summary sentence regarding SBFD: “Therefore, it is expected that new SBFD operators in AMBIT band or in C-Band will seek a fair coexistence with legacy TDD operating in CBRS band.”  Since RAN1 owns TR 38.858 and RAN4 owns sub-clause 12.2.1, RAN4 would need to endorse this CR before an LS is going out to RAN1 for final approval.

- References

- TR 38.858, “Study on Evolution of NR Duplex Operation”

- RP-240789 WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2409958 Way Forward for [111][313] NR_duplex_evo

- R4-2411019 “CR for Adding a summary sentence in sub-clause 12.2.1”