3GPP TSG-RAN WG4 Meeting #112												R4-2411023

Maastricht, The Netherlands, 19 – 23 August, 2024

Agenda item:			8.12.2.3

Source:	Huawei, HiSilicon

Title:	On MIMO dynamic OTA tests

Document for:	approval

# Introduction

Release 18 MIMO OTA work item uses fixed reference channel. While Release 19 dynamic MIMO OTA tests require variable reference channels. Test equipment plays the role of gNB and determines MCS (Modulation Coding Scheme) index among other parameters to be used in downlink transmission. In principle, test equipment from different manufacturers should behave in exactly the same way when selecting MCS index based on CSI (Channel State Information) and RI (Rank Indicator) feedback from UE. Otherwise UE performance might not be identical under the same channel condition but with test equipment from different suppliers.

This contribution proposes options for this issue and comment on channel composition among other issues.

# Discussion

The WF [5] in the last meeting reaches the following agreement.

Issue 4-2-2: Link adaption configuration for Dynamic FR1 MIMO OTA

Agreements

  - RAN4 adopt dynamic link adaptation. FFS details, e.g.,

    - mapping tables between CQI and MCS for available rank values

Issue 4-2-3: Test system Dynamic-link-adaption verification

Agreements

  - RAN4 should study a procedure to verify the consistency of different test equipment/test system on Scheduling Algorithm in dynamic channel model system. This can be considered as an additional aspect of system validation.

Most of the common parameters defined in Table 8.2-1 in TS 38.827 [1] can still be used in dynamic MIMO OTA test. However, an additional parameter is needed to determine the MCS index to be used in the downlink transmission. Tables in Annex A of TS 38.101-4 provide correspondence between UE CQI feedback and suggested MCS index to be used in the downlink transmission. However, in reality, such a correspondence between CQI and MCS is left to gNB implementation. Instead of defining a new mapping between CQI and MCS, e.g. based on BLER outer loop link adaptation [3], it would be simpler to use the mapping between CQI and MCS defined in Annex A of 38.101-4.

In TS 38.214 [4], Table A.4-1 is not suitable as it limits the modulation to 64 QAM, Table A.4-4 is based on Table 5.2.2.1-4 (aka CQI Table 3) and is for ULLRC, for example, it requires an initial BLER (Block Error Rate) of 0.00001 compared to a typical BLER of 0.1 for other CQI tables. As MIMO OTA tests are typical communication services, Table A.4-4, aka CQI Table 3 in [4], can be ruled out. Table A.4-5 includes 1024 QAM modulation, but limited to one MIMO layer. However, the percentage of commercial devices supporting 1024 QAM could be limited.

Table A.4-2 and Table A.4-3 in 38.101-4 [2] are based on Table 5.2.2.1-3, aka CQI Table 2 in [4], and are for rank 1, 2, 3 and 4 with modulations of up to 256QAM. These two tables are appropriate for MIMO OTA tests.

Proposal 1: test equipment shall implement the mapping between MCS and CQI/rank according to Table A.4-2 and Table A.4-3 in the annex of TS 38.101-4 to ensure consistency in downlink resources allocations among test equipment vendors.

Resource allocations can be recorded together with CQI and rank values in a file by test equipment. Deviation from information bit payload specified in Table A.4-2 or Table A.4-3 in the annex of TS 38.101-4 is then calculated per CQI value. The reason for using information bit payload rather than MCS index is that MCS index values are independent of rank values. This makes it difficult to ascertain if the rank values reported by UE is utilised or not. In commercial networks, gNB scheduler behaviour is not specified in 3GPP. For example, gNB could ignore the rank value reported by UE and assign a different rank in resource allocation. However, it is important to achieve test consistency across various test equipment vendor, hence the proposal 1 and 2.

The reason to use zero bit as pass/fail threshold is that the difference should be zero because it is a look-up table mapping arithmetic with no other uncertainty.

Proposal 2: use maximum of absolute differences in information bit payloads across all CQI/rank values between Table A.4-2 or Table A.4-3 in TS 38.101-4 and test equipment allocation as the validation metric. The pass/fail threshold is zero bit.

WF [5] agrees to use UMa and UMi channels as below.

Issue 4-1-1: Alignment of dynamic channel model parameters in different SDOs

Agreements

  - RAN4 adopt the UMa and the UMi dynamic channel models defined in CTIA. FFS whether minor modification is needed.

  - RAN4 develop few new channel models is not precluded.

UMi and Uma have different propagation pathloss models in terms base station height and shadow fading. However, the CDL (Cluster Delay Line) defined TR 38.901 [6] can be used for both UMi and UMa channel models. It is difficult to come up with a representative channel model suitable for all environments. Based on the above agreement, it would be convenient to use 50% UMi and 50% UMa along the dynamic route because it is difficult to come up with a mixture of UMi and UMa to satisfy all propagation scenarios.

Proposal 3: use 50% UMi and 50% UMa along the test route of dynamic channel

The equal share approach could be applied cluster models too.

Proposal 4: UMi and UMa have 5 segments each with the 5 segments represented by CDL-A, CDL-B, CDL-C, CDL-D and CDL-E, respectively.

Spatially consistent UT/BS mobility modelling in Section 7.6.3.2 of TS 38.901 [6] outlines two procedures for UE mobility modelling, viz. Procedure A and Procedure B. Procedure A maintains spatially consistent powers/delays/angles of clusters during UE mobility, while Procedure B generates channel realizations separately for all links to different UE positions with modified procedures to ensure that the spatial or time evolution of delays and angles are within reasonable limits.

Modelling methods [8, 9] published in open literature align with Procedure A in addition to the methods using Markov process [10, 11] to represent birth or death of clusters. However, method using Markov process has not been fully standardised in 3GPP. It is therefore prudent to use Procedure A to model UE mobility in order to maintain consistency with other standard organisations.

Proposal 5: use Procedure A in Section 7.6.3.2 of TS 38.901 to model UE mobility.

In addition, a limit in Procedure A is imposed on update distance as “The updated distance of UT/BS should be limited within 1 meter, i.e. v*t < 1 meter”.

Observation 1: Procedure A has the following limit “The updated distance of UT/BS should be limited within 1 meter, i.e. v*t < 1 meter”.

As used by other standard organisation [7], linear interpolation is a simple way to ensure continuity between segments with different number of clusters. For example, power of a non-existent cluster is assumed to be -100 dB.

Proposal 6: use linear interpolation between different segments

HARQ is a re-transmission mechanism to deal with packet loss in variable or fading channels and should be allowed under dynamic MIMO channel tests. Otherwise, it would not represent real performance if maximum number of HARQ transmissions is fixed to one. The current value for “maximum number of HARQ transmission” is one for FR1 in Table E.1-1 of TS 38.151 [12].

The proposed maximum number of HARQ transmissions of 8 is in square bracket and is up for discussion. Because the value of 8 maximum HARQ re-transmissions is a typical value used in networks.

Proposal 7: set the maximum number of HARQ transmissions to [8] for all configurations, namely TDD, FDD, 2x2 and 4x4 MIMO.

# Conclusion

This contribution provides the following proposals and observation.

Proposal 1: test equipment shall implement the mapping between MCS and CQI/rank according to Table A.4-2 and Table A.4-3 in the annex of TS 38.101-4 to ensure consistency in downlink resources allocations among test equipment vendors.

Proposal 2: use maximum of absolute differences in information bit payloads across all CQI/rank values between Table A.4-2 or Table A.4-3 in TS 38.101-4 and test equipment allocation as the validation metric. The pass/fail threshold is zero bit.

Proposal 3: use 50% UMi and 50% UMa along the test route of dynamic channel

Proposal 4: UMi and UMa have 5 segments each with the 5 segments represented by CDL-A, CDL-B, CDL-C, CDL-D and CDL-E, respectively.

Proposal 5: use Procedure A in Section 7.6.3.2 of TS 38.901 to model UE mobility.

Observation 1: Procedure A has the following limit “The updated distance of UT/BS should be limited within 1 meter, i.e. v*t < 1 meter”

Proposal 6: use linear interpolation between different segments.

Proposal 7: set the maximum number of HARQ transmissions to [8] for all configurations, namely TDD, FDD, 2x2 and 4x4 MIMO.

# Reference

- TR 38.827, v16.8.0, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- TS 38.101-4, v18.3.0, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

- Outer Loop Link Adaptation Enhancements for Ultra Reliable Low Latency Communications in 5G, E. Peralta, et al, IEEE 95th Vehicular Technology Conference, 2022, DOI: 10.1109/VTC2022-Spring54318.2022.9860717

- TS 38.214, v18.2.0, Physical layer procedures for data

- R4-2409931 WF for [111][338] TRP_TRS_MIMO_OTA

- TR 38.901 v18.0.0 Study on channel model for frequencies from 0.5 to 100 GHz

- Developing a MIMO Test Methodology using Dynamic Channel Models and Link Adaptation, M. Foegel, et al, 16th European Conference on Antenna and Propagation, 2022

- Novel Multi-Mobility V2X Channel Model in the Presence of Randomly Moving Clusters, B. Xiong, et al, IEEE TRANSACTIONS ON WIRELESS COMMUNICATIONS, VOL. 20, NO. 5, MAY 2021, pp.3180-3195

- Novel 3-D Nonstationary MmWave Massive MIMO Channel Models for 5G High-Speed Train Wireless Communications, Y. Liu, et al, IEEE TRANSACTIONS ON VEHICULAR TECHNOLOGY, VOL. 68, NO. 3, MARCH 2019, pp.2077-2086

- A 3D Non-Stationary Wideband GBSM for Low-Altitude UAV-to-Ground V2V MIMO Channels, H. Chang, et al, IEEE Access, Vo.7, 2019, pp.70719-70732

- A 3-D Dynamic Non-WSS Cluster Geometrical-Based Stochastic Model for UAV MIMO Channels, C. Ge, et al, IEEE TRANSACTIONS ON VEHICULAR TECHNOLOGY, VOL. 71, NO. 7, JULY 2022, pp.6886-6899

- TS 38.151 v18.1.0 Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs