3GPP TSG-RAN WG4 Meeting #112												R4-2411024

Maastricht, The Netherlands, 19 – 23 August, 2024

Agenda item:			8.12.2.2

Source:	Huawei, HiSilicon

Title:	on NTN tests

Document for:	approval

# Introduction

The WF [1] from the last meeting states to further study NTN IoT devices and usage scenarios.

Issue 3-1-4: UE type for IoT-NTN

Agreements

  - FFS detailed UE type for IoT-NTN.

Issue 3-1-5: Alignment on understanding of IoT-NTN devices

Agreements

  - Encourage OEMs/Satellite Operators to provide more device information for IoT-NTN device types, e.g., detailed definition, device size, weight, typical antenna patterns.

Issue 3-1-6: Usage scenarios for NR-NTN and IoT-NTN handheld UE

Agreements

  - RAN4 further discuss prioritized usage scenarios. Free space only is not considered.

    - Option 1: Prioritize the browsing mode (with hand phantom)

    - Option 2: Prioritize the browsing mode (with hand phantom) and Free Space

    - Option 3: Prioritize browsing mode (with hand phantom) and talk mode (Head+Hand)

    - Option 4: Prioritize Head+Hand talk mode, hand only browsing mode and talk mode (new positioning guideline), and Free Space

This contribution discusses the above topics.

# Discussion

As indicated in [2], the form factor of IoT NTN devices has a wide range, from a tag to a module mounted on vehicles. This makes difficult or almost impossible to define a single or a few form factors for IoT NTN devices. There are commercial IoT NTN modules on the markets. Performances of standalone modules may or may not be meaningful because those performances may not be representative for in situ scenarios. In addition, it is difficult to test such modules in situ, e.g. modules mounted on agricultural vehicles.

Even the above test issues can be overcome, it is still difficult to group IoT NTN devices into manageable categories.

Proposal 1: deprioritise IoT NTN devices in this work item

For smartphones, there is no agreed device orientation for a given beam direction pointing towards a satellite albeit manufacturers may provide user instructions on device orientation to achieve maximum directivity towards satellites. For example, the direction of maximum gain could be perpendicular or parallel to the screen. It is therefore prudent to measure all 360 degrees solid angle for TRP and TRS, then define a beam like metric. This would mean no change to the existing test procedures for both TRP and TRS. The only change is the post data processing.

This effectively rules out the following two issues involving partial measurement suggested in direction 3 and 5 of WF [1], namely (a) Issue 3-2-1: “Categorized performance metric for NR-NTN handheld”, and (b) Issue 3-3-5: “New positioning guideline of hand only talk mode”. This further narrows variations and edges towards convergence.

Proposal 2: measure all 360 degrees solid angle for NTN TRP and TRS because partial sphere measurement may not capture the main beam direction without prior knowledge from device manufacturers.

Satellites are in the upper hemisphere of NTN devices and operators are not interested in the performance of in the other hemisphere. Proposal 3 suggests to use the upper hemisphere, viz. 90 degrees solid angle, to calculate TRP and TRS. The value of [90] degrees solid angle is in square brackets and can be discussed.

Proposal 3: calculate TRP and TRS using measured data within [90] degrees solid angle containing the main beam if CDF/CCDF or maximum is not used as performance metrics.

Maximum does not normally represent the full picture while the weighted TRP/TRS approach needs to locate the main beam and carry out associated calculations. CDF/CCDF, on the other hand, possesses the characteristics of averaging at a given percentile without the additional computation such as finding the main beam in weighted TRP/TRS approach.

Observation 1: CDF/CCDF for TRP/TRS is an effective and simple approach for NTN OTA tests.

The maximum gain or main beam direction relative to device orientation is device dependent. This gives rise to the need to find the main beam direction from the 360 degrees solid angle measurement for TRP before calculating the NTN specific performance metric in proposal 3.

The value [60] degrees solid angle is a suggested one and can be further discussed. For example, if the value in proposal 4 is reduced to less than 15 degrees solid angle, it would be equivalent to find the maximum from all measured data point because 15 degree is the default measurement step size. However, if the value in proposal 4 is larger than the measurement step of 15 degrees, finding the main beam direction would involve integration over the specified solid angle value step by step in both theta and phi directions across the 360 degrees solid angle.

Proposal 4: main beam is found by averaging transmit and receive power within a solid angle of [60 degrees] for TRP and TRS, respectively, if CDF/CCDF or maximum is not used as performance metrics.

The NTN are in FDD mode and its frequency bands are copied below from Table 5.2.2-1 of TS 38.101-1 [3].

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |

The uplink and downlink frequency ranges of band n256 and n255 are relatively close. This implies that the antenna patterns for uplink and downlink should be very similar. In other words, the main beam directions for transmit and receive bands are closely correlated. However, band n254 has a widely separately uplink and downlink frequency ranges, different antennas or a single antenna with two resonances could be used for uplink and downlink as it is difficult to cover such wide frequency range with a single resonance. Such a wide separation could lead to difference in main beam directions of uplink and downlink for band n254.

In addition to the above frequency difference in uplink and downlink, the direction or location of the link antenna may also impact the main beam direction for TRS due to the fact that the best sensitivity is achieved when the main beam points towards the link antenna. These are the reasons to find main beam direction separately in proposal 4.

Observation 2: the main beam directions of uplink and downlink could be different, especially for band n254.

The available bandwidths in FR1 NTN bands are 30 MHz for n256, 34 MHz for n255 and 16.5 MHz for n254. Typically, those spectra are shared among two or three operators within a country, albeit there could be allocations where an operator is given an entire band. Without any prior knowledge of future NTN spectrum allocation by various administrations across the world, it would therefore be prudent to use the minimum bandwidth of 5 MHz specified in Table 5.3.2-1 in TS 38.101-5 [3] for NTN OTA tests.

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

Proposal 5: the bandwidth for NTN TRP and TRS test should be 5 MHz.

Browsing with hand phantom should be the most common scenario because (a) talk is normally via speaker or earphones nowadays, (b) some optimisation of device orientations may need to be carried out by hand for communicating with GEO satellites. In such cases, hand and head use scenario is not practical.

Proposal 6: prioritise browsing scenario with hand phantom.

Regarding Issue 3-2-3: Whether performance metric should be different for GEO and NGEO (including LEO and MEO), earth stations normally track the movement of LEO and MEO satellites. In future or possibly even now, hand held devices could possess such tracking capability. UE would track the direction of link antenna under such capability using techniques such as phased antenna array or antenna switching or both. Currently there is no reliable solution for antenna switching, let alone dealing with tracking functionality from phased antenna arrays. At this stage of the work item, it would be simpler to switch off NGEO satellite tracking capability if available in UE during OTA tests.

Proposal 7: use the same metric for NGEO and GEO satellites with NGEO satellites tracking functionality if available in UE switched off during NTN OTA measurement.

# Conclusion

This contribution put forwards the following proposals.

Proposal 1: deprioritise IoT NTN devices in this work item

Proposal 2: measure all 360 degrees solid angle for NTN TRP and TRS because partial sphere measurement may not capture the main beam direction without prior knowledge from device manufacturers.

Proposal 3: calculate TRP and TRS using measured data within [90] degrees solid angle containing the main beam if CDF/CCDF or maximum is not used as performance metrics.

Observation 1: CDF/CCDF for TRP/TRS is an effective and simple approach for NTN OTA tests.

Proposal 4: main beam is found by averaging transmit and receive power within a solid angle of [60 degrees] for TRP and TRS, respectively.

Observation 2: the main beam directions of uplink and downlink could be different, especially for band n254.

Proposal 5: the bandwidth for NTN TRP and TRS test should be 5MHz.

Proposal 6: prioritise browsing scenario with hand phantom.

Proposal 7: use the same metric for NGEO and GEO satellites NGEO satellites tracking functionality if available in UE switched off during NTN OTA measurement.

# References

- R4-2409931 WF for [111][338] TRP_TRS_MIMO_OTA

- IoT Guide: Hybrid Cellular/Non-Terrestrial Network (NTN), GSMA, April 2024

- TS 38.101-5, v18.5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements