3GPP TSG-RAN WG4 Meeting #112												R4-2411025

Maastricht, The Netherlands, 19 – 23 August, 2024

Agenda item:			8.12.2.1

Source:	Huawei, HiSilicon

Title:	On XR devices test

Document for:	Approval

# Introduction

The WF [1] in the last RAN4 meeting captures the following situation and this contribution further discusses the XR device types to be included in the OTA tests.

Issue 2-1-1: XR device type

Agreements

FFS The defined test methodologies of TRP/TRS for FR1 non-Redcap XR devices shall be applied to all possible 5G XR device types except the XR device form-factors based on smartphone UE i.e. XR5G-P1.

RAN4 further discuss prioritization of XR devices type, e.g.,

- Whether 1Tx XR device should be prioritized in Rel-19

- whether head-worn XR devices with external 5G (e.g. XR5G-V1/V2/V3, XR5G-A1/A2/A5) should be considered.

# Discussion

Figure 1 is a copy of Figure 4.8-1 from TR 26.928 [2] and summaries devices with various form factors. Detailed capability descriptions for all the form factors can be found in section 4.8.1 of TR 26.928 [2].

The devices can be divided into two categories based on their form factors, namely:

Category 1: head mounted displays, e.g. XR5G-V2/V3/V4, XR5G-A3/A4

Category 2: glasses, e.g. XR5G-A2/A5

Obviously, devices with wired connection, such as XR5G-A1 or XR5G-V1, are excluded from OTA tests.

The reason to group the devices into the above two categories is that the physical size of devices in category 1 is much larger than that in category 2. Typically, large device sizes would allow more space for antennas. Namely the antennas in category 1 devices would have better performance than those in category 2. As a result, it would be fair to conduct the OTA tests and define the associated performances according to the above categories.

In addition, the market shares of category 1 and 2 devices seem to roughly equal for the next 2 or 3 years according to some forecasts.

Proposal 1: define XR device performance according to their form factors, namely category 1 for head mounted displays and category 2 for glasses. Include both category 1 and 2 in this work item.

If both category 1 & 2 are included in this work item, there may be a need to communicate such agreement to CTIA regarding head phantom development because the LS to CTIA [8] does not define the above two categories, which could have different placement guidelines or MU values.

Observation 1: If both category 1 & 2 are included in this work item, there may be a need to communicate such agreement to CTIA regarding head phantom development.

According to the XR device capability descriptions in section 4.8.1 of TR 26.928 [2], wireless connections of some devices are Wi-Fi or 5G side link based. XR OTA tests for those types of devices would be outside the scope of this work item. Because (a) Wi-Fi OTA test is clearly outside the scope, (b) side link coverage distance would be much smaller than those of network coverage and requires a different level of performance.

Proposal 2: only test XR devices with 5G link to gNB in networks, not those with side link connections.

Examination of a typical website [3] comparing VR or AR head-mounted display or glasses shows that hardly any current products support 5G connectivity, let along more than one transmit antennas, albeit receive diversity seems to be readily implementable [4, 5]. As a result, it is prudent to prioritize 1Tx devices.

Proposal 3: prioritize 1Tx XR devices in this work item as XR devices with 2Tx may not be available before the end of 2025.

Figure 1 AR and VR devices of various form factors (Figure 4.8-1 from TR 26.928)

The parameter supportOf2RxXR-r18 in 38.306 [6] can identify XR devices with 2Rx. However, 3GPP does not appear to provide a mechanism to distinguish XR devices without receive diversity, namely 1Rx, and such devices could exist on various markets. It is unlikely that XR devices without receive diversity would set the parameter supportOf2RxX-r18 to {true} albeit such devices could meet the receiver sensitivity requirements for 2Rx as specified in 38.101-1 [7]. In this case, XR devices with 1Rx could be treated as 4Rx devices because 4Rx is the default.

Observation 2: XR devices with 1Rx could be treated as 4Rx devices if parameter supportOf2RxXR-r18 is not set on those devices.

# Conclusion

In this contribution, we put forward the following proposals and observation.

Proposal 1: define XR device performance according to their form factors, namely category 1 for head mounted displays and category 2 for glasses. Include both category 1 and 2 in this work item.

Observation 1: If both category 1 for glasses and category 2 for head mounted displays are included in this work item, there may be a need to communicate such agreement to CTIA regarding head phantom development.

Proposal 2: only test XR devices with 5G link to gNB in networks, not those with side link connections.

Proposal 3: prioritize 1Tx XR devices in this work item as XR devices with 2Tx may not be available before the end of 2025.

Observation 2: XR devices with 1Rx could be treated as 4Rx devices if parameter supportOf2RxXR-r18 is not set on those devices.

# Reference

[1] R4-2409931 WF for [111][338] TRP_TRS_MIMO_OTA

[2] TR 26.928 Extended Reality (XR) in 5G, v18.0.0

[3] https://vr-compare.com/

[4] 4 × 4 MIMO Antenna System for Smart Eyewear in Wi-Fi 5G and Wi-Fi 6e Wireless Communication Applications, M. Chung, et al, Electronics 2021, 10, 2936. https://doi.org/10.3390/electronics10232936

[5] Transparent and Flexible Antenna for Wearable Glasses Applications, S. Hong, et al, IEEE TRANSACTIONS ON ANTENNAS AND PROPAGATION, VOL. 64, NO. 7, July 2016, pp.2797-2804

[6] TS 38.306 User Equipment (UE) radio access capabilities, v18.1.0

[7] TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, v18.5.

[8] R4-2404654 LS on Head Phantoms for XR devices OTA testing