3GPP TSG-RAN WG4 Meeting #112	R4-2411026

Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Agenda item:	8.16.2

Source:	MediaTek Inc.

Title:	Discussion on UE requirements with 8Rx MMSE-IRC receiver

Document for:	Discussion

# Introduction

In RAN#104, Rel-19 WI on NR demodulation performance for 8Rx MMSE-IRC (MMSE Interference Rejection Combining) receiver was agreed [1]. Based on the WID, Rel-17 interference model for 2/4Rx UE will be used as a baseline to define 8Rx requirements.

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

In this contribution, we present our views for requirements on 8Rx MMSE-IRC receiver under the scenario of inter-cell interference and inter-user intra-cell interference.

# Requirements with inter-cell interference

## 2.1 PDSCH requirements

We summarize some important parameters used to define PDSCH requirements with inter-cell interference in Rel-17 in Table 1.

Table 1. Summary of parameters for Rel-17 PDSCH requirements with inter-cell interference

| Clause | Test | Duplex mode | Antenna configuration |  |  |
| --- | --- | --- | --- | --- | --- |
| 5.2.2.1.15 | 1-1 | FDD | 2T2R | - Two interfering cells - INRs: 7.77 dB and 2.29 dB - TDLC300-100 | - MCS13 in MCS Table 1 - Rank 1 - ULA low |
| 5.2.2.2.16 | 1-1 | TDD |  |  |  |
| 5.2.3.1.15 | 1-1 | FDD | 2T4R |  |  |
| 5.2.3.2.16 | 1-1 | TDD |  |  |  |
| 5.2.2.1.15 | 1-2 | FDD | 2T2R | - One interfering cell - INR: 7.58dB - TDLA30-10 |  |
| 5.2.2.2.16 | 1-2 | TDD |  |  |  |
| 5.2.3.1.15 | 1-2 | FDD | 2T4R |  |  |
| 5.2.3.2.16 | 1-2 | TDD |  |  |  |

Except duplex mode and number of Rx antenna, it can be shown that there are only two different settings in Rel-17 requirements. The test parameters tables for 2Rx (Table 5.2.2.1.15-2) and 4Rx (5.2.3.1.15-2) requirements in FDD mode are the same. Furthermore, the test parameters tables for 2Rx (Table 5.2.2.2.16-2) and 4Rx (Table 5.2.3.2.16-2) requirements in TDD mode are the same. The difference between parameters tables for FDD and TDD mode are only duplex mode and HARQ related settings. Therefore, we propose to reuse test parameters of Table 5.2.2.1.15-2 and Table 5.2.2.2.16-2 in TS38.101-4 as a starting point to define 8Rx IRC receiver requirements in FDD mode and TDD mode, respectively.

Proposal 1: Reuse test parameters of Table 5.2.2.1.15-2 and Table 5.2.2.2.16-2 in TS38.101-4 as a starting point to define 8Rx IRC receiver requirements in FDD mode and TDD mode, respectively.

Except test parameters in Annex, there are still some settings to be determined when defining 8Rx IRC receiver requirements, e.g., MCS, rank, propagation condition, MIMO correlation matrix and antenna configuration. As we are now discussing for 8Rx CPE/FWA/vehicle/industrial devices, we propose to use 2T8R ULA low as MIMO correlation matrix and antenna configuration.

Proposal 2: Use “2T8R ULA low” as MIMO correlation matrix and antenna configuration to define PDSCH requirements for 8Rx IRC receiver considering CPE/FWA/vehicle/industrial devices.

As shown in Table 1, in Rel-17, RAN4 defined requirements with two different propagation conditions, i.e., TDLC300-100 for two interfering cells and TDLA30-10 for one interfering cell. We propose to use the same setting of propagation condition as a starting point.

Proposal 3:  Use TDLC300-100 (for the case of two interfering cells) and TDLA30-10 (for the case of one interfering cell) as a starting point to define 8Rx IRC receiver requirements.

Based on the above proposals, we provide initial evaluation results in Table 2 for 8Rx PDSCH demodulation performance in the presence of inter-cell interference based on the following assumptions:

- Test parameters of Table 5.2.2.1.15-2 in TS38.101-4 (INRs 7.77 dB and 2.29 dB for two interfering cells and INR 7.58 dB for one interfering cell)

- MIMO correlation matrix and antenna configuration: 2T8R ULA low

- Rank: rank 1

- propagation condition: TDLC300-100 for two interfering cells and TDLA30-10 for one interfering cell

Table 2. Initial evaluation results for 8Rx PDSCH demodulation performance in the presence of inter-cell interference

| Scenario | MCS | SNR for 70% of Max T-put (MMSE-MRC baseline receiver) | SNR for 70% of Max T-put (MMSE-IRC baseline receiver) | SNR for 70% of Max T-put (MMSE-IRC simplified receiver) | MMSE-IRC gain | SNR difference between MMSE-IRC baseline and simplified receiver | Corresponding SINR for SNR (MMSE-IRC baseline receiver) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Two interfering cells | MCS 13 | 9.1 | 3.4 | 4.8 | 5.7 | 1.4 | -5.9 |
|  | MCS 16 | 11.5 | 6.2 | 7.5 | 5.3 | 1.3 | -3.1 |
| One interfering cell | MCS 13 | 6.9 | 0.9 | 2.2 | 6.0 | 1.3 | -7.3 |
|  | MCS 16 | 9.3 | 3.4 | 4.7 | 5.9 | 1.3 | -4.8 |

In Rel-18, RAN4 defined two sets of requirements for 8Rx by considering different UE implementation, which are baseline receiver and simplified receiver. Therefore, in Table 2, we provide SNR values achieving 70% of maximum throughput for MMSE baseline receiver, MMSE-IRC baseline receiver and MMSE-IRC simplified receiver. During the discussion in Rel-17 IRC receiver requirements, companies proposed lots of options to determine the MCS for requirements [2].

- Down selection between MCS 4 and MCS 13 based on results for agreed INR values based on the following criteria - Option 1: Testable performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-MRC - Option 2: SINR is not lower than -6 dB - Option 3: Consider the difference between SNR and INR to avoid possible handover (SNR-INR > -3dB) - Option 4: SNR > INR - Other options are not precluded - Using of multiple options is not precluded

From Table 2, it is obvious that MMSE-IRC receiver can provide a large gain compared to MMSE-MRC receiver for both MCS 13 and 16.

Observation 1: Based on our initial evaluations, the gain of 8Rx MMSE-IRC receiver over 8Rx MMSE-MRC is larger than 5 dB when considering the same setting as RAN4 used in Rel-17.

The corresponding SINR for SNR are provided in the last column of Table 2 based on Figure 1 and 2, it shows that the SINRs for MCS 13 are quite low for both one and two interfering cells. Therefore, we propose RAN4 can consider MCS 16 as a starting point to define requirements. Also, from our initial evaluations, the gap between 8Rx MMSE-IRC baseline receiver and 8Rx MMSE-IRC simplified receiver is over 1dB. Therefore, we suggest considering two sets of requirements as what RAN4 did in Rel-18 8Rx WI.

Observation 2: The SINR for both one and two interfering cells scenarios are quite low if considering MCS13 used in Rel-17 MMSE-IRC requirements.

Proposal 4: Consider rank 1 and MCS16 as a starting point to define 8Rx MMSE-IRC requirements with inter-cell interference.

Proposal 5: Consider two sets of PDSCH requirements for 8Rx MMSE-IRC receiver with inter-cell interference.

| Figure 1. SINR versus SNR for two interfering cells | Figure 2. SINR versus SNR for one interfering cell |
| --- | --- |

## 2.2 CQI requirements

In Rel-17, RAN4 use the following test metrics for CQI reporting requirements with inter-cell interference. The testing SINR point, throughput ratio g  and BLER X% are determined by simulations based on the Table 6.2.2.1.2.3-1, Table 6.2.2.2.2.3-1, Table 6.2.3.1.2.3-1 and Table 6.2.3.2.2.3-1 in TS38.101-4.

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g . b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

We propose to use Table 6.2.2.1.2.3-1 and Table 6.2.3.1.2.3-1 and assume 2T8R ULA as a starting point to define 8Rx CQI reporting requirements with inter-cell interference.

Proposal 6: Use the following test metrics to define 8Rx CQI reporting requirements with inter-cell interference. The testing SINR point, throughput ratio g  and BLER X% can be determined through simulation by using Table 6.2.2.1.2.3-1 and Table 6.2.3.1.2.3-1 with 2T8R ULA.

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g .

b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

- Requirements with inter-user intra-cell interference

Some important parameters used to define PDSCH requirements with inter-user intra-cell interference in Rel-17 are summarized in Table 3. We propose to use Table 5.2.3.1.16 and 5.2.3.2.17 in TS38.101-4 as a starting point to define requirements.

Table 3. Summary of parameters for Rel-17 PDSCH requirements with inter-user intra-cell interference

| Clause | Test | Duplex mode | Antenna configuration and propagation condition |  |
| --- | --- | --- | --- | --- |
| 5.2.2.1.16 | 1-1 | FDD | 2T2R ULA low TDLC300-100 | - Target UE - port 1000 - MCS13 - Random precoder - Co-scheduled UE - port 1001 - random 16 QAM - Any column of precoder matrix is not equal to any column of precoder matrix of Target UE |
| 5.2.2.2.17 | 1-1 | TDD |  |  |
| 5.2.3.1.16 | 1-1 | FDD | 2T4R ULA low TDLC300-100 |  |
| 5.2.3.2.17 | 1-1 | TDD |  |  |
| 5.2.3.1.16 | 2-1 | FDD | 4T4R ULA low TDLA30-10 | - Target UE - ports 1000 and 1001 - MCS13 - Random precoder - Co-scheduled UE - ports 1002 and 1003 - random 16 QAM - Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH |
| 5.2.3.2.17 | 2-1 | TDD |  |  |

Proposal 7: Reuse test parameters of Table 5.2.3.1.16 and Table 5.2.3.2.17 in TS38.101-4 as a starting point to define 8Rx MMSE-IRC receiver MU-MIMO requirements in FDD mode and TDD mode, respectively.

We provide initial evaluation results in Table 4 for 8Rx PDSCH demodulation performance with inter-user intra-cell interference based on the following assumptions:

- Target UE

  - Antenna configuration and propagation condition: 4T8R ULA low

  - DMRS port: ports 1000 and 1001

  - MCS: MCS13

  - Precoder selection: Random precoder

- Co-scheduled UE

  - DMRS port: ports 1002 and 1003

  - MCS: random 16 QAM

  - Precoder selection: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

Table 4. Initial evaluation results for 8Rx PDSCH demodulation performance with inter-user intra-cell interference

| SNR for 70% of Max T-put (MMSE-MRC baseline receiver) | SNR for 70% of Max T-put (MMSE-IRC baseline receiver) | SNR for 70% of Max T-put (MMSE-IRC simplified receiver) | MMSE-IRC gain | SNR difference between MMSE-IRC baseline and simplified receiver |
| --- | --- | --- | --- | --- |
| Inf | 6.9 | 10.7 | Inf | 3.8 |

From our initial evaluations, the gap between 8Rx MMSE-IRC baseline receiver and 8Rx MMSE-IRC simplified receiver is 3.8 dB. Therefore, we suggest considering two sets of requirements as what RAN4 did in Rel-18 8Rx WI.

Proposal 8: Consider two sets of requirements for 8Rx MMSE-IRC receiver with inter-user intra-cell interference.

# Conclusion

Proposal 1: Reuse test parameters of Table 5.2.2.1.15-2 and Table 5.2.2.2.16-2 in TS38.101-4 as a starting point to define 8Rx IRC receiver requirements in FDD mode and TDD mode, respectively.

Proposal 2: Use “2T8R ULA low” as MIMO correlation matrix and antenna configuration to define PDSCH requirements for 8Rx IRC receiver considering CPE/FWA/vehicle/industrial devices.

Proposal 3:  Use TDLC300-100 (for the case of two interfering cells) and TDLA30-10 (for the case of one interfering cell) as a starting point to define 8Rx IRC receiver requirements.

Observation 1: Based on our initial evaluations, the gain of 8Rx MMSE-IRC receiver over 8Rx MMSE-MRC is larger than 5 dB when considering the same setting as RAN4 used in Rel-17.

Observation 2: The SINR for both one and two interfering cells scenarios are quite low if considering MCS13 used in Rel-17 MMSE-IRC requirements.

Proposal 4: Consider rank 1 and MCS16 as a starting point to define 8Rx MMSE-IRC requirements with inter-cell interference.

Proposal 5: Consider two sets of PDSCH requirements for 8Rx MMSE-IRC receiver with inter-cell interference.

Proposal 6: Use the following test metrics to define 8Rx CQI reporting requirements with inter-cell interference. The testing SINR point, throughput ratio g  and BLER X% can be determined through simulation by using Table 6.2.2.1.2.3-1 and Table 6.2.3.1.2.3-1 with 2T8R ULA.

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ g .

b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Proposal 7: Reuse test parameters of Table 5.2.3.1.16 and Table 5.2.3.2.17 in TS38.101-4 as a starting point to define 8Rx MMSE-IRC receiver MU-MIMO requirements in FDD mode and TDD mode, respectively.

Proposal 8: Consider two sets of requirements for 8Rx MMSE-IRC receiver with inter-user intra-cell interference.

# Reference

- RP-241297, Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO

- R4-2115730, WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC, Intel