3GPP TSG-RAN WG4 #112	R4-2411032

August 19th ‒ 23rd, 2024

Maastricht, NL

Agenda item:	7.14.1

Source: 	Novamint, Ubiik, Semtech, Telit, Sequans Communications

Title: 	Regulation for LTE FDD new band in 1800–1830 MHz for Canada

Document for:	Discussion

# Introduction

A spectrum-related work item [1] was agreed to specify a new LTE band in the 1800-1830 MHz frequency range.  This contribution provides the background on the technical regulation as published in https://ised-isde.canada.ca/site/spectrum-management-telecommunications/en/devices-and-equipment/standard-radio-system-plans-srsp/srsp-3017-technical-requirements-fixed-radio-systems-operating-bands-1700-1710-mhz-and-1780-1850-mhz.

Moreover, a clarification letter from the Canadian Regulator (ISED) is attached.

# Current regulation

The following text is a cut and paste from the link https://ised-isde.canada.ca/site/spectrum-management-telecommunications/en/devices-and-equipment/standard-radio-system-plans-srsp/srsp-3017-technical-requirements-fixed-radio-systems-operating-bands-1700-1710-mhz-and-1780-1850-mhz. The text highlighted in yellow is particularly relevant to specification of the new band 1800-1830 MHz.

SRSP-301.7 — Technical Requirements for Fixed Radio Systems Operating in the Bands 1700-1710 MHz and 1780-1850 MHz

Issue 4
January 17, 2022

Contents

Preface

1. Intent

2. General

3. Related documents

4. Radio frequency channel arrangement description

5. Transmitter characteristics

6. Antenna characteristics

7. Maximum equivalent isotropically radiated power

8. Avoidance of the geostationary satellite orbit

9. Enhanced technical requirements for moderately congested and highly congested areas

10. International coordination

Preface

This Standard Radio System Plan (SRSP) replaces SRSP-301.7, issue 3. Issue 4 of SRSP-301.7 has been released to amend the antenna characteristics for fixed radio systems used for the management of the electricity supply.

The following are the main changes:

- The section on the antenna characteristics for fixed radio systems used for the management of the electricity supply has been modified to allow smaller omnidirectional antennas for terminal stations.

- References to low-capacity and very low capacity radio systems have been removed, based on policy decisions in SMSE-022-14, Decisions on Spectrum Utilization Policies and Technical Requirements Related to Backhaul.

- References to minimum hop lengths have been removed.

- Other editorial updates and improvements have been made throughout the document.

- Intent

This Standard Radio System Plan (SRSP) states the minimum technical requirements for the efficient use of the frequency bands 1700-1710 MHz and 1780-1850 MHz by radio systems in the fixed service, namely:

- fixed point-to-point line-of-sight digital radio systems in the frequency bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz;

- analog and digital radio systems, in the frequency band 1700-1710 MHz, used for aural broadcasting studio-to-transmitter link (STL) services; and

- fixed point-to-point and point-to-multipoint digital radio systems, in the frequency band 1800-1830 MHz, used for the management of the electricity supply.

In this standard, the phrase “fixed radio systems used for the management of the electricity supply” refers to radio systems carrying traffic solely for the management, operation and maintenance of the electricity supply.

This SRSP is intended to be used in the design and specification of radio systems and equipment, as well as in the technical evaluation of applications for new radio facilities or for modifications to radio systems submitted in accordance with the current issue of Radio Standards Procedure RSP-113, Application Procedures for Planned Radio Stations Above 960 MHz in the Fixed Service.

This SRSP specifies system characteristics related to efficient spectrum usage only and is not to be regarded as a comprehensive specification for equipment design and/or selection.

- General

This section includes information on the general aspects of this SRSP, such as licensing, resolution of interference conflicts and spectrum sharing.

#### 2.1 Licensing process

Existing radio systems operating in the frequency bands 1700-1710 MHz and 1780-1850 MHz that were licensed as standard prior to the issuance of this SRSP may continue to operate as standard. Requests to extend or expand these systems will be considered by Innovation, Science and Economic Development Canada (ISED) on a case-by-case basis. New systems deployed in the bands 1700-1710 MHz and 1780-1850 MHz shall conform to the requirements of this SRSP.

#### 2.2 Compliance of radio systems

Radio systems conforming to the technical requirements of this SRSP will be given priority in licensing over non-standard radio systems operating in the bands 1700-1710 MHz and 1780-1850 MHz.

The arrangements for non-standard systems are outlined in Spectrum Utilization Policy SP Gen, General Information Related to Spectrum Utilization and Radio System Policies.

The Geographical Differences Policy (GDP) guideline applies to radio systems in these bands. The GDP incorporates some technical requirements that are applied in areas having certain levels of congestion, which are further described in part B, section 1.6, of Spectrum Utilization Policy SP 1-20 GHz, Revisions to Microwave Spectrum Utilization Policies in the Range of 1-20 GHz.

Even though a radio system complies with the requirements of this SRSP, modifications may be required to that system whenever it causes harmful interference, as defined in the Radiocommunication Act.

Analog and digital radio systems used for aural broadcasting STL services will be permitted in the band 1700-1710 MHz and, on a case-by-case basis, in the bands 1780-1800 MHz and 1830-1850 MHz. All STL systems shall conform to the applicable provisions for point-to-point digital radio systems in the bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz in this SRSP.

#### 2.3 Resolution of interference conflicts between radio systems

When potential interference between radio systems cannot be resolved by the parties concerned, ISED should be advised. After consultation with these parties, ISED will determine the necessary modifications and schedule of modifications to resolve the conflict. Among other potential mitigation measures, ISED may require licensees and/or applicants to use a receiver with improved selectivity characteristics.

#### 2.4 Spectrum sharing

It should be noted that the fixed service shares the bands 1700-1710 MHz and 1780-1850 MHz with other services in accordance with the Canadian Table of Frequency Allocations. It may be necessary to coordinate with meteorological satellite (METSAT) earth stations operating in the band 1700-1710 MHz. For fixed systems operating near 1710 MHz and near 1850 MHz, coordination may be required with Advanced Wireless Services (AWS) and Personal Communications Services (PCS) systems, respectively, operating in the adjacent bands.

ISED’s Spectrum Management and Telecommunications website provides information on the stations of other services operating in these bands.

- Related documents

The current issues of the following documents are applicable and available on ISED’s Spectrum Management and Telecommunications website, under Official publications:

Arrangement D Arrangement Between the Department of Transport and the Interdepartment Radio Advisory Committee for the Exchange of Frequency Assignment Information and Engineering Comments on Proposed Assignments Along the Canada-United States Borders in Certain Frequency Bands Above 30 – Mc/s

CPC-2-0-03 Radiocommunications and Broadcasting Antenna Systems

CTFA Canadian Table of Frequency Allocations

RSP-113 Application Procedures for Planned Radio Stations Above 960 MHz in the Fixed Service

SP 1-20 GHz Revisions to Microwave Spectrum Utilization Policies in the Range of 1-20 GHz

SP 1-3 GHz Amendments to the Microwave Spectrum Utilization Policies in the 1-3 GHz Frequency Range

SP Gen General Information Related to Spectrum Utilization and Radio Systems Policies

TRC-43 Designation of Emissions, Class of Station and Nature of Service

CPC - Client Procedures Circular

RSP - Radio Standards Procedure

SP - Spectrum Utilization Policy

TRC - Telecommunications Regulation Circular

- Radio frequency channel arrangement description

This section describes the radio frequency (RF) channel arrangements for radio systems.

#### 4.1 RF channel arrangements for point-to-point radio systems in the frequency bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz

In order to facilitate assignment of a range of frequencies and bandwidths in the available spectrum, centre frequencies may be assigned on a 125 kHz grid. Channel bandwidths of 1 to 10 MHz, in 250 kHz increments, may be assigned as needed in the bands 1700-1710 MHz, 1780-1800 MHz or 1830-1850 MHz, with the exception of the channel bandwidths of STL systems, which shall not exceed 1 MHz. The centre frequency shall be chosen such that the occupied bandwidth of the emission is contained within the bands.

#### 4.1.1 RF channel centre frequencies for the band 1700-1710 MHz

Centre frequencies shall be selected from a 125 kHz grid, starting at 1700.5 MHz and ending at 1709.5 MHz, as expressed by the following relationship:

An = 1700.375 + 0.125n for n = 1 to 73

where n is an integer, and An is the centre frequency (in MHz) of the RF channel. The lowest available frequency shall be assigned first.

Unidirectional radio systems used for aural broadcasting STL services shall be licensed first in this band. If no frequencies are available in this band, new STL systems may be licensed in the bands 1780-1800 MHz and 1830-1850 MHz, in accordance with section 2.2.

#### 4.1.2 RF channel centre frequencies for the bands 1780-1800 MHz and 1830-1850 MHz

Equipment employing frequency division duplexing or time division duplexing is permitted in these bands. The minimum transmit/receive (T/R) spacing will be determined by the limitations of the equipment and the ability to coordinate.

Centre frequencies shall be selected from a 125 kHz grid, starting at 1780.5 MHz and ending at 1849.5 MHz, as expressed by the following relationship:

Bn = 1780.375 + 0.125n for n = 1 to 553

where n is an integer, and Bn is the centre frequency (in MHz) of the RF channel.

Frequencies in the bands 1780-1800 MHz and 1830-1850 MHz shall be used first. Where there are no available frequencies in these bands, frequencies anywhere in the band 1780-1850 MHz can be used; a point-to-point system operating in the band 1800-1830 MHz through this provision shall follow the technical requirements specified in sections 5.1 and 6.1 below.

#### 4.1.3 Frequency planning

Radio systems in the bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz are required to be designed using a two-frequency plan, whereby the same frequency pair is typically reused along a route of successive hops. Where reasonable justification is provided (e.g. where siting prevents adequate antenna discrimination), extra frequencies beyond the initial pair may be permitted to resolve the problem, at the discretion of ISED’s regional offices.

#### 4.1.4 Branching or spur route channels

The frequencies assigned to a main route system should be reused on the branching or spur routes, where possible. The siting of repeater stations should be planned with this requirement in mind to ensure sufficient antenna discrimination at the branch-off angle.

#### 4.2 RF channel arrangements for fixed radio systems used for the management of the electricity supply in the frequency band 1800-1830 MHz

Fixed point-to-point and point-to-multipoint radio systems used for the management, operation and maintenance of the electricity supply shall use robust modulation schemes tolerant of interference, and the radio system design shall include link budget margins for self-interference impairments. Network planning techniques shall be used to maximize frequency reuse. To increase spectrum utilization, the centre frequencies shall be reassigned to use the same frequencies to the maximum extent possible. Normally, the total bandwidth assigned to a given licensee should not exceed 20 MHz. At the discretion of ISED’s regional offices, requests for additional bandwidth may be considered if technical justification is provided.

In order to facilitate assignment of a range of frequencies and bandwidths in the available spectrum, centre frequencies may be assigned on a 125 kHz grid. The centre frequency shall be chosen such that the occupied bandwidth of the emission is contained within the frequency band 1800-1830 MHz.

#### 4.2.1 RF channel centre frequencies for the band 1800-1830 MHz

Centre frequencies shall be selected from a 125 kHz grid, starting at 1800 MHz and ending at 1830 MHz, as expressed by the following relationship:

Cn = 1799.875 + 0.125n for n = 1 to 241

where n is an integer, and Cn is the centre frequency (in MHz) of the RF channel.

#### 4.3 Protection channels

Protection channels are not permitted in the bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz. However, quad-path diversity applications will be considered on a hop-by-hop basis to solve special propagation problems. Quad-path diversity refers to the use of frequency diversity and space diversity on the same path.

- Transmitter characteristics

This section describes the transmitter characteristics for radio systems.

#### 5.1 Transmitter characteristics for radio systems in the frequency bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz

The transmitter power delivered to the antenna input shall not exceed the limits for each authorized bandwidth shown in table 1.

An increase in transmitter power over the above specified limit may be permitted if technical justification is provided. The power delivered to the antenna input shall not exceed 20 W (+13 dBW) per channel.

The centre frequency of the emission shall be maintained within ±0.001% of the assigned frequency.

#### 5.1.1 Spectral efficiency

Digital systems shall have a minimum spectral efficiency of 1 bit/s/Hz on a single polarization.

#### 5.1.2 Emission limits

In any 4 kHz band, where the centre frequency is removed from the assigned frequency by more than 50%, up to and including 250% of the authorized bandwidth, the mean power of emission shall be attenuated below the mean output power of the transmitter in accordance with the following equation:

A = 35 + 0.8 (P − 50) + 10 log10 (B)

where:

- A = attenuation (in dB) below the mean output power level

- P = percent removed from the centre frequency of the assigned RF channel

- B = authorized bandwidth (in MHz)

Notes:

- In no case shall the attenuation be less than 50 dB.

- Attenuation greater than 80 dB or to an absolute power of less than −13 dBm/MHz is not necessary.

In any 1 MHz band, where the centre frequency is removed from the assigned frequency by more than 250% of the authorized bandwidth, the mean power of emission shall be attenuated by 43 + 10 log10 (mean output power in watts) dB or 80 dB, whichever is the lesser attenuation.

#### 5.2 Transmitter characteristics for fixed radio systems used for the management of the electricity supply in the frequency band 1800-1830 MHz

The transmitter power delivered to the antenna input shall not exceed 2 W in any 1 MHz segment within the channel bandwidth.

An increase in transmitter power over the above specified limit may be permitted if technical justification is provided. However, the power delivered to the antenna input shall not exceed 20 W (+ 13 dBW) per channel.

The centre frequency of the emission shall be maintained within ±0.001% of the assigned frequency.

#### 5.2.1 Spectral efficiency

Digital systems shall normally have a minimum spectral efficiency of 1 bit/s/Hz on a single polarization. At the discretion of ISED’s regional offices, digital systems with a lower spectral efficiency may be permitted on a case-by-case basis if suitable technical justification is provided. Both the payload data and any overhead data are included in the total capacity bit rate used to determine if this requirement is met.

#### 5.2.2 Emission limits

In the 1 MHz bands immediately adjacent to the upper and lower limits of the authorized bandwidth, the mean power of emission in any band equal to 1% of the authorized bandwidth shall be attenuated by 43 + 10 log10 (mean output power in watts) dB.

Beyond the first 1 MHz bands immediately adjacent to the upper and lower limits of the authorized bandwidth, the mean power of emission in a bandwidth that is equal to 1 MHz shall be attenuated by 43 + 10 log10 (mean output power in watts) dB.

- Antenna characteristics

This section describes the antenna characteristics for radio systems.

#### 6.1 Antenna characteristics for radio systems in the frequency bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz

For radio systems in the frequency bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz, the co-polarized radiation pattern envelope in the horizontal plane of the antenna shall remain within envelope B, shown in figure 1 and table 2, for both vertical and horizontal polarizations.

Figure 1: Minimum antenna characteristics for radio systems in the frequency bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz

Description of figure 1

This line graph shows the antenna radiation pattern limits for radio systems operating in the frequency bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz. The x-axis plots azimuth, in degrees from the main lobe, from 0 to 180. The y-axis measures antenna directivity, in dB down from the main lobe, from 0 to 80.

#### 6.2 Antenna characteristics for fixed radio systems used for the management of the electricity supply in the frequency band 1800-1830 MHz

The use of directional antennas is encouraged to reduce interference and facilitate network planning; however, omnidirectional antennas are permitted in some cases, as outlined below.

If a point-to-multipoint radio system makes use of relay or repeater stations to improve or extend the coverage of a base station, these stations shall be treated as terminal stations for the purpose of antenna characteristics.

#### 6.2.1 Antenna characteristics for base stations in point-to-multipoint links

The gain of base station antennas for point-to-multipoint systems shall be greater than or equal to 7 dBi. Omnidirectional antennas are permitted if they respect this minimum gain.

#### 6.2.2 Antenna characteristics for terminal stations in point-to-multipoint links

If the transmitter power density at the antenna input is greater than or equal to 0.25 W/MHz, a terminal station shall make use of a directional antenna, and the antenna gain shall be greater than or equal to 12 dBi. In addition, the 3 dB beam width shall not exceed 30° in vertical and horizontal polarizations, and the front-to-back ratio shall be at least 20 dB.

If the transmitter power density at the antenna input is less than 0.25 W/MHz, omnidirectional and directional antennas are permitted.

#### 6.2.3 Antenna characteristics for point-to-point links

Point-to-point systems shall make use of a directional antenna, and the antenna gain shall be greater than or equal to 12 dBi. In addition, the 3 dB beam width shall not exceed 30° in vertical and horizontal polarizations, and the front-to-back ratio shall be greater than or equal to 20 dB.

- Maximum equivalent isotropically radiated power

The maximum equivalent isotropically radiated power (e.i.r.p.) from the antenna shall not, in any case, exceed +55 dBW per RF channel.

- Avoidance of the geostationary satellite orbit

As far as is practicable, sites for transmitting terrestrial stations operating in the fixed service should be selected so that the direction of maximum radiation of the antenna will be pointed at least 2° away from the geostationary satellite orbit, taking into account the effect of atmospheric refraction.

- Enhanced technical requirements for moderately congested and highly congested areas

The technical requirements in this section apply only to radio systems operating in moderately congested and highly congested areas, as defined in the Geographical Differences Policy guideline in part B, section 1.6, of SP 1-20 GHz. The requirements apply only to radio systems in the frequency bands 1700-1710 MHz, 1780-1800 MHz and 1830-1850 MHz, excluding fixed radio systems used for the management, operation and maintenance of the electricity supply in the band 1800-1830 MHz.

The co-polarized radiation pattern envelope in the horizontal plane of the antenna shall remain within envelope A, shown in figure 1 and table 2, for both vertical and horizontal polarizations.

Digital systems shall have a minimum spectral efficiency of 2.4 bits/s/Hz on a single polarization.

- International coordination

Stations operating in the band 1780-1850 MHz near the Canada-United States (U.S.) border are subject to the provisions of Arrangement D.

Canada does not currently have a formal arrangement with the U.S. government for the sharing of the 1700-1710 MHz band in the border regions. Licensees will be subject to any future agreements between Canada and the U.S. regarding use of these systems in the border regions, which may result in modifications of previously authorized stations.

# ISED clarification letter

The following text is a cut and paste from the letter from ISED available in attachment.

Question 1: “SRSP 301.7 is not compatible with the 3GPP channel raster which is based on 100kHz spacing.”

In your letter, you have correctly identified that the current channel plan for this frequency band is based on a 125 kHz channel raster. The original intent of this channel plan raster was to enable flexibility to licensees on what bandwidth to operate with and how to best accommodate multiple systems in the same area. Your proposal to adopt a 12.5 kHz raster appears to be aligned with this principle. Further, I note that an alternate possibility could also be to adopt a channel plan based on a 100 kHz raster, once it would be defined by 3GPP.

Therefore, I can confirm ISED is open to revising SRSP-301.7 to modify the channel plan to better support new standardized equipment, taking into account feedback from other stakeholders.

Question 2: “SRSP 301.7 is not paired spectrum does ISED permit FDD?”

Although the channel plan in SRSP-301.7 for the 1800-1830 MHz band is not formulated based on paired channels, there is no provision within the SRSP to mandate the use of either TDD or FDD duplexing. As such, our current rules would permit FDD operation as long as the channel centre frequencies fall within the channel raster identified in the current edition of the SRSP. As identified in our response to your previous question, we are open to revising the SRSP to better clarify this and better take into account the channel plans of new standardized equipment, taking into account feedback from other stakeholders.

Question 3: “Does ISED permit LTE and/or 5G in this band or is this a WiMAX band?”

Similarly to our response for the previous question, SRSP-301.7 does not mandate the use of any specific communication protocol. As such, any of the protocols you have specified in your letter – as well as any others – could be used in the 1800-1830 MHz frequency band, as long as the radio system deployed is fixed and the other provisions of the SRSP are met.

# Considerations

The current regulation (see Section 1 Intent) state it is relevant to fixed point-to-point and point-to-multipoint digital radio systems, in the frequency band 1800-1830 MHz

Observation 1 – the regulation is relevant to fixed point-to-point and point-to-multipoint digital radio systems

Section 4.2 states “Fixed point-to-point and point-to-multipoint radio systems used for the management, operation and maintenance of the electricity supply shall use robust modulation schemes tolerant of interference, and the radio system design shall include link budget margins for self-interference impairments.”

Since the regulation allows fixed services, in case of non IoT devices the design is based on large devices (CPEs), possibly with external antennas and more complex components. Moreover, the regulation requires robust techniques tolerant to interference and requires margins to take into account impairments. As such the definition of the channel arrangement (UL 1800-1810 MHz; DL 1820-1830 MHz) is considered feasible with state of the art components.

Observation 2 – the definition of the channel arrangement (UL 1800-1810 MHz; DL 1820-1830 MHz) is considered feasible with state of the art components, by assuming large devices (CPEs) and margins as requested by the regulation.

Proposal 1 – Specify the channel arrangement FDD, UL 1800-1810 MHz; DL 1820-1830 MHz

Section 4.2 also states “In order to facilitate assignment of a range of frequencies and bandwidths in the available spectrum, centre frequencies may be assigned on a 125 kHz grid. The centre frequency shall be chosen such that the occupied bandwidth of the emission is contained within the frequency band 1800-1830 MHz.”

This is not in line with the assumption in 3GPP of a 100 kHz channel raster. However, the Regulator in their clarification stated “I note that an alternate possibility could also be to adopt a channel plan based on a 100 kHz raster, once it would be defined by 3GPP.”

Therefore, it is proposed to specify for the new band a 100 kHz channel raster and subsequently request the Regulator to adapt the rules.

Observation 3 – the regulation is not in line with the 3GPP channel raster, but the Regulator indicated the willingness to adapt it after completion of the work in 3GPP

Proposal 2 – specify a 100 kHz channel raster

Section 5.2 states “The transmitter power delivered to the antenna input shall not exceed 2 W in any 1 MHz segment within the channel bandwidth. An increase in transmitter power over the above specified limit may be permitted if technical justification is provided. However, the power delivered to the antenna input shall not exceed 20 W (+ 13 dBW) per channel.”

Moreover Section 5.2.1 states “Digital systems shall normally have a minimum spectral efficiency of 1 bit/s/Hz on a single polarization. At the discretion of ISED’s regional offices, digital systems with a lower spectral efficiency may be permitted on a case-by-case basis if suitable technical justification is provided.”

Finally Section 7 states “The maximum equivalent isotropically radiated power (e.i.r.p.) from the antenna shall not, in any case, exceed +55 dBW per RF channel.”

Observation 4 – The recommended transmitter power allows the deployment of wide area base stations

Section 5.2.2 states “In the 1 MHz bands immediately adjacent to the upper and lower limits of the authorized bandwidth, the mean power of emission in any band equal to 1% of the authorized bandwidth shall be attenuated by 43 + 10 log10 (mean output power in watts) dB. Beyond the first 1 MHz bands immediately adjacent to the upper and lower limits of the authorized bandwidth, the mean power of emission in a bandwidth that is equal to 1 MHz shall be attenuated by 43 + 10 log10 (mean output power in watts) dB.”

Observation 5 – The regulation provides out-of-band emission limits

Proposal 3 -specify the most stringent values between the current regulation and 3GPP out-of-band limits for band 3

Section 6.2 states “The use of directional antennas is encouraged to reduce interference and facilitate network planning; however, omnidirectional antennas are permitted in some cases.”; “The gain of base station antennas for point-to-multipoint systems shall be greater than or equal to 7 dBi. Omnidirectional antennas are permitted if they respect this minimum gain.”; “If the transmitter power density at the antenna input is greater than or equal to 0.25 W/MHz, a terminal station shall make use of a directional antenna, and the antenna gain shall be greater than or equal to 12 dBi. In addition, the 3 dB beam width shall not exceed 30° in vertical and horizontal polarizations, and the front-to-back ratio shall be at least 20 dB. If the transmitter power density at the antenna input is less than 0.25 W/MHz, omnidirectional and directional antennas are permitted.”

Observation 6 – The regulation allows the use of UEs with omni directional antennas and power class of 23 dBm. The use of large devices (CPEs) with directional antennas is encouraged.

# Conclusion

This document provides the status of the current regulation and a clarification letter from the Canadian Regulator. Based on that, the following observation and proposals are made:

Observation 1 – the regulation is relevant to fixed point-to-point and point-to-multipoint digital radio systems

Observation 2 – the definition of the channel arrangement (UL 1800-1810 MHz; DL 1820-1830 MHz) is considered feasible with state of the art components, by assuming large devices (CPEs) and margins as requested by the regulation.

Proposal 1 – Specify the channel arrangement FDD, UL 1800-1810 MHz; DL 1820-1830 MHz

Observation 3 – the regulation is not in line with the 3GPP channel raster, but the Regulator indicated the willingness to adapt it after completion of the work in 3GPP

Proposal 2 – specify a 100 kHz channel raster

Observation 4 – The recommended transmitter power allows the deployment of wide area base stations

Observation 5 – The regulation provides out-of-band emission limits

Proposal 3 -specify the most stringent values between the current regulation and 3GPP out-of-band limits for band 3

Observation 6 – The regulation allows the use of UEs with omni directional antennas and power class of 23 dBm. The use of large devices (CPEs) with directional antennas is encouraged.

# Reference

- RP-241649, “New WID: Introduction of LTE FDD band in 1800 – 1830 MHz for Canada,” Novamint, Sequans, Semtech, Telit, Ubiik