3GPP TSG-RAN WG4 #112	R4-2411033

August 19th ‒ 23rd, 2024

Maastricht, NL

Agenda item:	7.14.2

Source: 	Novamint, Ubiik, Semtech, Telit, Sequans Communications

Title: 	UE RF Specification impact due to Introduction of LTE FDD new band in 1800–1830 MHz for Canada

Document for:	Discussion

# Introduction

A spectrum-related work item [1] was agreed to specify a new LTE band in the 1800-1830 MHz frequency range. In this contribution we discuss the necessary changes into Core specifications relevant to UE RF requirements listed in [1] due to introduction of the new band.

# Discussion

In Table 1 we go through all UE Tx and Rx requirements one by one and asses which requirements need modification with introduction of the 1800-1830 MHz LTE band for Canada.

In Table 2 we go through the Core specifications relevant to UE RF requirements listed in [1] (36.101) and identify which requirements need modification with introduction of the 1800-1830 MHz LTE band for Canada. An impact analysis on TS36.307 is also provided for information (see Table 3).

Moreover, based on the analysis of the current regulation [2], we also present proposals on the values to be considered.

Table 1: UE RF specification impact (TS36.101)

| NR UE Tx/Rx requirement | Proposal for NR 900 MHz LTE Band in the US |
| --- | --- |
| 6.2.2 UE maximum output power | 23 ±2 |
| 6.2.3 MPR | No specific requirement needed |
| 6.2.4 A-MPR | The UL for the new band (1800-1810 MHz) is far from band 2 and 25 (40 MHz). Therefore,it is assumed that no A-MPR requirement is needed. The UL for the new band (1800-1810 MHz) is far from band 4 (45 MHz) and 66 (20 MHz). Therefore,it is assumed that no A-MPR requirement is needed. |
| 6.2.5 Configured transmitted power | No specification impact |
| 6.3.2 Minimum output power | No specification impact |
| 6.3.3 Transmit OFF power | No specification impact |
| 6.3.4 Transmit ON/OFF time mask | No specification impact |
| 6.3.5 Power control | No specification impact |
| 6.5.1 Frequency error | No specification impact |
| 6.5.2.1 EVM | No specification impact |
| 6.5.2.2 Carrier leakage | No specification impact |
| 6.5.2.3 In-band emissions | No specification impact |
| 6.5.2.4 EVM equalizer spectrum flatness | No specification impact |
| 6.6.1 Occupied bandwidth | No specification impact |
| 6.6.2.1 Spectrum emission mask | Canadian regulation: In the 1 MHz bands immediately adjacent to the upper and lower limits of the authorized bandwidth, the mean power of emission in any band equal to 1% of the authorized bandwidth shall be attenuated by 43 + 10 log10 (mean output power in watts) dB. Beyond the first 1 MHz bands immediately adjacent to the upper and lower limits of the authorized bandwidth, the mean power of emission in a bandwidth that is equal to 1 MHz shall be attenuated by 43 + 10 log10 (mean output power in watts) dB. The 3GPP general specifications are more stringent than the Canadian regulation, therefore the general SEM defined in Table 6.6.2.1.1-1 are adopted |
| 6.6.2.2 Additional Spectrum emission mask | No additional SEM required The 3GPP general specifications are more stringent than the Canadian regulation, therefore the general SEM defined in Table 6.6.2.1.1-1 are adopted |
| 6.6.2.3.1 E-UTRA ACLR | No specification impact |
| 6.6.2.3.2 UTRA ACLR | No specification impact |
| 6.6.3.1 General spurious emissions | No specification impact |
| 6.6.3.2 Spurious UEtoUE co-ex | Since the agreement in [1] is not to impose new UE requirements with this new band as the protected band from the existing 3GPP bands (i.e., Table 6.6.3.2-1 in TS 36.101, and Table 6.5.3.2-1 in TS 38.101-1), the introduction of the new band implies the addition of the following row: |
| 6.6.3.3 Additional spurious emissions | Canadian regulation: In the 1 MHz bands immediately adjacent to the upper and lower limits of the authorized bandwidth, the mean power of emission in any band equal to 1% of the authorized bandwidth shall be attenuated by 43 + 10 log10 (mean output power in watts) dB. Beyond the first 1 MHz bands immediately adjacent to the upper and lower limits of the authorized bandwidth, the mean power of emission in a bandwidth that is equal to 1 MHz shall be attenuated by 43 + 10 log10 (mean output power in watts) dB. No new requirement is expected to be specified. The 3GPP general specifications are more stringent than the Canadian regulation, therefore the general values defined inTable 6.6.3.1-2 are adopted |
| 6.6.4 Transmit intermodulation | No specification impact |
| 7.3.1 Reference sensitivity | The regulation is relevant to fixed point-to-point and point-to-multipoint digital radio systems [2]. Since the regulation allows fixed services, in case of non Half-Duplex devices the design is based on large devices (CPEs), possibly with external antennas and more complex components to allow Full-Duplex operation. Moreover, the regulation requires robust techniques tolerant to interference and requires margins to take into account impairments. For categories NB1, NB2 and M1 Half-Duplex operation is assumed, which avoids the use of duplexers in the UE. It is proposed to adopt the values specified for band 3 |
| 7.4 Max input level | No specification impact |
| 7.5 Adjacent channel selectivity | No specification impact |
| 7.6.1 In-band blocking | New band needs to added into Table 7.6.1.1-2 |
| 7.6.2 Out of band blocking | New band needs to added into Table 7.6.2.1-2 |
| 7.6.3 Narrow band blocking | No specification impact |
| 7.7 Spurious response | No specification impact |
| 7.8 Intermodulation characteristics | No specification impact |

Table 2: TS36.101 impact analysis

| TS | Section | Table | Title table |  | Proposed value |
| --- | --- | --- | --- | --- | --- |
| 36.101 | 5.5 Operating bands | 5.5-1 | operating bands |  | add a new row with band [110] |
| 36.101 | 5.5E Operating bands for UE category 0, UE category M1 and M2 and UE category 1bis |  |  |  | add band [110] for cat 0, M1, M2 and 1bis UE category 0 is designed to operate in the E-UTRA operating bands 2, 3, 4, 5, 8, 13, 20, 25, 26 and 28 and [110] in both half duplex FDD mode and full-duplex FDD mode and in bands 39, 40 and 41 in TDD mode. The E-UTRA bands are defined in Table 5.5-1. UE category M1 and M2 is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 18, 19, 20, 21, 24, 25, 26, 27, 28, 31, 54, 66, 71, 72, 73, 74, 85, 87, 88, 106 and [110] in both half duplex FDD mode and full-duplex FDD mode, and in bands 39, 40, 41, 42, 43 and 48 in TDD mode. The E-UTRA bands are defined in Table 5.5-1. UE category 1bis is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 12, 13, 18, 20, 26, 28, 31, 66, 72 and [110] in full duplex FDD mode and in bands 34, 39, 40 and 41 in TDD mode. The E-UTRA bands are defined in Table 5.5-1 |
| 36.101 | 5.5F Operating bands for category NB1 and NB2 |  |  |  | add band [110] for cat NB1 and NB2 Category NB1 and NB2 are designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 28, 31, 41, 42, 43, 48, 54, 65, 66, 70, 71, 72, 73, 74, 85, 87, 88, 103, 106 and [110] which are defined in Table 5.5-1. Category NB1 and NB2 are designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n14, n18, n20, n24, n25, n26, n28, n31, n41, n54, n65, n66, n70, n71, n72, n74, n90. |
| 36.101 | 5.6.1 Channel bandwidths per operating band | 5.6.1-1 | E-UTRA channel bandwidth |  | add band [110] with channel bandwidth 1.4; 3; 5; 10 |
| 36.101 | 5.7.3 Carrier frequency and EARFCN | 5.7.3-1 | E-UTRA channel numbers |  | add band [110] with NOffs-DL = 73386; Range of NDL = (73386 - 73485) and NOffs-UL = 134342; Range of NUL = (134342 - 134441) |
| 36.101 | 5.7.4 TX–RX frequency separation | 5.7.4-1 | Default UE TX-RX frequency separation |  | add band [110] with 10 MHz TX-RX frequency separation |
| 36.101 | 6.2.2 UE maximum output power | 6.2.2-1 | UE Power Class |  | add band [110] with power class 23 dBm +-2 |
| 36.101 | 6.2.2E UE maximum output power for Category M1 and M2 UE | 6.2.2E-1 | UE Power Class |  | add band [110] with power class 23 dBm +-2 |
| 36.101 | 6.2.2F UE maximum output power for category NB1 and NB2 | 6.2.2F-1 | UE Power Class |  | add band [110] with power class 23 dBm +-2 |
| 36.101 | 6.6.3.2 Spurious emission band UE co-existence | 6.6.3.2-1 | Requirements |  | add a new row with band [110] to add the protected bands (for Region 2) - according to [1] band [110] does not require protection from other bands |
| 36.101 | 7.3.1 Minimum requirements (QPSK) | 7.3.1-1 | Reference sensitivity QPSK PREFSENS |  | The regulation is relevant to fixed point-to-point and point-to-multipoint digital radio systems [2]. Since the regulation allows fixed services, in case of non Half-Duplex devices the design is based on large devices (CPEs), possibly with external antennas and more complex components to allow Full-Duplex operation. Moreover, the regulation requires robust techniques tolerant to interference and requires margins to take into account impairments. For categories NB1, NB2 and M1 Half-Duplex operation is assumed, which avoids the use of duplexers in the UE. It is proposed to adopt the values specified for band 3 |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-1A | Reference sensitivity for FDD and TDD UE category 0 QPSK PREFSENS |  | add band [110] with the same values of band 3 |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-1B | Reference sensitivity for FDD and TDD UE category 0 QPSK PREFSENS |  | add band [110] with the same values of band 3 |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-2 | FDD and TDD UE category 0 Uplink configuration for reference sensitivity |  | add band [110] with the same values of band 3 |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-3 | Reference sensitivity for FDD and TDD UE category M1 QPSK PREFSENS | add band [110] with the same values of band 3 |  |  |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-4 | Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS |  | add band [110] with the same values of band 3 |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-5 | FDD and TDD UE category M1 Uplink configuration for reference sensitivity |  | add band [110] with the same values of band 3 |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-6 | Reference sensitivity for FDD and TDD UE category 1bis QPSK PREFSENS |  | add band [110] with the same values of band 3 |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-7 | FDD and TDD UE category 1bis Uplink configuration for reference sensitivity |  | add band [110] with the same values of band 3 |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-8 | Reference sensitivity for FDD /TDD UE category M2 QPSK PREFSENS | add band [110] with the same values of band 3 |  |  |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-9 | Reference sensitivity for HD-FDD category M2 QPSK PREFSENS |  | add band [110] with the same values of band 3 |
| 36.101 | 7.3.1E Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis | 7.3.1E-10 | FDD/HD-FDD and TDD UE category M2 Uplink configuration for reference sensitivity |  | add band [110] with the same values of band 3 |
| 36.101 | 7.6.1.1 In-band blocking Minimum requirements | 7.6.1.1-2 | In-band blocking |  | add band [110] |
| 36.101 | 7.6.2.1 Out-of-band blocking Minimum requirements | 7.6.2.1-2 | Out of band blocking |  | add band [110 |

Table 3: TS36.307 impact analysis

| TS | Section | Table | Title table | Proposed value |
| --- | --- | --- | --- | --- |
| 36.307 | A Frequency arrangement for overlapping operating bands | A-1 | Overlapping bands (multi-band environments) for each E-UTRA band | add band [110] overlapping with band 3 |

## Observations and Proposals

Observation 1: since the agreement in [1] is not to impose new UE requirements with this new band as the protected band from the existing 3GPP bands (i.e., Table 6.6.3.2-1 in TS 36.101, and Table 6.5.3.2-1 in TS 38.101-1), the introduction of the new band implies mainly the addition of a new row in existing table, using when needed the values defined for band 3.

Observation 2: Concerning the requirement in 36.101 ”6.2.4 A-MPR,” it is to be noted that:

The UL for the new band (1800-1810 MHz) is far from band 2 and 25 (40 MHz). Therefore,it is assumed that no A-MPR requirement is needed.

The UL for the new band (1800-1810 MHz) is far from band 4 (45 MHz) and 66 (20 MHz). Therefore,it is assumed that no A-MPR requirement is needed.

Proposal 1: it is proposed to endorse the analysis in Table 1

Proposal 2: it is proposed to agree on the values proposed in Table 2 for the modifications required to introduce the new LTE band 1800-1830 MHz for Canada

# Conclusion

In this contribution we have discussed the necessary changes into 3GPP specifications due to the introduction of a new LTE band in the 1800-1830 MHz frequency range.

Observation 1: since the agreement in [1] is not to impose new UE requirements with this new band as the protected band from the existing 3GPP bands (i.e., Table 6.6.3.2-1 in TS 36.101, and Table 6.5.3.2-1 in TS 38.101-1), the introduction of the new band implies mainly the addition of a new row in existing table, using when needed the values defined for band 3.

Observation 2: Concerning the requirement in 36.101 ”6.2.4 A-MPR,” it is to be noted that:

The UL for the new band (1800-1810 MHz) is far from band 2 and 25 (40 MHz). Therefore,it is assumed that no A-MPR requirement is needed.

The UL for the new band (1800-1810 MHz) is far from band 4 (45 MHz) and 66 (20 MHz). Therefore,it is assumed that no A-MPR requirement is needed.

Proposal 1: it is proposed to endorse the analysis in Table 1

Proposal 2: it is proposed to agree on the values proposed in Table 2 for the modifications required to introduce the new LTE band 1800-1830 MHz for Canada

# Reference

- RP-241649, “New WID: Introduction of LTE FDD band in 1800 – 1830 MHz for Canada,” Novamint, Sequans, Semtech, Telit, Ubiik

- R4-2411032, “Regulation for LTE FDD new band in 1800–1830 MHz for Canada,” Novamint, Ubiik