3GPP TSG-RAN WG4 Meeting#112							         			R4-2411049

Maastricht, Netherland, 19 – 23 August 2024

Title: 	On improved MPR for intra-band ULCA when only one CC has RBs allocated

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.1.1.2.2 MPR applicability for FR1 intra-band UL CA [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN4#111, a way forward [1] captured a number of agreements related to the Release 19 power domain enhancement. Notably, it covered the intra-band ULCA MPR improvement when only one of the two carriers is allocated with transmitted RBs. In this contribution, we address in particular whether the emissions requirement should be based on a single CC or on the configured ULCA.

# Discussion

## 2.1 Way forward inputs

In way forward [1], the following agreement was achieved on MPR improvements for intra-band contiguous ULCA.

Way forward:

- For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated.

  - FFS: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C.

  - FFS: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated.

  - FFS: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

  - FFS whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA

Furthermore, the way forward discussed the non-contiguous ULCA case in terms of the scope of work.

Way forward:

Enhancements not aligned with the WID will not be considered.

- Further clarify the scope for NC CA in next meeting based on the WID objectives.

In this contribution we address the last bullet of the contiguous ULCA way forward to understand if the single CC MPR can be used for all cases if the emissions are not based on the aggregated BW for CA.

Based on last RAN plenary version of the WI [2], non-contiguous ULCA is also in scope; we therefore further discuss the same issue for non-contiguous ULCA if this remains in scope.

## 2.2 Single PA and 2Tx implementations for ULCA without LO switching.

For intra-band ULCA implemented with two LO (DualPA IE), each carrier has its own LO thus when only one CC is allocated, one of Tx path can be off thus the UE is transmitting in the same mode than for a single carrier transmission. In that case, the single carrier emission (ACLR, SEM, spurious emissions) can be applied and the single carrier MPR is valid.

In the single PA or 2Tx cases however, both CCs are transmitted by one or two PAs using a single LO frequency placed in the middle of the total 2CC bandwidth. Assuming that the LO is not switched to avoid the transmit interruption, the transmitted signal is no longer the same as that of a single CC as the image and IMD products are in a different position.

To illustrate this, the two following figures show the contiguous and non-contiguous cases with TRX impairments for a single LO frequency intra-band ULCA implementations when only one of the carriers is allocated some RBs:

- Figure 1 for the intra-band contiguous case with two 100MHz contiguous carriers:

  - The fully allocated lower carrier: 100 full label (cyan)

  - The lower carrier allocated with lowest RB: 100 1RB0 label (dark blue)

  - The lower carrier allocated with highest RB: 100 1RB272 label (purple)

  - The intra-band contiguous ULCA SEM: 100+100 SEM label (orange dash)

  - The lower carrier single carrier SEM: 100 SEM label (red dash)

- Figure 2 for the intra-band non-contiguous case with 60MHz lower carrier and 40MHz upper carrier separated by 100MHz (allowable case for two CC per PA as the in-gap SEM requirement is -13dBm/MHz):

  - The fully allocated lower carrier: 50 full label (cyan)

  - The lower carrier allocated with lowest RB: 60 1RB0 label (dark blue)

  - The lower carrier allocated with highest RB: 60 1RB161 label (purple)

  - The intra-band non-contiguous ULCA SEM: 50+G100+50 SEM label (orange dash)

  - The lower carrier single carrier SEM: 50 SEM label (red dash).

Figure 1: Intra-band contiguous ULCA case

Figure 2: Intra-band non-contiguous ULCA case

Observations for intra-band contiguous ULCA case:

- When the single carrier SEM requirement is used, and assuming the 28dBc image allowance at MPR0:

  - The 1RB0 image fails the -13dBm/MHz SEM limit. Any RB allocation < 6.48MHz bandwidth would fail.

  - The fully allocated image passes thanks to the PSD spread.

- When the configured intra-band contiguous ULCA SEM is used, the image still falls in band of the two CCs thus there is no issue with the SEM requirement.

- Unless the LO is switched when there is allocation only one CC, the single carrier MPR is not sufficient to meet the single carrier SEM but using the configured contiguous ULCA SEM works.

Observations for intra-band non-contiguous ULCA case:

- When the single carrier SEM requirement is used, and assuming the 28dBc image allowance at MPR0: the image of any RB allocation fails the -30dBm/MHz spurious emissions.

- When the configured intra-band non-contiguous ULCA SEM is used:

  - The 1RB0 image still falls within the higher CCs thus there is no issue with the SEM requirement.

  - The image for the fully allocated case falls within the in-gap region and both fails the 60MHz single carrier SEM.

  - The image of the fully allocated cases pass the -13dBm/MHz in-gap SEM.

  - The image of the 1RB161 fails, as would any RB allocation < 6.48MHz bandwidth.

- If the LO is not switched, even when using the configured intra-band non-contiguous ULCA SEM, in the case of asymmetric CCs, some allocations would fail SEM if the single CC MPR is used.

## 2.3 Alternatives for contiguous intra-band ULCA.

In order to enable the use of the single carrier MPR, a solution shall be found for the issue with the image failing the single carrier SEM for the implementations with a single LO frequency (either single PA or 2Tx implementations that supports TxD and/or UL MIMO on top of UCCA).

One option is to always allow LO switching, but this may result in transmit interruptions. In that case, it may be worth allowing interruptions only when DualPA is not signalled or TxD or UL MIMO is signalled.

Another option is to always use the configured intra-band contiguous ULCA SEM even when the only one of the carriers is allocated with transmitted RBs. On top of the benefit of not requiring LO switching, this means that all allocation can be considered as inner since the SEM and ACLR is defined for the aggregated bandwidth.

Proposal for intra-band contiguous ULCA: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements.

## 2.4 Alternatives for non-contiguous intra-band ULCA.

For the non-contiguous ULCA case, using the configured intra-band non-contiguous ULCA SEM does prevent some allocation image to fail when the LO is not switched. In this case there would too much complexity to pick on which allocations would still benefit from the ULCA defined emissions and thus LO switching should be allowed but it only applies to the cases where DualPA is not signalled. In this case there is no additional benefit to use the ULCA defined emissions requirements versus the single CC defined emissions requirements, but there is no harm either.

Proposal for intra-band non-contiguous ULCA: When DualPA is not signalled or TxD or UL MIMO is signalled, transmit interruption may be needed to allow LO switching and the single carrier MPR can be sued when transmitted RBs are allocated in only one of the CC.

# Conclusion

In this contribution, we examined the intra-band contiguous and non-contiguous ULCA architectures and discussed the issues related to the image of the allocated RBs failing the single carrier SEM when only one carrier is allocated, and a single LO frequency is used and made the following proposals.

Proposal for intra-band contiguous ULCA: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements.

Proposal for intra-band non-contiguous ULCA: When DualPA is not signalled or TxD or UL MIMO is signalled, transmit interruption may be needed to allow LO switching and the single carrier MPR can be sued when transmitted RBs are allocated in only one of the CC.

# References

[1] R4-2410749 WF on power domain enhancements, Huawei, HiSilicon, RAN4#111

[2] RP‑241656 UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon, RAN#104