3GPP TSG-RAN WG4 Meeting #112                                                                                        R4-2411058

Maastricht, NL, Aug. 19 – Aug. 23, 2024

Source: 	CATT

Title: 	Discussion on SAN RF requirements for the NTN new S-band

Agenda item:	7.17.3

Document for:	Approval

- Introduction

In RAN #104 meeting, the work item to introduce a new NR-NTN FDD band was approved [1]. Part of the objective is shown as follows:

The objective of the core part is to: - Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario - Support channel bandwidths and SCS as presented in Table 4.1-1 below; - Mandatory support of channel raster points at step of 10 kHz; - Support UE Power class 3 (+23dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. Table 4.1-1: Channel bandwidth and SCS system parameters. SCS (kHz); Channel bandwidth (MHz). 15; 5; 10; 15; 20. 30; 10; 15; 20. 60; 10; 15; 20.

In this contribution, we would like to share some initial views on SAN requirements for introducing the NTN new S-band.

- Discussion

2.1 Operating bands

Per work item objectives, the intention of the new S-band is to introduce a new NR-NTN FDD band with a UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 – 2200 MHz. Following the Satellite band numbering scheme, the new NTN band should be numbered in descending order from n256. Since the number n253 has already been in use, for the new NR-NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the new band should be numbered as [n252] as illustrated in the table below.

Table 2.1-1: Satellite operating bands in FR1

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n252] | 2000 MHz - 2020 MHz | 2180 MHz - 2200MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the new band should be numbered as Table 2.1-1.

2.2 SAN channel bandwidth

According to the work item [1], the supported channel bandwidths and SCS of new NTN band are shown as follow:

Table 4.1-1: Channel bandwidth and SCS system parameters. SCS (kHz); Channel bandwidth (MHz). 15; 5; 10; 15; 20. 30; 10; 15; 20. 60; 10; 15; 20.

Therefore, the channel bandwidth and SCS of new NTN band could follow the existing framework of TS 38.108 and be specified as the following table [3]:

Table 2.2-1: SAN channel bandwidths and SCS per operating band in FR1

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| [n252] | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |

Proposal 2: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the channel bandwidth and SCS should be specified as Table 2.2-1 in specs.

2.3 Channel raster

According to the work item [1], the new NTN band needs to mandatorily support 10 KHz enhanced channel raster. Considering that the 10 KHz enhanced channel raster has been defined in TS 38.108, the new NTN band could follow the same approach. For new NTN band with 100 KHz channel raster, the step size could reuse the existing approach <20> from TS 38.108. For new NTN band with enhanced channel raster (10 KHz), the step size should be <2>. The NREF range will be calculated based on the existing global raster numerology. Therefore, the NR-ARFCN of new NTN band could be determined based on the existing conditions, as shown follow:

Table 2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

Table 2.3-2: Applicable NR-ARFCN per operating band for enhanced channel raster

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 |

Proposal 3: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the channel raster should be defined as Table 2.3-1 and Table 2.3-2.

2.4 Synchronization raster

For synchronization raster of NTN new band, it can be defined based on the same approach as the existing NTN bands. It is foreseeable that the SSB SCS and pattern of the new NTN band will reuse the existing S-band n256, and the step size of the GSCN will be <1>. Therefore, the GSCN entry is calculated in the same way as usual:

Table 2.4-1: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30kHz | Case C | 6218 – <1> – 6241 |
| [n252] | 15 kHz | Case A | 5456 – <1> – 5494 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Proposal 4: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster should be defined as Table 2.4-1.

2.5 RF requirements

Since the SAN RF requirements in TS 38.108 are defined in a band agnostic manner, for all SAN type 1-H and type 1-O requirements should be applicable to new NTN band [n252], which means that there is no SAN RF requirements impact by introducing new NTN band.

Proposal 5: For TS 38.108, no RF requirements impact by introducing the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz. The existing SAN type 1-H and 1-O RF requirements are applicable to new band.

- Conclusion

This contribution provides our initial views on SAN requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the new band should be numbered as Table 2.1-1.

Proposal 2: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the channel bandwidth and SCS should be defined as Table 2.2-1.

Proposal 3: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the channel raster should be defined as Table 2.3-1 and Table 2.3-2.

Proposal 4: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster should be defined as Table 2.4-1.

Proposal 5: For TS 38.108, no RF requirements impact by introducing the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz. The existing SAN type 1-H and 1-O RF requirements are applicable to new band.

- References

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- RP-241666, New WID on Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz), Inmarsat, Viasat, Thuraya

- TS 38.108, Satellite Access Node radio transmission and reception, V18.3.0, 3GPP Technical Specification