3GPP TSG-RAN WG4 Meeting #112                                                                                        R4-2411059

Maastricht, NL, Aug. 19 – Aug. 23, 2024

Source: 	CATT

Title: 	Discussion on UE RF requirements for the NTN new S-band

Agenda item:	7.17.2

Document for:	Approval

- Introduction

In RAN #104 meeting, the work item [1] on introduction of the new NR-NTN FDD band was approved. Part of the objective is shown as follows:

The objective of the core part is to: - Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario - Support channel bandwidths and SCS as presented in Table 4.1-1 below; - Mandatory support of channel raster points at step of 10 kHz; - Support UE Power class 3 (+23dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. Table 4.1-1: Channel bandwidth and SCS system parameters. SCS (kHz); Channel bandwidth (MHz). 15; 5; 10; 15; 20. 30; 10; 15; 20. 60; 10; 15; 20.

In this contribution, we would like to share some initial views on UE requirements for introduction of the NTN new S-band.

- Discussion

2.1 Operating bands

Per work item objectives, the intention of the new S-band is to introduce a new NR-NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 – 2200 MHz. Following the NTN Satellite band numbering scheme, the new NTN band should be numbered in descending order from n256. However, considering that the work item of NTN extended L-band [2] was approved as well, there is a possibility that the number n253 will be occupied. Therefore, for the new NR-NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the new band should be numbered as [n252].

Table 2.1-1: NTN Satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n252] | 2000 MHz - 2020 MHz | 2180 MHz - 2200MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the new band should be numbered as Table 2.1-1.

2.2 UE channel bandwidth

According to the work item [1], the supported channel bandwidths and SCS of new NTN band are shown as follow:

Table 4.1-1: Channel bandwidth and SCS system parameters. SCS (kHz); Channel bandwidth (MHz). 15; 5; 10; 15; 20. 30; 10; 15; 20. 60; 10; 15; 20.

Therefore, the channel bandwidth and SCS of new NTN band could follow the existing framework of TS 38.101-5 [3]:

Table 2.2-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| [n252] | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |

Proposal 2: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the channel bandwidth and SCS should be defined as Table 2.2-1.

2.3 Channel raster

According to the work item [1], the new NTN band needs to mandatory support 10 KHz channel raster. Considering that the 10 KHz channel raster has been defined in TS 38.101-5, the new NTN band could follow the same approach. For new NTN band with 100 KHz channel raster, the step size could reuse the existing approach <20> from TS 38.101-5. For new NTN band with enhanced channel raster (10 KHz), the step size should be <2>. The NREF range will be calculated based on the existing global raster numerology. Therefore, the NR-ARFCN of new NTN band could be determined based on the existing conditions, as shown follow:

Table 2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

Table 2.3-2: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n252] | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 |

Proposal 3: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the channel raster should be defined as Table 2.3-1 and Table 2.3-2.

2.4 Synchronization raster

For synchronization raster of NTN new band, it can be defined based on the same approach as the existing NTN bands. It is foreseeable that the SSB SCS and pattern of the new NTN band will reuse the existing S-band n256, and the step size of the GSCN will be <1>. Therefore, the GSCN entry is calculated in the same way as usual:

Table 2.4-1: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30kHz | Case C | 6218 – <1> – 6241 |
| [n252] | 15 kHz | Case A | 5456 – <1> – 5494 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Proposal 4: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster should be defined as Table 2.4-1.

2.5 TX-RX frequency separation

According to the work item, there is no description of the asymmetric channel bandwidths. Therefore, the fixed TX-RX frequency separation could be applied. The TX-RX frequency separation could be calculated based on the carrier centre frequency, as shown below:

Table 2.5-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [n252] | 180 MHz |

Proposal 5: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the TX-RX frequency separation should be defined as Table 2.5-1.

2.6 UE maximum output power

According to the work item objective, the new NR-NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz should support UE power class 3, hence the maximum output power requirements could be defined as follow:

Table 2.6-1: UE Power Class

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| [n252] | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

Proposal 6: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the maximum output power should be defined as Table 2.6-1.

2.7 UE RF Regional requirements

For NTN UE RF requirements, not all RF requirements are band agnostic. There are certain RF requirements are affected by regional regulations, as shown in the table below:

Table 2.6-1: NTN UE RF requirements affected by regional regulations

| Section | Requirement | Note |
| --- | --- | --- |
| Tx requirements |  |  |
| 6.2.3 | UE additional maximum output power reduction | Depends on regional regulation requirements |
| 6.5.2.3 | Additional spectrum emission mask | Depends on regional regulation requirements |
| 6.5.3.2 | Spurious emissions for UE co-existence | Depends on regional regulation requirements |
| 6.5.3.3 | Additional spurious emissions | Depends on regional regulation requirements |
| Rx requirements |  |  |
| 7.3 | Reference sensitivity | Depends on regional requirements |

According to the work item description, the proposed new NTN S-band will not be deployed world-wide but in North America. Therefore, it is beneficial for RAN4 to clarify the intended regions and/or countries where the new NTN bands are envisaged to be deployed, in order to finalize which regional requirements and NS signalling values apply.

Proposal 7: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, RAN4 should clarify the intended regions and/or countries where the new NTN bands deployed.

- Conclusion

This contribution provides our initial views on UE requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the new band should be numbered as Table 2.1-1.

Proposal 2: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the channel bandwidth and SCS should be defined as Table 2.2-1.

Proposal 3: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the channel raster should be defined as Table 2.3-1 and Table 2.3-2.

Proposal 4: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster should be defined as Table 2.4-1.

Proposal 5: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the TX-RX frequency separation should be defined as Table 2.5-1.

Proposal 6: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the maximum output power should be defined as Table 2.6-1.

Proposal 7: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, RAN4 should clarify the intended regions and/or countries where the new NTN bands deployed.

- References

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- RP-241666, New WID on Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz), Inmarsat, Viasat, Thuraya

- TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements, V18.5.0, 3GPP Technical Specification