3GPP TSG-RAN WG4 Meeting #112                                                                                        R4-2411062

Maastricht, NL, Aug. 19 – Aug. 23, 2024

Source: 	CATT

Title: 	Discussion on SAN RF requirements for NTN support spectrum less than 5 MHz

Agenda item:	8.8.3.3

Document for:	Approval

- Introduction

In RAN #104 meeting, the work item for enhanced requirements and test methodology for NR and IoT NTN was approved [1]. Part of the objective is shown as follows:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

In this contribution, we would like to share some initial views on SAN requirements for support spectrum less than 5 MHz.

- Discussion

Table 2.1 provides all SAN RF Tx and Rx requirements that might be affected by the introduction of 3 MHz channel bandwidth. Since the work item is FR1 oriented, no OTA requirements will be impacted. Meanwhile, Table 2.1 also provides the NR TN BS requirements that are affected by the introduction of 3 MHz channel bandwidth as a reference.

Table 2.1: SAN RF requirements impact by 3 MHz

| NR NTN SAN RF Tx / Rx requirements | NR TN BS spec affected by 3 MHz | Proposal for NTN 3 MHz |
| --- | --- | --- |
| Output power | Channel bandwidth agnostic, but additional requirements (regional) introduced for certain region such as n100 | No specification impact No additional regional requirement for NTN SAN were identified |
| Total power dynamic range | 3 MHz channel bandwidth and dynamic range minimum requirements introduced | Reuse the TN BS approach for 3 MHz channel bandwidth |
| Adjacent Channel Leakage Power Ratio | 3 MHz channel bandwidth introduced No ACLR limit impact | Introduce 3 MHz channel bandwidth No ACLR limit impact |
| Out of band emissions (OBUE for BS) | 3 MHz channel bandwidth and operating band unwanted emission minimum requirements introduced | No specification impact |
| Reference sensitivity level | 3 MHz channel bandwidth and REFSENSE minimum requirements introduced FRC for 3 MHz defined | Introduce 3 MHz channel bandwidth for REFSENSE requirement, both GEO and LEO are affected Introduce new FRC for 3 MHz REFSENSE |
| Dynamic range | 3 MHz channel bandwidth and dynamic range minimum requirements introduced FRC for 3 MHz defined | Introduce 3 MHz channel bandwidth for dynamic range requirement Introduce new FRC for 3 MHz dynamic range |
| Adjacent Channel Selectivity (ACS) | 3 MHz channel bandwidth introduced The wanted signal power, interfere signal offset and type of interfering signal was modified | Reuse the TN BS approach for 3 MHz channel bandwidth |
| In-channel selectivity (ICS) | 3 MHz channel bandwidth and ICS minimum requirements introduced | Introduce 3 MHz channel bandwidth for ICS requirement Introduce new FRC for 3 MHz ICS |

Therefore, we believe that RAN 4 could use the above table as the starting point for introducing 3 MHz channel bandwidth.

Proposal 1: To use the proposals in Table 2.1 for NTN SAN RF requirements for NR NTN support 3 MHz channel bandwidth.

- Conclusion

This contribution provides our initial views on SAN requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Proposal 1: To use the proposals in Table 2.1 for NTN SAN RF requirements for NR NTN support 3 MHz channel bandwidth.

- References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)