3GPP TSG-RAN WG4 Meeting #112                                                                                        R4-2411063

Maastricht, NL, Aug. 19 – Aug. 23, 2024

Source: 	CATT

Title: 	Discussion on UE RF requirements for NTN support spectrum less than 5 MHz

Agenda item:	8.8.3.2

Document for:	Approval

- Introduction

In RAN #104 meeting, the work item for enhanced requirements and test methodology for NR and IoT NTN was approved [1]. Part of the objective is shown as follows:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

In this contribution, we would like to share some initial views on UE requirements for support spectrum less than 5 MHz.

- Discussion

Table 2.1 provides all UE RF Tx and Rx requirements that might be affected by the introduction of 3 MHz channel bandwidth. Since the work item is FR1 oriented, no OTA requirements will be impacted. Meanwhile, Table 2.1 also provides the NR TN UE requirements that are affected by the introduction of 3 MHz channel bandwidth as a reference.

Table 2.1: UE RF requirements impact by 3 MHz

| NR NTN UE RF Tx / Rx requirements | NR TN UE spec affected by 3 MHz | Proposal for NTN 3 MHz |
| --- | --- | --- |
| UE maximum output power reduction | the Pi/2 BPSK edge allocation MPR for 3 MHz channel bandwidth was introduced | No specification impact The MPR requirements for NTN UE are refer to TS 38.101-1 |
| UE additional maximum output power reduction | 3 MHz channel bandwidth introduced A-MPR requirement for different network signalling value introduced | Introduce 3 MHz channel bandwidth and A-MPR requirements for all NTN network signalling values |
| Output power dynamic | 3 MHz channel bandwidth introduced No output power dynamic impact | Introduce 3 MHz channel bandwidth No output power dynamic requirements impact, but the measurement bandwidth need to be updated |
| Occupied bandwidth | 3 MHz channel bandwidth introduced | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Spectrum emissions mask | 3 MHz channel bandwidth introduced spectrum emissions limit requirements introduced | Introduce 3 MHz channel bandwidth for General spectrum emissions mask Introduce new spectrum emissions limit for 3 MHz CBW |
| Additional spectrum emissions mask | 3 MHz channel bandwidth introduced Additional spectrum emissions limit requirements introduced for different network signalling values | Further evaluation required Additional requirements for NS_04N and NS_05N are CBW agnostic |
| Adjacent Channel leakage ratio (ACLR) | 3 MHz channel bandwidth introduced No ACLR limit impact | Reuse the TN UE approach for 3 MHz channel bandwidth |
| General spurious emissions | 3 MHz channel bandwidth and correspondingly FOOB (6 MHz) introduced | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Additional spurious emissions | 3 MHz channel bandwidth introduced | Introduce 3 MHz channel bandwidth for all network signalling values |
| Reference sensitivity power level | 3 MHz channel bandwidth introduced REFSENS requirement for 3 MHz channel bandwidth introduced | Introduce 3 MHz channel bandwidth Introduce new REFSENS requirement for 3 MHz channel bandwidth |
| Maximum input level | 3 MHz channel bandwidth introduced No power limit impact | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Adjacent channel selectivity (ACS) | 3 MHz channel bandwidth introduced Bandwidth and offset of interferer signal was modified | Introduce 3 MHz channel bandwidth Introduce new bandwidth and offset of interferer signal for 3 MHz channel bandwidth |
| In-band blocking | 3 MHz channel bandwidth introduced Bandwidth and offset of interferer signal was modified | Introduce 3 MHz channel bandwidth Introduce new bandwidth and offset of interferer signal for 3 MHz channel bandwidth |
| Out-of-band blocking | 3 MHz channel bandwidth introduced No blocking requirement impact | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Narrow band blocking | 3 MHz channel bandwidth introduced Wanted signal power was modified | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Spurious response | 3 MHz channel bandwidth introduced No spurious response requirement impact | Reuse the TN UE approach for 3 MHz channel bandwidth |
| Wide band intermodulation | 3 MHz channel bandwidth introduced | No specification impact The IMD requirements for NTN UE are refer to TS 38.101-1 |

Therefore, we believe that RAN 4 could use the above table as the starting point for introducing 3 MHz channel bandwidth.

Proposal 1: To use the proposals in Table 2.1 for NTN UE RF requirements for NR NTN support 3 MHz channel bandwidth.

- Conclusion

This contribution provides our initial views on UE requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Proposal 1: To use the proposals in Table 2.1 for NTN UE RF requirements for NR NTN support 3 MHz channel bandwidth.

- References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)