3GPP TSG-RAN WG4 Meeting #112                                                                                        R4-2411066

Maastricht, NL, Aug. 19 – Aug. 23, 2024

Source: 	CATT

Title: 	Discussion on coexistence study for NTN support HPUE

Agenda item:	8.8.2.1

Document for:	Approval

- Introduction

In RAN #104 meeting, the work item for enhanced requirements and test methodology for NR and IoT NTN was approved [1]. Part of the objective is shown as follows:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, we would like to share some initial views on how to conduct the coexistence study for NR NTN HPUE.

- Discussion

  - Co-existence simulation scenario

Refer to the work item objectives, the example bands for introducing HPUE are bands n256 and 256, n255 and 255, and the impact on the uplink performance of TN network will be evaluated by co-existence study. It would be beneficial to leverage the approach of the FR1 NTN coexistence study with some modifications. Therefore, the proposed scenarios for NTN HPUE coexistence study are shown as below. Similar to the FR1-NTN scenario, dense urban will not be considered since the NTN UE will connect to TN network. Meanwhile, the coexistence study between HAPS is mainly focus on the HAPS ACLR and HAPS ACS, which is out of the scope.

Table 2.1-1: Proposed scenarios for NTN HPUE co-existence

| FR1: 2GHz |  | NTN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  | Set 2 |  |  |
|  |  | GEO | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |
| NR / NB-IoT | Rural | X | X | X | X | X | X |
|  | Urban macro | X | X | X | X | X | X |
| NOTE: Two sets of satellite parameters are listed in Table 6.2.2.1-2 and Table 6.2.2.1-3 according to TR 38.821 [2] |  |  |  |  |  |  |  |

Considering that the purpose of coexistence study is to evaluate the impact from high Tx power to TN network UL, the aggressor could be focus on the NTN HPUE ACLR and ACS, as shown below.

Table 2.1-2: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | ACS TN gNB fixed ACLR NTN UE to be varied/defined |
| 2 | TN with NTN | NTN UL | TN DL | ACS TN gNB fixed ACLR NTN UE to be varied/defined |
| 3 | TN with NTN | TN DL | NTN DL | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 4 | TN with NTN | TN UL | NTN DL | ACLR TN UE fixed ACS NTN UE to be varied/defined |

For frequency and bandwidth of the coexistence study, the FR1 NTN approach could be reused, as shown below.

Table 2.1-3: Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Rural | 2 GHz | 20MHz | FDD, TDD | 1 |
| TN Urban macro | 2 GHz | 20MHz | FDD, TDD | 1 |
| GEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| LEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| NOTE: Only FRF=1 has been used in co-existence studies for simplification. |  |  |  |  |

Proposal 1: For NR NTN HPUE coexistence study, the simulation scenario could consider the proposed Table 2.1-1 to Table 2.1-3 as a baseline.

2.2 Network layout model

For network layout model, it is foreseeable that the HPUE will be deployed in a similar manner to PC3 NTN UE, so the network layout model for FR1 NTN coexistence study could be considered as a reference baseline. However, referring to the past HPUE discussion, it may also be necessary to discuss NTN HPUE ratios for simulated network layout. It is obviously that for NTN cells, the UE of different power class might be deployed in a single cell, so the ratio of NTN HPUE should be clarified as well.

Proposal 2: For NR NTN HPUE coexistence study, the network layout model for FR1 NTN coexistent study could be considered as baseline. Meanwhile, the ratio of NTN HPUE may need to be clarified.

2.3 Simulation parameters assumptions

Similar to the discussion for network layout model above, for satellite parameters including set-1 and set-2 and NTN PC 3 UE (if any), the system parameters in TR 38.863 [3] could be considered as baseline. Meanwhile, if RAN4 confirms that the ratio of HPUEs in the NTN cell is not 100%, the number of active UEs needs to be updated accordingly:

Table 2.3-1: Other parameters for NTN satellite

| Parameters | NTN satellite | Remark |
| --- | --- | --- |
| Carrier frequency | 2GHz |  |
| The number of active UE (UL) | 9 UEs and 2RBs per UE for GEO and LEO | NOTE |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 6.2.6.2 |  |
| NTN satellite Noise Figure in dB | See Table 2.3-3-1 |  |
| Handover margin | 3dB |  |

For NR NTN HPUE parameters, since the power class of NR NTN HPUE will be supported up to PC1, the NR NTN HPUE characteristics should also include the non-handheld UE. For handheld NR NTN HPUE, the UE characteristics in TR 38.863 could be considered as the baseline, but for non-handheld NR NTN HPUE, the antenna type and antenna gain may need further discussion. Table 2.3-2 illustrates this issue below with PC2 as an example:

Table 2.3-2: NR NTN HPUE (PC2) characteristics for system level simulations

| Characteristics | Handheld | Non-handheld |
| --- | --- | --- |
| Frequency band | S band (i.e. 2 GHz) | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element | To be discussed |
| Polarisation | Linear: +/-45°X-pol | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element | To be discussed |
| Antenna temperature | 290 K | 290 K |
| Noise figure | 9 dB | 9 dB |
| Tx transmit power | 400 mW (26 dBm) | 400 mW (26 dBm) |
| Tx antenna gain | 0 dBi per element | To be discussed |

Proposal 3: For NR NTN HPUE coexistence study, the satellite parameters for FR1 NTN coexistent study could be considered as baseline.

Proposal 4: For NR NTN HPUE coexistence study, the handheld UE characteristic for FR1 NTN coexistent study could be considered as baseline. For Non-handheld HPUE, the UE characteristic may need further discussion.

- Conclusion

This contribution provides our initial views on how to conduct the coexistence study for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: For NR NTN HPUE coexistence study, the simulation scenario could consider the proposed Table 2.1-1 to Table 2.1-3 as a baseline.

Proposal 2: For NR NTN HPUE coexistence study, the network layout model for FR1 NTN coexistent study could be considered as baseline. Meanwhile, the ratio of NTN HPUE may need to be clarified.

Proposal 3: For NR NTN HPUE coexistence study, the satellite parameters for FR1 NTN coexistent study could be considered as baseline.

Proposal 4: For NR NTN HPUE coexistence study, the handheld UE characteristic for FR1 NTN coexistent study could be considered as baseline with 26 dBm Tx power. For Non-handheld HPUE, the UE characteristic may need further discussion.

- References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)

- TR 38.821, Solutions for NR to support Non-Terrestrial Networks (NTN), V16.2.0, 3GPP technical report

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects, V18.1.0, 3GPP technical report