3GPP TSG-RAN WG4 Meeting #112                                                                                        R4-2411067

Maastricht, NL, Aug. 19 – Aug. 23, 2024

Source: 	CATT

Title: 	Discussion on UE Tx requirement for NTN support HPUE

Agenda item:	8.8.2.1

Document for:	Approval

- Introduction

In RAN #104 meeting, the work item for enhanced requirements and test methodology for NR and IoT NTN was approved [1]. Part of the objective is shown as follows:

High power UE (HPUE) for NTN - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, we would like to share some initial views on UE Tx requirements for NR NTN HPUE.

- Discussion

According to the work item description, the power classes that will be introduced for NR NTN UE include PC2, PC1.5 and PC1. Since the HPUE feature can be traced back to LTE and involve several WIs, HPUE architecture are diverse and may have an impact on different requirements.

In general, HPUE operating under PC2 could be deployed via carrier aggregation/dual connectivity or single band TDD/FDD. However, it is clear that for the FR1 NTN UE, the CA/DC or TDD is not supported, so the NTN HPUE is more likely to be deployed by single band FDD approach. Therefore, for NTN HPUE operating PC2, 26dBm antenna port output power is required in the uplink single band.

For FR1 handheld NTN UE, omni-directional antenna is typically used, which means that a better performance PA may be required to achieve higher transmit power. For FR1 non-handheld NTN UE, it may be possible to use additional Tx chain, but in any case RAN4 should specify how to achieve the output power of 26 dBm by NTN HPUE.

For NTN HPUE support higher power class such as power class 1.5 and power class 1, only non-handheld UE are usually used under these power classes. However, with reference to the existing HPUE, PC1.5 is usually implemented via dual Tx. For higher power class NTN HPUE, RAN4 may also need to specify the architecture which could achieve higher transmit power.

Observation 1: RAN4 may need to discuss the NTN HPUE architecture for each of the supported power classes.

Proposal 1: For NTN HPUE, both 1Tx and 2Tx architecture could be considered for band n265, n255 and n254.

Observation 2: Different MPR and A-MPR requirements may need to be specified to support NTN HPUE 1Tx and 2Tx architectures.

- Conclusion

This contribution provides our initial views on UE Tx requirements for NR NTN HPUE. The following proposals are concluded as follows:

Observation 1: RAN4 may need to discuss the NTN HPUE architecture for each of the supported power classes.

Proposal 1: For NTN HPUE, both 1Tx and 2Tx architecture could be considered for band n265, n255 and n254.

Observation 2: Different MPR and A-MPR requirements may need to be specified to support NTN HPUE 1Tx and 2Tx architectures.

- References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)