3GPP TSG-RAN WG4 Meeting #112                                                                                        R4-2411068

Maastricht, NL, Aug. 19 – Aug. 23, 2024

Source: 	CATT

Title: 	Discussion on transient time for SAN

Agenda item:	8.25.3

Document for:	Approval

- Introduction

In RAN4 #111 meeting, A WF [1] on NR NTN phase 3 SAN RF requirements was agreed. However, there are still some remaining issues about SAN transient time need to be discussed in the next meeting.

In this contribution, we would like to share our views on transient time for NTN SAN.

- Discussion

The agreement on RF requirements enhancements for spatial domain techniques was achieved during the previous meetings, as shown below:

Issue 1-3-1: Cell DTX Issue 1-3-2: RF requirements enhancements for spatial domain techniques - Agreement: companies encourage to further investigate SAN beam hopping in DL coverage enhancements for both FR1 and FR2 - clarify “transient time” meaning, if for beam switching delay or PA switch ON/OFF or frequency carrier. - Synchronisation aspects: - between ground gNB and satellite payload for transparent architecture; - between two or more transparent satellites serving the same Earth-fixed cell. FFS for Earth-moving cell. - NOTE: FFS if above issue could be treated as requirement or implementation specific. - Working hypothesis: PA always on, phase shift (pre-)reconfiguration to perform beam-hopping. - NOTE1: [RAN1 agreement] For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to zero at least if SAN phase array antenna is assumed. Values different from zero beam steering latency can be optionally reported by companies if any potential issue identified. - NOTE2: Other implementation with analog beam steering technologies are not precluded. Companies to further discuss if analog/digital antenna assumptions and for which FR1/FR2 implementations.

The definition of transient period for TN network BS is defined in TS 38.104:

Transmitter transient period requirements apply only to TDD operation of the BS. The transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa.

According to the current NTN SAN spec TS 38.108, there is no transient period requirements defined as both FR1 and FR2 SAN will operate in FDD mode. For devices operating in TDD mode, the UL/DL switch is usually implemented by a circulator and the delay will be caused by the entire RF chain (For instance, when the UL/DL switching occurs, the LNA can only operate after the PA is completely switched off). In addition, the terminology "transient" in the current 3GPP specification is only used to describe the requirements for UL/DL switching for TDD device, and the introduction of “transient” related requirements may cause some confusion. Therefore, there is no need for additional "transient" discussion for NTN SAN operating in FDD mode.

Observation 1: The terminology “transient time” may cause some confusion.

Referring to the discussion in RAN4 #111 meeting, the discussion focused on where the “delay” comes from. However, for NTN SAN (FR1 and FR2) all requirements are defined on the TAB connector or RIB, as shown below:

Figure 4.3.1-1: Radiated and conducted reference points for SAN type 1-H

For the requirements defined in TAB connector, it is not able to characterise the antenna related performance. If the phase array antenna is considered to have some delay in beam switching action, the delay comes from the phase array antenna, not the left part of the reference point (RF component). As a result, the delay associated with phased array antennas can only be defined and tested in OTA manner, which causes some additional difficulties for NTN SAN testing.

Observation 2: The delay caused by phase array antenna may need to be tested in OTA manner.

Furthermore, the behaviour of NTN SAN with beam hopping could be considered as an implementation related topic. For SAN that supporting beam hopping feature, the transmission RF chain is usually stay turned on, so the served UE can get higher CBW and power with a focused beam. For phase array antennas, switching the beam usually takes only a few nano-seconds so no additional impact on the RF requirements. As for the delay caused by configuring the beam hopping, this will depend on the implementation such as different algorithms used by the SAN. In extreme cases, the SAN can use a completely pre-defined beam hopping pattern where there is no delay at all.

Therefore, we believed that for NTN SAN, the beam switching delay caused by phase array antenna has no effect on the RF requirements for NTN SAN.

Proposal 1: There is no RF requirements impact by the beam switching delay of phase array antenna.

- Conclusion

This contribution provides our initial views on transient time for NTN SAN. The following proposals are concluded as follows:

Proposal 1: There is no RF requirements impact by the beam switching delay of phase array antenna.

- References

- R4-2410009, Way Forward for [111][315] NR_NTN_Ph3, Thales, RAN4# 111