3GPP TSG-RAN WG4 Meeting #112                                                                                        R4-2411069

Maastricht, NL, Aug. 19 – Aug. 23, 2024

Source: 	CATT

Title: 	Discussion on RF requirements for NTN RedCap UE

Agenda item:	8.25.2.1

Document for:	Approval

- Introduction

In RAN4 #111 meeting, A WF [1] on NR NTN phase 3 UE RF requirements was agreed. However, there are still some remaining issues about NR NTN UE requirements need to be discussed in the next meeting.

In this contribution, we would like to share our views on NR NTN (e) RedCap UE requirements.

- Discussion

The agreement on NTN RedCap UE RF requirements was achieved during the previous meetings, as shown below:

##### Issue 1-4: Frequency error Agreement: Discuss changes common to non-Redcap NTN and (e)RedCap NTN NR UE frequency error in maintenance agenda. (e)RedCap UE frequency error requirement may be impacted by this maintenance discussion. Option for further discussionn, not an agreement: - As a starting point, consider (e)Redcap UE frequency error based on existing NR UE requirements. ##### Issue 1-5: Maximum output power options for further discussion: Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band. Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE ##### Issue 1-6: Specification structure and baseline changes Agreement: Use R4-2408816 as starting point for (e)RedCap NR NTN UE RF requirements specification structure. Further changes on requirement values and additional impacted clauses (e.g. according to issue 1-4 and 1-5) as well as other improvements can be added. FFS for suffix letter.

According to the current NTN UE spec TS 38.101-5, the frequency error requirements are shown as below:

### 6.4.1 Frequency error The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

NTN RedCap UE has not yet defined the frequency error requirements. However, for a NTN system, the frequency pre-compensation is required for NTN UE completing the demodulation. Therefore, the Frequency error is necessary for NTN RedCap UE. In addition, the current minimum requirement for frequency error are ± 0.1 PPM for NTN UE and TN UE, which is relatively relaxed, so we believe that reuse this requirements may not cause an additional problem.

Proposal 1: The NR UE frequency error requirements could be reused for NTN (e)RedCap UE.

According to the current NTN UE spec TS 38.101-5, the maximum output power requirements are shown as below:

### 6.2.1 UE maximum output power The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). Table 6.2.1-1: UE Power Class NR satellite band; Class 3 (dBm); Tolerance (dB). n256; 23; ±2. n255; 23; ±2. n254; 23; ±2. NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated.

For NTN UE operating at power class 3, the tolerance is ± 2 dB which means that the actual antenna port Tx power can be up to 25 dBm. The tolerance takes into account manufacturing tolerances, temperature and other factors that may have an effect on the transmitter power, meaning that if the power class is relaxed at the design stage (such as 25 dBm), the final transmit power may be much higher than that of PC3(probably 27 dBm).

In fact, the maximum transmission power of PC3 is already close to PC2 (after considering tolerace), which means that further relaxation may cause the NTN (e)RedCap UE to encounter the problems faced by PC2. For instance, additional SAR limit would require the introduction of additional signalling, and higher Tx power may cause some coexistence issues. Therefore, the relaxation of transmit power may require further evaluation. Considering that RAN4 is conducting NTN HPUE related evaluation, it may be possible to leverage the results of the NTN HPUE related evaluation to reduce the work load of RAN4.

Observation 1: The power level for PC3 HD-FDD (e) NTN RedCap could refer to the conclusions of the NTN HPUE evaluation.

For TS 38.101-5, there is no suffix table in current specification, as shown below:

## 4.3 Specification suffix information Specification suffix information is not defined for the time being in Release 17.

Therefore, we believed that the clause 4.3, specification suffix information should be updated.

Considering that no suffix has been defined in TS 38.101-5 yet, RAN4 may consider defining the definition table of suffix starting from suffix A, as shown below:

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | RedCap |

Proposal 2: RAN4 may consider the definition of suffixes for NTN RedCap / eRedCap UE shown in Table 4.3-1 as starting point.

- Conclusion

This contribution provides our initial views on NR NTN (e) RedCap UE requirements. The following proposals are concluded as follows:

Proposal 1: The NR UE frequency error requirements could be reused for NTN (e)RedCap UE.

Observation 1: Observation 1: The power level for PC3 HD-FDD (e) NTN RedCap could refer to the conclusions of the NTN HPUE evaluation.

Proposal 2: RAN4 may consider the definition of suffixes for NTN RedCap / eRedCap UE shown in Table 4.3-1 as starting point.

- References

- R4-2410574, WF on Rel-19 phase 3 NR-NTN UE RF requirements, Qualcomm, RAN4# 111.