3GPP TSG-RAN WG4 Meeting #112				                                                                            R4-2411070

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	CATT

Title: 	Discussion on SBFD general issues

Agenda item:	8.19.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for SBFD general issues including specification, operating bands, CLI impact on RF requirements, sub-band configuration and guard band size.

- Discussion

  - Specification

After two meetings discussion, there’re three options on the table for the specification,

- Embedded in TS 38.104, no new clauses are created.

- Embedded in TS 38.104, new clauses with suffixes are created.

- A new specification for SBFD BS RF.

There’re many new RF requirements for SBFD BS compared with non-SBFD BS, so both option 2 and option 3 work. Option 1 can be used when there’re very small different requirements like ATG and HAPS. Option2 has the pros that non-SBFD slots requirements and SBFD slots requirements can be found in the same spec. But there’re some concerns that when more and more features are introduced, 104 spec will be lengthy and not clean like the situation for UE RF specification TS 38.101-1. Our understanding is that there may not be as many new BS RF requirements for the new features like UE. The problem may not be that severe. Option 3 can avoid the concerned situation. The drawback maybe the non-SBFD requirements would need to refer to TS 38.104. This can be discussed further how to make the specifications clearer if new spec is agreed in the group. It seems neither of the options is perfect, we’re ok with both option 2 and option 3 and slightly prefer option 3.

Observation 1: A new specification for SBFD BS or new clauses with suffixes in TS 38.104 works for SBFD RF requirements. New specification is slightly preferred.

  - Operating bands for SBFD operation

Regarding the operating bands, there’s some discussion in last RAN4 meeting that if the band support should be declare based or some restriction such as  > BW xMHz can be added. In our understanding, the BW restriction in specification may not be that necessary because the bands and BW support for BS is declared based. There’s another reason that what’s the exact BW restriction may not have very good justifications at current stage. Operators may have different request for every specific bands. So we propose both the band and BW can be declaration based.

Proposal 1: Operating band and BW support for SBFD feature are declare based for the RF requirements.

  - CLI handling impact on RF requirement

There’s some discussion that if CLI handling should impact the RF requirement. Our understanding is that RF requirement is defined for single network node, but CLI is related to more than one node. So CLI handling should not impact RF requirement.

Proposal 2: RF requirements are defined without consideration of CLI handling.

  - Sub-band configuration and guard band size

For the sub-band configuration and guard band size, it should be common understanding that RAN4 RF requirements discussion will not impact RAN1 discussion. The following is the agreement in RAN1#117 meeting,

==========================================

Agreement

For cell-specific indication of SBFD subband frequency location, adopt the following option.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

==========================================

It can be seen from the agreement that RAN1 design is flexible enough that RAN4 RF requirements can define any feasible sub-band configuration and guard band size. How to define the requirements in RAN4 was discussed in previous meetings. There were some proposals that sub-band configuration and guard band size can be declaration based for the RF requirements. After more consideration, we think this proposal works for both the requirements and conformance test.

Proposal 3: the RF requirements can be defined based on the declared sub-band configuration and guard band size.

Some concerns were raised that the conformance test requirements need specific sub-band configuration and GB size to be defined. For non-SBFD single carrier test, the test model supports all of the possible BWs. For the Rx requirements, one FRC applies to several BWs. SBFD feature can also use this approach to solve the possible issues for the different GB declaration. For the sub-band configuration, if the Rx requirements can be defined to adapt to the possible BWs, there’re not much issues for the Tx requirements. The RB allocation for sub-band D can be full RB, the RB number is based on the vendor’s declaration. For sub-band U, a relative smaller BW FRC can be used to accommodate the different GB size. The test should be performed on each of B, M and T in sub-band U. With these analysis, we have the following high level proposal for the conformance test to address the concerns for conformance test.

Proposal 4: The following high level principles can be considered to solve the concerns of declaration based sub-band configuration and GB size.

1）Both DUD and DU patterns are supported by the conformance test.

2）RB allocation can be full RB for sub-band D, relative smaller specific BW for sub-band U FRC is used, for example, 10MHz for FR1 FRC and 20 MHz for FR2 FRC can be considered.

3) The test shall be performed on each of B, M and T in sub-band U.

- Conclusion

This contribution provides our further analysis for SBFD general issues, we have the following observation and proposals.

Observation 1: A new specification for SBFD BS or new clauses with suffixes in TS 38.104 works for SBFD RF requirements. New specification is slightly preferred.

Proposal 1: Operating band and BW support for SBFD feature are declare based for the RF requirements.

Proposal 2: RF requirements are defined without consideration of CLI handling.

Proposal 3: The RF requirements can be defined based on the declared sub-band configuration and guard band size.

Proposal 4: The following high level principles can be considered to solve the concerns of declaration based sub-band configuration and GB size.

1）Both DUD and DU patterns are supported by the conformance test.

2）RB allocation can be full RB for sub-band D, relative smaller specific BW for sub-band U FRC is used, for example, 10MHz for FR1 FRC and 20 MHz for FR2 FRC can be considered.

3) The test shall be performed on each of B, M and T in sub-band U.

- References

[1] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111

[2] R4-2410108, “Topic summary for [111][313] NR_duplex_evo”, Samsung, RAN4#111