3GPP TSG-RAN WG4 Meeting #112				                                                                            R4-2411072

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	CATT

Title: 	Discussion on AIoT device RF requirement

Agenda item:	8.20.3.2

Document for:	Discussion

- Introduction

This contribution provides our further consideration on AIoT device RF requirements.

- Discussion

  - General

According to RAN1 discussion, the three device types have different architecture assumption thus have different performance. The different architecture leads to different RF performance, such as maximum output power, REFSENS.  Passive devices have the advantage of low cost, size but with relatively lower performance compared with active devices. With more energy and higher cost, active device can have remarkably performance improvement, thus the requirements should be discussed separately according to the different architecture. Although from name perspective, device 2a seems more like device 2b, but device 2a is still a passive device. In our understanding, device 1 and device 2a can still be discussed together from RF requirements perspective. The same set of requirements with possibly different levels for device 1 and device 2a can be defined. So we have the following proposal.

Proposal 1: Separate the RF requirements discussions for passive and active devices.

  - Device 1 and Device 2a

Bothe Device 1 and Device 2a are passive devices. They are based on backscattering and are cost sensitive..

The following two architectures for Device 1 and Device 2a are discussed in RAN1 and captured in TR38.848.

Figure 1: A-IoT Device Type 1 architecture assumed in RAN1

Figure 2: A-IoT device Type 2a architecture assumed in RAN1

From Figures 1 and 2 for device Type 1 and 2a respectively, here are some main differences between device 1 and device 2a in  architecture  summarized as follows for the additional functions in Device 2a improvements,

- Device 2a has larger energy storage capability.

- [LNA may be used for D2R signal Rx chain.]

- BB amplifier is used for D2R signal Rx chain.

- N-bit ADC may be used compared with comparator.

- [Large frequency shifter may be used for Tx chain].

- Reflection amplifier is used for backscattering signal.

From the difference, Device 2a may have some RF performance improvement compared with Device 1, for example, larger maximum output power, better reference sensitivity if LNA is used, UL frequency shift compared with DL pending on RAN1 decision. So it seems the RF requirements approach for Device 1 and Device 2a may be considered to be the same with different specific levels.

Proposal 2: The same set of RF requirements, maybe with different specific levels, can be considered for Device 1 and Device 2a.

For device 1 and device 2a, the performance is highly related to the antenna. With another consideration is that defining RF requirement with respect to the antenna connector is not reasonable for the two types of devices. It’s reasonable that only define OTA requirements for Device 1 and Device 2a.

Proposal 3: Only define OTA requirements for Device 1 and Device 2a.

NR UE requirements including plenty of Tx and Rx requirements, it is not reasonable to define a lot of requirements for AIoT passive requirements considering the very low cost. So we have the following proposal,

Proposal 4: The number of requirements for Device 1 and Device 2a should be as small as possible to limit the costs.

With the principle of proposal 3, the requirements in proposal 4 are proposed.

Proposal 5: The following requirements are to be defined for Device 1 and Device 2a.

- Radiated power

- Reference sensitivity (the name can be discussed further)

- Unwanted emissions

There’s one point should be noted that RF BPF exists in the architecture figure but RAN1 noted that it depends on RAN4 decision if it’s needed. From our understanding, it may not be reasonable to assume RF BPF for passive devices for the reason of cost and size consideration. There may be some argument that how to handle the adjacent channel and possible blocking interference. TN UE uses single individual BPF for every supporting band to reject the blocking requirements. ACS requirement for TN UE relies on digital filter after high performance ADC. So the same approach of ACS and blocking requirements for TN UE can’t be supported by current assumed RF architecture. Our understanding is that the possible interference should be solved by deployment not by the device implementation. If this can’t be solved by deployment, low cost passive devices can’t be used in the corresponding scenarios. So we have the following proposal.

Proposal 6: No ACS and blocking requirements for device 1 and device 2a.

  - Device 2b

For device 2b, there’re three RF architectures under discussion in RAN1. Figure 3 ~ Figure 5 are copied from RAN1 summaries for reference.

Figure 3: A-IoT device Type 2b with RF-ED receiver architecture assumed in RAN1

Figure 4: A-IoT device Type 2b with ZIF receiver architecture assumed in RAN1

Figure 4: A-IoT device Type 2b with IF receiver architecture assumed in RAN1

The main differences for the three architectures are summarized as following based our understanding,

- Device 2b with RF ED receiver architecture has the same R2D Rx architecture with Device 2a. Tx chain is improved that the signal is generated interminably and LO is used. So this architecture has the same R2D Rx performance with Device 2a. Tx signal power and quality are be improved thus UL coverage can be enhanced.

- Device 2b with IF ED receiver architecture detects the envelope on IF frequency compared with the above architecture. With improved IF filter performance, relative nearer noise may be rejected effectively then ED performance may be improved compared with RF ED architecture. IF filter performance is better than RF filter but it still has limited performance for the noise near the wanted signal. For example ACS noise can’t be rejected by IF filter. The NR inband signal can’t be filtered either. So how much it can be improved depends on the deployment and system requirement, such as noise assumption (frequency and level), and the filter performance evaluation.

- Device 2b with zero IF architecture coverts the RF signal to baseband directly, no ED is used. This architecture may need some discussion that if comparator or ADC can decide OOK signal correctly for OFDM based OOK waveform. It may relate to the waveform discussion in RAN1. The similar noise assumption problem also exists because it seems no digital filter is assumed in this architecture.

From the above preliminary consideration, discussion for device 2b RF requirements is too premature now. The deployment assumption and system requirement should be decided first, then the baseline RF architecture should be decided for further RF requirement discussion.

Observation: Discussion for device 2b RF requirements is too premature at current stage.

Proposal 7: For Device 2b RF requirement discussion, the deployment assumption and system requirement should be decided first. Then the baseline RF architecture should be decided.

- Conclusion

This contribution provides our further discussion on AIoT device RF requirements. The following observations and proposals are provided.

Proposal 1: Separate the RF requirements discussions for passive and active devices.

Proposal 2: The same set of RF requirements, maybe with different specific levels, can be considered for Device 1 and Device 2a.

Proposal 3: Only define OTA requirements for Device 1 and Device 2a.

Proposal 4: The number of requirements for Device 1 and Device 2a should be as small as possible to limit the costs.

Proposal 5: The following requirements are to be defined for Device 1 and Device 2a.

- Radiated power

- Reference sensitivity (the name can be discussed further)

- Unwanted emissions

Proposal 6: No ACS and blocking requirements for device 1 and device 2a.

Observation: Discussion for device 2b RF requirements is too premature at current stage.

Proposal 7: For Device 2b RF requirement discussion, the deployment assumption and system requirement should be decided first. Then the baseline RF architecture should be decided.

- References

[1] R4-2410597, “WF on impacts of A-IoT on RF requirements”, CMCC, RAN4#111

[2] R4-2410567, “WF on co-existence study for ambient IoT and NR/LTE”, CMCC, RAN4#111