3GPP TSG-RAN WG4 Meeting #112                                                                              R4-2411077

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on RF requirements for 2U/2D CA_n5-n8

Source: 	CATT

Agenda item:	7.3.3

Document for:	Discussion

- Background

2UL CA_n5-n8 was discussed in the WI NR_700800900_combo_en [1] [2], but it was removed due to the limited time in R18. The study conclusion is included in TR 38.872 [3]. R19 CA basket [4] included this band combination again. This contribution summarizes the discussion status in WI NR_700800900_combo_en and proposes how to continue the discussion in R19.

- Discussion

When CA_n5-n8 was discussed in the SI and WI NR_700800900_combo_en, two architectures and 3 requirements options were discussed. The study output including RF architecture and RF requirements evaluation was captured in TR 38.872 for future reference.

The following is the two implementation architectures and the corresponding requirements options.

-------------------------------------------------------

Generally, in this study item, some high level implementations were discussed and shown as below.

1) Full band n5 and n8 RF filters implementation with option 1 and option2:

Option 1: Only support 1UL/2DL CA. Single UL in n5

Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL

Note: Potential impacts on RAN2 are observed

2) Dedicated RF filters implementation with partial frequency range

Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL

--------------------------------------------------------

Option 1 is already defined in R18 specification. To support 2UL/2DL CA, option 2 and option 3 should be discussed. For the potential impacts on RAN2 for option 2, 3 LS [5] [6] [7] were sent between RAN2 and RAN4. RAN2 didn’t continue the discussion because 2UL/2DL CA was removed in R18.

As the 2UL/2DL CA discussion is reopened, we propose to continue the discussion based on the two architectures. For the full band RF filter implementation, LS needs to be sent to RAN2. The content can refer [5] and [7], a draft is provided in the draft for discussion.

For the RF requirements, only one for the dedicated filter architecture needs to be defined if option 3 can be agreed. The MSD requirements for n8UL/n5 DL is converged in R18 WI as captured in TR 38.872,

Table 1: CA_n5-n8 MSD value for 1U/2D n8UL/n5DL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n8 | n5 | 909 | 10 | 15 | 25 (RBstart=0) | 877.5 | 5 | 12.3 | >ACLR2 |

Based on the output in R18 WI, we propose to continue the discussion as following,

Proposal 1: Continue the discussion on 2UL/2DL CA_n5A-n8A based on the output in R18 WI NR_700800900_combo_en. No new RF evaluations are expected if no critical technical issues are found.

Proposal 2:  Both the two RF architectures, i.e. the full band filter architecture (Non-concurrent n5 DL and n8 UL) and dedicated filter architecture (simultaneous n5 DL and n8 UL), are kept for further discussion.

Proposal 3: A LS is sent to RAN2 to retrigger the discussion on non-concurrent n5 DL and n8 UL. A draft is provided in the Annex.

- Summary

The study output for 2UL/2DL CA_n5A-n8A in R18 WI NR_700800900_combo_en, we have the following proposals to continue the discussion in R19.

Proposal 1: Continue 2UL/2DL CA_n5A-n8A based on the output in R18 WI NR_700800900_combo_en. No new RF evaluations are expected if no critical technical issues are found.

Proposal 2:  Both the two RF architectures, i.e. the full band filter architecture (Non-concurrent n5 DL and n8 UL) and dedicated filter architecture (simultaneous n5 DL and n8 UL), are kept for further discussion.

Proposal 3: A LS is sent to RAN2 to retrigger the discussion on non-concurrent n5 DL and n8 UL. A draft is provided in the Annex.

Reference

[1] RP-221464, “Revised SI: Study on enhancement for 700/800/900MHz band combinations for NR”, RAN#96

[2] RP-232680, “Revised WID on enhancement for 700/800/900MHz band combinations”, RAN#101

[3] TR 38.872

[4] RP-241674, “New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells”, RAN#104

[5] R4-2306465, “[Draft] LS on non-simultaneous UL and DL from different two bands during UL CA”, RAN4#106b

[6] R2-2306862, “Reply LS on non-simultaneous UL and DL from different two bands during UL CA”

[7] R4-2314656, “Reply LS on non-simultaneous UL and DL from different two bands during UL CA”, RAN4#108

Annex: Draft LS to RAN2

3GPP TSG-RAN WG4 Meeting #112                                                                              R4-240xxxx

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	(Draft) LS on non-simultaneous n5 DL and n8 UL for 2UL/2DL CA_n5-n8

Response to:

Release:	Rel-19

Work Item:	NR_CADC_SUL_R19

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:                  Huiping Shan

E-mail Address:   shanhuiping@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

The discussion on 2UL/2DL CA_n5A-n8A is restarted in RAN4 in the WI NR_CADC_SUL_R19. After RAN4 discussion, the questions asked in the LS R4-2306465 and R4-2314656 still need answers from RAN2 experts. In order to save the time to track the discussion history, the background and questions are re-written in this LS.

In the RAN4 discussion for 2UL/2DL CA_n5-n8, it is found that n8 UL produces severe emission interference on n5 DL. To mitigate the interference may need extra RF implementation efforts. A potential solution is proposed to see if it’s possible that n5 DL and n8 UL are scheduled non-concurrently when UE is configured by RRC with 2UL/2DL, i.e. not two UL and one DL.

For the questions asked in LS (R2-2306862), RAN4 has the following answers, as cited below from R4-2314656.

Question 1: Does RAN4 see problem if cross carrier scheduling is used in this scenario i.e. PCell (n8) scheduling SCell (n5)?

Answer from RAN4: Cross carrier scheduling is transparent from RAN4’s perspective. RAN4 didn’t see any impact to RAN4 specifications.

Question 2: What are RAN4 understanding regarding RRM measurements in this kind of scenario? Does UE need to measure the cell with UL only (n5) (e.g. for SCell addition/change/release purpose)?

Answer from RAN4:

- RAN4 expects measurement of the SCell (n5) in this scenario is needed.

- In RAN4 understanding, network would avoid the collision with the DL transmission of SCell (n5) when scheduling the UL in n8.

- No impact to existing RRM specifications is seen from RAN4 perspective.

RAN4 respectively asks RAN2 if RAN2 sees any problem to provide the configuration of 2U/2D CA_n5-n8 with RRC signaling.

2. Actions:

To RAN WG2:

RAN4 respectively asks RAN2 if RAN2 sees any problem to provide the configuration of 2U/2D CA_n5-n8 with RRC signaling.

3. Date of Next TSG-RAN Meetings:

TSG RAN WG4 Meeting #112bis	October 14 – October 18, 2024	Hefei, CN

TSG RAN WG4 Meeting #113	November 18 –November 22, 2024	Orlando, US