3GPP TSG-RAN WG4 Meeting # 112                                                               R4-2411080

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on modification of existing Tx requirements for SBFD operation

Source: 	CATT

Agenda item:	8.19.2.2

Document for:	Discussion

- Background

WF [1] of RAN4#111 concluded that further studies are needed for some TX RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Tx RF Requirement

  - Co-location ACLR

The co-location ACLR requirement is proposed for several meetings. However, it’s not clear if a stringent ACLR requirement for co-location scenario should be defined. For the general co-existence simulation, usually 100% grid shift is assumed to avoid the unrealistic requirement for ACIR. If co-location for adjacent channel BSs is needed, our understanding is that it should be solved by deployment. Furthermore, CLTA discussion is still ongoing in R19 BS RF evolution WI. 30 dB CL assumption is not applicable for many bands and the real deployment scenario. There’s another aspect that it seems co-location ACLR is not a specific discussion for SBFD only. So in our understanding, it’s very difficult to have a conclusion in this WI for this general requirement. We propose not to continue this discussion in SBFD WI.

Proposal 1: No new co-location ACLR requirement is defined in SBFD WI.

  - OBUE

Although this requirement is called OBUE, the intention is to define some new and more stringent requirement at the 2nd adjacent channel frequency range. The motivation is to enable two SBFD-capable BSs to be deployed co-location, and the operating frequency band of the two BSs is separated by one channel bandwidth. A preliminary for the link analysis is provided in Table 1. For FR1 aggressor WA BS, the output power is 49dBm, bandwidth of channel is 100MHz, if assume the degradation of victim BS sensitivity is 1dB, the required total isolation between the two BSs is 144dB, that is, the sum of ACLR, couple loss of co-location and the rejection of RF BPF used after PA must larger than 144dB.

Table 1: Feasibility analysis of SBFD BS co-location

| Parameters | Value |
| --- | --- |
| Output power(dBm) | 49 |
| BW(MHz) | 100 |
| Degradation of victim BS sensitivity(dB) | [1] |
| Required total isolation(dB) | 144 |
| 2nd ACLR(dB) | [45] |
| Coupling loss of co-location (dB) | [30] |
| Rejection of RF BPF used after PA (dB) | 69 |

Couple loss of co-location is depended with operating frequency and distance between two BSs. According to [2], coupling loss increases with the increase of BS operating frequency. Existing 30 dB coupling loss is suitable for frequency range 0.45 – 2 GHz. For higher operating frequencies, other antenna port-to-port isolation should be considered.  RF BPF used after PA must be high power filter, and rejection of BPF is depended with operating frequency, passband width and filter type. For large channel bandwidths, the 2nd ACLR will not improve much and may even become worse.

If ACLR is 45dB, coupling loss of co-location is 30dB，rejection from RF BPF at victim BS UL operating frequency range must be larger than 69dB, if 69 dB rejection can’t be achieved, couple loss of co-location may be improved by increase distance between two BSs, and the frequency impact on coupling loss can also be accounted for.

Observation: For WA BS, if victim BS DESENS is 1dB and CL for co-location BS is 30 dB, RF BPF rejection performance should be larger than 69dB which is a stringent requirement for some RF filters.

Proposal 2: The assumption for the victim BS DESENS and CL of co-location BS should be decided to further discuss the new OBUE (or the 2nd ACLR) requirement.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: No new co-location ACLR requirement is defined in SBFD WI.

Observation: For WA BS, if victim BS DESENS is 1dB and CL for co-location BS is 30 dB, RF BPF rejection performance should be larger than 69dB which is a stringent requirement for some RF filters.

Proposal 2: The assumption for the victim BS DESENS and CL of co-location BS should be decided to further discuss the new OBUE (or the 2nd ACLR) requirement.

- Reference

[1] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung RAN4#111

[2] R4-1706766, “Co-location isolation between different frequency bands”, CATT, RAN4 NR#2