3GPP TSG-RAN WG4 Meeting # 112                                                               R4-2411081

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on modification of existing Rx requirements for SBFD operation

Source: 	CATT

Agenda item:	8.19.2.3

Document for:	Discussion

- Background

WF [1] and topic summary [2] of RAN4#111 concluded that further studies are needed for some Rx RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS Rx RF requirement

  - OTA sensitivity

OTA test environment only consider SI is relatively easy to achieve, comparing the accuracy of test results considering only SI and self-interference, inter-site interference, and inter-sector interference at the same time, the former is better. According to the SI feasibility analysis results of each company in [1], 1.0dB is a more appropriate value.

Proposal 1: 1.0 dB degradation is agreed for OTA sensitivity requirement.

  - Dynamic range requirement

In [3], dynamic range simulation considered both uplink signal and gNB-to-gNB CLI signal. When the grid shift is 100% and the CDF is 99.9%, the IoTs of scenario1and 3 are less than 20dB, and the IoT of the scenario 5 is slightly greater than 20dB, and the IoT of scenario 6 is 1.07dB and the IoT of scenario 9 is -5.84dB.  IoTs of both FR1 and FR2-1 can reuse the existing requirements.

When the grid shift is 10% and the CDF is 99.9%, IoTs of both FR1 and FR2-1 can reuse the existing requirements.

Proposal 2: FR1 dynamic range can reuse the existing requirements.

Proposal 3: FR2 dynamic range can reuse the existing requirements.

  - OTA ACS requirement

For ACS requirement, there’re two views if the non-SBFD or SBFD REFSENS should be the baseline REFSENS level. The decision may depend on if the SBFD sub-band DL is switched on for the ACS test. This discussion is related to all of the requirements which use DESENS LEVEL as the requirement. Our understanding is that non-SBFD BS approach is reused that Tx is switched on for the Rx requirements. So the OTA sensitivity degradation should be taken into account for the baseline REFSENS.

Proposal 4: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement.

  - In-band blocking

According to [3], the simulation results of in-band blocking for FR1 and FR2-1 are shown in table 1 and table 2, and the interference are from TDD DL to SBFD UL.

Table 1: SBFD conducted in-band blocking simulation results (FR1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | Conducted in-band blocking simulation results (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 100% grid-shift |  | 10% grid-shift |  |
|  |  |  | 99% CDF | 99.9% CDF | 99% CDF | 99.9% CDF |
| 1 | FR1 | Urban Macro -> Urban Macro | -67.18 | -53.39 | -70.01 | -61.97 |
| 3 | FR1 | Indoor -> Indoor | -76.45 | -71.24 | -77.44 | -73.86 |
| 5 | FR1 | Micro -> Micro | -58.27 | -49.88 | -48.45 | -40.06 |

Table 2: SBFD OTA in-band blocking simulation results (FR2-1, TDD DL to SBFD UL subband)

| Scenario | Frequency | Aggressor -> Victim | OTA in-band blocking simulation results (dBm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 100% grid-shift |  | 10% grid-shift |  |
|  |  |  | 99% CDF | 99.9% CDF | 99% CDF | 99.9% CDF |
| 6 | FR2-1 | Urban Macro -> Urban Macro | -88.68 | -73.56 | -86.84 | -75.47 |
| 9 | FR2-1 | Indoor -> Indoor | -88.67 | -82.64 | -75.67 | -69.64 |

The simulation results of in-band blocking level for FR1 and FR2-1 are less than the interference signal level in existing requirements.

Observation1: The simulation shows that the interference from TDD DL to SBFD UL subband for FR1 and FR2-1 are less than the interference signal level in existing in-band blocking requirements.

Proposal 5: For FR1 and FR2-1, existing in-band blocking can be reused.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: 1.0 dB degradation is agreed for OTA sensitivity requirement.

Proposal 2: FR1 dynamic range can reuse the existing requirements.

Proposal 3: FR2 dynamic range can reuse the existing requirements.

Proposal 4: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement.

Observation1: The simulation shows that the interference from TDD DL to SBFD UL subband for FR1 and FR2-1 are less than the interference signal level in existing in-band blocking requirements.

Proposal 5: For FR1 and FR2-1, existing in-band blocking can be reused.

- Reference

[1] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111

[2] R4-2410108, “Topic summary for [111][313] NR_duplex_evo”, Samsung, RAN4#111

[3] R4-2411083, “SBFD in-band blocking and dynamic range simulation results”, CATT, RAN4#112