3GPP TSG-RAN WG4 Meeting # 112                                                               R4-2411082

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on potentially new requirements for SBFD operation

Source: 	CATT

Agenda item:	8.19.2.1

Document for:	Discussion

- Background

WF [1] of RAN4#111 and TR [2] concluded that further studies are needed for potentially new RF requirements, in this contribution, we provide our further analysis of them.

- New requirements for SBFD operation

2.1 Transient period

In WF[1], agreements of transmitter transient period are shown following.

- Issue 4-3-1: Requirement for transient period between SBFD and non-SBFD

- Agreement:

  - FFS requirement and cases for transient periodbetween SBFD and non-SBFD:

    - Option 1: The existing TDD BS transmitter transient period, i.e., 10us for FR1 and 3us for FR2-1

    - Other options are not precluded.

  - FFS location of transient period between SBFD and non-SBFD:

    - Option 1: The transmitter transient period shall be located within the SBFD slot

    - Other options are not precluded.

- Issue 4-3-2: Requirement for transient period between different SBFD configurations

- Agreement:

  - FFS requirement for transient period between different SBFD configurations

The transient period for SBFD is a little confusing from our understanding. Existing transient period is a pure Tx requirement for TDD BS. The on/off power is measured for this requirement test. The following figure is copied from TS 38.104.

Figure 6.4.2.1-1: Illustration of the relations of transmitter ON period, transmitter OFF period and transmitter transient period

Observation: Existing transient period is a pure Tx requirement for TDD BS. The on/off power is measured for this requirement test.

However, if transient period is defined for SBFD, we’re not sure how to define the requirement, i.e. if only DL SB is switched on or both DL SB and UL SB should be switched on. Especially for the different SBFD configuration, it’s difficult to define or test the transient period as a RF requirement. So from our understanding, we’re not sure if the intention is to have a capability similar with Tx/Rx time for NR which is defined in TS 38.211 but not a RF requirement.

Proposal 1: The intention and the detail for the SBFD transient period requirement should be clarified before this requirement is defined.

For the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot if it needs be decided

Proposal 2: Location of transient period between SBFD and non-SBFD shall be located within the SBFD slot if it needs to be decided.

2.2In channel new requirements

For the following new RF requirements,

  - In-channel adjacent subband leakage ratio

  - In-channel adjacent subband selectivity

  - In-channel adjacent subband blocking

Considering the performances of the potential new SBFD in channel requirements are actually included in the SI performance, it may be possible not defining these new requirements.

Proposal 3: The following new requirements are not defined for SBFD BS.

  - In-channel adjacent subband leakage ratio

  - In-channel adjacent subband selectivity

  - In-channel adjacent subband blocking

- Summary

In this contribution, we have the following proposals.

Observation: Existing transient period is a pure Tx requirement for TDD BS. The on/off power is measured for this requirement test.

Proposal 1: The intention and the detail for the SBFD transient period requirement should be clarified before this requirement is defined.

Proposal 2: Location of transient period between SBFD and non-SBFD shall be located within the SBFD slot if it needs to be decided.

Proposal 3: The following new requirements are not defined for SBFD BS.

  - In-channel adjacent subband leakage ratio

  - In-channel adjacent subband selectivity

  - In-channel adjacent subband blocking

Reference

[1] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111

[2] TR 38.858

[3] R4- 2407554, “Discussion on SBFD system parameters”, CATT, RAN4 #111