3GPP TSG-RAN WG4 Meeting # 112                                                             R4-2411084

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	                 A-IoT BS feasibility and requirements

Source: 	CATT

Agenda item:	8.20.3.1

Document for:	Discussion

- Introduction

This contribution provides our further analysis for A-IoT BS feasibility and requirements according to the WF [1] [2].

- A-IoT BS feasibility analysis

For the A-IoT system design, R2D and CW/D2R are transmitted in TDM mode. Then the reader interference analysis focuses on the scenario that reader receives CW and D2R signals at the same time. The analysis is the same for different spectrum usage.

A preliminary analysis similar with SBFD self-interference analysis is shown in Table 1. If the REFSENS desense is 1 dB, the required SI noise cancellation capability is up to 150 dB.

Table 1: Analysis of A-IoT BS required CW cancellation capability

| Parameters | Calculation | Value |
| --- | --- | --- |
| Power of CW PCW (dBm) |  | 33 |
| Bandwidth of D2R BWD2R (KHz) |  | [180] |
| NF (dB) |  | 10 |
| Noise power floor Pfloor (dBm/BWD2R) | Pfloor=-174dBm+10*log10(BWD2R*10^3)+NF | -111.4 |
| Receiving path capability degradation x (dB) |  | [1] |
| Total noise power Ptotal noise (dBm/BWD2R) | Ptotal noise= Pfloor+xdB | -110.4 |
| power of interference signal Pinterference (dBm/BWD2R) | Pinterference=10*log10(10^((Pfloor+xdB)/10)-10^(Pfloor/10)) | -117.3 |
| Required CW cancellation capability (dB) | Iso=PCW-Pinterference | 150.3 |

This analysis is not perfect because the CW noise power is added to the whole Rx BW which may not be correct.  BB DEMOD capability is not known because AIoT physical channel discussion in RAN1 is not concluded yet. If BB DEMOD can endure some level of CW signal, then the required CW cancellation capability can be relaxed. Besides the CW signal interference to BB DEMOD, the ADC dynamic range is also impacted if the received CW level is large. So the feasibility of the reader especially to the CW cancellation may need more analysis inputs from companies.

Observation 1: If SBFD SI analysis approach is used, required CW cancellation capability is up to 150 dB for 33 dBm CW power level.

Proposal 1: How CW impacts AIoT reader Rx BB DEMOD performance needs some evaluation or assumption for the feasibility study.

- A-IoT BS RF requirements analysis

It’s a little earlier to discuss BS RF requirements in SI stage before the system design in RAN1 is not complete. Some preliminary views from our side are provided in Table 2 to Table 4.

Table 2: A-IoT BS potential Tx applicable requirements

| No. | Requirement list | Requirement necessary? | Comments |
| --- | --- | --- | --- |
| 1 | Maximum output power | Yes |  |
| 2 | Output power dynamic | FFS | More inputs from AIoT physical channel design is needed |
| 3 | Transmit OFF power | Yes (?) |  |
| 4 | Transmitter transient period | Yes |  |
| 5 | Frequency error | Yes |  |
| 6 | EVM | Yes | AIoT waveform is different with NR, new requirement and test approach is needed. |
| 7 | TAE | No | No CA and MIMO features |
| 8 | Occupied bandwidth | Yes |  |
| 9 | ACLR | Yes |  |
| 10 | Operating band unwanted emissions | Yes |  |
| 11 | Transmitter spurious emissions | Yes |  |
| 12 | Transmitter intermodulation | FFS | IMD scenario needs more discussion. |

Table 3: A-IoT BS potential Rx requirements

| No. | Requirement list | Requirement necessary? | Comments |
| --- | --- | --- | --- |
| 1 | Reference sensitivity level | Yes |  |
| 2 | Dynamic range | Yes |  |
| 3 | In-channel selectivity | FFS | The scenario needs more discussion. |
| 4 | Adjacent Channel Selectivity | Yes |  |
| 5 | In-band blocking | FFS | The scenario needs more discussion. |
| 6 | Narrow-band blocking | No |  |
| 7 | Out-band blocking | FFS | The scenario needs more discussion. |
| 8 | Receiver spurious emissions | Yes |  |
| 9 | Receiver intermodulation | FFS | The scenario needs more discussion. |
| 10 | Narrowband intermodulation | FFS | The scenario needs more discussion. |

Table 4: A-IoT BS CW requirements

| No. | Requirement list | Requirement necessary? | Comments |
| --- | --- | --- | --- |
| 1 | Output power | yes |  |
| 2 | CW signal quality | FFS | If phase noise or emissions should be defined FFS. |

- Summary

This contribution provides our further analysis for A-IoT BS feasibility and requirements. We have the following observations and proposals.

Observation 1: If SBFD SI analysis approach is used, required CW cancellation capability is up to 150 dB for 33 dBm CW power level.

Proposal 1: How CW impacts AIoT reader Rx BB DEMOD performance needs some evaluation or assumption for the feasibility study.

For the AIoT BS RF requirements, preliminary views from our side are provided in Table 2 to Table 4.

Reference

[1] R4-2410567, “WF on co-existence evaluation for ambient IoT”, CMCC, RAN4#111

[2] R4-2410597, “WF on impacts of A-IoT on RF requirements”, Huawei, RAN4#111

[3] R4-2407525, “Discussion on deployment scenarios and spectrum usage for A-IoT”, CATT, RAN4#111

[4] R4-2407522, “Discussion on RF requirements of Ambient IoT BS”, CATT, RAN4#111